Text consolidated by Tulkošanas un terminoloģijas centrs (Translation and Terminology Centre) with amending regulations of:

28 July 2009 (No. 825).

If a whole or part of a paragraph has been amended, the date of the amending regulation appears in square brackets at the end of the paragraph. If a whole paragraph or sub-paragraph has been deleted, the date of the deletion appears in square brackets beside the deleted paragraph or sub-paragraph.

Republic of Latvia

Cabinet

Regulation No. 143

Adopted 14 February 2006

# Regulations regarding the Safe Loading of Bulk Carriers

Issued pursuant to

Section 11, Paragraph four of the

Maritime Administration and Marine Safety Law

I. General Provisions

1. These Regulations prescribe safety requirements of bulk carriers that are related to the loading and unloading of bulk cargoes and that have to be observed in the terminals of the Republic of Latvia in order to reduce the risk of excessive stress and the risk of physical damage to the structure of the ship that occurs at the moment of loading or unloading, and in order to ensure the harmonisation of mutual suitability of ships and terminals, as well as prescribe procedures for harmonised mutual co-operation and communication between ships and terminals.

2. These Regulations apply to:

2.1. bulk carriers (irrespective of their flags) which call at the terminals in order to load or unload bulk cargoes; and

2.2. terminals at which bulk carriers, to which these Regulations apply, call.

3. These Regulations shall not restrict the application of Paragraph VI/7 of the International Convention for the Safety of Life at Sea of 1974, and the Protocols and Amendments of 1978 and 1988 thereof (hereinafter – the SOLAS Convention), shall not apply to equipment that is used only in exceptional circumstances for loading of bulk cargoes into ships or unloading them out, as well as occasions when bulk cargoes are loaded or unloaded using only the relevant bulk carrier equipment.

4. For the purpose of these Regulations:

4.1. bulk carrier, in accordance with Paragraph 1X71.6 of the SOLAS Convention, shall bear the meaning of a ship for the carriage of bulk cargoes that conforms to the following conditions:

4.1.1. a ship constructed with a single deck, top-side tanks and hopper-side tanks in cargo spaces, intended primarily for the carriage of bulk cargoes;

4.1.2. an ore carrier - a sea-going single deck ship, having two longitudinal bulkheads and a double bottom throughout the cargo region and that is intended for the carriage of ore cargoes in the centre holds only; and

4.1.3. a combination carrier shall bear the meaning given to it in Paragraph II-2/3.14 of the SOLAS Convention;

4.2. bulk cargo – cargo which conforms to the definition indicated in Paragraph XII/1.4 of the SOLAS Convention;

4.3. terminal – any fixed, floating or mobile equipment that is equipped and used for the loading or unloading of bulk cargoes into or from bulk carriers;

4.4. terminal operator - the owner of a terminal or any organisation or a person to whom the owner has transferred the responsibility for loading or unloading operations that are conducted at the terminal and are related to a particular bulk carrier;

4.5. terminal representative – any person appointed by the terminal operator, who is responsible for the preparation, the conduct and the completion of loading and unloading operations that are conducted in the terminal and that are related to a particular bulk carrier;

4.6. ship representative – a person, who has command over a bulk carrier or a ship’s officer designated by the master for the conduct of loading or unloading operations;

4.7. administration of the flag State - the competent authority of the State whose flag the bulk carrier is entitled to fly;

4.8. cargo information – the cargo information specified in Paragraph VI/2 of the SOLAS Convention;

4.9. loading and unloading plan – the plan specified in Paragraph VI/7.3 of the SOLAS Convention, the format of which is provided in Annex 2 to the Code of Practice for the Safe Loading and Unloading of Bulk Carriers (hereinafter - the BLU Code) adopted by the International Maritime Organisation (hereinafter – the IMO) by Assembly Resolution A.862(20) of 27 November 1997;

4.10. ship/shore check list – the check list which has a reference in Chapter 4 of the BLU Code and the format of which is provided in Annex 3 to the BLU Code; and

4.11. bulk cargo density declaration – information regarding the density of the cargo to be provided in accordance with Paragraph XII/10 of the SOLAS Convention.

[28 July 2009]

## II. Requirements for the Suitability of Bulk Carriers and Terminals

5. The readiness of the bulk carriers for the loading and unloading of bulk cargoes shall be examined by the terminal operator, by ascertaining whether the ship observes the requirements regarding operational suitability of the bulk carriers for loading and unloading (Annex 1).

6. The terminal operator shall ensure in the terminal the performance of the following requirements:

6.1. the terminal shall conform to the requirements regarding the suitability of the terminal for the conduct of loading and unloading works of bulk cargoes (Annex 2);

6.2. the terminal operator has appointed the terminal representative (representatives);

6.3. informative materials that contain the requirements of the terminal and regulatory enactments of the Republic of Latvia regarding safe loading of bulk carriers, as well as the information (with amendments) regarding the port and the terminal, indicated in Paragraph 1.2 of Annex 1 to the BLU Code (Content of the Terminal Information Book) have been prepared, and these materials are available for those representatives of bulk carriers, who call at the terminal in order to load or unload bulk cargoes; and

6.4. a quality management system is developed, implemented and maintained in the terminal that is certified in accordance with the State Standard LVS EN ISO 9001: 2001 L, Quality Management Systems and it is audited in accordance with the State Standard LVS EN ISO 19011: 2002 Instructions for the Auditing of Quality and/or Environment Management Systems.

III. Duties of the Ship Representatives and the Terminal Representatives

7. The duties of the ship representative:

7.1. to ensure safe loading and unloading of the bulk carrier under the command thereof;

7.2. in accordance with the formalities intended in regulatory enactments that are related to ships arriving at port and departing from port, prior to the estimated time of arrival at the terminal, to report in writing (using fax, Internet, through agents) information indicated in Annex 3 to these Regulations to the terminal;

7.3. prior to the loading of bulk cargo, to ascertain whether the cargo information specified in Paragraph VI/2.2 of the SOLAS Convention and bulk cargo density declaration (if such is intended) are received from the consignor, by including this information in a cargo declaration form in accordance with Appendix 5 of the BLU Code; and

7.4. in addition to the requirements referred to in Sub-paragraphs 7.1, 7.2, and 7.3 of these Regulations, to observe the duties of the ship representative, prior to and during loading or unloading operation, specified in Annex 4 to these Regulations.

8. The duties of the terminal representative:

8.1. upon the receipt of the initial notification of the estimated time of arrival of the ship (ETA), to report in writing (using fax, Internet, through agent) information referred to in Annex 5 to these Regulations to the ship representative;

8.2. to ensure that the information included in the cargo declaration form would be reported in writing (using fax, Internet, through agent) to the ship representative as soon as possible (at least 24 hours prior to the arrival of the ship at port);

8.3. immediately in writing (using fax, Internet, through agent) to report to the ship representative and the Maritime Safety Inspectorate  of the State stock company the Maritime Administration of Latvia (hereinafter – the Maritime Safety Inspectorate) regarding the determined deficiencies on the bulk carrier if they may endanger the safe loading and unloading of bulk cargoes;

8.4. in addition to the requirements referred to in Sub-paragraphs 8.1, 8.2 and 8.3 of these Regulations, to observe the duties of the terminal representative, prior to the start of or during the loading or unloading operation, specified in Annex 6 to these Regulations; and

8.5. to present the master of the ship moored at the berths of the terminal with the instruction on action in emergency situation in accordance with the requirements of SOLAS Convention.

[28 July 2009]

IV. Mutual Procedures of Bulk Carriers and Terminals

9. Prior to loading or unloading of bulk cargoes, the ship representative shall agree with the terminal representative regarding the loading or unloading plan (Annex 7) in accordance with the requirements referred to in Paragraph VI/7.3 of the SOLAS Convention. The loading or unloading plan of the cargo shall be prepared in the form specified in Annex 2 to the BLU Code, indicating the IMO number of the relevant bulk carrier. The agreed loading and unloading plan shall be approved with a signature by the ship representative and the terminal representative.

10. Amendments to the loading or unloading plan, which according to either party may affect the safety of the ship or crew, shall be prepared, accepted and agreed by both parties in the form of a revised loading or unloading plan.

11. The agreed loading or unloading plan specified in Paragraph 9 of these Regulations and any subsequent agreed and revised variants shall be kept by the ship and the terminal for a period of six months in order to ensure a possibility of verification thereof by the Maritime Safety Department of the State stock company the Maritime Administration of Latvia (hereinafter – the Maritime Safety Department).

12. Prior to loading or unloading of bulk cargoes, the ship representative together with the terminal representative shall complete the ship/shore check list (Annex 8) and sign it in accordance with the guidelines of Annex 4 to these Regulations.

13. In order to be capable of responding to requests for information on the loading or unloading process and to ensure prompt compliance of the instructions of the ship representative and the terminal representative, if the loading and unloading operations are suspended, efficient and continuous communication shall be established and maintained between the ship and the terminal.

14. The ship representative and the terminal representative shall conduct the loading or unloading operations in accordance with the cargo loading or unloading plan. The terminal representative shall ensure that during the loading or unloading of bulk cargo, the hold order, quantity and rate of loading or unloading would be observed as stated in the loading or unloading plan. The terminal representative shall not deviate from the agreed loading or unloading plan, except for the cases when it has been previously co-ordinated with the representative of the ship and a written agreement is reached.

15. On completion of the loading and unloading operations, the ship representative and the terminal representative shall confirm in writing that loading or unloading has been carried out in accordance with the cargo loading or unloading plan, including any co-ordinated amendments. In addition on completion of the unloading a record shall be made in the cargo loading or unloading plan that the cargo holds have been emptied and cleaned in accordance with the requirements of the ship representative, as well as any damages of the ship or the repairs carried out shall be registered.

V. Operation of the Maritime Safety Department

16. The Maritime Safety Department shall control whether the requirements for the terminal operators, the ship representatives and the terminal representatives, specified in these Regulations, are observed.

17. The inspectors of the Maritime Safety Department shall carry out the following inspections of bulk cargo terminals:

17.1. initial inspection– an inspection to evaluate for the first time the conformity of the terminal with the requirements specified in Paragraphs 6 and 8, as well as in Chapter IV of these Regulations;

17.2. periodical inspection – an inspection that is carried out not less than once every three months in order to ascertain the introduction and maintenance of the requirements included in Paragraphs 6 and 8, as well as in Chapter IV of these Regulations; and

17.3. unannounced inspection – any inspection that has not been referred to in Sub-paragraph 17.1 or 17.2 of these Regulations and that is carried out to ascertain the continuous observance of the requirements included in Paragraphs 6 and 9, as well as in Chapter IV of these Regulations, including the inspection that is carried out on the basis of credible information, received from other persons regarding the non-conformity of the specified requirements for the terminal.

18. During the inspection referred to in Paragraph 17 of these Regulations the inspectors of the Maritime Safety Department shall ascertain the observance of the requirements specified in Paragraph 7 and in Chapter IV of these Regulations. An inspection deed of the bulk carrier regarding the results of the inspection of the bulk carrier shall be drawn up (Annex 9).

[28 July 2009]

19. The inspections referred to in Paragraph 17 of these Regulations (except the unannounced inspections) shall be a paid service in accordance with Section 6, Paragraph two of the Maritime Administration and Marine Safety Law.

20. The Maritime Safety Department every three years shall provide the Ministry for Transport with a written report regarding the results of the inspections referred to in Paragraphs 17 and 18 of these Regulations. The efficiency of co-operation and communication procedures between the bulk carriers and the terminals, which have been specified in these Regulations, shall be evaluated in the report. The Ministry for Transport shall send the referred to report to the European Union Commission. The report regarding the period of the previous three years shall be sent until 30 April of the current year.

21. Without prejudice to the rights and duties of the ship representative, which have been determined in Paragraph VI/7.7 of the SOLAS Convention, the Maritime Safety Department shall prevent or halt the loading or unloading of bulk cargoes if the conduct of such operations may endanger the safety of the ship or crew. The operations may be commenced or resumed after the prevention of the relevant deficiencies. The deficiencies shall be prevented immediately after determination thereof during the Maritime Safety Department inspection.

22. If the ship representative or the terminal representative cannot come to an agreement regarding the application of the procedures specified in Chapter IV of these Regulations and it may cause a threat to maritime safety or the protection of the marine environment, the application of the procedures shall be determined by the Maritime Safety Department in accordance with the requirements specified in Chapter IV of these Regulations.

VI. Repair of Damage Incurred During Loading or Unloading

23. The application of the requirements of this Chapter shall not restrict the performance of the requirements of the international and Latvian regulatory enactments which determine the procedures for port State control.

24. The terminal representative in writing (using fax, Internet, through agent) shall report to the ship representative regarding the damage to the structure of the ship or equipment that have occurred during loading or unloading and, if necessary, shall repair it.

25. If the damage referred to in Paragraph 24 of these Regulations may affect the structural capability, watertight integrity, or the essential engineering systems of the ship, the terminal representative or the ship representative shall inform in writing (using fax, Internet, through agent) the administration of the flag State or an organisation recognised thereof, which has been recognised in accordance with the regulatory enactments regulating the operation of recognised organisations (hereinafter – recognised organisation) and the Maritime Safety Inspectorate. The decision regarding the commencement of repair or deferment thereof shall be taken by the Maritime Safety Inspectorate, taking into account the opinion (if such has been delivered) of the administration of the flag State or a recognised organisation thereof, as well as the representative of the ship. If it is necessary to commence an immediate repair, it shall be carried out before the ship leaves the port.

26. Taking a decision referred to in Paragraph 25 of these Regulations, the Maritime Safety Inspectorate may rely upon a recognised organisation to undertake the inspection of the damage and to advise on the necessity regarding immediate commencement of repair or deferment thereof.

VII. Closing Provisions

27. For terminals, which have been established until the day of coming into force of these Regulations, the permits issued for the operation of a bulk cargo terminal shall be valid until the end of the term of validity indicated therein.

28. If a plan for the implementation of the terminal quality management system has been developed in accordance with the State Standard LVS EN ISO 9001: 2001 L Quality Management Systems, one year after the establishment thereof, the requirements specified in Sub-paragraph 6.4 of these Regulations shall not be applied to the terminal and the Maritime Safety Department shall issue a temporary permit for the operation of a bulk cargo terminal (Annex 10). The permit shall be valid not longer than 12 months. The inspectors of the Maritime Safety Department shall inspect the terminal and the results of the inspection shall be indicated in the inspection deed of the bulk cargo terminal (Annex 11).

Informative Reference to European Union Directives

These Regulations contain legal norms arising from:

1) Directive 2002/84/EC of the European Parliament and of the Council of 5 November 2002 amending the Directives on maritime safety and the prevention of pollution from ships; and

2) Directive 2001/96/EC of the European Parliament and of the Council of 4 December 2001 establishing harmonised requirements and procedures for the safe loading and unloading of bulk carriers.

Prime Minister 		A. Kalvītis

Minister for Transport 	A.Šlesers

Annex 1

Cabinet Regulation No. 143

14 February 2006

Requirements in Relation to the Operational Suitability of Bulk Carriers 
for Loading and Unloading Bulk Cargoes

A bulk carrier, calling at the terminals of the Republic of Latvia in order to conduct loading or unloading of bulk cargoes, shall conform to the following requirements:

1. Cargo holds shall be provided with hatch openings of sufficient size (commensurable with the loading mechanisms) the design of which enables the bulk cargo to be loaded, stowed, trimmed and unloaded satisfactorily.

2. Cargo hatch openings shall be marked with identification numbers that conform to the numbers used in the loading or unloading plan. The location, size and colour of these numbers shall be clearly visible to and identifiable by the operator of the terminal loading or unloading equipment.

3. Cargo hold hatches, hatch operating systems and safety devices shall be in good functional order and used only for their intended purpose.

4. List indicating lights of ships, if there are such, shall be tested prior to loading or unloading and proved to be operational.

5. If a loading instrument is required on the ship, this instrument shall be certified and operational in order to carry out stress calculations during loading or unloading.

6. Main propulsion and auxiliary machinery shall be in good functional order.

7. Deck equipment related to mooring and berthing operations shall be operable and in good order and condition.

Minister for Transport 	A.Šlesers

Annex 2

Cabinet Regulation No. 143

14 February 2006

Requirements in Relation to the Suitability of Terminals 
for Loading or Unloading Bulk Cargoes

1. The terminal shall only accept bulk carriers for loading or unloading of bulk cargoes that can safely berth alongside the loading or unloading equipment, taking into account the water depth at the berth, maximum permitted size of the ship, mooring arrangements, fenders, safe access and all possible obstructions to the loading or unloading operations.

2. Terminal loading and unloading equipment shall be properly certified and maintained in good order. They shall be only operated by qualified and appropriately certified personnel.

3. The terminal personnel, in conformity with competence, shall be trained in all aspects of safe loading and unloading of bulk carriers. The purpose of training shall be to inform the personnel regarding the risk that exists in loading and unloading of bulk cargoes and the adverse effect improper loading and unloading operations may have on the safety of the ship.

4. Terminal personnel, who are involved in the loading and unloading operations, shall be provided with and use personal protective equipment, as well as rest shall be ensured in order to avoid accidents due to fatigue.

Minister for Transport 	A.Šlesers

[28 July 2009]

Annex 3

Cabinet Regulation No. 143

14 February 2006

Information to be Provided by the Ship Representative to the Terminal

1. In order to plan prompt servicing of the ship at the port in accordance with the loading/unloading plan of the ship, the ship representative shall notify the terminal regarding the estimated time of arrival of the ship (ETA) at the port (if necessary, the information shall be updated).

2. The ship representative shall, in notifying the estimated time of arrival of the ship (ETA), provide the following information at the same time:

2.1. the name of the ship, call sign, IMO number, flag, port of registry;

2.2. the loading or unloading plan, stating the quantity of cargo, stowage by hatches, loading or unloading sequence and the quantity to be loaded or unloaded at once;

2.3. the arrival draught and proposed departure draught;

2.4. the time required for ballasting or de-ballasting;

2.5. the length, beam of the ship and length of the cargo area from the forward coaming of the forward-most hatch to the after coaming of the aft-most hatch into which cargo is to be loaded or from which cargo is to be unloaded;

2.6. the distance from the waterline to the first hatch through which it is intended to conduct loading or unloading and the distance from the side of the ship to the hatch opening;

2.7. location of the ship’s accommodation ladder;

2.8. maximum air draught;

2.9. details and capacity of cargo-handling gear of the ship (if any);

2.10. the number and type of mooring tows;

2.11. specific requests (for example, trimming of the cargo or continuous measurement of the water content therein);

2.12. details of such necessary repairs that may delay berthing, the commencement of loading or unloading, or may delay the ship sailing after the completion of loading or unloading;

2.13. other information related to the ship that is requested by the terminal.

Minister for Transport 	A.Šlesers

Annex 4

Cabinet Regulation No. 143

14 February 2006

Duties of the Ship Representative Prior 
to and During Loading or Unloading Operations

The ship representative shall ensure that:

1) the loading or unloading of cargo and ballast regulation is under the control of the ship’s officer in charge;

2) the disposition of cargo and ballast water is monitored throughout the loading or unloading process in order to ensure that the structure of the ship is not overstressed;

3) the ship does not have a list or if a list is required for operational reasons, it shall be kept as small as possible;

4) the ship is securely moored, taking into account local weather conditions and forecasts;

5) a sufficient number of officers and crew are retained on the ship in order to ensure the adjustment of tows, as well as the necessary action in any situation, taking into account the need of the crew to have sufficient rest periods to avoid fatigue;

6) the terminal representative is made aware of the cargo trimming requirements, which are determined in IMO Resolution MSC.193(79) of 3 December 2004 Safe Practice for Bulk Cargoes (BC Code);

7) the terminal representative is informed regarding the way in which de-ballasting or ballasting has to be co-ordinated on the ship with cargo loading or unloading rates and regarding any deviation from the de-ballasting or ballasting plan, as well as regarding any other matter that is related to the cargo loading or unloading;

8) the ballast water is discharged in conformity with the agreed loading plan, as well as in order not to flood the berth or the adjacent craft (if it is not practical for the ship to completely discharge its ballast water prior to the trimming stage in the loading process, the ship representative shall agree with the terminal representative on the time at which loading would have to be suspended and the duration thereof);

9) there is an agreement with the terminal representative regarding the actions to be taken in the event of rain, or other changes in the weather, when the nature of the cargo might cause a hazard due to such a change;

10) no hot work is carried out on board or in the vicinity of the ship while the ship is alongside the berth, except when permission of the terminal representative is received. The work referred to shall be carried out in accordance with the regulatory enactments regulating the performance of hot work;

11) close supervision of the loading or unloading operation and of the ship during the final stages of loading or unloading;

12) the terminal representative is warned immediately if the loading or unloading process has caused damage, has created a hazardous situation, or is likely to do so;

13) the terminal representative is informed when the final trimming of the ship has to commence in order to allow for the conveyor system to run-off;

14) the unloading of the port side closely matches that of the starboard side in the same hold in order to avoid twisting the ship’s structure;

15) during ballasting, before permission is given to carry out hot works in the vicinity of holds or above them, the relevant precautions are taken, taking into account the possibility of the discharge of flammable vapours from the holds.

Minister for Transport 	A.Šlesers

Annex 5

Cabinet Regulation No. 143

14 February 2006

Information to be Provided by the Terminal to the Ship Representative

1. The name of the berth at which loading or unloading shall take place and the estimated time for berthing or completion of loading or unloading.

2. Characteristics of the loading or unloading equipment, including the nominal loading or unloading rate of the terminal and the number of loading or unloading heads to be used, as well as the estimated time required to complete each stage of loading or unloading.

3. Features on the berth or pier of which the ship representative must be aware, including fixed and mobile obstruction, fenders, bollards and mooring arrangements.

4. Minimum depth of water alongside the berth and in ship channels.

5. Water density at the berth.

6. Maximum distance between the water line and the top of the cargo hatch covers or coaming, depending on whichever size is relevant to the loading and unloading operation, and the maximum air draught.

7. Arrangements for gangways and access.

8. Side of the ship which is to be alongside the berth.

9. Maximum allowable speed of approach to the pier and the availability of tugs, their type and capacity.

10. The loading sequence for different parcels of cargo and any other restrictions if it is not possible to take the cargo in any order or any hold to suit the ship.

11. Any properties of the cargo to be loaded which may cause a hazard when placed in contact with existing cargo or residue thereof.

12. Information regarding the expected loading or unloading operations and changes in the existing plans for loading or unloading.

13. Whether the loading or unloading equipment in the terminal is fixed and, whether the movement thereof is limited.

14. Mooring lines required.

15. Warning regarding any unusual mooring arrangements.

16. Restrictions on ballasting or de-ballasting.

17. Maximum sailing draught of the ship, which is determined by the port authority.

18. Other information regarding the terminal, which has been requested by the ship representative.

Note. The information referred to in Paragraphs 1 and 4 of this Annex regarding the estimated time for berthing and departure and minimum water depth at the berth shall be systematically updated and passed to the ship representative after receipt of the current notification regarding the estimated time of arrival (ETA). Depending on the circumstances, information on the minimum water depth in ship channels shall be provided by the terminal or by the port authority.

Minister for Transport 	A.Šlesers

[28 July 2009]

Annex 6

Cabinet Regulation No. 143

14 February 2006

Duties of the Terminal Representative Prior 
to and During Loading or Unloading Operations

Prior to commencement of and during loading or unloading operations the terminal representative shall:

1) in writing (for example, using fax, Internet, through agent) provide the ship representative with the names of the terminal crew or name of the consignor representative, who shall have the responsibility for the loading or unloading operation and with whom the ship representative shall have contact, as well as the procedures on how to contact them;

2) take all precautionary measures in order to avoid damage to the ship by the loading or unloading equipment and inform the ship representative if damage has occurred;

3) ensure that the ship does not have a list or if a list is required due to operational reasons, it shall be kept as small as possible;

4) the unloading of the port side closely matches that of the starboard side in the same hold to avoid twisting the ship’s structure;

5) in the case of high density cargoes or when the individual grab loads are large, notify the ship representative regarding a possibility of high, localised impact loads on the structure of the ship until the double bottom cover is completely covered by cargo, (especially when high free-fall drops are permitted and special care has to be taken at the start of the loading operation in each cargo hold).

6) ensure that there is a mutual harmonisation between the ship representative and the terminal representative at all stages of co-operation and in relation to all processes of loading or unloading and the ship representative is informed regarding any change to the rate (that is related to the loading), and regarding the weight of the cargo loaded at the end of each stage of loading;

7) maintain a record of the weight and disposition of the cargo loaded or unloaded and ensure that the weight of the cargo in the holds conforms to the agreed loading or unloading plan;

8) ensure that the cargo is trimmed, when loading or unloading, in accordance with the requirements of the ship representative;

9) ensure that the quantities of cargo, which are necessary to achieve the departure draught and trim, is calculated so that all the terminal conveyor system would be empty on completion of a loading (for that purpose the terminal representative shall inform the ship representative in writing of the nominal tonnage contained on the terminal conveyor system and all requirements for clearing the conveyor system on completion of the loading);

10) during the unloading immediately give the ship representative a warning regarding the expected increase or reduction of the number of unloading mechanisms, as well as notify the ship representative when unloading is considered to be completed in the relevant hold;

11) ensure that no hot work is carried out on board or in the vicinity of the ship while the ship is alongside the berth, except for the case when permission of the ship representative has been received;

12) ensure that prior to the commencement of loading or unloading operations the master of the ship is acquainted with the instruction on action in emergency situations in accordance with the requirements of SOLAS Convention.

Minister for Transport 	A.Šlesers

Annex 7

Cabinet Regulation No. 143

14 February 2006

Kravas iekraušanas vai izkraušanas plāns

| Datums | Kuģa vārds |  |  |  | Reisa Nr. |  |
| --- | --- | --- | --- | --- | --- | --- |
| Iekraušanas vai izkraušanas osta | Krava(-as) | Kravas(-u) ietilpības faktors | Balasta iesūknēšanas vai izsūknēšanas ātrums | Ūdens blīvums | Kuģa maksimāli iespējamā iegrime pie termināļa piestātnes | Lielākais kuģa virsūdens daļas gabarītu augstums |
| Mērķosta | Iepriekšējā krava | Kraušanas mehānismu Nr. | Iekraušanas/izkraušanas ātrums |  |  |  |

Iepriekšējais kravas iekraušanas vai izkraušanas plāns

| Bēruma Nr. | Krava |  | Balasta operācijas | Operācijas veikšanai nepieciešamais laiks (stundas) | Komentāri | Aprēķinātie lielumi |  |  |  | Aprēķinātie lielumi |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | tilpnes Nr. | tonnas |  |  |  | iegrime |  | maksimālie lielumi |  | kuģa virsūdens augstums | vidējā iegrime | kuģa galsvere |
|  |  |  |  |  |  | priekšgals | pakaļgals | BM* | SF** |  |  |  |

* Korpusa lieces moments (BM), ** Korpusa šķērsspēks (SF)

Termināļa pārstāvja paraksts ____________	Kuģa pārstāvja paraksts ______________

Noslēguma iekraušanas vai izkraušanas plāns

|  | Tilpnes Nr. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Kravas daudzums |  |  |  |  |  |  |

Kuģis ir iekrauts (izkrauts) saskaņā ar iepriekš saskaņoto iekraušanas/izkraušanas plānu. Pretenziju pusēm nav.

Termināļa pārstāvja paraksts _____________ 	Kuģa pārstāvja paraksts _____________

Loading or Unloading Plan

| Date | Vessel |  |  |  | Voyage No. |  |
| --- | --- | --- | --- | --- | --- | --- |
| Load/unload Port | Cargo(es) | Assumed stowage factor of cargo(es) | Ballast pumping rate | Water density | Max draught available | Max. air draught in berth |
| To Port | Last cargo | No. of loaders | Load rate |  |  |  |

Preliminary Loading or Unloading Plan

| Pour No. | Cargo |  | Ballast operations | Time Required (hours) | Comments | Calculated values |  |  |  | Calculated values |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Hold No. | Tonnes |  |  |  | Draught |  | Maximum |  | Air draught | Draught mid | Trim |
|  |  |  |  |  |  | Fwd | Aft | BM* | SF** |  |  |  |

*Bending moments (BM), ** Shear forces (SF)

### Signed Terminal ____________________	 Signed Ship ___________________

Final Loading or Unloading plan

|  | Hold No |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Cargo |  |  |  |  |  |  |

The ship is loaded (unloaded) in accordance with preliminary agreed Loading/Unloading Plan. No claims were made.

### Signed Terminal __________________ 	Signed Ship __________________

Minister for Transport 	A.Šlesers

[28 July 2009]

Annex 8

Cabinet Regulation No. 143

14 February 2006

Kuģa/krasta drošības kontroles lapa
beramkravu kuģu iekraušanai vai izkraušanai

| Datums | Osta |
| --- | --- |
| Kuģa vārds | Terminālis/piestātne |
| Dziļums pie piestātnes | Maksimālais kuģa virsūdens augstums |
| Kuģa iegrime pienākot | Kuģa virsūdens augstums pienākot |
| Kuģa iegrime izejot | Kuģa virsūdens augstums izejot |

|  | Kuģis | Terminālis |
| --- | --- | --- |
| 1. Vai dziļums pie piestātnes un kuģa virsūdens augstums ir atbilstošs kravas operāciju veikšanai? | [ ] | [ ] |
| 2. Vai ir atbilstošs tauvošanās aprīkojums, ņemot vērā straumes, laikapstākļus, kuģu satiksmi un pie borta pietauvotos citus kuģus? | [ ] | [ ] |
| 3. Vai ārkārtas gadījumos kuģis ir spējīgs atstāt piestātni jebkurā laikā? | [ ] | [ ] |
| 4. Vai ir nodrošināta droša nokļūšana no kuģa uz piestātni? Trapa stāvokli uzmana kuģis/terminālis (nevajadzīgo svītrot) | [ ] | [ ] |
| 5. Vai akceptētā sakaru nodrošinājuma sistēma ir operatīva? | [ ] | [ ] |
| Sakaru metodes Valoda |  |  |
| Radio kanāli Tālrunis |  |  |
| 6. Vai kontaktpersonas kravas operāciju laikā ir reāli sasniedzamas? | [ ] | [ ] |
| Kuģa kontaktpersona Atrašanās vieta |  |  |
| Termināļa kontaktpersona Atrašanās vieta |  |  |
| 7. Vai uz kuģa un terminālī ir atbilstošs personāla sastāvs darbībai ārkārtas situācijās? | [ ] | [ ] |
| 8. Vai ir paredzētas un saskaņotas degvielas uzpildes operācijas? | [ ] | [ ] |
| 9. Vai ir paredzēti un saskaņoti piestātnes vai kuģa remontdarbi iekraušanas laikā? | [ ] | [ ] |
| 10. Vai ir saskaņota rīcība par kravas operāciju laikā radušos bojājumu ziņošanu un attiecīgu dokumentēšanu? | [ ] | [ ] |
| 11. Vai kuģis ir saņēmis ostas un termināļa noteikumus, kas satur arī drošības un piesārņojuma novēršanas prasības, kā arī informāciju par ārkārtas situāciju novēršanas iespējām? | [ ] | [ ] |
| 12. Vai kravas nosūtītājs ir sniedzis informāciju kuģa pārstāvim par kravas īpašībām saskaņā ar SOLAS konvencijas VI nodaļas nosacījumiem? | [ ] | [ ] |
| 13. Vai gaiss kravas tilpnēs un citās slēgtās telpās, kur nepieciešams piekļūt, nav kaitīgs elpošanai, ja krava apstrādāta ar fumigācijas gāzi, un vai terminālis un kuģis ir vienojušies par gaisa analīžu veikšanas nepieciešamību? | [ ] | [ ] |
| 14. Vai noteiktie kravas pārkraušanas jaudas limiti un arī iekraušanas/izkraušanas mehānismu pārvietošanās ierobežojumi ir saskaņoti starp kuģi un termināli? | [ ] | [ ] |
| Kraušanas mehānisms Nr. Kraušanas ātrums t/st. |  |  |
| Kraušanas mehānisms Nr. Kraušanas ātrums t/st. |  |  |
| Kraušanas mehānisms Nr. Kraušanas ātrums t/st. |  |  |
| 15. Vai kravas iekraušanas/izkraušanas plāns ir izstrādāts visiem posmiem, kuģi iekraujot un izsūknējot balastu vai izkraujot un iesūknējot balastu? Iekraušanas/izkraušanas plāna kopija iesniegta | [ ] | [ ] |
| 16. Vai tilpnes, kuras tiek iekrautas vai izkrautas, ir skaidri atzīmētas iekraušanas/izkraušanas plānā, norādot pārkraušanas darbu secību, kā arī pārkraujamo kravas daudzumu tajās attiecīgajos posmos? | [ ] | [ ] |
| 17. Vai kravas izlīdzināšanas nepieciešamība, metodes un pakāpe tika apspriesta? | [ ] | [ ] |
| 18. Vai kuģa un termināļa pārstāvji saprot un akceptē to, ka gadījumā, ja balasta izsūknēšana vai iesūknēšana nenotiek saskaņā ar kravas iekraušanas vai izkraušanas secību, ir nepieciešams pārtraukt kravas operācijas līdz minēto procesu saskaņošanai? | [ ] | [ ] |
| 19. Vai plānotās darbības par kravas atlikumu aizvākšanu no kuģa tilpnēm pēc kravas izkraušanas kuģa pārstāvjiem tikušas izskaidrotas un viņi tās ir akceptējuši? | [ ] | [ ] |
| 20. Vai veicamie pasākumi, lai piekrautam kuģim nodrošinātu nepieciešamo beigu galsveri, ir izskatīti un pieņemti? | [ ] | [ ] |
| 21. Kravas daudzums, kas aizturēts termināļa konveijeru sistēmā |  |  |
| 22. Vai terminālis ir informēts par nepieciešamo laiku kuģa sagatavošanai iziešanai jūrā pēc kravas operāciju beigām? | [ ] | [ ] |

Piezīme. Atbildes atzīmē šādi:

| X | Jā (derīgs) | • | Nē (nederīgs) | N/A | Nav piemērojams |
| --- | --- | --- | --- | --- | --- |

Par norādīto informāciju ir vienojušies:

| Datums |  | Laiks |  |
| --- | --- | --- | --- |

| Kuģa pārstāvis |  |  |  |
| --- | --- | --- | --- |
|  | (amats, vārds, uzvārds) |  | (paraksts) |

| Termināļa pārstāvis |  |  |  |
| --- | --- | --- | --- |
|  | (amats, vārds, uzvārds) |  | (paraksts) |

Ship/Terminal Safety Checklist
for Loading or Unloading Dry Bulk Cargo Carriers

| Date | Port |
| --- | --- |
| Ship’s name | Terminal/Quay |
| Available depth of water in berth | Maximum air draught |
| Arrival draught (read/calculated) | Air draught |
| Calculated departure draught | Air draught |

|  |  | Ship |  | Terminal |
| --- | --- | --- | --- | --- |
| 1. Is the depth of water at the berth, and the air draught, adequate for the cargo operations to be completed? |  | [ ] |  | [ ] |
| 2. Are mooring arrangements adequate for all local effects of tide, current, weather, traffic and craft alongside? |  | [ ] |  | [ ] |
| 3. In emergency, is the ship able to leave the berth at any time? |  | [ ] |  | [ ] |
| 4. Is there safe access between the ship and the wharf? Tended by ship/terminal (cross out the appropriate) |  | [ ] |  | [ ] |
| 5. Is the agreed ship or terminal communications system operative? |  | [ ] |  | [ ] |
| Communication method: Language |  |  |  |  |
| Radio channels Phone numbers |  |  |  |  |
| 6. Are the liaison contact persons during operations positively identified? | [ ] |  | [ ] |  |
| Ship contact persons |  |  |  |  |
| Location |  |  |  |  |
| Shore contact person(s) |  |  |  |  |
| Location |  |  |  |  |
| 7. Are adequate crew on board and adequate staff in the terminal, for emergency? | [ ] |  | [ ] |  |
| 8. Have any bunkering operations been advised and agreed? | [ ] |  | [ ] |  |
| 9. Have any intended repairs to wharf or ship whilst alongside been advised and agreed? | [ ] |  | [ ] |  |
| 10. Has a procedure for reporting and recording damage from cargo operations been agreed? | [ ] |  | [ ] |  |
| 11. Has the ship been provided with copies of port and terminal regulations, including safety and pollution requirements and details of emergency services? | [ ] |  | [ ] |  |
| 12. Has the shipper provided the Master with the properties of the cargo in accordance with the requirements of Chapter VI of SOLAS Convention? | [ ] |  | [ ] |  |
| 13. Is the atmosphere safe in holds and enclosed spaces to which access may be required, have fumigated cargoes been identified, and has the need for monitoring of atmosphere been agreed by ship and terminal? | [ ] |  | Š |  |
| 14. Have the cargo-handling capacity and any limits of travel for each loader/unloader been passed to the ship/terminal? | [ ] |  | [ ] |  |
| Loader No. Rate tonnes/hr |  |  |  |  |
| Loader No. Rate tonnes/hr |  |  |  |  |
| Loader No. Rate tonnes/hr |  |  |  |  |
| 15. Has a cargo loading/unloading plan been calculated for all stages of loading/deballasting or unloading/ballasting? | [ ] |  | [ ] |  |
| Copy lodged with |  |  |  |  |
| 16. Have the holds to be worked been clearly identified in the loading or unloading plan, showing the sequence of work, and the grade and tonnage of cargo to be transferred each time the hold is worked? | [ ] |  | [ ] |  |
| 17. Has the need for trimming of cargo in the holds been discussed and have the method and extent been agreed? | [ ] |  | [ ] |  |
| 18. Do both ship and terminal understand and accept that if the ballast programme becomes out of step with the cargo operations, it will be necessary to suspend cargo operations until the ballast operation has caught up? | [ ] |  | [ ] |  |
| 19. Have the intended procedures for removing cargo residues lodged in the holds while unloading been explained to the ship and accepted? | [ ] |  | [ ] |  |
| 20. Have the procedures to adjust the final trim of the loading ship been decided and agreed? | [ ] |  | [ ] |  |
| Tonnage held by the terminal conveyor system |  |  |  |  |
| 21. Has the terminal been advised of the time required for the ship to prepare for sea, on completion of cargo work? | [ ] |  | [ ] |  |

| X | Fit or Yes | • | Unfit or No | N/A | Not applicable |
| --- | --- | --- | --- | --- | --- |

THE ABOVE HAS BEEN AGREED:

| Date |  | Time |  |
| --- | --- | --- | --- |

| For ship |  |  |  |
| --- | --- | --- | --- |
|  | Rank/title |  | Signature |

| For terminal |  |  |  |
| --- | --- | --- | --- |
|  | Position/title |  | Signature |

Minister for Transport 	A.Šlesers

[28 July 2009]

Annex 9

Cabinet Regulation No. 143

14 February 2006

Beramkravu kuģa pārbaudes akts

Latvijas Jūras administrācija

Kuģošanas drošības departaments

Rekvizīti _______________________________

Adresāts:

| Nosaukums |  |
| --- | --- |
| Adrese |  |
| Reģistrācijas Nr. |  |

| osta |  | Datums |  |
| --- | --- | --- | --- |
| Kuģa vārds | Kuģa karogs | SJO numurs | Tilpība |

Saskaņā ar Ministru kabineta 2006.gada 14.februāra noteikumu Nr.143 "Beramkravu kuģu drošas kraušanas noteikumi" ____.punktu valsts akciju sabiedrības "Latvijas Jūras administrācija" Kuģošanas drošības departamenta inspektori veica kuģa pārbaudi un pārbaudīja šādus objektus:

| Nr. | Pārbaudāmie objekti | Rezultāti |
| --- | --- | --- |
| 1. | Beramkravu kuģa stabilitātes informācijas grāmata (saskaņā ar SOLAS konvencijas noteikumu VI/7; Beramkravas kuģu drošas kraušanas kodeksu) |  |
| 2. | Atbilstības dokuments (piemērojams kuģiem, uzbūvētiem 1984.g. 1.septembrī vai vēlāk) |  |
| 3. | Kuģa/krasta drošības kontroles lapa |  |
| 4. | Iespējas efektīvai informācijas apmaiņai starp termināli un kuģi |  |
| 5. | Informācija par kravu uz kuģa (SOLAS konvencijas noteikums VI/2.2) |  |
| 6. | Pārbaudes lapa kuģa piemērotības noteikšanai iekraušanas/izkraušanas operācijām |  |
| 7. | Iekraušanas/izkraušanas plāns |  |
| 8. | Piemērotas kuģa iekārtas esība (sertificētas datorprogrammas nodrošinājums) lieces momenta (LM) un pārbīdes spēku (PS) noteikšanai iekraušanas/debalastēšanas vai izkraušanas/balastēšanas operāciju konkrētā stadijā |  |
| 9. | Iekraušanas/debalastēšanas vai izkraušanas/balastēšanas operāciju secības saistība ar saskaņoto iekraušanas/izkraušanas plānu |  |

Izmantotie simboli:

| X | Jā | – | Nē | N/P | Nav piemērojams |
| --- | --- | --- | --- | --- | --- |

Piezīmes.

__________________________________________________________________________

__________________________________________________________________________

Atzinums.

Saskaņā ar __________________________________________________________________

___________________________________________________________________________

un pamatojoties uz iepriekš minētajiem pārbaudes rezultātiem, secināts:

___________________________________________________________________________

___________________________________________________________________________

___________________________________________________________________________

Lēmumu saskaņā ar Administratīvā procesa likumu var apstrīdēt mēneša laikā pēc tā spēkā stāšanās, iesniedzot iesniegumu Latvijas Jūras administrācijas direktoram. Latvijas Jūras administrācijas direktora lēmumu var pārsūdzēt tiesā.

Latvijas Jūras administrācijas rekvizīti:

__________________________________________________________________________

| Kuģa pārstāvis |  |
| --- | --- |
|  | (paraksts un tā atšifrējums) |

| Termināļa pārstāvis |  |
| --- | --- |
|  | (paraksts un tā atšifrējums) |

| Latvijas Jūras administrācijas Kuģošanas drošības departamenta atbilstoši pilnvarotie inspektori |  |
| --- | --- |
|  | (paraksts un tā atšifrējums) |
|  | (paraksts un tā atšifrējums) |
|  | (paraksts un tā atšifrējums) |

MARITIME ADMINISTRATION OF LATVIA

MARITIME SAFETY DEPARTMENT

Requisites: ___________________________________

Addressee:

Name: _________________________________

Address: _______________________________

Identification No.: ________________________

BULK CARRIER INSPECTION REPORT

| Port of |  | Date |  |
| --- | --- | --- | --- |
| Name of ship | Flag of ship | ### IMO number | Gross tonnage |

In conformity with article ____ of regulations No.143 of Cabinet of Ministers of the Republic of Latvia "Safe stowage of bulk ships" from February 14, 2006, inspectors of Maritime Safety Department of Maritime Administration of Latvia performed the verification of the ship.

| No | Inspected Items | Results |
| --- | --- | --- |
| 1. | Bulk Carrier Stability Booklet (SOLAS reg. VI/7; the Code of Practice for the Safe Loading and Unloading of Bulk Carriers) |  |
| 2. | Document of compliance (applicable to ships constructed on or after 1 September 1984) |  |
| 3. | Ship/shore safety checklist |  |
| 4. | Effective communication between terminal and ship |  |
| 5. | Cargo information on board (SOLAS 1974 Reg.VI/2.2) |  |
| 6. | Checklist to show suitability for loading/unloading solid bulk cargoes |  |
| 7. | Loading/Unloading plan |  |
| 8. | Ship devices (loading instrument) in adequate operation for BM and SF determination in accordance with recent stage of loading/deballasting or unloading/ballasting operation |  |
| 9. | Succession of loading/deballasting or unloading/ballasting operation in accordance with confirmed loading/unloading plan |  |

Used symbols:

|  | X | Yes | – | No | N/A | Not applicable |
| --- | --- | --- | --- | --- | --- | --- |

Remarks

_____________________________________________________________________

_____________________________________________________________________

Conclusion

In conformity with _____________________________________________________

____________________________________________________________________

and on the basis of the results of verification listed above, concluded:

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

The decision can be challenged in one month term after coming into effect in conformity with administrative process with lodging the application to the director of Maritime Administration of Latvia. The decision of the director of Maritime Administration of Latvia can be appealed in court.

Requisites of Maritime Administration of Latvia:

Name and signature of Master

Name and signature of Terminal’s representative

Names and Signatures of Duly Authorized inspectors of the Maritime Safety Department of MAL

Minister for Transport 	A.Šlesers

Annex 10

Cabinet Regulation No. 143

14 February 2006

Temporary Authorisation for the Operation of Bulk Cargo Terminal

Maritime Administration of Latvia Maritime Safety Department

| I certify that: |  |  | port bulk cargo terminal/berth |
| --- | --- | --- | --- |
|  | (name of the port) |  |  |
|  |  | overall condition inspection, it is determined |  |
| (name or number of terminal/berth) |  |  |  |

that the overall condition and operational activity thereof conforms to _______________________________________________________ requirements.

The terminal has provided the implementation plan of the quality management system in accordance with the Standard. Taking into account the results of the inspection (see ____ _____________ date _________ year terminal inspection deed) and on the basis of

|  | determined, |  |
| --- | --- | --- |
|  |  | (name or number of terminal/berth) |

it is permitted to conduct operations related to bulk cargo loading.

| The temporary authorisation shall be valid until |  |
| --- | --- |
|  | (date) |

| Place for a seal |  | Issued to |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | (date) |

| No. |  |
| --- | --- |

| Authorised person |  |
| --- | --- |
|  | (position, signature and full name) |

Minister for Transport 	A.Šlesers

[28 July 2009]

Annex 11

Cabinet Regulation No. 143

14 February 2006

Inspection Deed of Bulk Cargo Terminals

Maritime Administration of Latvia Maritime Safety Department

Requisites _______________________________________

Addressee

| Name |  |  |  |
| --- | --- | --- | --- |
| Address |  |  |  |
| Registration No. |  |  |  |

| (name of the port) |  | (date) |
| --- | --- | --- |

In accordance with Cabinet Regulation No. 143 of 14 February 2006 Regulations regarding the Safe Loading of Bulk Carriers, Maritime Administration of Latvia Maritime Safety Department (hereinafter – KDD) inspectors

| carried out |  |  | bulk cargo terminal/berth |  |
| --- | --- | --- | --- | --- |
|  | (name of the port) |  |  |  |

|  | inspection. |
| --- | --- |
| (name or number of the terminal/berth) |  |

Determined in the inspection:

| 1. Performance of overall requirements |  |
| --- | --- |
| 1.1. An authorisation for the performance of Categories A and B polluting activities in the terminal, issued by the relevant State Environmental Service |  |
| 1.2. An inspection deed compiled by certified organisations that attests certification of equipment, as well as conformity of the state thereof with regulatory enactments of the Republic of Latvia |  |
| 1.3. An inspection deed of fire safety of the terminal compiled by the State Fire-fighting and Rescue Service or the Inspectorate that attests the conformity thereof with regulatory enactments |  |
| 1.4. Inspection deeds compiled by certified organisations that attest the conformity of electrical equipment, as well as insulation measurements of the terminal with regulatory enactments |  |
| 2. Requirements for the suitability of bulk terminal and bulk carrier |  |
| 2.1. The terminal representative is appointed for co-operation with ship |  |
| 2.2. Readiness control of the bulk carrier prior to commencement of loading or unloading works and acquaintance of the master of the ship with the instruction on action in emergency situations in accordance with the requirements of SOLAS Convention |  |
| 2.3. The terminal personnel is trained in conformity with the performance of duties to be carried out |  |
| 2.4. The service personnel of the terminal loading equipment is qualified and accordingly certified |  |
| 2.5. The terminal personnel is provided with personal protective equipment |  |
| 2.6. The terminal personnel is provided with appropriate rest possibilities |  |
| 3. Performance of mutual duties of the bulk carrier representative and the terminal representative |  |
| 3.1. Information of the ship representative to the terminal regarding the ship – conformity of the amount and quality thereof |  |
| 3.2. Information of the terminal to the ship representative – conformity of the amount and quality thereof |  |
| 3.3. Information of the terminal to the ship representative regarding the conformity of cargoes |  |
| 3.4. Supervision of the bulk carrier during loading/unloading of bulk cargo |  |
| 3.5. Readiness of the terminal for loading/unloading of bulk cargo |  |
| 3.6. Conformity of the supervision of the terminal during loading/unloading of bulk cargo |  |
| 4. Observance of mutual procedures of the terminal and the bulk carrier |  |
| 4.1. The amount and conformity of the Terminal Information Book |  |
| 4.2. Provision of means of communication for information exchange between the terminal and the ship |  |
| 4.3. Completion of the ship/shore check list |  |
| 4.4. Preliminary loading or unloading plan of bulk carriers |  |
| 4.5. Final loading or unloading plan of bulk carriers |  |
| 4.6. A deed signed jointly by the ship representative and the terminal representative regarding the conformity of loading |  |
| 5. A quality management system is developed, implemented and maintained in the terminal |  |

|  | X | Yes (Conforms) | – | No (Does not conform) | N/A | Not applicable |
| --- | --- | --- | --- | --- | --- | --- |

#### Deficiencies discovered

___________________________________________________________________________

___________________________________________________________________________

___________________________________________________________________________

___________________________________________________________________________

Conclusion

Taking into account the results of the terminal overall state inspection, as well as the operational activity thereof ensuring execution of the conditions of Cabinet Regulation No. 143 of 14 February 2006 Regulations regarding the Safe Loading of Bulk Carriers, it is concluded that the referred to terminal overall state, as well as operational activity thereof conforms/does not conform (delete as appropriate) with the specified criteria.

___________________________________________________________________________

___________________________________________________________________________

___________________________________________________________________________

___________________________________________________________________________

| Date of the next inspection |  |
| --- | --- |

The decision may be challenged within a time period of one month after coming into effect in conformity with administrative process with lodging the application to the Director of Maritime Administration of Latvia. The decision of the Director of Maritime Administration of Latvia can be appealed in court.

Requisites of the Maritime Administration of Latvia:

| The following officials of the terminal participated in the inspection |  |
| --- | --- |
|  | (position, signature and full name) |
|  | (position, signature and full name) |
|  | (position, signature and full name) |

| The inspection was carried out by the KDD inspectors |  |
| --- | --- |
|  | (signature and full name) |
|  | (signature and full name) |
|  | (signature and full name) |

Place for a seal

Minister for Transport 	A.Šlesers