Republic of Latvia

Cabinet

Regulation No 197

Adopted 14 March 2006

# Procedures for Port State Control

Issued pursuant to

Section 15, Paragraph three of the

Maritime Administration and Marine Safety Law

I. General Provisions

1. These Regulations prescribe the procedures for port State control.

2. Port State control shall be performed by the Maritime Safety Inspectorate of the State joint stock company the Maritime Administration of Latvia (hereinafter – the Maritime Safety Inspectorate) in order to prevent the shipping traffic which does not conform to the international standards and to increase the compliance of ships with the requirements of International Maritime Organisation, the European Union and regulatory enactments of the Republic of Latvia in respect of maritime safety, the marine environment protection and seafarers’ living and working conditions.

3. Terms used in these Regulations:

3.1. off-shore installation - fixed or floating platforms operating on or over the continental shelf of Latvia;

3.2. port State control inspector – the inspector of the Maritime Safety Inspectorate, authorised in accordance with Paragraph 9 of these Regulations to carry out ship inspections and other measures referred to in these Regulations for the implementation of port State control on the ships specified in Paragraph 5 of these Regulations;

3.3. inspection – an examination in order to check both the validity of the relevant certificates and other documents and the condition of the ship, its equipment and crew, as well as the living and working conditions of the crew;

3.4. more detailed inspection - an inspection where the ship, its equipment and crew as a whole or, as appropriate, the parts thereof are subjected to an in-depth inspection covering the construction, equipment of the ship, living and working conditions of the crew, their compliance with the operational rules on board and the compliance of the ship with the requirements regarding the performance of on-board operations;

3.5. ship detention - the prohibition of a ship to proceed to sea due to deficiencies established during the inspection that (individually or together) make the ship unseaworthy;

3.6. stoppage of an operation - the prohibition of a ship to continue an operation due to deficiencies established during the inspection that (individually or together) would render the continued ship operation hazardous;

3.7. SIReNAC database – a database established in accordance with the Paris Memorandum of Understanding on Port State Control of 26 January 1982 (hereinafter – the Paris Memorandum), which is maintained and managed by the Information System department of the Ministry of Transport of France and in which the information regarding all port State inspections of ships in the ports of the Member States of the Paris Memorandum is placed;

3.8. target factor – a number indicated in the SIReNAC database and calculated in accordance with the methodology of the Paris Memorandum, which characterises the necessity to carry out an inspection and which is used for the selection of ships for port State control inspections (the greater the target factor indicated, the greater the necessity to carry out the inspection of the ship); and

3.9. complaint – a report or any information submitted by the master of a ship, a member of the crew, a professional maritime association, a trade union or any other natural or legal person with an interest in the safety of the ship, including an interest in the safety or health hazards to its crew.

4. The requirements of these Regulations shall not be applied to fishing vessels, ships of war, naval auxiliaries, wooden ships of a primitive build, government ships used for non-commercial purposes (for example, harbour master’s service boats, pilot boats), as well as pleasure yachts not engaged in trade.

5. These Regulations shall apply to any ship, which complies with the following criteria:

5.1. a ship flying a flag of a foreign state;

5.2. a ship calling at a port of Latvia or at an off-shore installation, or anchored off such ports or off-shore installations; and

5.3. one or several of the following Conventions may be applied to a ship:

5.3.1. 1) the 1966 International Convention on Load Lines and the 1988 Protocol thereof (hereinafter – the LL 66 Convention);

5.3.2. the International Convention for the Safety of Life at Sea of 1974, and the Protocols 1978 and 1988 thereof (hereinafter – the SOLAS Convention);

5.3.3.  the International Convention for the Prevention of Pollution from Ships of 1973 and the 1978 Protocol thereof (hereinafter – the MARPOL Convention);

5.3.4. the International Convention on Standards of Training, Certification and Watchkeeping for Seafarers of 1978 with amendments (hereinafter – the STCW Convention);

5.3.5. the Convention on the International Regulations for Preventing Collisions at Sea, 1972 (hereinafter – the COLREG Convention);

5.3.6. the International Convention on Tonnage Measurement of Ships, 1969 (hereinafter – the TONNAGE Convention);

5.3.7. the Merchant Shipping (Minimum Standards) Convention No. 147 of International Labour Organisation, 1976 (hereinafter – the ILO No 147);

5.3.8. the Protocol of 1996 to the Merchant Shipping (Minimum Standards) Convention No. 147 of International Labour Organisation, 1976 (hereinafter – the ILO PROT 96); and

5.3.9. the International Convention on Civil Liability for Oil Pollution Damage, 1992 (hereinafter - the CLC 92 Convention).

6. If the Conventions referred to in Sub-paragraph 5.3 of these Regulations do not contain the requirements for the ships the gross tonnage of which does not exceed 500, the requirements, which ensure that a ship is not dangerous to maritime safety, human health or environment, shall be applied to them. In applying this Paragraph, Annex 1 Port State Control procedures to the Paris Memorandum shall be taken into account.

7. When carrying out an inspection on board a ship of a flag State to which any of the Conventions referred to in Sub-paragraph 5.3 of these Regulations are not binding, the port State control inspector shall not apply more favourable requirements to such ship than to a ship for the flag State of which this Convention is binding.

8. The application of these Regulations shall be ensured by the port State control inspectors.

9. The right of the port State control inspector to carry out port State control shall be approved by the Port State Control Inspector Certificate (Annex 1). The inspector shall present the certificate upon request.

II. Selection of Ships for Inspection

10. The Maritime Safety Inspectorate shall carry out inspections of ships referred to in Paragraphs 11, 12 and 20 of these Regulations to at least 25% of the average total number of those ships which entered the ports of Latvia during 1 calendar year, calculated on the basis of the 3 previous calendar years for which statistics are available.

11. When selecting ships for inspection, the Maritime Safety Inspectorate shall observe the specified priority in accordance with the criteria by which a ship is selected for an inspection (Annex 2).

12. The Maritime Safety Inspectorate shall ensure that an inspection in accordance with Chapter III of these Regulations is carried out on any ship not subject to an expanded inspection with the target factor greater than 50 and if a period of at least 1 month has elapsed since the last inspection carried out in a port in the region of the Paris Memorandum.

13. The Maritime Safety Inspectorate shall refrain (in order not to cause the excessive administrative load and not to interfere with the operation of ships by carrying out too frequent inspections) from inspecting ships which have been inspected by any Member State of the European Union within the previous 6 months, provided that:

13.1. the criteria referred to in Paragraph 1 of Annex 2 to these Regulations are not applied to the ship;

13.2. no deficiencies have been reported, following the previous inspection carried out on board;

13.3. no clear grounds exist for carrying out an inspection in accordance with Paragraph 18 of these Regulations; and

13.4. Paragraph 12 of these Regulations is not applied to the ship.

14. Paragraph 13 of these Regulations shall not apply to examinations during which the readiness of the crew and technical means to operate in case of emergency are evaluated, which are specifically provided for in the Conventions referred to in Sub-paragraph 5.3 of these Regulations.

III. Inspection Procedures

15. The port State control inspector shall carry out the following operations during an inspection:

15.1. check the certificates and other documents (Annex 3) regarding the particular ship; and

15.2. check the compliance of the overall condition of the ship with the general safety requirements, including the engine room, accommodation and service spaces, as well as the hygienic conditions of the ship premises.

16. The port State control inspector may also examine documents other than those referred to in Annex 3 to these Regulations, if they are on the relevant ship in accordance with the requirements of the Conventions referred to in Sub-paragraph 5.3 of these Regulations.

17. If after the operations specified in Paragraphs 15 and 16 of these Regulations there are clear grounds to believe that the conditions of a ship, the equipment or crew thereof significantly differ from the requirements of the Conventions referred to in Sub-paragraph 5.3 of these Regulations and other regulatory enactments in the field of maritime safety, marine environment protection and seafarers’ living and working conditions, the port State control inspector shall carry out a more detailed inspection.

18. Clear grounds to carry out a more detailed inspection of a ship, its equipment or its crew (Annex 4) are the evidence of the port State control inspector.

19. Ship inspections shall be carried out in accordance with the procedures specified in the international documents referred to in Annex 5 to these Regulations.

IV. Expanded Inspections

20. A ship, complying with the categories specified in Chapter I of Annex 6 to these Regulations, is subject to an expanded inspection in accordance with Chapter III of Annex 6 to these Regulations after a period of 12 months since the last expanded inspection carried out in a port of a State signatory of the Paris memorandum.

21. If the ship referred to in Paragraph 20 of these Regulations is selected for inspection in accordance with the criteria specified in Paragraph 12 of these Regulations, an expanded inspection shall be carried out by the port State control inspector.  An inspection in accordance with Chapter III of these Regulations may be carried out in the period between two expanded inspections.

22. If a ship, to which Paragraph 20 of these Regulations applies, plans to enter a port of Latvia after a period of 12 months since the last expanded inspection, the master or operator thereof shall communicate all the information listed in Chapter II of Annex 6 to these Regulations to the Maritime Safety Inspectorate.  The information shall be provided at least 3 days before the expected time of arrival at the port of Latvia or before leaving the previous port if the voyage is expected to take fewer than 3 days.

23. An expanded inspection of a ship shall be carried out mandatory in the following cases:

23.1. if the ship has not fulfilled the requirements referred to in Paragraph 22 of these Regulations; and

23.2. if the requirements referred to in Paragraphs 20, 21 and 22 of these Regulations apply to the ship, the target factor of the ship is seven or more and if a period of 12 months has elapsed since the last expanded inspection.

V. Report Regarding an Inspection and Safety Control Measures

24. On completion of an inspection, a more detailed inspection or an expanded inspection, the port State control inspector shall draw up a report of inspection specified in the Paris Memorandum. The report shall be submitted to the ship master. A copy of the report shall be handed over to the co-ordinator of ship control of the Maritime Safety Inspectorate, which shall enter the data of the inspection into the SIReNAC database and into the database of the Maritime Safety Inspectorate. A copy of the report shall be kept in the Maritime Safety Inspectorate.

25. If during an inspection, a more detailed inspection or an expanded inspection deficiencies hazardous to human life, safety, other ships or environment have been discovered, the port State control inspector may take the following decisions:

25.1. regarding the detention of the ship taking into account the criteria for the detention of a ship (Annex 7); and

25.2. regarding the stoppage of the operation.

26. In addition to the specified in Annex 7 to these Regulations a ship shall be detained, if the ship does not have a voyage data recorder or it is out of order, if the use of the referred to device is mandatory in accordance with Paragraph 27 of these Regulations and international regulatory enactments regarding the voyage data recorder systems. If it is not possible to rectify this deficiency immediately after detention at a port, the port State control inspector may permit the ship to proceed until the nearest port, which is appropriate for the rectification of the deficiency. Deficiencies shall be rectified within 30 days and the procedures specified in Paragraph 39 of these Regulations shall be applied for the rectification thereof.

27. Ships of the following classes shall, inasmuch as they call at a port of a Member State of the European Union, be fitted with a voyage recorder system meeting the performance standards specified in International Maritime Organisation (hereinafter – IMO) Resolution A.861(20) and the testing standards specified in Standard No. 61996 of the International Electronics Commission (IEC):

27.1. passenger ships; and

27.2. ships other than passenger ships of 3 000 gross tonnage and upwards, built after 1 July 2002.

28. In addition to the specified in Annex 7 to these Regulations a ship shall be detained, if the port State control inspector during the inspection discovers, that on board the ship, to which the International Safety Management Code (hereinafter – ISM code) is applied within the territory of the Member States of the European Union, does not have a copy of the document of compliance of the shipping company or the Safety Management Certificate of the ship issued in accordance with ISM code.

29. If the overall condition of a ship fails to comply with the requirements of these Regulations, the port State control inspector may suspend the inspection of such ship until the person responsible for the ship has ensured the compliance of the ship with the requirements of the Conventions referred to in Sub-paragraph 5.3 of these Regulations.

30. The implementation of a decision regarding the detention of a ship or the stoppage of an operation shall be ensured by such harbour master, in which the ship is located. The oral indication of the port State control inspector regarding the ensuring of the relevant safety control measures or release of the ship shall also be binding upon the harbour master. In order to detain a ship, the port State control inspector shall:

30.1. complete A and B forms of the Paris Memorandum report regarding the inspection of the ship, where the information shall be provided in accordance with the specified in the Paris Memorandum; and

30.2. complete the notification to the master regarding the detention of the ship specified in the Paris Memorandum, attach the completed forms referred to in Sub-paragraph 30.1 of these Regulations thereto and submit a notification to the master of the ship to be detained, but the copy thereof – to the harbour master.

31. Where the grounds for detention are the result of an accidental damage suffered on the ship voyage to a port, no notification to the master regarding the detention of the ship shall be issued, provided that:

31.1. the requirements contained in Regulation I/11(c) of SOLAS have been duly fulfilled;

31.2. prior to entering the port, the master or ship owner has submitted to the Maritime Safety Inspectorate the details on the circumstances of the accident and the damage suffered, as well as informed what information has been provided to the flag State Administration;

31.3. the appropriate remedial actions, to the satisfaction of the Maritime Safety Inspectorate, are being taken by the ship, and

31.4. the Maritime Safety Inspectorate has ascertained, after having been notified of the completion of the remedial actions, that the deficiencies, which were hazardous to safety, human health or the environment, have been rectified.

32. If a ship is being detained after carrying out the inspections specified in Paragraphs 11, 12 and 20 of these Regulations, the Maritime Safety Inspectorate shall immediately inform in writing the flag State administration of all the circumstances in which the intervention of the relevant administration is necessary, including the information in the notification regarding the detention of the ship specified in the Paris Memorandum. The Maritime Safety Inspectorate shall also send the referred to notification to the nearest diplomatic representative, if it is not possible to establish contacts with the ship flag State administration, and to the classification society or another authorised organisation, if the reason for the detention of the ship is a deficiency regarding which the classification society or another authorised organisation is to be held liable. If the Maritime Safety Inspectorate receives a complaint or evidence that the ship fails to comply with the requirements of regulatory enactments regarding the organisation of the working time of seafarers, the Maritime Safety Inspectorate shall inform in writing the flag State administration, including such information in the report.

33. When on board the ship, which has been detained or the operation of which has been stopped, all the deficiencies determined during the inspection have been rectified, the ship master shall inform the Maritime Safety Inspectorate thereof in order to carry out the re-inspection.

34. If after the inspection referred to in Paragraph 33 of these Regulations the port State control inspector determines that the ship may proceed to sea or resume the operation of the ship, without endangering human health, safety, other ships, as well as environment, the port State control inspector shall lift the decision regarding the ship delay, detention or stoppage of an operation (ship release).

35. When releasing a detained ship, the port State control inspector shall complete the notification regarding the ship release specified in the Paris Memorandum. The notification shall be submitted to the ship master, but a copy of the notification – to the relevant harbour master, ship flag State administration and classification society, which has issued the ship certificates, if the reason for the detention of the ship was a deficiency regarding which the classification society is responsible.

36. If the documents referred to in Paragraph 28 of these Regulations are not present on board a ship, but during the inspection of the ship other deficiencies have not been discovered in accordance with Paragraphs 25, 26 or 28 of these Regulations, which could justify the detention, the port State control inspector may take a decision regarding the ship release, in order to prevent the excessive workload of the port of Latvia (workload that causes harm to the economic activity of the port). After taking such decision, the Maritime Safety Inspectorate shall immediately inform the competent authorities of the Member States of the European Union thereof.

37. When carrying out port State control, it is not allowed without reason (not taking into account the requirements of these Regulations and good navigation practice in relation to the measures of ship safety control) to detain or delay a ship, to stop the operations thereof or refuse it to enter another port. A shipowner, operator or the representative thereof shall have the right to contest the decision, taken by the Maritime Safety Inspectorate regarding the ship detention or delay, stoppage of the ship operations or refusal of access to a port, within a month after taking of the decision in accordance with the procedures specified in the Administrative Procedures Law, by submitting the relevant submission to the Director of the Maritime Administration of Latvia. The contestation or appeal of the decision of the Maritime Safety Inspectorate in a court shall not suspend the detention of the ship.

VI. Permission for a Ship to Proceed to a Repair Yard

38. The port State control inspector may allow a ship to proceed to the nearest repair yard, including the permission in the report referred to in Paragraph 24 of these Regulations or in a separate document, if the following conditions exist and the following provisions have been complied with:

38.1. the deficiencies discovered during an inspection, a more detailed inspection or an expanded inspection can not be rectified at the port, where the inspection was carried out;

38.2. the repair yard has been selected upon the mutual agreement of the ship master and the Maritime Safety Inspectorate and taking into account the provisions, which the flag State has intended for such cases; and

38.3. the observance of the conditions referred to in Sub-paragraph 38.2 of these Regulations ensure that the further navigation of the ship does not endanger the safety and health of the passengers and crew members, other ships, as well as marine environment.

39. The Maritime Safety Inspectorate shall notify the competent authority of that State in which the repair yard is situated regarding the ship repair and the conditions for proceeding specified in Paragraph 38 of these Regulations, as well as the authorities referred to in Paragraph 35 of these Regulations and any other authority which is connected with such proceeding (for example, if it is necessary to call at any intermediate port during the proceeding, the competent authority of such state shall be notified). The information shall include the final report regarding the inspection, as well as the data regarding the time and place of the ship arrival. Additional information shall be provided, using the methods provided for in the SIReNAC system.

40. If a ship proceeds to a repair yard in accordance with Paragraph 38 of these Regulations, but fails to comply with the conditions specified in the report regarding the inspection of the Maritime Safety Inspectorate, the requirements of the Conventions referred to in Sub-paragraph 5.3. of these Regulations and does not call at the repair yard indicated in order to carry out the repair of the ship, the Maritime Safety Inspectorate shall take the measures in accordance with Annex 8 to these Regulations to deny the entry of this ship into any port of the European Union until the Maritime Safety Inspectorate in accordance with Annex 8 to these Regulations has ascertained the complete compliance of the ship with the requirements of the Conventions referred to in Sub-paragraph 5.3 of these Regulations.

41. If a ship, which in accordance with Paragraph 38 of these Regulations proceeds to a repair yard, fails to comply with the conditions specified in the report regarding the inspection of the Maritime Safety Inspectorate during this voyage, the Maritime Safety Inspectorate shall immediately notify thereof the competent authorities of other Member States of the European Union.

42. In order to take a decision regarding the refusal to call at a port, the Maritime Safety Inspectorate shall consult with the administration of the flag State of the relevant ship.

43. If the Maritime Safety Inspectorate receives the information form another State that a ship has been sent to a repair yard in Latvia within the framework of port State control, the Maritime Safety Inspectorate shall control the arrival of the ship in Latvia and implementation of repair works, as well as shall notify the State, which has sent the ship, regarding the action within the framework of this control.

44. If the refusal to call at the ports of the European Union applies to a ship in accordance with Paragraph 40 or Chapter VIII of these Regulations, but the shipowner, operator or ship master has implemented the adequate measures to ensure safe entry into the particular port, the Maritime Safety Inspectorate may give a permission for the ship to enter the port, provided the following conditions exist:

44.1. force majeure conditions;

44.2. safety considerations;

44.3. threats of larger pollution; and

44.4. the necessity to rectify the deficiencies discovered during the inspection.

VII. Procedure in Case Certain Ships Cannot Be Inspected

45. In cases where, for operational reasons (for example, lack of human resources, lack of information), the Maritime Safety Inspectorate is unable to carry out an inspection of ships with a target factor of more than 50 in accordance with Paragraph 12 of these Regulations, or a mandatory expanded inspection in accordance with Sub-paragraph 23.2 of these Regulations, the Maritime Safety Inspectorate shall, without delay, place the information in the SIReNAC database that such inspection has not taken place.

46. The cases referred to in Paragraph 45 of these Regulations, when for operational reasons the Maritime Safety Inspectorate has not carried out the inspections of ships, the Maritime Safety Inspectorate shall notify the European Commission at intervals of 6 months, indicating the reasons for not inspecting the relevant ships.

47. The number of ships, which have not been inspected during the relevant calendar year in accordance with Paragraph 45 of these Regulations shall not exceed 5% from the average annual number of ships that entered the ports of Latvia and are subject to inspections. The basis for the calculations shall be the last 3 years for which statistics are available.

VIII. Access Refusal to the Ports of Latvia

48. The Maritime Safety Inspectorate shall ensure that a ship complying with one of the categories referred to in Paragraph 1 of Annex 8 to these Regulations, is refused access to the ports of Latvia, except in the situations described in Paragraph 44 of these Regulations, if the ship:

48.1. flies the flag of a State included in the black list as published in the annual report of the Paris Memorandum, and has been detained more than twice in the course of the preceding 24 months in a port of a State signatory of the Paris Memorandum; and

48.2. flies the flag of a State described as ‘very high risk’ or ‘high risk’ in the black list as published in the annual report of the Paris Memorandum, and has been detained more than once in the course of the preceding 36 months in a port of a State signatory of the Paris Memorandum (the refusal of access shall come into effect immediately the ship has been permitted to leave the port where it has been the subject of a second or third detention as appropriate).

IX. Professional Requirements for the Port State Control Inspectors

49. Port State control shall be carried out by the port State control inspectors the qualification of which complies with the minimum requirements for the port State control inspectors (Annex 9).

50. If the Maritime Safety Inspectorate can not take a decision without assistance due to the lack of competence in a specific field (for example, regarding the atomic energy in relation to the inspection of such ship, which carries dangerous substances), prior to taking of a decision it shall invite a competent person (for example, an instructor of an institution of higher education) for the provision of consultations.

51. The same limitations in respect of the conflict of interest prevention, which are applied to the port State inspectors in accordance with Section 11 and 12 of the Law On Prevention of Conflict of Interest in Activities of Public Officials, shall be applied to a person who consults the Maritime Safety Inspectorate in accordance with Paragraph 50 of these Regulations.

X. Exchange of Information and Co-operation

52. If pilots, engaged in berthing or unberthing ships, as well as performing other official duties, observe the deficiencies of ships which endanger or may endanger the safety of the ship or crew, or which may pose a threat of harm to the marine environment, they shall immediately inform the Maritime Safety Inspectorate regarding the deficiencies observed.

53. If a State or Port Authority official, when exercising the official duties thereof, detects that a ship within the port has deficiencies, which may prejudice the safety of the ship and crew or poses a threat of harm to the marine environment, such person shall immediately inform the Maritime Safety Inspectorate thereof.

54. The Maritime Safety Inspectorate shall organise co-operation with all competent authorities and merchants in order to receive information regarding the ships entering the ports of Latvia, which is necessary for effective port State control.

55. The Maritime Safety Inspectorate shall exchange information and co-operate with the competent authorities of other States, authorities of the European Union which are related to the fields of maritime safety, marine environment protection and seafarers’ living and working conditions, including the European Commission (Annex 10) and the authority which maintains SIReNAC database (Annex 11). In order to select ships for the inspections referred to in Paragraphs 11, 12 and 20 of these Regulations, the port State control inspectors shall use the SIReNAC database, as well as other public and private databases, which are connected with ship inspections.

56. At least once a month the Maritime Safety Inspectorate shall publish on the Internet home page of the Maritime Administration of Latvia (www.jurasadministracija.lv) the information regarding the ships, which were detained in the ports of Latvia in the previous month or to which the entry therein is refused (Annex 11).

XI. Reimbursement of Costs

57. A shipowner, operator or the representative thereof in Latvia shall cover the following expenses:

57.1. all the expenses of the Maritime Administration of Latvia relating to the repeated inspections specified in Paragraph 33 of these Regulations ;

57.2. all the expenses of the Maritime Administration of Latvia relating to the inspections specified in Paragraph 40 of these Regulations; and

57.3. all the expenses caused in relation to the detention of the ship in a port of stay of the ship (for example, the disbursement of wages for the crew of the ship).

58. A decision regarding the detention of a ship shall not be repealed until the payment for the ship inspection, more detailed inspection or expanded inspection is made, or sufficient security for the reimbursement of costs is provided.

XII. Closing Provisions

59. Annex 1 to these Regulations shall come into force on 1 May 2006. Until 30 April 2006 the right of the port State control inspectors to carry out port State control shall be attested with the service certificate, in which at least the following information and requisites shall be indicated:

59.1. the issuing authority of the certificate – the Maritime Administration of Latvia;

59.2. the inspector’s given name, surname;

59.3. the date of issue;

59.4. the term of validity of the certificate;

59.5. a sample of the inspector’s signature; and

59.6. the photograph (3 x 4 cm).

60. Ships of the classes referred to in Paragraph 27 of these Regulations, which were built until 1 July 2002, shall be equipped with the voyage data recording system that complies with the relevant IMO requirements referred to in the SOLAS Convention, within the following periods of time:

60.1. cargo ships of 20 000 gross tonnage and upwards – not later than until 1 January 2007; and

60.2. cargo ships of 3 000 gross tonnage and upwards, but not less than 20 000 – not later than until 1 January 2008.

Informative Reference to European Union Directives

These Regulations contain legal norms arising from:

1) Council Directive 95/21/EC of 19 June 1995 concerning the enforcement, in respect of shipping using Community ports and sailing in the waters under the jurisdiction of the Member States, of international standards for ship safety, pollution prevention and shipboard living and working conditions (port State control);

2) Commission Directive 96/40/EC of 25 June 1996 establishing a common model for an identity card for inspectors carrying out port State control (Text with EEA relevance);

3) Council Directive 98/25/EC of 27 April 1998 amending Directive 95/21/EC concerning the enforcement, in respect of shipping using Community ports and sailing in the waters under the jurisdiction of the Member States, of international standards for ship safety, pollution prevention and shipboard living and working conditions (port State control);

4) Commission Directive 98/42/EC of 19 June 1998 amending Council Directive 95/21/EC concerning the enforcement, in respect of shipping using Community ports and sailing in the waters under the jurisdiction of the Member States, of international standards for ship safety, pollution prevention and shipboard living and working conditions (port State control) (Text with EEA relevance);

5) Commission Directive 1999/97/EC of 13 December 1999 amending Council Directive 95/21/EC concerning the enforcement, in respect of shipping using Community ports and sailing in the waters under the jurisdiction of the Member States, of international standards for ship safety, pollution prevention and shipboard living and working conditions (port State control) (Text with EEA relevance);

6) Directive 2001/106/EC of the European Parliament and of the Council of 19 December 2001 amending Council Directive 95/21/EC concerning the enforcement, in respect of shipping using Community ports and sailing in the waters under the jurisdiction of the Member States, of international standards for ship safety, pollution prevention and shipboard living and working conditions (port State control);

7) Directive 2002/84/EC of the European Parliament and of the Council of 5 November 2002 amending the Directives on maritime safety and the prevention of pollution from ships (Text with EEA relevance); and

8) Directive 1999/95/EC of the European Parliament and of the Council of 13 December 1999 concerning the enforcement of provisions in respect of seafarers' hours of work on board ships calling at Community ports.

Prime Minister 									A. Kalvītis

Minister for Transport 								A. Šlesers

Annex 1

Cabinet Regulation No. 197

14 March 2006

## Certificate of the Port State Control Inspector

Logo of the Maritime Administration of Latvia

Director of the Maritime Administration of Latvia

## Minister for Transport 							A. Šlesers

Annex 2

Cabinet Regulation No. 197

14 March 2006

## Priority and Sequence Criteria for Selection of Ships to Be Inspected

1. Regardless of the value of the target factor, the following ships shall be considered for priority inspection:

1.1. the ships which have been reported by pilots or port authorities as having deficiencies which may prejudice their safe navigation (in accordance with Paragraphs 52 and 53 of these Regulations);

1.2. the ships which have failed to comply with the requirements of the regulatory enactments in respect of ships which are on their way to the ports of the European Union or leave them carrying dangerous or environment polluting products;

1.3. the ships which have been the subject of a report or notification by another Member State;

1.4. the ships which have been the subject of a complaint (the identity of the person lodging the complaint must not be revealed to the master or the owner of the ship concerned);

1.5. ships that have been:

1.5.1. involved in a collision, grounding or standing on their way to the port;

1.5.2. accused of an alleged violation of the provisions on discharge of harmful substances including oil and oil products or effluents;

1.5.3. manoeuvred in an erratic or unsafe manner whereby routing measures, adopted by the IMO, or safe navigation practices and procedures have not been followed; and

1.5.4. otherwise operated in such a manner as to pose a danger to persons, property or the environment; and

1.6. the ships which have been suspended or withdrawn from their class for safety reasons in the course of the preceding 6 months.

2. When deciding on the sequence of inspection of the ships to which Paragraph 1 of these Annex does not apply, the Maritime Safety Inspectorate shall take into account the general target factor in accordance with Paragraph 1 of Annex 1 to the Paris Memorandum.

Minister for Transport 							A. Šlesers

Annex 3

Cabinet Regulation No. 197

14 March 2006

### Certificates and Documents to Be Checked when Carrying out Port State Control

During an inspection the port State control inspector shall check the following certificates and documents, if in accordance with the Conventions referred to in Sub-paragraph 5.3 of these Regulations they shall be present on board:

1. The International Tonnage Certificate.

2. The Passenger Ship Safety Certificate.

3. The Cargo Ship Safety Construction Certificate.

4. The Cargo Ship Safety Equipment Certificate.

5. The Cargo Ship Safety Radio Certificate.

6. The Exemption Certificate.

7. The Cargo Ship Safety Certificate.

8. The International Certificate of Fitness for the Carriage of Liquefied Gases in Bulk.

9. The Certificate of Fitness for the Carriage of Liquefied Gases in Bulk.

10. The International Certificate of Fitness for the Carriage of Dangerous Chemicals in Bulk.

11. The Certificate of Fitness for the Carriage of Dangerous Chemicals in Bulk.

12. The International Oil Pollution Prevention Certificate.

13. The International Pollution Prevention Certificate for the Carriage of Noxious Liquid Substances in Bulk.

14. The International Load Line Certificate.

15. The International Load Line Exemption Certificate

16. The oil record book, parts I and II.

17. The cargo record book.

18. The Minimum Safe Manning Certificate.

19. The qualification documents of the ship crew members.

20. The medical certificates of the ship crew members.

21. The stability information.

22. A copy of the document of compliance of the company and the certificate of the safe administration of the ship.

23. The certificates as to the ship hull strength and machinery installations issued by the classification society in question (only to be checked by the inspector if the ship maintains its class with a classification society).

24. The document of compliance with the special requirements for ships carrying dangerous goods.

25. The high speed craft safety certificate and permit to operate high speed craft.

26. The dangerous goods special list or manifest, or detailed stowage plan.

27. The ship log book with respect to the records of tests and drills and the log for records of inspection and maintenance of lifesaving appliances and arrangements.

28. The special purpose ship safety certificate.

29. The mobile offshore drilling unit safety certificate.

30. For oil tankers – the record of oil discharge monitoring and control system for the last ballast voyage.

31. The muster list, fire control plan, and for passenger ships - the damage control plan.

32. The shipboard oil pollution emergency plan.

33. Survey report files (in case of bulk carriers and oil tankers).

34. The reports of the previous port State control inspections.

35. For ro-ro passenger ships - information on the A/A-maximum ratio.

36. The document of the authorisation for the carriage of grain.

37. The cargo securing manual.

38. The garbage management plan and garbage record book.

39. The decision support system for the masters of passenger ships.

40. The plans for co-operation between the search and rescue services and passenger ships trading on fixed routes.

41. The list of operational limitations on passenger ships.

42. The bulk carrier (bulker) booklet.

43. The bulk cargo loading and unloading plans.

44. A document which attests insurance or any other financial security in respect of civil liability regarding the damages caused by oil pollution in accordance with the CLC Convention.

45. The table of the working hours organisation.

46. The record book of the seafarers’ working and rest hours.

47. The International Ship Security Certificate.

48. Continuous Synopsis Records.

Minister for Transport 							A. Šlesers

Annex 4

Cabinet Regulation No. 197

14 March 2006

Clear Grounds Indicating the Necessity to Carry out a More Detailed Inspection (Examples)

1. The ships referred to in Paragraph 1, Annex 2 to these Regulations, as well as the ships complying with the following criteria:

1.1. the ship statutory certificates on the ship construction and equipment in accordance with the requirements of the Conventions referred to in Sub-paragraph 5.3 of these Regulations and the classification certificates, which have been issued by the classification societies, which are not recognised in the European Union;

1.2. in the report of the current year of the Paris Memorandum regarding the ships detained, the flag State of the ship is referred to as the State whose number of ships detentions in relation to the number of ships inspected during the previous year exceeds the average number of ships detentions within the framework of the Paris Memorandum in proportion to the number of ships inspected during the last 3 years;

1.3. the ship has been given permission to leave the port of a Member State of the European Union on condition that the deficiencies discovered shall be rectified at the next port of entry;

1.4. the ship has been given permission to leave the port of a Member State of the European Union on condition that the deficiencies discovered shall be rectified within 14 days; and

1.5. the ship flying the flag of a country that has not ratified the Conventions referred to in Sub-paragraph 5.3 of these Regulations.

2. The oil record book has not been properly kept.

3. During examination of the certificates and documentation specified in Annex 3 of these Regulations in accordance with Sub-paragraph 15.1 and Paragraph 16 of these Regulations, inaccuracies have been revealed.

4. Indications that the crew members are unable to comply with the requirements of international and national regulatory enactments regarding the minimum level of training of seafarers (for example, a person can not reply to the questions related to the performance of his or her official duties, can not use the equipment in relation to the performance of these duties).

5. When performing cargo and other operations, the requirements of the Conventions referred to in Sub-paragraph 5.3. of these Regulations and other regulatory enactments in the field of maritime safety, marine environment protection and seafarers’ living and working conditions or good navigation practice, including IMO guidelines have been violated, for example, the content of oxygen in the inert-gas main supply to the cargo tanks is above the prescribed maximum level.

6. Failure of the master on an oil tanker to present the record of the oil discharge monitoring and control system for the last ballast voyage.

7. Absence of an up-to-date muster list (cases of alert), or crew members not aware of their duties in the event of fire or an order to abandon the ship.

8. The emission of false distress alerts not followed by proper cancellation procedures.

9. Absence of equipment on board the ship specified in the Conventions referred to in Sub-paragraph 5.3 of these Regulations and other regulatory enactments in the field of maritime safety, marine environment protection and seafarers’ life and working conditions or the procedures intended therein are not carried out.

10. Unsanitary conditions on board the ship.

11. Evidence from the inspector’s observations that hull or structural deterioration or other deficiencies exist that may place at risk the structural, watertight or weathertight integrity of the ship.

12. Information or evidence that the master or crew is not familiar with essential shipboard operations relating to the safety of the ship or the prevention of pollution, or that such operations have not been carried out.

13. Absence of the schedule of the organisation of work on board the ship and the record book of work and rest hours of the seafarers.

14. Evidence from the inspector’s observations that the seafarers are extremely tired (cannot concentrate on the performance of duties).

15. The garbage record book of the ship or the cargo operation logbook has not been completed in accordance with the requirements of the regulatory enactments in the field of maritime safety and environmental protection.

Minister for Transport 							A. Šlesers

Annex 5

Cabinet Regulation No.197

14 March 2006

## Procedures for the Ship Inspections

The ship inspections shall be carried out in accordance with the procedures specified in the following international documents:

1. Annex 1, Port State Control Procedures, to the Paris Memorandum.

2. The provisions of the International Maritime Dangerous Goods Code.

3. The International Labour Organisation publication Inspection of Labour Conditions on Board Ship: Guidelines for procedures.

4. The principles of safe manning in accordance with IMO resolution A.890(21) specified in the SOLAS Convention and Annexes thereof – Minimum Safe Manning Document (Annex 1) and Guidelines for the Application of Principles of Safe Manning (Annex 2).

Minister for Transport 							A. Šlesers

Annex 6

Cabinet Regulation No. 197

14 March 2006

## Expanded Inspections

I. Categories of Ships Subject to Expanded Inspections

1. Gas and chemical tankers older than 10 years of age, as determined on the basis of the date of construction indicated in the ship safety certificate.

2. Bulk carriers older than 12 years of age, as determined on the basis of the date of construction indicated in the ship safety certificate.

3. Oil tankers with the gross tonnage of more than 3 000 gross tonnes and older than 15 years of age, as determined on the basis of the date of construction indicated in the ship safety certificate.

4. Passenger ships older than 15 years of age other than ro-ro passenger ships and high speed passenger crafts specified in Paragraph 4 of Cabinet Regulation No. 145 of 14 February 2006, Regulations Regarding Safety of Ro-Ro Passenger Ships and High Speed Passenger Crafts.

II. Information to be Notified to the Maritime Safety Inspectorate

5. A ship master or operator shall provide to the Maritime Safety Inspectorate the following information in accordance with the conditions of Paragraph 22 of these Regulations:

5.1. the name of the ship;

5.2. the flag;

5.3. IMO identification number (if any);

5.4. the dead-weight tonnage;

5.5. the date of construction of the ship, as determined on the basis of the date indicated in the ship safety certificates;

5.6. for tankers:

5.6.1. the configuration: single hull, single hull with segregated ballast tanks, double hull;

5.6.2. the condition of the cargo and ballast tanks: full, empty, inert; and

5.6.3. the volume and nature of the cargo;

5.7. the probable time of arrival at the port of destination or pilot station, as required by the Maritime Safety Inspectorate;

5.8. the planned duration of the call;

5.9. the planned operations at the port of destination (loading, unloading, other); and

5.10. the planned statutory survey inspections and substantial maintenance and repair work to be carried out whilst in the port of destination.

III. Procedures Relating to an Expanded Inspection of Certain Categories of Ships

6. Subject to their practical feasibility or any constraints relating to the safety of persons, the ship or the port, the following items must be included in the programme of an expanded inspection:

6.1. ships in general (the categories of ships in accordance with Chapter I of this Annex):

6.1.1. black-out and start of the emergency generator;

6.1.2. inspection of emergency lighting;

6.1.3. operation of the emergency fire-pump with two fire hoses connected to the fire main-line;

6.1.4. operation of the bilge pumps;

6.1.5. closing of the watertight doors;

6.1.6. lowering of a lifeboat to the water;

6.1.7. test of the ship systems (remote emergency stop for, e.g., boilers, ventilation and fuel pumps);

6.1.8. testing of the steering gear including the auxiliary steering gear;

6.1.9. inspection of the emergency source of power to radio installations; and

6.1.10. inspection and, to the extent possible, the test of the separator of an oil/water mixture;

6.2. in addition to the items referred to in Sub-paragraph 6.1. of this Annex, the following items shall also be inspected for gas and chemical tankers:

6.2.1. cargo tank monitoring and safety devices relating to the temperature, pressure and ullage;

6.2.2. oxygen analysing and explosimeter devices, including their calibration, availability of chemical detection equipment (bellows) with an appropriate number of suitable gas detection tubes for the specific cargo being carried;

6.2.3. whether cabin escape sets ensure suitable respiratory and eye protection for every person on board (if required by the products listed on the International Certificate of Fitness or Certificate of Fitness for the Carriage of Dangerous Chemicals in Bulk or Liquefied Gases in Bulk, as applicable);

6.2.4. whether the product being carried is listed in the International Certificate of Fitness for the Carriage of Dangerous Chemicals in Bulk or Certificate of Fitness for the Carriage of Dangerous Chemicals in Bulk or Liquefied Gases in Bulk, as applicable; and

6.2.5. the fixed fire-fighting installations on deck (they may be foam or dry chemical or other as required by the product carried);

6.3. in addition to the items referred to in Sub-paragraph 6.1 of this Annex, the following items shall be inspected for bulk carriers:

6.3.1. possible corrosion of deck machinery mountings;

6.3.2. possible deformation or corrosion of hatch covers;

6.3.3. possible cracks or local corrosion in transverse bulkheads;

6.3.4. access to cargo holds; and

6.3.5. the following documents approved by the flag State or classification society:

6.3.5.1. reports of structural surveys;

6.3.5.2. condition evaluation reports;

6.3.5.3. thickness measurement reports; and

6.3.5.4. a descriptive document;

6.4. in addition to the items referred to in Sub-paragraph 6.1 of this Annex, the following items shall be inspected for oil tankers:

6.4.1. the fixed deck foam system;

6.4.2. fire-fighting equipment in general;

6.4.3. inspection of fire dampers in the engine room, pump room and accommodation;

6.4.4. control of the pressure of the inert gas and the oxygen content thereof;

6.4.5. ballast tanks (at least one of the ballast tanks within the cargo area to be examined from the tank manhole or deck access in the first instance and entered if the inspector establishes clear grounds for the further inspection); and

6.4.6. the following documents approved by the flag State or classification society:

6.4.6.1. reports of structural surveys;

6.4.6.2. condition evaluation reports;

6.4.6.3. thickness measurement reports; and

6.4.6.4. a descriptive document; and

6.5. in addition to the items referred to in Sub-paragraph 6.1 of this Annex, the following items shall be inspected for passenger ships:

6.5.1. fire detection and alarm systems;

6.5.2. closing of fire doors;

6.5.3. the ship general emergency radio transmission system;

6.5.4. the fire drill where, as a minimum, all sets of the firemen's outfits must be demonstrated and a part of the catering crew take part; and

6.5.5. knowledge or key crew members regarding the ship management plan.

7. Depending on the type of a ship, the navigation area and ship operation conditions in the programme of an expanded inspection, additional checks may be included, which are not specified in Paragraph 6 of this Annex.

8. If an expanded inspection of a ship in accordance with the programme referred to in Paragraph 6 of this Annex is implemented during the ship operations, the port State control inspectors, when carrying out the inspection, may not endanger the safe performance of these operations.

9. If deemed appropriate (for example, in order not to prevent the operation of the ship, to check whether the crew practically acts in the case of emergency) the inspection may be continued while the ship is on passage from one port to another, with the consent of the ship master or the operator. The inspectors may not obstruct the operation of the ship, nor may they induce the situations that, in the master's judgement, could endanger the safety of the passengers, the crew and the ship.

Minister for Transport 							A. Šlesers

Annex 7

Cabinet Regulation No. 197

14 March 2006

## Criteria for the Detention of a Ship

1. Before taking a decision regarding the detention of a ship, the port State control inspector shall apply the criteria referred to in Paragraphs 2, 3 and 4 of this Annex.

2. In taking a decision regarding the detention of a ship, the port State control inspector shall take into account the following main criteria:

2.1. the ships which are unsafe to proceed to sea shall be detained upon the first inspection irrespective of how much time the ship will stay in the port; and

2.2. the ship shall be detained if its deficiencies are sufficiently serious to merit the port State control inspector returning to satisfy himself that they have been rectified before the ship sails.

3. When deciding whether the deficiencies found in a ship are sufficiently serious to merit detention, the port State control inspector shall assess whether:

3.1. all the documents, specified in the Conventions referred to in Sub-paragraph 5.3 of these Regulations and in other regulatory enactments related to maritime safety, marine environment protection and seafarers’ living and working conditions, are valid; and

3.2. the ship has the crew required in the Minimum Safe Manning Document and whether the crew is able during the forthcoming voyage:

3.2.1. to navigate safely;

3.2.2. safely handle, carry and monitor the condition of the cargo;

3.2.3. to operate the engine room safely;

3.2.4. to maintain proper propulsion and steering;

3.2.5. to fight fires effectively in any part of the ship if necessary;

3.2.6. to abandon the ship speedily and safely and effect rescue if necessary;

3.2.7. to prevent pollution of the environment;

3.2.8. to maintain adequate stability of the ship;

3.2.9. to maintain adequate watertight integrity;

3.2.10. to communicate in distress situations;

3.2.11. to provide safe and healthy conditions on board; and

3.2.12. to provide maximum information in the case of an accident (using the voyage data recording equipment).

4. If the answer to any of the assessments referred to in Paragraph 3 of this Annex is negative, taking into account the dangerousness of all deficiencies found, the port State control inspector shall consider, whether the ship shall be subject to detention. A combination of deficiencies of a less serious nature (deficiencies the combination of which poses a risk to maritime safety, marine environment protection or the seafarers’ living or working conditions may also warrant the detention of the ship.

5. Deficiencies due to which the ship may be detained:

5.1. deficiencies of a general nature: the lack of valid certificates or documents required by the Conventions referred to in Sub-paragraph 5.3 of these Regulations or other regulatory enactments in the field of maritime safety, marine environment protection and seafarers’ living and working conditions. However, ships flying the flag of States not party to the Conventions referred to in Sub-paragraph 5.3 of these Regulations or not having implemented another regulatory enactment in the field of maritime safety, marine environment protection and seafarers’ living and working conditions, do not have a duty to receive the certificates intended in the Conventions or other regulatory enactments, therefore, the absence of the referred to certificates may not by itself constitute the reason to detain such ships. However, when applying the “no more favourable treatment” clause, substantial compliance with the requirements of the referred to documents is required before the ship sails;

5.2. the areas under the SOLAS Convention (references are given in brackets):

5.2.1. failure of the proper operation of the propulsion and other essential machinery, as well as electrical installations;

5.2.2. insufficient cleanliness of the engine room, an excess amount of oily-water mixtures in the bilges, insulation of the piping including the exhaust pipes in the engine room contaminated by oil products, improper operation of the bilge pumping arrangements;.

5.2.3. failure of the proper operation of the emergency generator, lighting, batteries and switches;

5.2.4. failure of the proper operation of the main and auxiliary steering gear;

5.2.5. the absence of personal life-saving appliances, collective survival craft and the launching arrangements thereof, an insufficient number of survival craft or the serious deterioration thereof;

5.2.6. the absence of the fire detection system, fire alarms, fire fighting equipment, fixed fire-extinguishing installation, ventilation valves, fire dampers and quick-closing devices, they do not comply with the requirements or they are substantially deteriorated and cannot fulfil the functions thereof;

5.2.7. the absence, substantial deterioration or failure of proper operation of the cargo deck area fire protection on tankers;

5.2.8. the absence, non-compliance with the requirements or substantial deterioration of the navigation lights, shapes or sound signals

5.2.9. the absence or failure of the proper operation of the radio equipment for distress and safety communication;

5.2.10. the absence or failure of the proper operation of the navigation equipment, taking into the provisions of SOLAS Regulation V/12(o) into account;

5.2.11. the absence of corrected navigation charts, or all other relevant nautical publications necessary for the intended voyage, taking into account that electronic charts may be used as a substitute for the charts;

5.2.12. sparking exhaust ventilation for the cargo pump rooms (SOLAS Regulation II-2/59.3.1);

5.2.13. serious deficiencies in the organisations of the ship operation specified in Section 5.5 of Annex 1 to the Paris Memorandum;

5.2.14. the number, composition or certification of the crew members is not in compliance with the Minimum Safe Manning Document;

5.2.15. the extended survey programme in accordance with Chapter XI, rule 2 of the SOLAS Convention is not carried out; and

5.2.16. the absence or failure of the proper operation of the voyage data recorder (VDR), if the usage thereof is mandatory;

5.3. the areas under the International Bulk Chemical (IBC) Code (references are given in brackets):

5.3.1. transport of a substance not mentioned in the Certificate of Fitness or missing cargo information (16.2);

5.3.2. missing or damaged high-pressure safety devices (8.2.3);

5.3.3. electrical installations not safe or not corresponding to the code requirements (10.2.3);

5.3.4. the sources of ignition in hazardous locations (11.3.15);

5.3.5. contraventions of special requirements (15);

5.3.6. the exceeding of maximum allowable cargo quantity per tank (16.1); and

5.3.7. insufficient heat protection for sensitive products (16.6.);

5.4. the areas under the International Gas Carrier Code (IGC) (references are given in brackets):

5.4.1. transport of a substance not mentioned in the Certificate of Fitness for the Carriage of Liquefied Gases in Bulk, the International Certificate of Fitness for the Carriage of Dangerous Chemicals in Bulk, the Certificate of Fitness for the Carriage of Dangerous Chemicals in Bulk or missing cargo information (18.1);

5.4.2. missing closing devices for the accommodations or service spaces (3.2.6);

5.4.3. bulkheads not gas tight (3.3.2);

5.4.4. defective air locks (3.6);

5.4.5. missing or defective quick-closing valves (5.6);

5.4.6. missing or defective safety valves (8.2);

5.4.7. electrical installations not safe or not corresponding to the code requirements (10.2.4);

5.4.8. ventilators in the cargo area not operable (12.1);

5.4.9. pressure alarms for cargo tanks not operable (13.4.1);

5.4.10. the gas detection plant or toxic gas detection plant defective (13.6); and

5.4.11. transport of substances to be inhibited, slow down, if the ship does not have a valid inhibitor certificate (17/19);

5.5. the areas under the LL 66 Convention;

5.5.1. significant areas of damage or corrosion, or pitting of plating and associated stiffening in decks and hull affecting seaworthiness or strength to take local loads, unless proper temporary repairs for a voyage to a port for permanent repairs have been carried out;

5.5.2. a recognised case of insufficient stability;

5.5.3. the absence of sufficient and reliable information, in an approved form, which by rapid and simple means enables the master to arrange for the loading and ballasting of the ship in such a way that a safe margin of stability is maintained at all stages and at varying conditions of the voyage, and that the creation of any unacceptable stresses in the ship structure are avoided;

5.5.4. the absence, substantial deterioration or defective closing devices, hatch closing arrangements and watertight doors;

5.5.5. the ship is overloaded; and

5.5.6. the absence of the draft mark or the draft mark impossible to read;

5.6. the areas under the MARPOL Convention, Annex 1 (references are given in brackets):

5.6.1. the absence, serious deterioration or failure of the proper operation of the oily-water filtering equipment, the oil discharge monitoring and control system or the 15 ppm alarm arrangements;

5.6.2. the remaining capacity of the slop or sludge tank insufficient for the intended voyage;

5.6.3. the Oil Record Book not available (20 (5));

5.6.4. an unauthorised discharge bypass fitted; and

5.6.5. the absence of the inspection report or its failure to comply with rule 13. G(3)(b) of the MARPOL Convention;

5.7. the areas under the MARPOL Convention, Annex II (references are given in brackets):

5.7.1. the absence of the procedures manual in accordance with rule 5, Annex II to the MARPOL Convention (P&A Manual);

5.7.2. the cargo is not categorised (3 (4));

5.7.3. no cargo record book available (9 (6));

5.7.4. transport of oil-like substances without satisfying the requirements or without an appropriately amended certificate (14); and

5.7.5. an unauthorised discharge bypass fitted;

5.8. the areas under the STCW Convention:

5.8.1. the crew members do not have appropriate certificates, valid dispensations or they can not provide any documentary proof that an application for endorsement has been submitted to the flag State administration;

5.8.2. failure to comply with the applicable safe manning requirements of the flag State administration;

5.8.3. failure of the navigational or engineering watch arrangements to conform to the requirements specified for the ship by the flag State administration;

5.8.4. the absence in a watch of a person qualified to operate equipment essential to safe navigation, safety radio communications or the prevention of marine pollution;

5.8.5. failure to provide proof of professional proficiency for the duties assigned to seafarers for the safety of the ship and the prevention of pollution; and

5.8.6. inability to provide for the first watch at the commencement of a voyage and for subsequent relieving watches persons who are sufficiently rested and otherwise fit for duty; and

5.9. the areas under the ILO Convention No.147 and ILO PROT 96:

5.9.1. insufficient food for the voyage to the next port;

5.9.2. insufficient potable water for the voyage to the next port;

5.9.3. excessively unsanitary conditions on board;

5.9.4. no heating in the accommodation of a ship operating in the areas where the temperatures may be excessively low;

5.9.5. excessive garbage non-complying with the hygiene norms, blockage by equipment or cargo or otherwise unsafe conditions in the passageways and accommodations; and

5.9.6. evidence that the crew members, who shall ensure the first watch, are too tired (for example, can not concentrate on the performance of duties).

6. Ships shall not be detained, if the inert gas system or cargo-related gear or machinery fail to operate properly or are not maintained in accordance with the operating instructions thereof. It shall be considered as sufficient grounds for stopping cargo operations.

7. The list of deficiencies referred to in Paragraph 5 of this Annex is not complete. Only the deficiencies, which are the basis for detention in the areas in which the STCW Convention is applied and which are referred to in Sub-paragraph 5.8 of this Annex, shall be the only basis for detention in accordance with the referred to Convention.

Minister for Transport 							A. Šlesers

Annex 8

Cabinet Regulation No. 197

14 March 2006

## Refusal of Access to Ports

1. The categories of ships subject to refusal of access to the ports of the European Union:

1.1. gas and chemical tankers;

1.2. bulk carriers;

1.3. oil tankers; and

1.4. passenger ships.

2. Procedures relating to refusal of access to the ports of the European Union:

2.1. if the compliance with Paragraph 48 of these Regulations is determined and the ship is detained in a port of Latvia for the second or third time, as appropriate, the Maritime Safety Inspectorate shall take a decision regarding the access refusal for the ship, informing the master and the owner or the operator of the ship in writing thereof, the Maritime Safety Inspectorate shall also inform the flag State administration, the relevant classification society, the Member States of the European Union, the European Commission, the Centre administratif des affaires maritimes and the Secretariat of the Paris Memorandum; and

2.2. the decision regarding the access refusal for the ship shall take effect as soon as the ship has received permission to leave the port after the deficiencies leading to the detention in the ports of Latvia have been remedied.

3. In order to have the decision regarding the access refusal for the ship revoked, the owner or the operator shall submit a formal request to the Maritime Safety Inspectorate attaching the certificate issued by the flag State administration attesting that the ship fully conforms to the relevant provisions of the Conventions. If the reason of the access refusal was a deficiency regarding which a classification society is to be held liable, the shipowner or operator shall, together with the request for the revocation of the decision regarding the access refusal for the ship, also submit the certificate issued by the classification society, attesting that the ship conforms to the class standards stipulated by that society and to the class assigned for the ship.

4. The decision regarding the access refusal for a ship shall be revoked only after the re-inspection of the ship at any of the ports, if it is determined that the vessel fully complies with the requirements of the relevant Conventions.

5. If the port at which it is intended to carry out the re-inspection of a ship in accordance with Paragraph 4 of this Annex is located within the territory of the European Union, the competent State authority of this port with the agreement of the Maritime Safety Inspectorate may give permission for the ship to proceed to the relevant port for the sole purpose of carrying out the re-inspection and verifying that the ship meets the conditions referred to in Paragraph 3 of this Annex.

6. The re-inspection shall be carried out within the scope of an expanded inspection. All costs of such expanded inspection will be covered by the shipowner or the operator.

7. If the Maritime Safety Inspectorate has determined after the expanded inspection that the ship complies with the requirements specified in Sub-paragraph 5.3 of these Regulations and other regulatory enactments in the field of maritime safety, marine environment protection and the seafarers’ living and working conditions, the decision regarding the access refusal for the ship shall be revoked in accordance with Paragraph 4 of this Annex. The shipowner or operator shall be informed thereof in writing.

8. The Maritime Safety Inspectorate shall also notify the flag State administration, the relevant classification society, other Member States of the European Union, the European Commission, the Centre administratif des affaires maritimes and the Secretariat of the Paris Memorandum of the decision thereof.

9. Information relating to the ships that have been refused access to the ports of the European Union shall be entered into the SEReNAC database and published in accordance with Paragraph 56 of these Regulations and Annex 10.

Minister for Transport 								A. Šlesers

Annex 9

Cabinet Regulation No. 197

14 March 2006

## Minimum Requirements for the Port State Control Inspectors

1. The port State control inspector shall be a person, which is authorised by the Maritime Administration of Latvia to carry out port State control.

2. Minimum requirements for the port State control inspector:

2.1. work experience of at least 1 year as the flag-State inspector carrying out inspections and certification in accordance with the Conventions referred to in Sub-paragraph 5.3 of these Regulations and at least one of the following qualifications:

2.1.1. the certificate of competency as a master, enabling that person to take command of a ship of 1 600 GT or more in accordance with the requirements of Regulation II/2 of the STCW Convention;

2.1.2. the certificate of competency as a chief engineer, enabling that person to take up that task on board a ship whose main power plant has a power equal or superior to 3 000 kW in accordance with the requirements of Regulation III/2 of the STCW Convention; and

2.1.3. the passed examination for a naval architect, mechanical engineer or an engine engineer related to the maritime fields and work experience in that capacity of at least 5 years;

2.2. the inspector the qualification of which complies with the referred to in Sub-paragraph 2.1.1 and 2.1.2 of this Annex shall have work experience of at least 5 years at sea as an officer in the deck or engine department respectively;

2.3. the appropriate higher education or other equivalent training, the education and diploma of a school for ship safety inspectors and experience of at least 2 years in the position of the control inspector of the flag State, carrying out inspections and certification in accordance with the requirements of the Conventions referred to in Sub-paragraph 5.3 of these Regulations;

2.4. knowledge of the English language; and

2.5. knowledge of the Conventions referred to in Sub-paragraph 5.3 of these Regulations and other regulatory enactments related to maritime safety, marine environment protection and seafarers’ living and working conditions, as well as knowledge of the procedures for carrying out port State control.

3. If a person has been hired for the position of the port State control inspector before 1 May 2004 and does not comply with any of the criteria specified in Paragraph 2 of this Annex, he or she may continue to fulfil the duties of the inspector no longer than up to 1 May 2025.

Minister for Transport 								A. Šlesers

Annex 10

Cabinet Regulation No. 197

14 March 2006

## Data Provided by the Maritime Safety Inspectorate to the European Commission

1. The Maritime Safety Inspectorate shall provide the European Commission with the following data for the preceding year every year by 1 April:

1.1. the number of the inspectors acting on its behalf within the framework of port Sate control of shipping. This information shall be communicated completing the following table:

| Port/area | Number of full-time inspectors | Number of part-time inspectors1 | Conversion to full-time |
| --- | --- | --- | --- |
| Port X … |  |  |  |
| Port Y … |  |  |  |
| Total |  |  |  |
| Note. 1 Where the inspections carried out in the context of port State control represent only a part of the inspector’s work, the total number of the inspectors shall be indicated as a number equivalent to the full-time inspectors. |  |  |  |

This information shall be provided for each port of Latvia. For the purposes of this Annex, a port is taken to mean an individual port or the geographical area covered by the inspector or a team of the inspectors, comprising several individual ports where appropriate. The same inspector may work in more than one port or geographical area; and

1.2. the number of individual ships that entered the ports of Latvia.

2. The Maritime Safety Inspectorate shall, every 6 months, provide a list of the ships that entered the ports of Latvia to the European Commission, containing the IMO number of the ship and the date of arrival. This list shall not include ships performing regular ferry services. The information regarding the ships that entered the ports of Latvia, other than performing regular ferry services, shall be entered into the SIReNAC database indicating the IMO number and the date of arrival.

3. The Maritime Safety Inspectorate shall provide the European Commission with the lists of regular ferry services referred to in Paragraph 2 of this Annex, as well as notify regarding all changes in such services.

Minister for Transport 								A. Šlesers

Annex 11

Cabinet Regulation No. 197

14 March 2006

## Publication of Information Regarding Inspections, Ship Detention and Refusal of Access

1. The information published in accordance with Paragraph 56 of these Regulations shall include the following data:

1.1. the name of the ship;

1.2. the IMO number;

1.3. the type of the ship,

1.4. the gross tonnage;

1.5. the year of construction indicated in the Ship Safety Certificate;

1.6. the name or designation and address of the shipowner or the operator of the ship;

1.7. in the case of ships carrying liquid or solid cargoes in bulk, the name and address of the charterer, and the type of charter;

1.8. the flag State;

1.9. one or several classification societies, which have issued the class certificates (if any) to the particular ship;

1.10. one or several classification societies or any other party, which has issued the certificates to the particular ship in accordance with the Conventions on behalf of the flag State;

1.11. the port and date of the previous expanded inspection, if necessary, indicating, whether the order for detention has been given;

1.12. the port and date when the last special inspection was performed, as well as the name of the organisation which carried out the inspection;

1.13. the number of detentions during the previous 24 months;

1.14. the country and port of detention;

1.15. the date when the detention order was repealed;

1.16. the duration of detention in days;

1.17. the number of deficiencies found and the reason for detention, in clear and explicit terms;

1.18. the description of the measures performed by the flag State administration and classification society, as a follow-up to the detention of the ship;

1.19. if the ship has been refused access to any port within the European Union – the reasons for such refusal in clear and explicit terms;

1.20. the indication, whether the classification society or any other private body that carried out the survey has a responsibility in relation to the deficiencies which, alone or in combination, led to detention; and

1.21. the description of the measures taken in the case of the ship which has been allowed to proceed to the nearest appropriate repair yard, or which has been refused access to the European Union ports.

2. The following information shall be included in the SIReNAC database regarding the ships inspected:

2.1. the name of the ship;

2.2. the IMO number;

2.3. the type of the ship,

2.4. the gross tonnage;

2.5. the year of construction;

2.6. the name or designation and address of the shipowner or the operator of the ship;

2.7. in the case of the ship carrying liquid or solid cargoes in bulk, the name and address of the charterer, and the type of charter;

2.8. the flag State;

2.9. one or several classification societies, which have issued to the particular ship the class certificate (if any) ;

2.10. one or several classification societies or any other party, which has issued the certificates to the particular ship in accordance with the Conventions on behalf of the flag State, stating also the relevant certificates;

2.11. the country, port and date of the inspection; and

2.12. the number and nature of the deficiencies.

Minister for Transport 								A. Šlesers