Republic of Latvia

Cabinet

Regulation No. 253

Adopted 25 June 2002

Labour Protection Requirements for Investigation and Extraction of Minerals

Issued pursuant to Section 25, Clause 8, 9 and 18 of the Labour Protection Law

I. General Provisions

1. These Regulations prescribe the labour protection requirements in work of geological search, investigation and extraction of minerals, surface extraction of minerals, as well as the underground extraction of minerals through drilling.

2. The requirements of these Regulations apply to all undertakings involved in:

2.1. the search for minerals;

2.2. investigation of minerals;

2.3. the extraction of minerals; and

2.4. the preparation of extracted minerals for sale, including the storage of natural gas in an underground storage facility, except for the further processing of minerals.

3. Requirements specified in Chapter III of these Regulations shall be applied to the underground extraction of minerals.

4. The employer shall be responsible for compliance with these Regulations.

5. Compliance with these Regulations shall be controlled by the State Labour Inspection.

II. Obligations of Employers

6. For the labour protection of employees the employer shall ensure the following:

6.1. such design, construction, equipment, complementation, utilisation and maintenance of workplaces as the employees shall be able to perform work duties without endangering the safety or health of themselves or other employees;

6.2. utilisation of the workplace only under the supervision of such an employee as appointed by the employer who has the necessary knowledge and skills in the field related to the provision of labour protection at the relevant workplace (hereinafter – appointed employee);

6.3. work of increased risk shall be entrusted to appropriately trained employees and shall be performed in accordance with the instructions of labour protection;

6.4. opportunities to receive appropriate first aid;

6.5. regular drills of employees regarding labour protection issues;

6.6.  providing information on a regular basis to employees and trusted representatives thereof regarding all labour protection measures of employees taken at the workplace, including:

6.6.1. risk assessment and preventative measures;

6.6.2. collective and personal protective equipment.

7. The employer shall ensure the development, and preparation of a labour protection plan (hereinafter – plan) and updating thereof in conformity with changes in the process of work. The plan shall particularly indicate:

7.1. an assessment of the work environment risk;

7.2. the labour protection measures to be taken in order to ensure the safety and health protection of employees;

7.3. a certification that the construction, utilisation and maintenance of the workplace and equipment is safe; and

7.4. the procedures for work organisation, management and labour protection.

8. The plan shall be prepared prior to the commencement of work and it shall be reviewed if the work organisation, technology and work equipment is changed, the workplace has been enlarged or modified or an accident at work has occurred after which the work equipment has been changed and measures have been taken to avoid repetition of such cases.

9. If employees of several employers are employed in one minerals extraction site, the employers shall co-operate by preparing plans and measures to be performed in order to ensure the labour protection of employees.

10. If employees of several employers concurrently work at the same workplace:

10.1. each employer shall be responsible for issues within his or her competence;

10.2. an employer responsible for the workplace shall take the necessary measures for the employer of employees involved in his or her undertaking from another undertaking to receive information regarding the work environment risk prior to such involvement, harmonise and co-ordinate all measures related to labour protection and specify the purpose, measures and procedures of harmonisation in his or her plan; and

10.3. the harmonisation of labour protection measures shall not affect the liability of each employer in this field.

11. For the protection from risk of causing fire, explosion and harmful atmosphere taking into account the specific nature of work, amount and potential number of employees the employer shall ensure the following:

11.1. necessary labour protection measures to prevent, detect and eliminate the cause and extension of fire and explosions, as well as to prevent the occurrence of explosive or harmful atmosphere;

11.2. emergency and rescue measures in order for the employees to be able to leave the workplaces safely and quickly;

11.3. clear emergency routes and exits;

11.4. a sufficient amount of appropriate equipment for first aid, fire-fighting, explosion safety and evacuation;

11.5. a sufficient number of specially trained employees with the necessary equipment who in case of fire or other hazard are able to provide help and act in accordance with the plan; and

11.6. appropriate warning and other information systems so that, if necessary, employees may be informed of the safety measures to be taken and evacuation possibilities, as well as the assistance, evacuation and rescue operations to be commenced.

III. General Labour Protection Requirements at Workplaces

12. Requirements specified in this Chapter shall be applied to work related to the search, investigation, extraction of minerals and preparation for sale thereof taking into account the special features of the workplace, type of activity, circumstances and the work environment risk.

13. Workplaces shall be designed, constructed, equipped, utilised, supervised and maintained so as to withstand the environmental forces anticipated and to conform with the type of utilisation thereof.

14.  Requirements determined for the organisation and utilisation of workplaces shall be the following:

14.1. workplaces shall be organised so as to provide adequate protection against the harmful impact of the work environment. Workplaces shall be maintained in order, any hazardous substances or sediment shall be removed or controlled in order not to endanger the safety and health of employees;

14.2. workplaces shall be planned and organised (equipped) in accordance with the principles of ergonomics;

14.3. danger areas in which serious and direct hazard exists shall be demarcated and indicated by warning signs; and

14.4. isolated workplaces shall be provided with appropriate means of communications.

15. In order to ensure the safety and health protection of employees during the work shift at each workplace, the employer shall:

15.1. appoint employees (appointed employees) who performs the supervision; and

15.2. specify the procedures for supervision;

16. The employer may personally assume the supervision and responsibility referred to in Paragraph 15 of these Regulations for the workplace if he or she has been trained in conformity with requirements specified in regulatory enactments.

17. There shall be a specified number of employees at the workplace trained in conformity with the specific nature and amount of work.

18. Employees shall be provided with necessary information and instructions, as well as the following training of employees shall be performed:

18.1. initial training upon commencing of work;

18.2. periodic training, not less than once a year; and

18.3. additional training, if changes of the workplace, work procedure (technologies) or work equipment, disasters and accidents have occurred or there have been other reasons due to which the work environment risk has increased.

19. Instructions and other regulatory documents for each workplace shall be prepared, as well as instructions and training shall be performed in accordance with requirements prescribed by the labour protection regulatory enactments.

20. Safe working methods shall be used at each workplace performing any work.

21. A working procedure in accordance with the plan shall be developed and documented for the performance of dangerous work and for the performance of such simple work which together with other activities may cause serious and direct danger.  The appointed employee shall issue a work permit before the commencement of work. Regulations, as well as safety measures to be taken before work, during the work and after the work shall be specified in the work permit.

22. In using mechanical and electrical equipment and tools (hereinafter – work equipment) the following conditions shall be complied with:

22.1. work equipment shall be installed, prepared for work, operated and the maintenance thereof shall be performed in compliance with the labour protection requirements and the relevant regulatory enactments;

22.2. work equipment shall be relevantly appropriate for utilisation in a zone in which there is or may occur an explosive environment, there is or may be vapour or fast-evaporating liquid; and

22.3. if necessary, shall be equipped with appropriate protective equipment and appropriate (fail-safe) security systems.

23. Requirements for the technical maintenance of work equipment shall be as follows:

23.1. work equipment shall be examined, maintained and controlled regularly in accordance with a prepared scheme; and

23.2. any part of work equipment shall be examined, maintained and controlled by a trained employee.

24. Safety equipment shall be technically maintained ready to use and in working order, and in performing the maintenance the operation principles of safety equipment shall be taken into account.

25. During drilling work appropriate control devices shall be used to prevent an unforeseen discharge taking into account the conditions of drilling and servicing work.

26. General safety measures for protection against the impact of harmful atmosphere, the risk of explosion and fire are as follows:

26.1. measures have been taken to assess the presence of hazardous and potentially explosive substances in the atmosphere and to measure the concentration of such substances;

26.2. automatic and continuous gas concentration measuring devices, automatic alarm devices and devices which ensure automatic power cut off from electrical installations and internal combustion engines shall be placed at specified places in conformity with the plan;

26.3. if automatic measuring is performed, the results thereof shall be registered and stored in accordance with the plan;

26.4. smoking is prohibited in places which are subject to the risk of fire or explosion and such places are indicated in conformity with regulatory enactments regarding the use of safety signs at workplaces; and

26.5. it is prohibited to use an open flame and perform work that may cause a danger of ignition, except in cases when the relevant safety measures have been taken to prevent the ignition or explosion.

27. Safety measures for the protection against the impact of harmful atmosphere are as follows:

27.1. if hazardous substances accumulate or may accumulate in the atmosphere, measures have been taken in order to:

27.1.1. prevent the release of hazardous substances;

27.1.2. ensure collection thereof at the place of release or relocation; and

27.1.3. ensure safe accumulation or dispersion of hazardous substances as not to endanger employees;

27.2. for the safety and health protection of employees respiratory and resuscitation equipment shall be placed in places where the impact of the harmful atmosphere is possible. Taking into account the number of employees who may be subjected to the effect of hazardous substances, employees trained to handle the referred to equipment shall be appointed. The equipment shall be stored and maintained in working order in conformity with the specified requirements; and

27.3. if hydrogen sulphide or other toxic gases are or may be present in the atmosphere, required protective equipment shall be specified in a separate chapter of the plan and prevention measures to be taken shall be determined.

28. Safety measures for protection against the risk of explosion are as follows:

28.1. all necessary measures have been taken to prevent the formation of an explosive atmosphere and the increase of concentration of hazardous substances;

28.2. at places where the risk of explosion exists the necessary measures shall be taken in order to prevent the ignition of explosive atmosphere or the explosion of unexploded projectiles; and

28.3. procedures for the prevention of an explosion have been developed in which the necessary equipment and measures to be taken shall be specified in detail.

29. Requirements for fire alarm systems, automatic fire detectors and fire-fighting equipment are as follows:

29.1. taking into account the specific nature of the workplace, size of premises and purposes of use, equipment located therein, physical and chemical properties of materials used, as well as the maximum potential number of persons present, workplaces shall be provided with appropriate fire-fighting equipment in sufficient quantity and, if necessary, with fire detectors and alarm systems;

29.2. fire-fighting equipment, fire detectors and fire alarm systems shall be examined regularly and maintained in accordance with a specified order;

29.3. means of fire-fighting shall be easily accessible and simple to use and the location thereof shall be indicated by signs located in conformity with regulatory enactments regarding the use of safety signs at workplaces; and

29.4. procedures of fire safety measures have been developed in which the fire safety measures to be taken shall be specified.

30. Explosives and explosion-initiating devices shall be stored, transported and utilised only by employees trained for such work. Such work shall be organised and performed without employees being subject to risk.

31. Traffic routes shall conform to the following requirements:

31.1. traffic routes shall ensure the possibility for employees to access workplaces safely and leave them quickly in case of emergency;

31.2. employees working in the vicinity of traffic routes shall be provided with a possibility of easy, safe and appropriate access to the traffic routes, stairs, including secured and fixed ladders, loading and unloading platforms and ramps;

31.3. the number and size of pedestrian walkways and traffic routes shall correspond to the number of users and the activities to be carried out;

31.4. if traffic routes are utilised by vehicles and pedestrians, safety clearance of sufficient width shall be installed or adequate protective devices shall be provided for each user of the construction site;

31.5. traffic routes shall be located at a specified distance from doors, gates, thoroughfares, pedestrian walkways, corridors and staircase landings;

31.6. traffic routes shall be clearly identified, regularly monitored and maintained; and

31.7. if vehicles enter workplaces, traffic regulations shall be complied with.

32. Requirements for danger areas are as follows:

32.1. danger areas shall be clearly indicated in conformity with regulatory enactments regarding the use of safety signs at workplaces;

32.2. if workplaces have danger areas where employees or objects may fall, such areas, if possible, shall be equipped so as to prevent the entrance of unauthorised persons; and

32.3. necessary measures have been taken to protect employees who are permitted to be present in a danger area.

33. Emergency routes and exits shall conform to the following requirements:

33.1. emergency routes and exits shall ensure access to the safe zone (by the shortest way), assembly or evacuation area as quickly as possible. Exits shall be clear and in case of danger shall ensure an evacuation of employees that is rapid and as safe as possible from all workplaces to a safe zone, assembly or evacuation area;

33.2. number, distribution and dimensions of emergency routes shall be determined taking into account the purpose of utilisation of workplaces, equipment located therein and dimensions thereof, as well as the maximum potential number of persons present;

33.3. emergency routes and exits shall be indicated in conformity with the regulatory enactments regarding requirements for the use of safety signs at workplaces;

33.4. emergency routes and exits requiring illumination shall be provided with emergency lighting of adequate intensity if the work lighting is damaged;

33.5. emergency doors shall not be locked during the presence of employees, and shall be immediately opened outwards in case of danger; and

33.6. accommodation and rest rooms shall have at least two separate emergency exits located as far apart as possible and through which the safe zone is accessible.

34. Workplaces outdoors shall conform to the following requirements:

34.1. workplaces, traffic routes and other areas or installations outdoors which are used by employees during the course of work or where employees are located, shall be organised in such a way as to enable pedestrians and vehicles to move safely;

34.2. if necessary, workplaces outdoors shall be provided with artificial lighting;

34.3. employees shall be protected against unfavourable weather conditions and from falling objects;

34.4. employees shall be provided with appropriate protective equipment against slipping and falling;

34.5. employees shall have the possibility to leave the workplace in cases of an accident, fire or other situations dangerous to health and life, as well as the provision of first aid shall be ensured; and

34.6. employees shall be protected against the effect of a harmful working environment (for example, gases, vapour, dust, noises or vibration).

35. Means of evacuation and rescue shall be located at special places, maintained in working order and indicated in conformity with regulatory enactments regarding the use of safety signs at workplaces. Employees shall be trained in the use of the means of evacuation and rescue.

36. Training in safety measures shall be held at regular intervals at all workplaces where employees are usually present. The purpose of the training is to provide knowledge and examine the abilities and skills of employees to whom specific duties have been assigned in case of emergency, particularly related to the use, handling or operation of the rescue equipment. If necessary, other employees shall be also trained to use, handle or operate correctly the relevant equipment.

37. The first aid facilities are as follows:

37.1. the equipment and means of first aid shall be available at all workplaces in conformity with working conditions and such equipment and means shall be indicated in accordance with regulatory enactments regarding the use of safety signs at workplaces;

37.2. taking into account the type of activity, extent of work and work environment risk, one or more first aid rooms shall be organised in which first aid equipment and means is provided, and for which easy access to stretchers can be ensured;

37.3.  instructions regarding first aid in cases of an accident shall be placed at clearly visible places in the first aid rooms;

37.4. the first aid rooms shall be indicated in conformity with regulatory enactments regarding the use of safety signs at workplaces;

37.5. first aid kits shall be available at all workplaces where, due to working conditions, they are necessary, and the location thereof shall be indicated; and

37.6. sufficient number of employees shall be trained in the provision of the first aid.

38. Requirements for lighting of workplaces are as follows:

38.1. workplaces shall be provided with lighting sufficient to the safety and health of employees;

38.2. illuminating devices at workplaces and exits shall be located in such a manner that employees are protected from risk caused by accidents and occupational diseases due to insufficient lighting;

38.3. workplaces in which sudden switching off of lighting may cause harm to the safety and health of employees, shall be equipped with sufficient emergency lighting;

38.4. if necessary employees shall be provided with individual illuminating devices;

38.5. illuminating devices shall be located so as to provide lighting in working rooms, emergency routes, loading zones and danger areas at times when there are employees present.

39. Requirements for the ventilation of enclosed workplaces and microclimate are as follows:

39.1. a sufficient supply of fresh air shall be ensured in workplaces, taking into account the nature of the work and the physical load of employees;

39.2. air-conditioning or ventilation systems shall be maintained in good order and regularly cleaned, and the efficiency of operation thereof shall be inspected;

39.3. a ventilation control system shall be installed, which indicates disturbances in the operation of the ventilation system;

39.4.  operation of mechanical ventilation systems and air-conditioning equipment shall ensure the required microclimate;

39.5.  concentration of such substances which upon coming into contact with the human body cause an injury, an occupational disease or other health problems (harmful substance) may not exceed the maximum permissible level of occupational exposure to harmful substances;

39.6.  the ventilation system shall be linked with fire-detectors or fire-fighting equipment, so that, during a fire, the ventilation is switched off, preventing the fire from spreading to other premises;

39.7. a temperature appropriate for the nature of work and physical load of employees shall be ensured in working premises; and

39.8. the temperature in rest areas, rooms for duty staff, changing rooms, lavatories, showers, canteens and first aid rooms shall be appropriate for the intended use for such areas.

40. Floors, walls, ceilings and roofs of rooms shall conform to the following requirements:

40.1. floors of the rooms shall be stable and secured, they must not be slippery or have dangerous bumps, holes or slopes that may endanger the safety and health of employees;

40.2. workplaces shall have adequate thermal insulation, taking into account the nature of the work and the physical load of employees;

40.3. floors, walls and ceiling in the rooms shall be easy to clean or refurbished in conformity with the standards of hygiene;

40.4. transparent or translucent walls, in particular all-glass partitions in work premises and in the proximity to traffic routes shall be clearly marked, made from safe material or demarcated in such a way as to prevent workers from coming into contact with walls or being injured in case the walls shatter; and

40.5. measures shall be taken to prevent performance of work on or access to roofs by employees, if the roofs are made of insufficiently durable materials or may otherwise create a hazard to the safety and health of workers (for example, if not fenced off), unless such equipment is provided that allows the work to be performed safely.

41. Windows and skylights shall conform to the following requirements:

41.1. windows, skylights and glass partitions shall not allow excessive effects of sunlight in the workplace, taking into account the specific nature of the work and the workplace;

41.2. employees shall have the possibility to open, close, adjust or secure windows, skylights and ventilation wings in a safe manner;

41.3. open windows, skylights or ventilation wings shall not create a hazard to the employees; and

41.4. windows and skylights shall be designed together with facilities for cleaning thereof, or shall be fitted with devices allowing such to be cleaned without the risk to the employees carrying out such work or to employees in the proximity of the windows.

42. Doors and gates shall conform to the following requirements:

42.1. the location, number and dimensions of doors and gates, and the materials used in construction thereof, shall be determined by the characteristics and use of the premises, and the nature of work;

42.2. transparent doors shall be labelled at a conspicuous level;

42.3. swing doors and gates shall be transparent or have translucent elements;

42.4. transparent or translucent parts in doors and gates, which parts are not made of durable material or may otherwise cause a risk of incurring injuries or traumas, shall be protected against breakage;

42.5. sliding doors shall be fitted with a safety device to prevent them from being derailed and falling over;

42.6. doors and gates opening upwards shall be fitted with a mechanism to prevent such from falling back;

42.7. safety signs shall mark doors opening to the emergency routes and exits and at any they can be opened from the inside without assistance;

42.8. doors for pedestrians shall be organised in proximity to the gates intended for vehicles, if passage of pedestrians through the gates can not be ensured;

42.9. doors for pedestrians shall be marked and kept unobstructed;

42.10. the operation of mechanical doors and gates shall not endanger employees, and the doors and gates shall be fitted with easily identifiable and accessible emergency shut-down devices, which allow such to be opened manually if due to failure of the power supply the doors and gates do not open automatically;

42.11. gates intended for vehicles shall be marked in warning colours from the inside; and

42.12. if chains or similar devices are utilised for the demarcation of any territory, they shall be visible and marked by signs denoting prohibition or warning.

43. Requirements of dimensions and space to ensure the freedom of movement of employees at workplaces:

43.1. work premises shall be sufficiently large and high and with sufficient air as the employees can perform the work without risk to the safety and health thereof; and

43.2.  unoccupied sections of work premises shall be large enough as to allow sufficient freedom of movement to employees and to enable them to perform their work safely.

44. Rest rooms shall conform to the following requirements:

44.1. in conformity with the specific nature of work and number of employees comfortable and accessible rest rooms shall be provided, if that has been determined by the employer or trusted representatives of employees;

44.2. the rooms shall be sufficiently spacious and with an adequate number of tables and seats with backrests;

44.3. if due to the specific nature of work frequent and regular breaks are necessary, but rest rooms have not been organised, other rooms shall be organised in which employees can stay during breaks;

44.4. measures shall be taken to protect non-smokers against the discomfort caused by smoking; and

44.5. the requirements of this Paragraph may not be applied to persons employed in offices or other similar workplaces, who have adequate possibilities for rest in their work premises during interruptions of work.

45. Changing rooms and lockers shall conform to the following requirements:

45.1. changing rooms shall be organised if it is intended that special work clothes are to be worn during the process of work and where, for reasons of health or propriety, employees are not to be expected to change in another place;

45.2. changing rooms shall be easily accessible, sufficiently spacious, and equipped with seating and lockers.

45.3. if working conditions are related to the exposure of harmful substances, humidity, dirt or other factors the storing of personal clothing and work clothes in separate lockers shall be ensured;

45.4. a possibility to dry the wet work clothes shall be ensured;

45.5. separate changing rooms for men and women shall be provided or, if that is not possible, separate use of changing rooms for men and women shall be provided; and

45.6. if changing rooms are not required in accordance with Sub-paragraph 45.1 of these Regulations, each employee shall be provided with a place to lock and store his or her personal clothing and belongings.

46. Showers, washbasins and lavatories shall conform to the following requirements:

46.1. the employer in co-operation with trusted representatives of employees shall determine the necessity to organise shower rooms in conformity with the nature of work to be performed or the health protection of employees.  If possible, separate shower rooms shall be organised for men and women, or, if this is not possible, separate use of shower rooms for men and women shall be ensured;

46.2. shower rooms shall be sufficiently spacious to permit workers to wash without hindrance to each other in conformity with hygiene requirements;

46.3. the showers shall have a hot and cold water supply;

46.4. if showers are not organised in accordance with Sub-paragraph 46.1 of these Regulations, washbasins with a supply of hot and cold running water shall be installed in the proximity of the work premises and the changing rooms;

46.5. in the cases specified in Sub-paragraph 46.4 of these Regulations, if necessary, the washbasins shall be separated or used separately by men and women;

46.6. if shower rooms or washbasins are installed separately from the changing rooms, there must be easy communication between the two;

46.7. the number of lavatories and washbasins shall be sufficient, taking into account the number of employees, and such shall be equipped with personal hygiene products (toilet paper, soap, towels or other types of devices for hand drying);

46.8. lavatories and washbasins shall be installed near the work premises and rest rooms, changing rooms and shower rooms; and

46.9. separate lavatories shall be provided or separate use of lavatories for men and women and a separate cabin for handicapped persons shall be ensured.

47. If pregnant women or nursing mothers are employed, they shall be ensured the opportunity to lie down and rest in appropriate conditions.

48. Workplaces shall be organised so as it is possible to employ handicapped persons therein, if necessary, especially fitting the doors, pedestrian walkways, stairs, showers, washbasins and lavatories.

49. Mineral spoil created during the operation of quarries shall be maintained such as to ensure the safety and health of employees.

IV. Special Labour Protection Requirements for Extraction of Minerals in Coastal Areas by Drilling Thereof

50. Fire-safety requirements are as follows:

50.1. whenever workplaces are designed, constructed, equipped and operated or maintained, measures shall be taken to prevent fire from starting and spreading from the sources identified in the plan.  Fast and effective fire fighting shall be ensured;

50.2. workplaces shall be equipped with the relevant means of fire-fighting or automatic fire-fighting equipment, if necessary, with automatic fire detectors and fire alarm systems.  The places of location thereof shall be indicated in accordance with the procedures specified in regulatory enactments;

50.3. fire-fighting devices (non-automatic) shall be easily accessible and simple to use, as well as protected from potential damage;

50.4. fire-safety regulations in which safety measures to be performed for protection against fire, detection thereof, combat with outbreak and spreading thereof shall be specified in accordance with these Regulations, and shall be located in a visible place.

51. In accordance with the plan a possibility to control the specific equipment with remote devices in case of emergency shall be provided. Such equipment shall include systems that isolate an unexpected discharge from wells (for example, petroleum, gas escape), machinery and pipelines, as well as ensure the discontinuation of the discharge.

52. Requirements for means of communication in normal conditions and in an emergency situation are as follows:

52.1. if it has been specified in the plan, each workplace where employees are present shall be provided with:

52.1.1. an acoustic and optical system transmitting an alarm indication to every employee of the relevant workplace; and

52.1.2. an acoustic system capable of being heard distinctly in all places where employees are frequently present;

52.2. facilities for raising an alarm shall be located at suitable places; and

52.3. employees shall be provided with special communication equipment at rarely utilised workplaces.

53. In accordance with the plan the employer shall specify safe assembly areas and prepare a list in which employees gathering in such areas shall be specified.

54. Means of evacuation and rescue shall be easily accessible and located at special places, maintained in working order and indicated in conformity with regulatory enactments regarding the use of safety signs at workplaces.  Employees shall be trained in the use of means of evacuation and rescue.

55. If emergency routes are complicated and air unsuited for respiration is present or may be present, the workplace shall be provided with self-contained escape apparatus for immediate use.

56. Safety training measures shall be taken at regular intervals at all workplaces where employees are usually present. The purpose of the training is to provide knowledge and examine the abilities and skills of such employees to whom specific duties have been assigned in case of emergency, also the skills related to the use, handling or operation of the rescue equipment. If necessary other employees shall be also trained to use, handle or operate correctly the referred to equipment.

57. If necessary, traffic regulations shall be determined for workplaces crossed by traffic routes.

V. Special Labour Protection Requirements for Extraction of Minerals at Sea by Drilling Thereof

58. In addition to requirements referred to in Paragraph 8 of these Regulations the employer who is responsible for the workplace referred to in this Section in accordance with the regulatory enactments shall ensure that the measures to be taken in order to protect the safety and health of employees in normal conditions and emergency situation shall be specified in the plan.

59. In accordance with Paragraph 58 of these Regulations the plan shall:

59.1. specify the risk factors (sources) related to the workplace, as well as any supplementary activities that may cause accidents with serious consequences to health and safety of employees;

59.2. assess the threat caused by the risk factors referred to in Sub-paragraph 59.1 of these Regulations;

59.3. specify that all necessary safety measures have been taken to prevent the possibility of accidents and disasters caused by the risk factors (sources) referred to in Sub-paragraph 59.1 of these Regulations, to restrict the spreading of danger and to ensure effective and controlled evacuation of the workplace in case of an emergency situation;

59.4. indicate that the supervision system in normal conditions and in case of emergency situation conforms to the requirements specified in the Labour Protection Law and these Regulations; and

59.5. specify the parameters of meteorological conditions, during the existence of which the performance of work at sea shall not be permitted .

60. In planning and implementing all the stages referred to in these Regulations, the employer shall comply with the procedures and methods specified in the plan.

61. Fire-safety requirements are as follows:

61.1. safety measures have been taken in accordance with the plan to prevent the outbreak and spreading of fire, to detect and combat thereof;

61.2. fire walls (where necessary) to separate the fire-risk zones;

61.3. in accordance with the risks specified in the plan all workplaces shall be provided with relevant fire detection and protection systems, fire-fighting systems and alarm systems, for example:

61.3.1. fire detection systems;

61.3.2. fire alarm systems;

61.3.3. fire water mains;

61.3.4. fire hydrants and hoses;

61.3.5. water deluge systems and water monitors;

61.3.6. automatic sprinkler systems;

61.3.7. gas extinguishing systems;

61.3.8. foam systems;

61.3.9. portable fire extinguishers; and

61.3.10. fireman's equipment;

61.4. fire-fighting devices (non-automatic) shall be easily accessible and simple to use, as well as protected from possible damage;

61.5. fire-safety regulations in which the safety measures to be performed for protection against fire, detection thereof, combat with outbreak and spreading thereof in accordance with these Regulations are specified, shall be located in a visible place.

61.6. Emergency systems shall be demarcated or otherwise protected to ensure the operation thereof during an accident. If necessary, such systems shall be duplicated; and

61.7. locations of fire-fighting equipment shall be indicated in conformity with regulatory enactments regarding the use of safety signs at workplaces.

62. In accordance with the plan the remote control system shall be set up for emergency situations:

62.1. such system shall include control points at appropriate places which may be utilised in an emergency situation, if necessary including the control points in safe assembly areas and evacuation points;

62.2. the remote control system shall include equipment for ventilation, emergency shutdown of flammable equipment, for the prevention of the escape of flammable fluids and gas, fire protection and well control.

63. Requirements for means of communication in normal conditions and in an emergency situation are as follows:

63.1. if it has been determined in the plan, each workplace where employees are present shall be provided with:

63.1.1. an acoustic and optical system transmitting an alarm indication to every employee of the relevant workplace;

63.1.2. an acoustic system capable of being heard distinctly in all places where employees are frequently present, and

63.1.3. a system maintaining communication with the coast and rescue services;

63.2. the systems referred to in Sub-paragraph 63.1 of these Regulations shall be capable of operating in an emergency situation;

63.3. an acoustic system, if necessary, shall be supplemented by a communication system that is not dependent on a possible power supply failure;

63.4. facilities for raising an alarm shall be located at suitable places; and

63.5. communication systems appropriate to the circumstances shall be provided at workplaces where employees are very rarely present.

64. Requirements for safe assembly areas and the list of employees are as follows:

64.1. the employer shall take adequate measures to protect evacuation areas and safe assembly areas from heat, smoke and, as far as possible, the effects of an explosion, and to ensure that escape routes to and from evacuation areas and safe assembly areas remain usable.

64.2. these measures shall provide protection to employees for a sufficient period in order if necessary to organise and perform safe evacuation, escape and rescue;

64.3. in accordance with the plan a possibility to control the equipment referred to in Sub-paragraph 62.2 of these Regulations by means of remote control devices and to communicate with the coast and rescue services shall be ensured at one of the protected places referred to in Sub-paragraph 64.1 of these Regulations;

64.4. safe assembly areas and evacuation areas must be easily accessible from accommodation premises and workplaces;

64.5. each safe assembly area shall have a list in which employees who will gather at the relevant place shall be specified. The list shall be kept up to date;

64.6. the list of such employees who have special duties in an emergency situation shall be prepared and placed in appropriate places at the workplace.  Their names shall be noted in the written instructions referred to in Paragraph 19 of these Regulations.

65. For evacuation and rescue of employees the following measures have been taken:

65.1. employees have been trained for action in an emergency situation;

65.2. in addition to the general training employees have been trained in conformity with the specific nature of the workplace and such training for each particular workplace shall be determined in the plan;

65.3. employees have been trained how to act in emergency situations taking into account the criteria specified in the plan;

65.4. each workplace shall be provided in sufficient quantity with appropriate means of evacuation for emergency situations and means for direct escape to the sea and further to the coast;

65.5. procedures for rescue operations in the sea and in situations when the evacuation of the workplace is necessary. It shall provide the utilisation of standby vessels and helicopters and specify the criteria related to the capacity and arrival time of the standby vessels and helicopters;

65.6. the shortest time possible necessary for the arrival to each platform shall be specified in the plan;

65.7. standby vessels shall be designed and equipped in conformity with evacuation and rescue requirements;

65.8. the minimum requirements for every lifeboat, survival raft, life-buoy and life-jacket which are specified:

65.8.1. they shall be suitable and equipped so as to ensure the maintenance of human life by the end of evacuation;

65.8.2. they shall correspond to the number of employees;

65.8.3. they shall conform to the particular workplace;

65.8.4. they shall be constructed of suitable materials having regard to their life-saving function and the circumstances in which they may be used and kept ready for use, and

65.8.5. they shall be clearly visible (in appropriate colour) during use and equipped with devices that the user can use to attract the attention of rescuers; and

65.9. life-saving devices shall be ready for immediate use.

66. Safety drills shall be performed in accordance with the following procedure:

66.1. at all workplaces where employees are usually present the safety drills shall be organised regularly for such employees whose duty is to use, control or operate the emergency equipment or handle the equipment and the execution of their duties shall be examined taking into account the referred to criteria in the plan;

66.2. the emergency equipment utilised in drills shall be examined, cleaned and, if necessary, refilled or changed.  Portable equipment utilised in drills shall be transferred to the place where it is usually kept; and

66.3. lifeboats shall be checked for operation.

67. Taking into account the size and type of operation of the platform, one or several first aid posts shall be organised.

68. Accommodation rooms shall be designed and constructed in compliance with the following requirements:

68.1. if in conformity with the nature, amount and duration of work it is required, the employer shall provide employees with accommodation.  Accommodation shall be:

68.1.1. in accordance with the plan, protected against the consequences of explosions, the infiltration of smoke and gas and the outbreak and spread of fire;

68.1.2. equipped with ventilation, heating and lighting;

68.1.3. provided at each floor with at least two independent exits leading to escape routes;

68.1.4. protected against noise, smells and fumes likely to be hazardous to health from other areas, and against inclement weather conditions; and

68.1.5. separated from all workplaces and located away from danger areas;

68.2. the accommodation rooms shall have suitable beds or bunks in conformity with the number of employees;

68.3. any room provided as sleeping accommodation shall have a place for the occupants to store their clothes;

68.4. separate sleeping rooms for men and women shall be provided;

68.5. accommodation rooms shall have a sufficient number of showers and washbasins with the supply of hot and cold running water;

68.6. separate shower rooms for men and women shall be provided or separate use of shower rooms shall be ensured;

68.7. shower rooms shall be sufficiently spacious to permit workers to wash without hindrance to each other in conformity with hygiene requirements;

68.8. accommodation rooms shall have a sufficient number of lavatories and washbasins;

68.9. separate lavatories shall be provided or separate use of lavatories for men and women and a separate cabin for handicapped persons shall be ensured; and

68.10. accommodation rooms and equipment thereof shall comply with the hygiene requirements.

69. When utilising a helicopter the following requirements shall be complied with:

69.1. helicopter decks at workplaces shall be sufficiently large and located as to provide an opportunity to use the helicopter in the most severe conditions in which the helicopter can operate;

69.2. the helicopter deck shall be designed and constructed in conformity with the intended work;

69.3. equipment required in case of an accident of the helicopter shall be provided and it shall be kept in the immediate vicinity of the helicopter landing area; and

69.4. on platforms where the employees reside, a sufficient number of employees trained on how to act in case of emergency shall be present on the helicopter deck during helicopter movements.

70. Safety and stability requirements for location of platforms at sea are as follows:

70.1. all the necessary measures have been taken to ensure the labour protection of employees in the extraction of minerals by drilling while the offshore platforms are at sea;

70.2. preparatory operations for the positioning of off-shore platforms shall be performed so as to ensure stability and safety thereof; and

70.3. equipment and procedures in performing the activities referred to in Sub-paragraph 70.1 of these Regulations shall be such as to reduce the risk to employees in the extraction of minerals by drilling taking into account normal conditions and emergency situations.

VI. Special Labour Protection Requirements for Surface Extraction of Minerals

71. An employer shall provide all entries in plans regarding all the necessary measures, which have been taken to provide labour protection of employees in normal conditions and emergency situations.

72. The plan, if necessary, shall be updated and it shall be available when performing an inspection at the workplace.

73. The work shall be performed in accordance with the labour protection plan.

74. The work shall be organised taking into account the indication in the plan regarding possible falls and slips of rocks.

75. Prior to the performance of work it shall be ascertained that the characteristics of the place of mineral resource extraction and the height and slope of extraction surface conforms to the nature, stability of rocks, work methods and the nature of the mechanisms used.

76. Transportation roads shall be sufficiently stable, constructed, organised and maintained so as the equipment and vehicles can be moved freely. Road slopes shall be stable and without overhangs.

77. Before commencing or restarting of work the density of surface or the possibility of rock or stone falls or slopes from surfaces located above the work zone or transportation roads shall be examined. If necessary, the surfaces shall be scaled.

78. The surface of a quarry and spoil heaps shall be utilised so as not to cause instability.

VII. Closing Provisions

79. These Regulations shall come into force on 1 July 2002.

Prime Minister									A. Bērziņš

Minister for Welfare									V. Jaksons