Republic of Latvia

Cabinet

Regulation No. 318

Adopted 19 September 2000

Regulations on the Requirements Regarding the Organising of Work Safety and Health Protection and the Equipping of Workplaces at Construction Sites

Issued pursuant to Section 16 of the

Law On Protection at Work

I. 	General Provisions

1. These Regulations determine the requirements regarding the organising of work safety and health protection and the equipping of workplaces at construction sites, as well as the obligations and liability of persons participating in construction for failure to comply with these requirements.

2. These Regulations apply to the following construction works:

2.1. underground works;

2.2. earthworks including the installation of artificial earthworks;

2.3. construction of external engineering networks;

2.4. general construction works, including:

2.4.1. concrete work;

2.4.2. masonry work;

2.4.3. insulating and roofing work;

2.4.4. carpentry and joinery work;

2.5. assembly and dismantling of pre-fabricated (ready-made) building structures;

2.6. finishing work, including:

2.6.1. plaster work;

2.6.2. tiling work;

2.6.3. painting work;

2.6.4. wallpapering work;

2.6.5. glazing work;

2.7. installation and assembly of internal plumbing networks and equipment for buildings;

2.8. demolition and pulling down of structures;

2.9. building repair work;

2.10. reconstruction, renovation and restoration of structures;

2.11. maintenance of structures, if this is associated with any of the aforementioned types of work;

2.12. drilling work; and

2.13. dredging and underwater work.

3. Construction work that poses increased risk to the safety and health of workers is the following:

3.1. work where workers are exposed to the risk of:

3.1.1. being buried underground in earthfalls;

3.1.2. being flooded by water;

3.1.3. falling into water and drowning; or

3.1.4. falling from a height exceeding one metre;

3.2. work where workers come into contact with harmful chemical or biological substances, which create hazards to the safety and health of workers or which, in accordance with the requirements of regulatory enactments, are subject to special supervision;

3.3. work where workers are exposed to the risk of being irradiated with ionizing radiation and the performance of which is regulated by regulatory enactments for protection against radiation;

3.4. work in high voltage power line protective zones;

3.5. work performed underground (for example, in wells or tunnels);

3.6. work where a system of air supply is necessary  for workers;

3.7. work where workers are exposed to increased air pressure (for example, in caissons);

3.8. work involving the use of explosives; and

3.9. work involving the assembly or dismantling of building structures, building elements or equipment weighing more than two tons.

4. These Regulations do not apply to the equipping of workplaces in mineral-extracting industries.

5. The implementation of these Regulations shall not affect the liability of the contractor as an employer as determined in the Law On Protection at Work and other regulatory enactments.

II.	Project Supervisor; Appointment of Co-ordinators; Prior Notice

6. On a construction site on which construction is or will be carried out by several contractors, commissioning parties or project supervisors, one or more co-ordinators for work safety and health protection at the project preparation stage and co-ordinators for work safety and health protection at the project execution stage shall be appointed.

7. A project is a process comprised of an aggregate of co-ordinated and controlled procedures for which a starting date and an end date have been specified and an objective corresponding to its specific requirements has to be achieved within a set time period, and within cost and resource limits.

8. A project supervisor is a natural or legal person who is responsible for the preparation, execution and supervision of the execution of construction work and acts on behalf of the commissioning party.

9. The co-ordinator for work safety and health protection at the project preparation stage is a natural or legal person to whom the commissioning party or project supervisor has entrusted the performing of the duties referred to in Paragraph 16 of these Regulations during the project preparation stage (hereinafter – project preparation co-ordinator).

10. The co-ordinator for work safety and health protection at the project execution stage is a natural or legal person to whom the commissioning party or project supervisor has entrusted the performing of the duties referred to in Paragraph 17 of these Regulations during the project execution stage (hereinafter – project execution co-ordinator).

11. The appointment of a project preparation co-ordinator and a project execution co-ordinator shall not relieve the commissioning party and the project supervisor of their liability regarding compliance with the requirements of work safety and health protection.

12. The commissioning party or the project supervisor shall ensure that prior to the commencement of construction work, a work safety and health protection plan is formulated in accordance with Sub-paragraph 16.2 of these Regulations.

13. It is the duty of the commissioning party or the project supervisor, before the construction work starts, to forward a prior notice to the State Labour Inspection regarding the carrying out of construction work (annex) in any of the following cases:

13.1. if the scheduled duration of the construction work exceeds 30 working days and more than 20 workers are employed simultaneously in the construction work; or

13.2. if the planned volume of construction work exceeds 500 person-days (working days worked by a worker).

14. The commissioning party or the project supervisor shall display the information included in the prior notice regarding the commencement of construction work in a visible place on the construction site and, where necessary, shall periodically update it.

III.	Project Preparation and Execution Stages; Duties of Co-ordinators

15. Where the project supervisor or the commissioning party is performing the duties of a project supervisor, he or she shall comply with the requirements for work safety and health protection set out in the Law On Protection at Work during the various stages of preparation and execution of  the project, in particular:

15.1. when architectural, technical and organisational aspects are being decided in planning the work or stages of work which are to take place simultaneously or in succession; and

15.2. when the volume of construction work to be carried out simultaneously and the time required for completing the stages is being calculated, taking into account the work safety and health protection plan and all the documents formulated or corrected in accordance with Sub-paragraphs 16.2, 16.3 and 17.3 of these Regulations.

16. The project preparation co-ordinator has the following duties:

16.1. to co-ordinate the fulfilling of the requirements of Paragraph 15 of these Regulations with the project supervisor, contractors and self-employed persons;

16.2. to formulate a work safety and health protection plan in accordance with Chapter IV of these Regulations, including in it also specific measures related to the construction work referred to in Paragraph 3 of these Regulations; and

16.3. to prepare records appropriate to the nature of the project, which contain information concerning the requirements of work safety and health protection that are to be taken into account in carrying out the work.

17. The project execution co-ordinator has the following duties:

17.1. to co-ordinate the implementation of the general principles of work safety and health protection when decisions are being made regarding technical and/or organisational measures, in the planning of the carrying out of various construction works simultaneously or in succession and in estimating the time  required for their execution;

17.2. to co-ordinate and supervise the execution of the work safety and health protection plan and the plan for carrying out the construction work in order to ensure that contractors and self-employed persons comply with the requirements of Paragraph 27 of these Regulations and the execution of the work safety and health protection plan;

17.3. to make necessary adjustments to the work safety and health protection plan and to the records referred to in Sub-paragraph 16.3 of these Regulations, taking into account the progress of the work and changes in the construction site, as well as in the construction project (if such have been made);

17.4. to organise the co-operation of contractors (also, of those contractors, who work on the same construction site in succession), co-ordinate their activities in order to protect workers and prevent accidents at work and occupational diseases, ensure mutual exchange of information in accordance with the requirements of the Law On Protection at Work and, if necessary, involve self-employed persons;

17.5. to co-ordinate work safety and health protection measures of contractors and monitor their appropriate implementation; and

17.6. to carry out necessary measures to ensure that unauthorised persons are not allowed to be on the construction site.

IV.	Work Safety and Health Protection Plan

18.A safety and health plan (hereinafter – the plan) shall be formulated in order to provide all persons participating in the construction with necessary information regarding work safety and health protection.

19. The plan shall include specific information related only to the particular project and it is necessary for organising work safety and health protection.

20. The plan shall be formulated before the commencement of construction work. If the project circumstances change, the plan may be reviewed. The project execution co-ordinators shall have the opportunity to become acquainted with the plan before the selection of a relevant contractor.

21. The plan shall have two phases:

21.1. the first phase  – during the project preparation stage, which is connected with the project development until the selection of a contractor, the plan may be incorporated into the organisational portion of the work on the construction project; and

21.2. the second phase  – during the project execution stage, which is connected with the performance of construction work, the plan may be incorporated in the projected schedule for carrying out the construction work.

22. If necessary information is included in other construction documents or the construction design drawings, the plan may contain only references to the location of such information.

23. The following information shall be included in the project preparation stage of the plan:

23.1. use of parcels of land adjacent to the area intended for the construction site:

23.1.1. buildings and facilities, which may restrict the performance of construction, work with respect to work safety and health protection;

23.1.2. surface and underground engineering communications;

23.1.3. traffic routes and restrictions related to them (for example, access to fire-fighting devices, time limitations for the supply of materials and parking of vehicles);

23.1.4. demolition or reconstruction of structures connected with materials harmful to health, unsteady or unstable structures that require special work safety and health protection measures;

23.1.5. the nature of the earth, indicating its composition, state of pollution or instability, and old pits or other underground obstructions; and

23.1.6. existence of technical records for structures to be demolished (may be information submitted by the commissioning party);

23.2. the risk factors contained in the construction plan, which cannot be avoided, as well as work safety and health protection measures that must be observed in regard to such risk factors;

23.3. detailed recommendations for particular work safety and health protection measures, regarding which the contractor may request explanations in resolving further matters;

23.4. potential risk factors, which, in accordance with the construction plan, may arise during the construction work process due to materials harmful to health, and necessary information regarding the requirements of laws and other regulatory enactments;

23.5. information related to the area of the intended construction site:

23.5.1. how the construction site can be accessed; emergency exits;

23.5.2. location of temporary structures and individual workplaces;

23.5.3. loading and unloading areas for building materials and warehouses;

23.5.4. transport routes and pedestrian walkways; and

23.5.5. driveways and thoroughfares for fire-engines;

23.6. co-ordination of the work safety and health protection measures to be taken on the construction site and the exchange of relevant information between the commissioning party, project supervisor, contractors and self-employed persons; and

23.7. specific measures in relation to the construction work referred to in Paragraph 3 of these Regulations.

24. The following shall be included in the construction execution stage of the plan:

24.1. general procedures for organising work safety and health protection on the construction site;

24.2. changes, which during the execution of the construction work take place in:

24.2.1. work plans and schedules;

24.2.2. nomenclature and supply schedules of construction products, building materials, technological systems and equipment; and

24.2.3. work schedules for workforce movement and for the main building machinery.

25. For building projects which are socially significant or which are technically complex, the plan may be uncompleted, but in such cases, in the construction execution stage the plan shall include:

25.1. general management procedures with regard to the organisation of work safety and health protection;

25.2. general safety requirements on the construction site;

25.3. explanations regarding accessibility of information during the project preparation stage and explanations regarding how it will be managed and controlled; and

25.4. procedures in accordance with which changes in the project will be included and incorporated in the plan, in the carrying out of various stages of the construction work for the project, or in other technological documents regarding execution of the construction work.

V.	Obligations of Contractors

26. In order to ensure that the safety and health of workers on the construction site is protected, contractors shall:

26.1. in conformity with the nature of the construction site and the construction work, working conditions and risk factors, take measures in order that the workplaces comply with the requirements referred to in Chapters VI, VII and VIII of these Regulations; and

26.2. comply with the instructions of the project preparation co-ordinator and the project execution co-ordinator.

27. In carrying out construction work, the contractor shall ensure that the requirements of the Law On Protection at Work are fulfilled, in particular as regards:

27.1. demarcating the construction site and maintaining it in a clean state and in good order;

27.2. choosing the location of workplaces, taking into account the necessity for free access to these workplaces, as well as determining  relocation and movement routes and areas for the location of equipment;

27.3. conditions under which various materials are handled;

27.4. technical servicing and inspection of machines, equipment and installations when commencing operations, as well as regular inspections during operation in order to eliminate defects which might create hazards to the safety and health of workers;

27.5. installation of storage areas and labelling of various materials (in particular as regards dangerous materials or substances);

27.6. collection and removal of used dangerous materials and substances;

27.7. storage, collection, movement and disposal of waste and construction debris;

27.8. changes in the time intended for the execution of the various types of work and work stages, based on work progress at the construction site;

27.9. co-operation with self-employed persons; and

27.10. co-operation and co-ordination of work with other industrial facilities on the construction site or in its proximity.

28. The contractor shall inform workers or their trusted representatives regarding all the measures to be taken on the construction site with respect to their work safety and health protection in accordance with the requirements of the Law On Protection at Work. The information must be comprehensible to the workers.

29. The contractor shall ensure consultations with workers or their trusted representatives and their participation in all matters related to work safety and health protection in accordance with the Law On Protection at Work.

VI.	Minimum Requirements regarding Equipping of

Workplaces at Construction Sites

30. Stability and solidity requirements:

30.1. materials, equipment and any component which may, when moving in any way, affect the safety and health of workers must be stable and safe; and

30.2. access to any surface involving insufficiently resistant materials is not permitted without appropriate equipment or ancillary means to enable the work to be carried out safely.

31. Energy distribution installations (systems, equipment and networks) shall be designed and installed so:

31.1. as not to create the risk of a fire or an explosion;

31.2. that people may be adequately protected from electrocution caused by direct or indirect contact with energy distribution installations; and

31.3. that in designing, installing and choosing equipment and protective devices, the type and power of the energy distributed, external conditions and the competence of such persons as are able to access parts of the energy distribution installations, are taken into account.

32. Emergency routes, passageways and exits shall be designed and installed in compliance with the following requirements:

32.1. emergency routes, passageways and exits shall be clear and ensure the most rapid passage to a safe area;

32.2. emergency routes, passageways and exits shall ensure rapid and, as far as possible, safe evacuation of workers from all workplaces in the event of danger;

32.3. the number, location and dimensions of emergency routes, passageways and exits shall be determined on the basis of the use of the construction site, buildings, structures and premises (also, temporary construction structures, including sheds), the equipment thereon or therein and its dimensions, and on the maximum potential number of persons present;

32.4. emergency routes, passageways and exits shall be indicated by signs conforming with the regulatory enactments regarding requirements for the use of safety signs at workplaces; and

32.5. emergency lighting of adequate intensity shall be provided for emergency routes, passageways and exits requiring illumination in case the work lighting fails.

33. Requirements regarding fire alarm and automatic fire-fighting systems and fire-fighting devices:

33.1. depending on the location of the construction site, the specific nature of the buildings and structures, the dimensions and use of the premises, the equipment thereon or therein, the physical and chemical properties of the materials used, and the maximum potential number of persons present, workplaces shall be provided with an adequate number of appropriate fire-fighting devices and, where necessary, fire detectors and alarm systems;

33.2. fire-fighting devices, fire detectors and alarm systems shall be regularly inspected and maintained;

33.3. appropriate inspections and worker drills shall be conducted at regular intervals; and

33.4. fire-fighting devices shall be conveniently accessible and simple to use and their location shall be indicated by signs located in compliance with regulatory enactments regarding the use of safety signs at workplaces, fire safety, and others.

34. Requirements for ventilation:

34.1. a sufficient supply of fresh air shall be ensured in workplaces, taking into account the nature of the work and the physical demands placed on workers;

34.2. ventilation systems shall be maintained in good order, regularly cleaned and the efficiency of their operation inspected;

34.3. the operation of mechanical ventilation systems and air-conditioning equipment may not cause draughts as may harm the health of workers;

34.4. if it is required for the protection of the health of the workers, a ventilation control system shall be installed to detect malfunctions in the operation of the ventilation system; and

34.5. waste created during the work process which, by polluting the air, may create an immediate hazard to the health of workers, shall be removed without delay.

35. Protection against noise and harmful external influences:

35.1. workers shall not be exposed to harmful levels of noise, vibrations or harmful external influences (for example, gases, vapours, and dust);

35.2. if workers have to enter an area where the atmosphere might contain toxic or harmful substances or have an insufficient oxygen level or there are highly inflammable substances, appropriate measures shall be taken in order to monitor this area and prevent any hazards; and

35.3. workers may not be exposed to a high-risk confined atmosphere working environment.

36. The temperature at a workplace shall be suitable for the working methods and the physical demands placed on the workers.

37. Requirements regarding natural and artificial lighting of workplaces, premises and traffic passageways shall be as follows:

37.1. workplaces, premises and traffic passageways shall be provided with sufficient natural light and be equipped with artificial lighting in such a way that the total lighting intensity is adequate for the safety and health of workers;

37.2. where necessary, portable light sources that are impact resistant shall be used;

37.3. the colour of artificial light used must not alter or affect the visibility of signals or signposts;

37.4. Lighting installations in premises, workplaces and traffic passageways shall be so placed that workers are not exposed to the risk of an accident; and

37.5. emergency lighting of adequate intensity shall be installed in artificially lit workplaces, premises and traffic passageways in which workers are especially exposed to risks if the artificial lighting fails.

38. Doors and gates shall be designed and installed in compliance with the following requirements:

38.1. sliding doors must be fitted with a safety device to prevent them from being derailed and falling over;

38.2. doors and gates opening upward must be fitted with a mechanism to prevent them from falling downward;

38.3. doors and gates that open to emergency routes, passageways and exits shall be indicated by signs conforming with regulatory enactments regarding requirements for the use of safety signs at workplaces;

38.4. if pedestrian traffic can not be ensured through the gates intended for motor vehicles, in the proximity of these gates doors for pedestrians shall be installed, and shall be clearly marked and kept unobstructed; and

38.5. the operation of mechanical doors and gates must not pose any risk to workers, and the doors and gates must be fitted with easily identifiable and accessible emergency shut-off devices, which allow them to be opened manually, if due to failure of the power supply the doors and gates do not open automatically.

39. Traffic routes and danger areas within these shall be designed and installed in compliance with the following requirements:

39.1. it shall be ensured that there is easy, safe and suitable access to traffic routes, and to stairs and fixed ladders, and loading and unloading platforms and ramps, without workers working in proximity to these traffic routes being endangered;

39.2. the number and dimensions of pedestrian walkways and traffic routes shall correspond to the number of users and the activities to be carried out;

39.3. if traffic routes are utilised by both motor vehicles and pedestrians, sufficient safety clearance must be installed or adequate protective devices must be provided for each user of the construction site;

39.4. traffic routes shall be clearly identified, marked, and regularly monitored and maintained;

39.5. traffic routes shall be a sufficient distance away from doors, gates, thoroughfares, pedestrian walkways, corridors and staircase landings;

39.6. traffic routes and doors opening on to them, shall be free from obstruction so that they can be used at any time without delay;

39.7. if there are limited-access danger areas in the construction site, these shall be demarcated or equipped with devices preventing unauthorised persons from entering such areas;

39.8. workers employed in danger areas shall be protected with protective equipment; and

39.9. danger areas shall be clearly indicated and marked in conformity with regulatory enactments regarding the use of safety signs at workplaces.

40. Loading ramps and platforms shall be designed and installed in compliance with the following requirements:

40.1. they must comply with the dimensions and loading weight of the freight to be transported;

40.2. they must have at least one exit point; and

40.3. loading ramps shall be equipped so as to prevent workers from falling from them.

41. Workplaces shall be spacious enough so that after the required equipment and installations are put in place, the workers can move freely while performing their work.

42. First aid:

42.1. the contractor shall ensure the provision of first aid and that personnel trained in providing first aid can be called upon at any time;

42.2. the contractor shall ensure that workers who have suffered injury from an accident or have suddenly become ill can be taken to a place where they can be provided with emergency medical care;

42.3. depending on the type of activity, extent of work and risk of the working environment, one or more first aid rooms must be provided that are furnished with first aid equipment and facilities, and for which easy access to stretchers can be ensured;

42.4. first aid rooms shall be clearly marked in conformity with regulatory enactments regarding the use of safety signs at workplaces;

42.5. first aid kits shall be available at all places where, due to working conditions, they are necessary, and the location thereof shall be clearly marked; and

42.6. the addresses and telephone numbers of the local emergency medical care and rescue services shall be indicated in a clearly visible place.

43. Changing rooms and lockers shall be designed and installed in compliance with the following requirements:

43.1. changing rooms shall be installed if it is intended that special work clothes are to be worn during the work process and where, for reasons of health or propriety, workers cannot be expected to change in another place;

43.2. changing rooms shall be easily accessible, sufficiently spacious, and fitted with lockers, seats and facilities that enable workers, where necessary, to dry their work clothes and their own clothing and personal effects;

43.3. separate changing rooms shall be installed for men and women or, if this is not feasible, separate use of changing rooms shall be ensured for men and women;

43.4. if working conditions are related to activation of harmful factors (for example, dangerous substances, humidity, dirt), separate lockers shall be installed for the keeping of the workers own clothing and their work clothes and

43.5. if installation of changing rooms is not necessary, each worker shall be provided with a place to lock away and store his or her own clothes and personal effects.

44. Showers, washbasins and lavatories shall be designed and installed in compliance with the following requirements:

44.1. if it is necessary due to the nature of the work or for the health protection of the workers, separate shower rooms shall be installed for men and women or, if this is not feasible, separate use of shower rooms for men and women shall be ensured at workplaces;

44.2. shower rooms shall be sufficiently spacious so that workers can wash without hindering each other and in compliance with hygiene requirements;

44.3. showers shall be equipped with a supply of hot and cold water;

44.4. where, due to the working conditions, shower rooms are not necessary, a sufficient number of suitable washbasins with running water (including hot water, where necessary) shall be installed in the proximity of the workrooms and the changing rooms;

44.5. separate wash basins shall be installed for men and women, or, if this is not feasible, separate use of washbasins for men and women shall be ensured;

44.6. where installation of shower rooms or washbasins is separate from changing rooms, easy access from one room to the other shall be ensured;

44.7. taking into account the number of workers, an adequate number of lavatories and washbasins with personal hygiene products (toilet paper, soap, towels or other devices for hand drying) shall be installed;

44.8. lavatories and washbasins shall be installed close to the workrooms and rest rooms, changing rooms and shower rooms; and

44.9. separate lavatories shall be installed for men and women or, if this is not feasible, separate use of lavatories for men and women shall be ensured.

45. The following requirements shall be complied with in designing and installing rest rooms or living quarters:

45.1. if safety and health requirements so prescribe, depending on the specifics of the work to be done and the number of persons employed on the construction site, and if the construction site is located far from populated places or from the place of residence of workers, the workers shall be provided comfortable, accessible rest rooms and/or living quarters;

45.2. rooms shall be sufficiently spacious and equipped with an adequate number of tables and seats with backrests;

45.3. if providing of appropriate rest rooms is not feasible, other rooms in which workers can stay during work breaks shall be ensured for workers if such is necessary for the safety or health of the workers; and

45.4. if the living quarters are utilised permanently, rest rooms and an adequate number of lavatories, washbasins and shower rooms shall be installed. The living quarters shall be equipped with beds, closets, tables and seats with back rests in conformity with the number of workers, and where necessary, living quarters shall be allocated separately for women and men.

46. If pregnant women and nursing mothers are employed, they shall be ensured the opportunity to lie down, in order to rest, in suitable conditions.

47. Where necessary, workplaces shall be so equipped as to enable the employing of disabled persons (suitable doors, pedestrian walkways, staircases, showers, washbasins and lavatories shall be installed).

48. Additional requirements are as follows:

48.1. clearly visible and identifiable signs regarding the performance of construction work shall be placed in the vicinity of the construction site and on its border or its fencing;

48.2. at the construction site workers shall be provided with drinking water and, if feasible, with some non-alcoholic beverage both in workrooms and in proximity to workplaces; and

48.3. workers shall be provided with facilities enabling then to eat and, if necessary, prepare meals in suitable conditions.

VII.	Additional Minimum Requirements for the Organising of Indoor Workplaces

49. The structure and stability of premises shall correspond to the nature of their use.

50. Requirements for emergency exit doors shall be as follows:

50.1. emergency exit doors shall open outwards;

50.2. emergency exit doors must not be locked or fastened, in order that any person who needs to use them in an emergency situation can open them easily and without delay; and

50.3. sliding or revolving doors shall not be utilised as emergency exits.

51. The temperature in rest rooms, rooms for personnel on guard duty, sanitary facilities, canteens and first aid rooms shall be appropriate to the nature of use of the relevant premises.

52. Windows, skylights and glass partitions shall be fitted in such a way as to prevent excessive influx of sunlight, taking into account the nature of the work and the use of the premises.

53. Floors, walls and ceilings of rooms shall be designed and installed in compliance  with the following requirements:

53.1. the floors of rooms must have no dangerous projections, holes or slopes and shall be stable and not slippery;

53.2. floors, walls and ceilings in rooms shall be such that they can be conveniently cleaned or maintained in compliance with hygiene requirements; and

53.3. translucent or transparent walls, in particular all-glass partitions in work premises and in proximity to traffic routes, shall be clearly indicated and made of safety materials or be demarcated in such a way as to prevent workers from pushing against walls, or being injured if the wall shatters.

54. Windows, skylights and ventilation hatches shall be designed and installed in compliance with the following requirements:

54.1. so that workers can safely open, close, regulate or secure windows, skylights and ventilation hatches;

54.2. open windows, skylights or ventilation hatches must not endanger workers; and

54.3. windows and skylights shall be fitted with appropriate devices allowing them to be cleaned without exposing the workers carrying out this work or workers located in proximity to the windows, to risk.

55. Doors and gates shall be designed and installed in compliance with the following requirements:

55.1. the placement, number and dimensions of doors and gates, as well as the materials used in their manufacture shall be determined by the nature and use of the premises;

55.2. revolving doors and gates, shall be translucent or have transparent elements;

55.3. transparent doors shall be labelled at such height where the labelling is clearly visible; and

55.4. transparent or translucent parts of doors and gates, which are manufactured from non-durable material, therefore putting workers at risk of being injured, shall be protected against breakage.

56. Traffic routes shall be clearly marked, taking into account the use of premises and the equipment they contain.

57. Escalators and travelators shall operate safely and they shall be fitted with safety devices as well as with clearly visible and accessible emergency stop devices.

58. Workrooms shall be sufficiently spacious to allow workers to perform their work without endangering their safety and health.

VIII.	Additional Minimum Requirements for the Equipping of Outdoor Workplaces

59. Mobile or stationary workplaces located at a height or at depth must be stable and solid, taking into account:

59.1. the number of workers in them;

59.2. the maximum loads they may have to bear and the load distribution; and

59.3. the influence of external environmental factors they may be subjected to.

60. If structures, support devices and other elements of the workplaces referred to in Section 59 are not stable, their stability shall be safely fixed by appropriate means in order to avoid any untimely or sudden movement of the whole or parts of the workplaces.

61. Stability and solidity of workplaces shall be regularly inspected, especially after any change in the height or depth of the workplace.

62. Energy distribution installations on the construction site shall be regularly inspected and maintained in good order, especially those subject to influences of external environmental factors.

63. Prior to establishing a construction site, on-site energy distribution installations shall be identified, inspected and clearly marked.

64. Above ground electric power lines, as far as possible, shall be relocated outside the construction site or disconnected from the power supply, or if this is not feasible, protective enclosures with signs shall be installed, thus demarcating them from means of transport  and other mechanisms.

65. If means of transport have to pass beneath electric power lines on a construction site, relevant warning signs and suspended shields shall be provided.

66. Workers shall be protected from adverse atmospheric influences.

67. Protection against falling objects:

67.1. wherever technically feasible, workers shall be protected against falling objects by containment methods;

67.2. materials and equipment shall be so arranged or placed in such a way as to prevent their possible collapsing or overturning; and

67.3. where necessary, covered passageways shall be installed on the construction site, or alternatively access to danger areas shall be prevented.

- 68. Protection against falls from a height:

68.1. falls from a height shall be prevented by suitable equipment, the utilisation of durable movable platforms with sufficiently high protective enclosures at sides and ends, which consist at least of a main handrail and an intermediate handrail, or, also, by utilisation of other equivalent equipment;

68.2. work at a height may be carried out only with suitable equipment or by utilising means of collective protection (for example, scaffolding, platforms, safety nets); and

68.3. if the use of appropriate equipment is not feasible due to the nature of the work, workers shall be ensured with safe access to the workplace and supplied with safety harnesses, belts or other anchoring safety devices.

- 69. Requirements for scaffolding, ladders, gangways and work platforms shall be as follows:

69.1. scaffolding shall be designed, erected  and maintained in good order, in order to prevent possible collapse or accidental movement;

69.2. work platform, gangway and scaffolding ladders shall be erected, their dimensions shall be selected, and they shall be secured and used in such a way as to prevent people from falling and to protect them from falling objects;

69.3. a responsible person shall inspect scaffolding:

69.3.1. prior to putting the scaffolding into operation;

69.3.2. during operation, at specified intervals;

69.3.3. after relocation or modification, after a thunderstorm or earthquake, or any other circumstances which could affect the durability or stability of scaffolding, and if they have not been used for a longer period of time;

69.4. ladders shall be sufficiently durable and maintained in working order; and

69.5. ladders shall be utilised in suitable places and solely for the work for which they were intended.

70. Requirements for lifting equipment shall be as follows:

70.1. all lifting equipment and the accessory equipment, component parts, anchorages and supports thereof must be:

70.1.1. properly designed and correctly erected, sufficiently durable and appropriate for the use to which they are put;

70.1.2. maintained in good working order;

70.1.3. subject to regular tests and inspections in accordance with the requirements of regulatory enactments;

70.2. lifting equipment and the accessory equipment thereof may be operated only by qualified workers who have received training in the use of such equipment;

70.3. maximum load values shall be clearly indicated on lifting equipment and the accessory equipment thereof; and

70.4. lifting equipment and the accessory equipment thereof may only be used for the work for which they were intended.

- 71. Requirements for excavation works and transport of earth:

71.1. means of transport for materials transportation and machinery intended for earthworks shall be:

71.1.1. designed in conformity with the requirements of work safety; constructed and equipped taking into account, as far as possible, ergonomic requirements;

71.1.2. maintained in good working order;

71.1.3. used solely for the work for which they were intended;

71.2. drivers of means of transport and machinery operators shall be specially trained;

71.3. relevant security measures shall be taken in order to prevent means of transport and machinery from falling into excavations, trenches or into water; and

71.4. where necessary, means of transport and machinery shall be fitted with special structures to protect the service personnel from being crushed in case they overturn, as well as from falling objects.

- 72. Requirements for installations, equipment and tools shall be as follows:

72.1. installations, equipment and tools, including hand tools (whether power-driven or not) shall be:

- 72.1.1. designed and manufactured taking into account, as far as possible,  ergonomic requirements;

72.1.2. maintained in good working order;

- 72.1.3. used solely for the work for which they were intended;

72.2. workers who use installations, equipment and tools, including hand tools (whether power-driven or not) shall be specially trained in their use; and

72.3. installations and equipment, which are operated in conditions of increased pressure, shall be subject to regular inspections and tests in accordance with the requirements of regulatory enactments.

73. Requirements for earthworks and underground works, as well as for work in wells and tunnels shall be as follows:

73.1. appropriate protective measures shall be taken when performing work in wells or tunnels, or underground:

73.1.1.suitable supports shall be used, or sides, walls or slopes shall be reinforced;

73.1.2. risks relating to persons, materials or objects falling, and to flooding shall be prevented;

73.1.3. sufficient ventilation at all workplaces shall be ensured in order that the air in the working environment is not dangerous or harmful to the health of workers; and

73.1.4. it shall be ensured that workers can reach safety in the event of fire or an influx of water, as well as in other dangerous situations;

73.2. before excavation work starts, measures shall be carried out to identify and reduce to a minimum any risks associated with underground cables and other distribution systems;

73.3. safe routes leading into excavation areas and leading out from such shall be installed; and

73.4.excavated earth, building materials and means of transport shall be kept at a safe distance from the excavation and, if necessary, shall be separated by appropriate protective enclosures.

- 74. Where the dismantling (demolition) of a building or construction could be dangerous:

74.1. appropriate safety measures shall be taken and relevant working methods be used; and

74.2. the work shall be planned and undertaken only under the supervision of a responsible person.

- 75. Requirements for work involving metal or concrete frameworks, shutterings and heavy prefabricated building elements shall be as follows:

75.1. metal or concrete frameworks and their components, shutterings, prefabricated building elements and temporary supports and buttresses shall be erected and dismantled only under the supervision of a competent person;

75.2. appropriate safety measures shall be taken to protect workers against risks arising from the instability or inadequate strength of a structure; and

75.3. shutterings, temporary supports and support structures shall be manufactured, erected, and maintained so that they will safely withstand any loads and deformations as it is foreseen will be placed on them during the work process.

76. Requirements for the use of cofferdams and caissons shall be as follows:

76.1. all cofferdams and caissons must be:

76.1.1. well constructed, of appropriate, durable materials of suitable strength;

- 76.1.2. appropriately equipped so that workers can shelter in them in the event of an irruption of water or materials;

76.2. cofferdams and caissons shall be constructed, installed, reconstructed and dismantled only under the supervision of a competent person; and

76.3. a responsible person shall periodically inspect all cofferdams and caissons.

- 77. Requirements for work on roofs shall be as follows:

77.1. if the height or the slope of a roof exceeds the norms set in regulatory enactments, collective protective measures shall be taken to prevent workers, or materials, work tools or other objects from falling; and

77.2. if workers work on (or near) a roof or any other surface made of fragile materials through which it is possible to fall, safety measures shall be taken to ensure prevention of falls or inadvertent stepping on fragile surfaces.

IX.	Obligations of Other Persons

- 78. Other persons (the self-employed, persons from supervisory and control services) have the following duties:

78.1. prior to the commencement of work or activities on the construction site, to inform the project execution co-ordinator or other responsible persons of the form of activity they are to carry out, and co-ordinate their activities in the sphere of work safety and health protection; and

78.2. to comply with regulatory enactments regarding the use of work equipment and personal protective equipment.

X.	Closing Provision

79. These Regulations shall come into force on 1 August 2001.

Prime Minister								A.Bērziņš

The Minister for Welfare							A. Požarnovs