Republic of Latvia

Cabinet

Regulation No 342

Adopted 25 June 2003

Regulations regarding the Latvian Construction Standard LBN 304-03 “Regulations for the Supervision of Construction Works by Author”

Issued pursuant to

Section 2, Paragraph four

of the Construction Law

1. These Regulations approve the attached Latvian construction standard LBN 304-03 “Regulations for the Supervision of Construction Works by Author”.

2. Cabinet Regulation No. 167 of 6 May 1997, Regulations for the Supervision of Construction Works by Author (Latvian construction standard LBN 304-97) (Latvijas Vēstnesis [the official Gazette of the Government of Latvia], 1997, No. 114) is repealed.

3. These Regulations shall come into force on 1 July 2003.

Prime Minister 									E. Repše

Acting for the Minister for Economics,

Minister for Transport 								R. Zīle

Approved by

Cabinet Regulation No. 342

25 June 2003

Latvian Construction Standard LBN 304-03 “Regulations for the Supervision of Construction Works by Author”

I. General Provisions

1. This construction standard shall prescribe the procedures for the supervision of construction works by the author (hereinafter – supervision by author).

2. The author of a construction plan shall perform supervision by author in the cases specified in Paragraph 161 of the 1 April 1997 Cabinet Regulation No. 112, General Construction Regulations (hereinafter – General Construction Regulations).

3. The purpose of supervision by author is to ensure the right of the author of a construction plan to carry out an authentic on-site implementation of the construction plan, preventing unauthorised deviations by construction participants from the accepted construction plan, as well as violations of binding regulatory enactments and standards during construction work.

4. Supervision by author shall not release a contracting authority and a contractor from liability for the quality of construction works, compliance with the construction plan and with the requirements of Latvian construction standards.

II. Contract of Supervision by Author

5. Supervision by author shall be performed during the entire period of construction work in accordance with a contract between a contracting authority and the author of a construction plan regarding supervision by author.

6. An author-supervisor, who may be the author of a construction plan or his or her authorised representative – an accordingly certified expert (group of experts), shall be indicated in a contract of supervision by author.

7. If the expenditures of supervision by author are not specified in a contract regarding construction planning or in the section of the calculation of the costs of a construction plan, the referred to expenditures and the payment procedures thereof shall be determined in a contract of supervision by author.

III. Supervision by Author Log Book

8. An author-supervisor shall inspect a construction object in accordance with a plan of supervision by author and enter the inspection results in a supervision-by-author logbook of a specific form (Annex).

9. A construction board, which issues a building permit, shall register a supervision-by-author logbook. The pages of a supervision-by-author logbook shall be numbered and self-copying. A supervision-by-author logbook shall be bound together and sealed with a stamp of the relevant construction board or local government.

10. A supervision-by-author logbook shall be stored with a construction work manager, but copies of completed pages – with an author-supervisor.

11. After the putting into service of a construction object, a supervision-by-author logbook and the documentation of the fulfilment of construction work shall be given to the contracting authority for storage.

IV. Duties of an Author-supervisor

12. An author-supervisor has the following duties:

12.1. to examine the compliance of the architectonic dimensions of a construction object with the architectural solutions of the construction plan during construction work;

12.2. to examine in a timely manner the compliance of structures, technological and other equipment, construction products and materials used in the structure of an object with the construction plan and prevent the incorporation of non-complying structures, technological and other equipment, construction products and materials into the structure, if they are not quality substitutes for those which are provided for in the construction plan;

12.3. to participate in the work of a commission when putting a construction object into service;

12.4. to examine whether a construction plan includes the appropriate documentation of the fulfilment of the construction plan and construction work;

12.5. to submit a motivated written request to suspend construction works to the contracting authority, the construction board, the State Construction Inspection or the State Labour Inspection, if unauthorised deviations from a construction plan have been determined, or if the requirements of Latvian construction standards or regulatory enactments regarding labour protection have not been observed;

12.6. to propose to the institution which issued a certificate of construction practice to a responsible construction work manager the cancellation thereof, if the instructions of the author-supervisor regarding the elimination of determined deviations and violations have not been carried out within the specified time limits; and

12.7. when performing supervision by author in State-protected cultural monuments and the protective zone thereof, to submit a report on the works performed to the State Inspection for Heritage Protection and the local government, if such is requested in the planning and architectural task.

Rights of an Author of a Construction Plan

13. An author of a construction plan has the following rights:

13.1. to request a contracting authority to suspend construction work, if a contractor does not observe the justified requirements of an author-supervisor;

13.2. to breach a contract of supervision by author, if a contracting authority does not observe the requirement of the author of a construction plan to suspend construction works, and propose to a construction board or a construction inspection the suspension of construction works, regarding which the author of the construction plan shall inform the contracting authority in writing; and

13.3. to perform supervision by author, exercising all the rights specified in this construction standard, even if, in accordance with the General Construction Regulations, supervision by author is not necessary and a contract of supervision by author has not been entered into. In the referred to case, the author of a construction plan shall cover the expenses of supervision by author.

VI. Liability for Supervision by Author

14. The author of a construction plan, or his or her authorised person with whom a contract of supervision by author has been entered into, shall be liable for authentic implementation of the relevant construction plan in accordance with the requirements specified in this construction standard and other regulatory enactments regulating construction.

15. The author of a construction plan shall not be liable for the quality of construction plan solutions, if the deviations from the construction plan entered in a supervision by author log book are not eliminated during the course of construction works.

16. An author-supervisor shall be liable for losses incurred by a contracting authority or a contractor, which have resulted from the inactivity or fault of the author-supervisor.

17. The State Construction Inspection shall settle disagreements between the author of a construction plan and other participants of a construction upon the request of such persons. The opinion of the State Construction Inspection may be appealed in court.

VII. Alterations to a Construction Plan

18. If co-ordination of a construction plan is not necessary, the author of a construction plan or an author-supervisor may carry out alterations to the construction plan during the course of construction work on the basis of a written agreement with the other participants of the construction in accordance with the requirements specified in Paragraph 107 of the General Construction Regulations.

19. If construction is intended to be performed using State or local government funds and the performer of the works is selected in accordance with competition procedures, the alterations made to a construction plan during construction work shall not alter the amount of the contract price by more than 10 % from the previously specified construction costs.

Acting for the Minister for Economics,

Minister for Transport 							R. Zīle

Annex

Latvian Construction Standard LBN 304-03

“Regulations for the Supervision of Construction Works by Author”

(approved with Cabinet Regulation No. 342 of 25 June 2003)

# Supervision by Author Log book No. ___

Registered at the Construction Board _____________

Date: ___ ___________ 20___

| 1. | Name of the construction object |  |
| --- | --- | --- |

| 2. | Address of the construction object |  |
| --- | --- | --- |

| 3. | Name of the structure |  |
| --- | --- | --- |

| 4. | Contracting authority |  |
| --- | --- | --- |
|  |  | (given name, surname, |
| personal identity number, address and telephone number of a natural person, or |  |  |
| name of a legal person, registration number, |  |  |
| legal address and telephone number) |  |  |

| 5. | Contractor |  |
| --- | --- | --- |
|  |  | (given name, surname, |
| identity number, address and telephone number of a natural person, or |  |  |
| name of a legal person, registration number, |  |  |
| legal address and telephone number) |  |  |

| 6. | Log book begun (date) |  |
| --- | --- | --- |

| 7. | Log book completed (date) |  |
| --- | --- | --- |

| 8. | Author of the construction plan |  |
| --- | --- | --- |
|  |  | (signature and full name) |

| 9. | Contracting authority |  |
| --- | --- | --- |
|  |  | (signature and full name) |

10. List of Author-supervisors

| No. | Author of the construction plan | Name, surname, position and telephone number of the author-supervisor | Section of the construction plan for which supervision by author is performed | Number and date of issue of the assignment document of the author-supervisor |
| --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 |
| 1. |  |  |  |  |
| 2. |  |  |  |  |

11. Entries of Author-supervisors

| No. | Date | Determined deviations from the construction plan and violations of construction standards | Instructions regarding the elimination of determined deviations and violations, and the time period for the execution thereof | Entry made by (surname of author-supervisor) | I have read the entry |  |  | Notation regarding the fulfilment of instructions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | manager of construction work (signature, full name and date) | contracting authority or construction supervisor (signature, full name and date) |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1. |  |  |  |  |  |  |  |
| 2. |  |  |  |  |  |  |  |

Acting for the Minister for Economics,

Minister for Transport 								R. Zīle