Text consolidated by Tulkošanas un terminoloģijas centrs (Translation and Terminology Centre) with amending regulations of:

7 June 2005 (No. 396).

If a whole or part of a paragraph has been amended, the date of the amending regulation appears in square brackets at the end of the paragraph. If a whole paragraph or sub-paragraph has been deleted, the date of the deletion appears in square brackets beside the deleted paragraph or sub-paragraph.

Republic of Latvia

Cabinet

Regulation No. 376

Adopted 23 August 2001

# Regulations regarding the Latvian Building
Code LBN 003-01 “Construction Climatology”

Issued pursuant to

Section 2, Paragraph four

of the Construction Law

1. These Regulations shall approve the Latvian Building Code LBN 003-01 “Construction Climatology”.

2. The revision of building design documentation in conformity with the requirements specified in the Latvian Building Code LBN 003-01 “Construction Climatology” shall not be mandatory for building designs approved until 31 August 2001 in accordance with the procedures prescribed by law and the technical solutions of which conform with the requirements of the regulatory enactments applied during the relevant period of time.

3. The Construction Norms and Regulation SNiP 2.01.01.-82 “Construction Climatology and Geophysics” of the former U.S.S.R. (approved by the State Committee for Construction Matters of the former U.S.S.R. by Decision No. 188 of 21 July 1982) shall not be applied in the territory of the Republic of Latvia starting from 1 September 2001.

4. These Regulations shall come into force on 1 September 2001.

Prime Minister 				A. Bērziņš

Minister for Environmental Protection

and Regional Development 			V. Makarovs

Approved by

Cabinet

Regulation No. 376

23 August 2001

Latvian Building Code LBN 003-01 “Construction Climatology”

1. This Building code contains the climatological indicators, applicable in construction.

2. The climatological indicators included in Annex 1 to the Building Code shall be applied in engineering research, construction design and in the performance of construction work, also in the repair, renovation and reconstruction of construction works.

3. The Building Code shall not be applied to buildings to which special procedures for the construction process have been determined by the Cabinet in accordance with Section 6 of the Construction Law that provides for the application of other norms of climatological indicators.

[7 June 2005]

4. The climatological indicators of any geographic point for the needs of construction in the territory of Latvia shall be determined according to the climatological indicators of the nearest geographical point included in the tables of Annex 1 to this Building Code.

5. The characteristics of wind for the geographical points of an open territory of the coastal areas of the Baltic Sea within the range of five kilometres shall be determined taking into consideration the climatological indicators of Liepāja referred to in Tables 13, 14 and 15 of Annex 1 to this Building Code.

5.1 If construction works and the structures thereof are designed in conformity with the requirements of the applicable standards:

5.1 1. the values of the maximum wind speed and the maximum wind gusts shall be determined in accordance with Table 15 of Annex 1 to this Building Code;

5.1 2. the fundamental basic speed value of the wind (with a probability once per 50 years) shall be determined in conformity with the geographical location in accordance with Figure 1.1 of Table 15 of Annex 1 to this Building Code;

5.1 3. the basic value of the wind pressure shall be determined in conformity with the geographical location in accordance with Figure 1.2 of Table 15 of Annex 1 to this Building Code;

5.1 4. the characteristic values of the snow load on the surface of the ground (with a probability of 0.02) shall be determined in accordance with Figure 2.1 of Table 16.1 of Annex 1 to this Building Code;

5.1 5. emergency snow load on the surface of the ground shall be determined in accordance with Figure 2.2 of Table 16.2 of Annex 1 to this Building Code;

5.1 6. the variation coefficient of the maximum snow load during the winter period shall be determined in accordance with Table 16.3 of Annex 1 to this Building Code; and

5.1 7. the characteristic values of the snow load on the surface of the ground (with a probability once per 150 years) shall be determined in accordance with Table 16.4 of Annex 1 to this Building Code.

[7 June 2005]

5.2 Upon the recommendation of the Ministry of Economics, the limited liability company Latvijas standarts shall publish in the newspaper Latvijas Vēstnesis [the Official Gazette of the Government of Latvia].the list of such standards that may be applied for the fulfilment of this Building Code.

[7 June 2005]

6. Explanations regarding the application of climatological indicators are provided in Annex 2 to this Building Code.

7. The Ministry of Economics in co-operation with technical standardisation committees shall organise the development of Latvian National Standards and the adaptation of standards of the international standardisation organisations necessary for the fulfilment of these Regulations, as well as shall issue recommendations for the fulfilment of this Building Code.

[7 June 2005]

8. The hydrometeorological information, which is not included in this Building Code, shall be prepared by the State Hydrometeorology Board in accordance with the technical task of the client, on the basis of a conforming contract.

Minister for Environmental Protection

and Regional Development V.Makarovs

[7 June 2005]

Annex 1

to Latvian Building Code LBN 003-01

Construction Climatology

(approved by Cabinet

Regulation No. 376

23 August 2001)

Table 1

Average Air Temperature (°C)

| No. | Place | Month |  |  |  |  |  |  |  |  |  |  |  | On average per year |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII |  |
| 1. | Ainaži | -5.2 | -5.0 | -1.6 | 3.9 | 10.2 | 14.5 | 16.5 | 15.9 | 11.8 | 7.3 | 2.2 | -2.2 | 5.7 |
| 2. | Alūksne | -7.6 | -6.8 | -2.5 | 4.0 | 11.0 | 14.8 | 16.1 | 15.0 | 10.2 | 5.2 | -0.4 | -4.9 | 4.5 |
| 3. | Daugavpils | -6.7 | -5.9 | -1.8 | 5.2 | 12.1 | 15.7 | 16.9 | 15.9 | 11.2 | 6.1 | 1.0 | -3.8 | 5.5 |
| 4. | Dobele | -5.0 | -4.7 | -1.0 | 4.9 | 11.4 | 15.2 | 16.5 | 15.9 | 11.5 | 6.8 | 1.7 | -2.6 | 5.9 |
| 5. | Liepāja | -3.0 | -3.0 | -0.2 | 4.6 | 10.3 | 14.3 | 16.4 | 16.4 | 12.9 | 8.5 | 3.7 | -0.3 | 6.7 |
| 6. | Mērsrags | -3.8 | -4.0 | -1.0 | 3.9 | 9.9 | 14.7 | 16.3 | 15.9 | 11.9 | 7.4 | 2.5 | -1.4 | 6.0 |
| 7. | Priekuļi | -6.2 | -5.6 | -1.6 | 4.6 | 11.3 | 15.0 | 16.3 | 15.4 | 10.9 | 6.1 | 0.7 | -3.8 | 5.3 |
| 8. | Rīga | -4.7 | -4.3 | -0.6 | 5.1 | 11.4 | 15.4 | 16.9 | 16.2 | 11.9 | 7.2 | 2.1 | -2.3 | 6.2 |
| 9. | Stende | -4.5 | -4.4 | -1.1 | 4.4 | 10.7 | 14.5 | 15.9 | 15.3 | 11.2 | 6.8 | 1.8 | -2.3 | 5.7 |
| 10. | Zīlāni | -6.7 | -5.9 | -1.5 | 5.0 | 11.9 | 15.3 | 16.6 | 15.6 | 11.0 | 6.0 | 0.7 | -4.0 | 5.3 |

Table 2

Absolute Minimum of Air Temperature and the Probabilities thereof (°C)

| No. | Place | Month |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Annual minimum air temperature, the excess of which is possible once per |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | I | II | III |  | IV | V | VI | VII | VIII | IX | X | XI | XII |  | Within a year |  |  |
|  |  | 50 years | 10 years |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. | Ainaži | -35.1 | -35.3 | -34.4 |  | -18.8 | -7.5 | -1.3 | 2.7 | 0.2 | -4.8 | -13.8 | -18.7 | -37.1 |  | -37.1 | -36.3 | -33.0 |
| 2. | Alūksne | -37.4 | -36.8 | -28.8 |  | -15.9 | -5.8 | -0.3 | 2.9 | 0.7 | -5.1 | -10.6 | -20.6 | -36.8 |  | -37.4 | -37.4 | -32.7 |
| 3. | Daugavpils | -42.7 | -43.2 | -32.0 |  | -18.6 | -5.5 | -1.3 | 2.1 | -1.5 | -5.0 | -14.7 | -24.1 | -38.7 |  | -43.2 | -41.0 | -35.5 |
| 4. | Dobele | -34.1 | -35.9 | -25.7 |  | -13.2 | -3.7 | 0.3 | 4.7 | 1.9 | -3.7 | -9.2 | -21.7 | -31.9 |  | -35.9 | -35.9 | -32.5 |
| 5. | Liepāja | -32.9 | -31.6 | -23.8 |  | -10.1 | -4.3 | 0.5 | 4.8 | 4.6 | -1.7 | -7.3 | -17.5 | -25.8 |  | -32.9 | -31.5 | -26.1 |
| 6. | Mērsrags | -33.7 | -36.2 | -28.9 |  | -16.2 | -5.4 | -2.2 | 3.3 | 1.4 | -3.9 | -10.0 | -18.1 | -25.4 |  | -36.2 | -34.2 | -29.9 |
| 7. | Priekuļi | -36.5 | -38.0 | -26.4 |  | -12.6 | -5.4 | -1.4 | 3.5 | -0.1 | -4.5 | -12.9 | -20.6 | -39.0 |  | -39.0 | -38.2 | -31.8 |
| 8. | Rīga | -33.7 | -34.9 | -30.3 |  | -13.1 | -5.5 | -2.3 | 4.0 | 0.0 | -4.1 | -8.7 | -18.9 | -31.9 |  | -34.9 | -34.8 | -31.0 |
| 9. | Stende | -34.5 | -36.1 | -27.9 |  | -15.9 | -5.4 | -3.1 | 2.0 | -0.2 | -4.5 | -11.8 | -16.7 | -26.0 |  | -36.1 | -34.9 | -30.0 |
| 10. | Zīlāni | -38.5 | -36.7 | -32.3 |  | -14.4 | -6.0 | -0.4 | 2.7 | 0.1 | -6.4 | -10.9 | -22.0 | -35.2 |  | -38.2 | -38.0 | -33.4 |

Table 3

Absolute Maximum of Air Temperature and the Probabilities thereof (°C)

| No. | Place | Month |  |  |  |  |  |  |  |  |  |  |  |  | Annual maximum air temperature, the excess of which is possible once per |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII | Within a year |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 years | 10 years |
| 1. | Ainaži | 7.3 | 10.8 | 17.5 | 25.1 | 29.5 | 33.2 | 32.7 | 33.2 | 30.0 | 22.0 | 15.4 | 10.5 | 33.2 | 33.1 | 31.1 |
| 2. | Alūksne | 7.9 | 10.9 | 18.2 | 26.5 | 30.2 | 30.8 | 33.3 | 32.9 | 29.4 | 21.1 | 13.1 | 10.0 | 33.3 | 33.2 | 31.1 |
| 3. | Daugavpils | 7.6 | 13.1 | 18.4 | 27.6 | 31.8 | 32.8 | 35.1 | 36.4 | 31.3 | 24.0 | 16.3 | 10.4 | 36.4 | 36.0 | 33.0 |
| 4. | Dobele | 9.5 | 13.8 | 20.0 | 26.1 | 30.6 | 33.1 | 35.8 | 34.4 | 30.7 | 23.5 | 16.8 | 11.1 | 35.8 | 35.8 | 32.8 |
| 5. | Liepāja | 7.9 | 15.5 | 17.8 | 25.4 | 30.0 | 33.0 | 33.7 | 33.6 | 30.7 | 22.2 | 15.4 | 10.1 | 33.7 | 33.0 | 31.5 |
| 6. | Mērsrags | 9.4 | 13.6 | 20.8 | 26.8 | 31.6 | 34.6 | 34.4 | 33.8 | 29.1 | 24.9 | 15.5 | 13.9 | 34.6 | 34.7 | 32.8 |
| 7. | Priekuļi | 7.9 | 10.9 | 17.2 | 25.9 | 30.3 | 32.0 | 34.0 | 32.7 | 29.5 | 21.5 | 14.3 | 9.8 | 34.0 | 33.5 | 31.6 |
| 8. | Rīga | 9.4 | 13.5 | 20.5 | 26.5 | 30.4 | 32.5 | 33.6 | 32.5 | 29.4 | 23.4 | 17.2 | 11.5 | 33.6 | 33.3 | 32.2 |
| 9. | Stende | 8.5 | 12.0 | 18.3 | 25.8 | 29.0 | 32.6 | 33.6 | 34.3 | 29.7 | 23.5 | 16.1 | 10.5 | 34.3 | 34.0 | 31.9 |
| 10. | Zīlāni | 7.8 | 11.8 | 18.0 | 26.3 | 30.0 | 31.9 | 34.5 | 34.3 | 30.5 | 22.6 | 16.6 | 9.9 | 34.5 | 34.4 | 31.9 |

Table 4

Average Maximum Air Temperature (°C) of the Hottest Month and the Probabilities thereof

| No. | Place | Average maximum air temperature of the hottest month | Average maximum temperature of the hottest month, the excess of which is possible once per |  |
| --- | --- | --- | --- | --- |
|  |  |  | 50 years | 10 years |
| 1. | Ainaži | 21.3 | 24.4 | 23.2 |
| 2. | Alūksne | 21.9 | 24.8 | 23.7 |
| 3. | Daugavpils | 23.3 | 26.5 | 25.3 |
| 4. | Dobele | 23.0 | 26.3 | 25.0 |
| 5. | Liepāja | 20.9 | 24.3 | 23.0 |
| 6. | Mērsrags | 21.5 | 24.3 | 23.5 |
| 7. | Priekuļi | 22.3 | 25.3 | 24.2 |
| 8. | Rīga | 22.4 | 25.5 | 24.3 |
| 9. | Stende | 22.0 | 25.3 | 24.2 |
| 10. | Zīlāni | 22.9 | 26.2 | 25.0 |

Table 5

Average Minimum Air Temperature (°C) of the Coldest Month and the Probabilities thereof

| No. | Place | Average minimum air temperature of the coldest month | Average minimum temperature of the coldest month, the excess of which is possible once per |  |
| --- | --- | --- | --- | --- |
|  |  |  | 50 years | 10 years |
| 1. | Ainaži | -10.8 | -19.6 | -16.8 |
| 2. | Alūksne | -12.4 | -20.3 | -17.7 |
| 3. | Daugavpils | -12.3 | -21.8 | -18.5 |
| 4. | Dobele | -10.2 | -18.5 | -15.7 |
| 5. | Liepāja | -7.5 | -15.1 | -12.5 |
| 6. | Mērsrags | -8.8 | -18.0 | -14.5 |
| 7. | Priekuļi | -11.0 | -19.6 | -16.4 |
| 8. | Rīga | -9.7 | -18.5 | -15.3 |
| 9. | Stende | -9.7 | -17.8 | -14.6 |
| 10. | Zīlāni | -11.4 | -20.0 | -16.8 |

Table 6

Average Air Temperature (°C) of the Five Coldest Days and the Probabilities thereof

| No. | Place | Average air temperature of the five coldest days | Average air temperature of the five coldest days and the probability thereof |  |
| --- | --- | --- | --- | --- |
|  |  |  | 0.98 | 0.92 |
| 1. | Ainaži | -23.2 | -26.8 | -22.7 |
| 2. | Alūksne | -25.1 | -29.5 | -24.0 |
| 3. | Daugavpils | -24.0 | -28.4 | -24.1 |
| 4. | Dobele | -22.3 | -24.4 | -20.9 |
| 5. | Liepāja | -18.3 | -20.5 | -17.3 |
| 6. | Mērsrags | -19.6 | -22.0 | -19.3 |
| 7. | Priekuļi | -23.8 | -28.2 | -22.7 |
| 8. | Rīga | -20.7 | -24.6 | -20.7 |
| 9. | Stende | -19.9 | -22.5 | -19.5 |
| 10. | Zīlāni | -23.8 | -26.8 | -23.2 |

Table 7

Duration of the Heating Period and Average Air Temperature (°C)

| No. | Place | Daily average air temperature £ 8°C |  |
| --- | --- | --- | --- |
|  |  | duration of the period (days) | average temperature (°C) |
| 1. | Ainaži | 205 | -0.5 |
| 2. | Alūksne | 214 | -1.9 |
| 3. | Daugavpils | 205 | -1.3 |
| 4. | Dobele | 204 | -0.4 |
| 5. | Liepāja | 193 | 0.6 |
| 6. | Mērsrags | 211 | 0.4 |
| 7. | Priekuļi | 208 | -1.1 |
| 8. | Rīga | 203 | 0.0 |
| 9. | Stende | 209 | -0.2 |
| 10. | Zīlāni | 206 | -1.3 |

Table 8

Average Amplitude of Air Temperature (°C)

| No. | Place | Month |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII |
| 1. | Ainaži | 6.0 | 6.3 | 6.9 | 7.8 | 9.6 | 10.2 | 8.2 | 7.9 | 7.2 | 5.8 | 4.7 | 5.5 |
| 2. | Alūksne | 5.4 | 6.1 | 7.0 | 8.5 | 10.4 | 10.0 | 9.5 | 8.7 | 7.5 | 5.7 | 4.1 | 4.9 |
| 3. | Daugavpils | 6.2 | 6.9 | 8.1 | 9.7 | 11.8 | 11.3 | 10.7 | 10.6 | 8.9 | 7.0 | 4.5 | 5.4 |
| 4. | Dobele | 5.9 | 6.2 | 7.1 | 9.1 | 11.2 | 11.0 | 10.8 | 10.3 | 9.2 | 6.9 | 4.8 | 5.4 |
| 5. | Liepāja | 5.2 | 5.4 | 6.0 | 6.9 | 8.3 | 7.7 | 6.8 | 6.8 | 6.3 | 5.4 | 4.4 | 4.8 |
| 6. | Mērsrags | 5.3 | 6.0 | 5.5 | 7.9 | 9.5 | 9,8 | 9.2 | 8.1 | 8.1 | 6.6 | 3.9 | 4.9 |
| 7. | Priekuļi | 5.3 | 6.1 | 6.7 | 8.4 | 10.6 | 10.3 | 9.7 | 9.1 | 7.6 | 5.8 | 4.2 | 4.8 |
| 8. | Rīga | 5.3 | 5.8 | 7.0 | 8.8 | 12.2 | 9.9 | 9.2 | 8.9 | 8.0 | 6.4 | 4.4 | 4.8 |
| 9. | Stende | 5.6 | 6.0 | 6.9 | 8.7 | 10.9 | 11.0 | 10.0 | 9.7 | 8.3 | 6.4 | 4.6 | 4.9 |
| 10. | Zīlāni | 5.5 | 6.0 | 7.2 | 9.0 | 11.0 | 10.8 | 10.2 | 10.0 | 8.5 | 6.6 | 4.4 | 4.7 |

Table 9

Daily Average Partial Pressure of Water Vapour in the Air (hPa)

| No. | Place | Month |  |  |  |  |  |  |  |  |  |  |  | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII | per year |
| 1. | Ainaži | 4.0 | 3.9 | 4.4 | 6.2 | 9.2 | 12.8 | 15.0 | 14.8 | 11.6 | 8.8 | 6.4 | 4.8 | 8.5 |
| 2. | Alūksne | 3.5 | 3.5 | 4.2 | 5.9 | 8.8 | 11.8 | 13.7 | 13.4 | 10.5 | 7.9 | 5.7 | 4.2 | 7.8 |
| 3. | Daugavpils | 3.7 | 3.7 | 4.5 | 6.5 | 9.8 | 12.6 | 14.2 | 13.9 | 11.0 | 8.3 | 6.1 | 4.4 | 8.2 |
| 4. | Dobele | 4.2 | 4.1 | 4.7 | 6.6 | 9.5 | 12.6 | 14.5 | 14.1 | 11.3 | 8.8 | 6.5 | 4.8 | 8.5 |
| 5. | Liepāja | 4.6 | 4.5 | 5.2 | 6.7 | 9.4 | 12.6 | 14.9 | 14.9 | 12.0 | 9.5 | 7.0 | 5.5 | 8.9 |
| 6. | Mērsrags | 4.3 | 4.2 | 4.9 | 6.4 | 9.2 | 12.4 | 14.7 | 14.5 | 11.6 | 8.9 | 6.7 | 5.0 | 8.6 |
| 7. | Priekuļi | 3.7 | 3.7 | 4.4 | 6.1 | 9.0 | 12.0 | 14.0 | 13.8 | 10.9 | 8.3 | 6.0 | 4.4 | 8.0 |
| 8. | Rīga | 4.1 | 4.0 | 4.8 | 6.4 | 9.3 | 12.5 | 14.5 | 14.3 | 11.4 | 8.7 | 6.4 | 4.8 | 8.4 |
| 9. | Stende | 4.2 | 4.1 | 4.8 | 6.2 | 8.7 | 11.7 | 13.8 | 13.7 | 11.1 | 8.7 | 6.4 | 4.9 | 8.2 |
| 10. | Zīlāni | 3.7 | 3.7 | 4.6 | 6.5 | 9.8 | 12.7 | 14.4 | 14.1 | 11.2 | 8.4 | 6.1 | 4.5 | 8.3 |

Table 10

Daily Average Relative Air Humidity (%)

| No. | Place | Month |  |  |  |  |  |  |  |  |  |  |  | Average |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII |  | per year |  |
| 1. | Ainaži | 84 | 83 | 80 | 76 | 74 | 78 | 80 | 82 | 83 | 84 | 86 | 86 |  |  | 81 |
| 2. | Alūksne | 87 | 84 | 78 | 71 | 68 | 71 | 75 | 79 | 84 | 87 | 90 | 90 |  |  | 80 |
| 3. | Daugavpils | 85 | 83 | 79 | 73 | 70 | 72 | 75 | 78 | 83 | 85 | 88 | 88 |  |  | 80 |
| 4. | Dobele | 87 | 84 | 81 | 75 | 70 | 73 | 77 | 80 | 83 | 86 | 89 | 89 |  |  | 81 |
| 5. | Liepāja | 85 | 84 | 83 | 79 | 76 | 78 | 80 | 80 | 80 | 83 | 85 | 86 |  |  | 82 |
| 6. | Mērsrags | 85 | 84 | 82 | 79 | 75 | 75 | 79 | 81 | 83 | 84 | 86 | 86 |  |  | 82 |
| 7. | Priekuļi | 85 | 82 | 77 | 72 | 68 | 71 | 76 | 79 | 85 | 86 | 89 | 88 |  |  | 80 |
| 8. | Rīga | 85 | 82 | 79 | 73 | 69 | 72 | 76 | 78 | 81 | 83 | 86 | 86 |  |  | 79 |
| 9. | Stende | 87 | 85 | 81 | 74 | 69 | 72 | 77 | 79 | 83 | 86 | 89 | 87 |  |  | 81 |
| 10. | Zīlāni | 87 | 84 | 80 | 75 | 71 | 73 | 78 | 80 | 85 | 87 | 90 | 90 |  |  | 82 |

Table 11

Amplitude of Relative Air Humidity (%)

| No. | Place | Month |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII |
| 1. | Ainaži | 4 | 6 | 12 | 18 | 22 | 21 | 20 | 20 | 17 | 11 | 5 | 3 |
| 2. | Alūksne | 3 | 10 | 19 | 26 | 32 | 31 | 32 | 31 | 24 | 15 | 5 | 2 |
| 3. | Daugavpils | 6 | 11 | 19 | 30 | 37 | 36 | 35 | 36 | 26 | 18 | 8 | 4 |
| 4. | Liepāja | 3 | 6 | 12 | 18 | 21 | 20 | 19 | 19 | 16 | 12 | 5 | 3 |
| 5. | Mērsrags | 3 | 8 | 12 | 20 | 25 | 25 | 25 | 25 | 20 | 14 | 6 | 4 |
| 6. | Priekuļi | 4 | 9 | 16 | 24 | 31 | 31 | 31 | 31 | 23 | 14 | 6 | 4 |
| 7. | Rīga | 4 | 8 | 16 | 25 | 28 | 28 | 28 | 27 | 22 | 15 | 6 | 3 |
| 8. | Stende | 3 | 9 | 17 | 27 | 34 | 34 | 32 | 32 | 25 | 16 | 6 | 3 |
| 9. | Zīlāni | 5 | 10 | 18 | 27 | 34 | 34 | 34 | 34 | 26 | 18 | 8 | 3 |

Table 12

Monthly and Annual Sum of Precipitation (mm)

| No. | Place | Month |  |  |  |  |  |  |  |  |  |  |  | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII | per year |
| 1. | Ainaži | 37 | 25 | 30 | 35 | 43 | 54 | 65 | 76 | 80 | 70 | 71 | 55 | 641 |
| 2. | Alūksne | 40 | 32 | 36 | 40 | 53 | 74 | 84 | 85 | 70 | 62 | 61 | 54 | 691 |
| 3. | Daugavpils | 36 | 27 | 34 | 40 | 51 | 73 | 83 | 74 | 66 | 52 | 52 | 46 | 634 |
| 4. | Dobele | 31 | 22 | 28 | 39 | 43 | 51 | 79 | 76 | 59 | 52 | 55 | 39 | 574 |
| 5. | Liepāja | 46 | 31 | 36 | 35 | 40 | 46 | 74 | 80 | 78 | 74 | 83 | 67 | 690 |
| 6. | Mērsrags | 38 | 25 | 30 | 37 | 40 | 52 | 72 | 75 | 75 | 60 | 65 | 48 | 618 |
| 7. | Priekuļi | 31 | 22 | 31 | 40 | 51 | 66 | 90 | 92 | 81 | 64 | 61 | 46 | 674 |
| 8. | Rīga | 33 | 25 | 31 | 39 | 43 | 61 | 79 | 79 | 76 | 60 | 61 | 49 | 636 |
| 9. | Stende | 40 | 26 | 35 | 37 | 42 | 56 | 83 | 79 | 77 | 68 | 70 | 53 | 666 |
| 10. | Zīlāni | 39 | 30 | 38 | 42 | 52 | 65 | 75 | 79 | 69 | 58 | 59 | 55 | 662 |

Table 13

Recurrence of the Wind Directions and Wind Lull (%)

1. Ainaži

| No. | Month | Wind direction |  |  |  |  |  |  |  | Wind lull |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | N | NE | E | SE | S | SW | W | NW |  |
| 1. | I | 8 | 9 | 14 | 19 | 23 | 13 | 9 | 5 | 1 |
| 2. | II | 7 | 7 | 16 | 18 | 23 | 13 | 8 | 8 | 2 |
| 3. | III | 8 | 7 | 16 | 14 | 25 | 14 | 8 | 8 | 2 |
| 4. | IV | 10 | 9 | 14 | 11 | 18 | 14 | 12 | 12 | 2 |
| 5. | V | 11 | 12 | 15 | 9 | 13 | 13 | 13 | 14 | 2 |
| 6. | VI | 11 | 9 | 11 | 7 | 12 | 15 | 18 | 17 | 2 |
| 7. | VII | 10 | 6 | 11 | 9 | 13 | 18 | 18 | 15 | 2 |
| 8. | VIII | 8 | 8 | 11 | 11 | 15 | 18 | 17 | 12 | 3 |
| 9. | IX | 8 | 7 | 11 | 14 | 17 | 18 | 14 | 11 | 2 |
| 10. | X | 8 | 4 | 11 | 16 | 21 | 18 | 12 | 10 | 1 |
| 11. | XI | 8 | 6 | 12 | 19 | 23 | 14 | 10 | 8 | 1 |
| 12. | XII | 8 | 7 | 13 | 20 | 21 | 14 | 10 | 7 | 1 |
| 13. | Within a year | 9 | 7 | 13 | 14 | 19 | 15 | 12 | 11 | 2 |

2. Alūksne

| No. | Month | Wind direction |  |  |  |  |  |  |  | Wind lull |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | N | NE | E | SE | S | SW | W | NW |  |
| 1. | I | 5 | 5 | 12 | 13 | 21 | 14 | 19 | 11 | 6 |
| 2. | II | 4 | 6 | 13 | 15 | 20 | 11 | 20 | 11 | 7 |
| 3. | III | 3 | 7 | 13 | 14 | 20 | 13 | 20 | 10 | 9 |
| 4. | IV | 6 | 10 | 16 | 13 | 15 | 9 | 20 | 11 | 10 |
| 5. | V | 8 | 13 | 19 | 9 | 13 | 7 | 18 | 13 | 12 |
| 6. | VI | 9 | 9 | 14 | 9 | 11 | 9 | 25 | 14 | 11 |
| 7. | VII | 7 | 7 | 12 | 8 | 13 | 12 | 26 | 15 | 13 |
| 8. | VIII | 6 | 7 | 11 | 10 | 15 | 14 | 23 | 14 | 14 |
| 9. | IX | 5 | 4 | 10 | 10 | 19 | 15 | 27 | 10 | 11 |
| 10. | X | 4 | 3 | 8 | 11 | 20 | 15 | 27 | 12 | 7 |
| 11. | XI | 4 | 3 | 9 | 13 | 25 | 16 | 21 | 9 | 5 |
| 12. | XII | 4 | 5 | 9 | 11 | 24 | 15 | 21 | 11 | 7 |
| 13. | Within a year | 5 | 7 | 12 | 11 | 18 | 13 | 22 | 12 | 9 |

3. Daugavpils

| No. | Month | Wind direction |  |  |  |  |  |  |  | Wind lull |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | N | NE | E | SE | S | SW | W | NW |  |
| 1. | I | 6 | 7 | 10 | 14 | 20 | 19 | 18 | 6 | 13 |
| 2. | II | 6 | 8 | 12 | 18 | 17 | 14 | 18 | 7 | 15 |
| 3. | III | 5 | 8 | 11 | 16 | 20 | 15 | 18 | 7 | 17 |
| 4. | IV | 8 | 11 | 11 | 14 | 16 | 12 | 17 | 11 | 16 |
| 5. | V | 11 | 13 | 14 | 12 | 13 | 10 | 16 | 11 | 18 |
| 6. | VI | 12 | 11 | 10 | 7 | 12 | 13 | 22 | 13 | 20 |
| 7. | VII | 10 | 7 | 7 | 8 | 12 | 16 | 27 | 13 | 22 |
| 8. | VIII | 8 | 7 | 7 | 8 | 16 | 18 | 25 | 11 | 24 |
| 9. | IX | 6 | 6 | 6 | 9 | 17 | 21 | 26 | 9 | 21 |
| 10. | X | 5 | 4 | 5 | 13 | 19 | 20 | 25 | 9 | 16 |
| 11. | XI | 6 | 3 | 7 | 13 | 23 | 20 | 21 | 7 | 11 |
| 12. | XII | 6 | 5 | 6 | 12 | 22 | 19 | 23 | 7 | 14 |
| 13. | Within a year | 7 | 8 | 9 | 12 | 17 | 17 | 21 | 9 | 17 |

4. Dobele

| No. | Month | Wind direction |  |  |  |  |  |  |  | Wind lull |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | N | NE | E | SE | S | SW | W | NW |  |
| 1. | I | 9 | 8 | 6 | 12 | 17 | 21 | 19 | 8 | 5 |
| 2. | II | 13 | 7 | 9 | 15 | 15 | 16 | 16 | 9 | 5 |
| 3. | III | 13 | 6 | 7 | 14 | 16 | 18 | 18 | 8 | 6 |
| 4. | IV | 19 | 9 | 8 | 12 | 11 | 14 | 16 | 11 | 6 |
| 5. | V | 20 | 12 | 8 | 9 | 10 | 13 | 17 | 11 | 8 |
| 6. | VI | 19 | 9 | 6 | 7 | 9 | 15 | 22 | 13 | 7 |
| 7. | VII | 14 | 7 | 4 | 6 | 9 | 19 | 26 | 15 | 8 |
| 8. | VIII | 12 | 7 | 4 | 6 | 12 | 21 | 26 | 12 | 8 |
| 9. | IX | 9 | 4 | 5 | 8 | 14 | 21 | 26 | 13 | 8 |
| 10. | X | 5 | 3 | 5 | 12 | 20 | 23 | 22 | 10 | 6 |
| 11. | XI | 6 | 6 | 6 | 11 | 22 | 21 | 20 | 8 | 4 |
| 12. | XII | 5 | 6 | 6 | 12 | 20 | 22 | 20 | 9 | 5 |
| 13. | Within a year | 12 | 7 | 6 | 10 | 14 | 12 | 21 | 11 | 6 |

5. Liepāja

| No. | Month | Wind direction |  |  |  |  |  |  |  | Wind lull |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | N | NE | E | SE | S | SW | W | NW |  |
| 1. | I | 7 | 10 | 12 | 20 | 16 | 13 | 15 | 7 | 1 |
| 2. | II | 9 | 12 | 14 | 17 | 17 | 11 | 12 | 8 | 1 |
| 3. | III | 8 | 11 | 13 | 15 | 16 | 17 | 12 | 8 | 1 |
| 4. | IV | 14 | 13 | 11 | 10 | 13 | 18 | 11 | 10 | 1 |
| 5. | V | 15 | 13 | 12 | 9 | 10 | 17 | 13 | 11 | 1 |
| 6. | VI | 16 | 12 | 8 | 6 | 10 | 20 | 16 | 12 | 1 |
| 7. | VII | 12 | 8 | 7 | 6 | 9 | 21 | 23 | 14 | 1 |
| 8. | VIII | 12 | 10 | 7 | 8 | 10 | 17 | 23 | 13 | 1 |
| 9. | IX | 10 | 8 | 8 | 13 | 12 | 14 | 24 | 11 | 1 |
| 10. | X | 7 | 6 | 10 | 17 | 14 | 15 | 21 | 10 | 1 |
| 11. | XI | 7 | 8 | 9 | 18 | 17 | 13 | 18 | 10 | 1 |
| 12. | XII | 8 | 8 | 10 | 19 | 16 | 12 | 18 | 9 | 1 |
| 13. | Within a year | 11 | 10 | 10 | 13 | 13 | 16 | 17 | 10 | 1 |

6. Priekuļi

| No. | Month | Wind direction |  |  |  |  |  |  |  | Wind lull |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | N | NE | E | SE | S | SW | W | NW |  |
| 1. | I | 6 | 9 | 7 | 14 | 26 | 22 | 10 | 6 | 6 |
| 2. | II | 6 | 8 | 8 | 17 | 24 | 19 | 12 | 6 | 7 |
| 3. | III | 5 | 8 | 7 | 15 | 25 | 19 | 14 | 7 | 7 |
| 4. | IV | 8 | 11 | 8 | 15 | 18 | 15 | 16 | 9 | 7 |
| 5. | V | 9 | 14 | 11 | 13 | 15 | 13 | 15 | 10 | 8 |
| 6. | VI | 10 | 11 | 8 | 10 | 14 | 16 | 19 | 12 | 9 |
| 7. | VII | 9 | 8 | 6 | 9 | 16 | 20 | 20 | 12 | 11 |
| 8. | VIII | 7 | 7 | 5 | 10 | 21 | 21 | 19 | 10 | 11 |
| 9. | IX | 5 | 5 | 6 | 12 | 22 | 24 | 17 | 9 | 9 |
| 10. | X | 5 | 4 | 5 | 13 | 24 | 26 | 15 | 8 | 7 |
| 11. | XI | 6 | 5 | 5 | 12 | 30 | 25 | 10 | 7 | 5 |
| 12. | XII | 5 | 6 | 6 | 12 | 29 | 23 | 11 | 8 | 5 |
| 13. | Within a year | 7 | 8 | 7 | 13 | 22 | 20 | 15 | 8 | 8 |

7. Rīga

| No. | Month | Wind direction |  |  |  |  |  |  |  | Wind lull |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | N | NE | E | SE | S | SW | W | NW |  |
| 1. | I | 7 | 6 | 9 | 20 | 30 | 14 | 9 | 5 | 6 |
| 2. | II | 8 | 6 | 12 | 20 | 23 | 13 | 10 | 8 | 7 |
| 3. | III | 8 | 5 | 11 | 22 | 25 | 13 | 8 | 8 | 7 |
| 4. | IV | 18 | 7 | 10 | 12 | 17 | 11 | 10 | 15 | 10 |
| 5. | V | 23 | 9 | 9 | 12 | 13 | 9 | 9 | 16 | 10 |
| 6. | VI | 23 | 5 | 6 | 8 | 15 | 13 | 12 | 18 | 11 |
| 7. | VII | 16 | 4 | 6 | 8 | 17 | 16 | 14 | 19 | 13 |
| 8. | VIII | 14 | 6 | 5 | 9 | 20 | 18 | 13 | 15 | 12 |
| 9. | IX | 10 | 5 | 5 | 12 | 25 | 20 | 13 | 10 | 9 |
| 10. | X | 6 | 5 | 8 | 18 | 28 | 19 | 10 | 6 | 5 |
| 11. | XI | 7 | 5 | 6 | 17 | 34 | 17 | 9 | 5 | 4 |
| 12. | XII | 7 | 6 | 10 | 17 | 32 | 15 | 8 | 5 | 5 |
| 13. | Within a year | 12 | 6 | 8 | 15 | 23 | 15 | 10 | 11 | 8 |

8. Stende

| No. | Month | Wind direction |  |  |  |  |  |  |  | Wind lull |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | N | NE | E | SE | S | SW | W | NW |  |
| 1. | I | 7 | 5 | 9 | 19 | 24 | 18 | 13 | 5 | 7 |
| 2. | II | 7 | 6 | 12 | 21 | 20 | 14 | 13 | 7 | 8 |
| 3. | III | 6 | 6 | 12 | 20 | 21 | 17 | 11 | 7 | 8 |
| 4. | IV | 11 | 8 | 12 | 16 | 15 | 14 | 13 | 11 | 8 |
| 5. | V | 12 | 12 | 15 | 13 | 12 | 13 | 13 | 10 | 9 |
| 6. | VI | 13 | 10 | 10 | 10 | 13 | 16 | 16 | 12 | 11 |
| 7. | VII | 10 | 6 | 8 | 9 | 16 | 19 | 20 | 12 | 12 |
| 8. | VIII | 9 | 7 | 7 | 11 | 19 | 20 | 18 | 9 | 16 |
| 9. | IX | 6 | 4 | 7 | 15 | 21 | 20 | 19 | 8 | 11 |
| 10. | X | 4 | 3 | 7 | 17 | 22 | 22 | 18 | 7 | 8 |
| 11. | XI | 5 | 3 | 7 | 17 | 29 | 19 | 15 | 5 | 6 |
| 12. | XII | 5 | 3 | 6 | 18 | 28 | 19 | 15 | 6 | 8 |
| 13. | Within a year | 8 | 6 | 9 | 16 | 20 | 18 | 15 | 8 | 9 |

9. Zīlāni

| No. | Month | Wind direction |  |  |  |  |  |  |  | Wind lull |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | N | NE | E | SE | S | SW | W | NW |  |
| 1. | I | 5 | 7 | 12 | 18 | 18 | 19 | 13 | 8 | 5 |
| 2. | II | 5 | 8 | 14 | 21 | 15 | 16 | 14 | 7 | 5 |
| 3. | III | 4 | 7 | 12 | 20 | 19 | 17 | 14 | 7 | 5 |
| 4. | IV | 7 | 10 | 14 | 16 | 13 | 14 | 14 | 12 | 5 |
| 5. | V | 10 | 12 | 15 | 15 | 12 | 11 | 14 | 11 | 7 |
| 6. | VI | 10 | 11 | 11 | 11 | 12 | 14 | 18 | 13 | 7 |
| 7. | VII | 8 | 7 | 8 | 10 | 14 | 20 | 20 | 13 | 10 |
| 8. | VIII | 7 | 7 | 8 | 12 | 16 | 20 | 20 | 10 | 10 |
| 9. | IX | 6 | 5 | 8 | 13 | 19 | 22 | 18 | 9 | 8 |
| 10. | X | 4 | 3 | 8 | 17 | 19 | 23 | 18 | 8 | 6 |
| 11. | XI | 5 | 5 | 8 | 17 | 21 | 23 | 15 | 6 | 4 |
| 12. | XII | 4 | 6 | 9 | 16 | 21 | 23 | 15 | 6 | 6 |
| 13. | Within a year | 6 | 7 | 11 | 15 | 17 | 19 | 16 | 9 | 7 |

Table 14

Average Wind Speed (m/s)

| No. | Place | Month |  |  |  |  |  |  |  |  |  |  |  | On average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII | per year |
| 1. | Ainaži | 5.1 | 4.7 | 4.7 | 4.3 | 4.0 | 3.8 | 3.8 | 4.0 | 4.5 | 5.1 | 5.4 | 5.2 | 4.5 |
| 2. | Alūksne | 3.1 | 3.0 | 2.9 | 2.7 | 2.4 | 2.3 | 2.1 | 2.2 | 2.6 | 2.9 | 3.1 | 3.0 | 2.7 |
| 3. | Daugavpils | 3.5 | 3.4 | 3.4 | 3.3 | 3.0 | 2.9 | 2.7 | 2.6 | 2.9 | 3.3 | 3.7 | 3.5 | 3.2 |
| 4. | Dobele | 4.2 | 4.0 | 4.0 | 3.9 | 3.7 | 3.5 | 3.2 | 3.1 | 3.4 | 3.8 | 4.2 | 4.3 | 3.8 |
| 5. | Liepāja | 5.9 | 5.3 | 5.4 | 5.1 | 4.6 | 4.5 | 4.6 | 4.7 | 5.3 | 5.7 | 6.3 | 6.0 | 5.3 |
| 6. | Priekuļi | 4.3 | 4.0 | 4.0 | 3.7 | 3.3 | 3.2 | 3.0 | 3.1 | 3.7 | 4.1 | 4.4 | 4.3 | 3.8 |
| 7. | Rīga | 4.9 | 4.4 | 4.5 | 4.2 | 3.9 | 3.9 | 3.6 | 3.7 | 4.3 | 4.8 | 5.2 | 5.1 | 4.4 |
| 8. | Stende | 3.9 | 3.7 | 3.7 | 3.4 | 3.2 | 2.9 | 2.7 | 2.7 | 3.1 | 3.6 | 3.9 | 3.8 | 3.4 |
| 9. | Zīlāni | 3.8 | 3.6 | 3.6 | 3.4 | 3.1 | 3.0 | 2.8 | 2.7 | 3.1 | 3.5 | 3.8 | 3.7 | 3.3 |

Table 15

Maximum Wind Speed and Maximum Wind Gusts (m/s)

1. Ainaži

| Characteristics of the wind |  | I |  | II |  | III |  | IV |  | V |  | VI |  | VII |  | VIII |  | IX |  | X |  | XI |  | XII |  | Year |  | Period of observations |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts |  |
| Maximum wind speed | from 2 min. average | 20* |  | 20* |  | 18* |  | 18* |  | 17* |  | 17* |  | 17* |  | 17* |  | 20 |  | 20* |  | 34 |  | 20* |  | 34 |  | II 1945–V 1976 |
|  | from 10 min. average | 17 |  | 13 |  | 14 |  | 13 |  | 14 |  | 13 |  | 15 |  | 13 |  | 16 |  | 15 |  | 15 |  | 16 |  | 17 |  | VI 1976–XII 2003 |
| Maximum wind gust |  |  | 26 |  | 22 |  | 23 |  | 20 |  | 21 |  | 18 |  | 28 |  | 21 |  | 28 |  | 26 |  | 40 |  | 23 |  | 40 | VII 1959–XII 2003 |

* Maximum wind speed observed during a time period when the observations of wind gusts were not yet carried out.

2. Alūksne

| Wind characteristics |  | I |  | II |  | III |  | IV |  | V |  | VI |  | VII |  | VIII |  | IX |  | X |  | XI |  | XII |  | Year |  | Period of observations |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts |  |  |
| Maximum wind speed | from 2 min. average | 24 |  | 17* |  | 17* |  | 17* |  | 17* |  | 17* |  | 17* |  | 17 |  | 17* |  | 17* |  | 17* |  | 24 |  | 24 |  | VI 1945–III 1975 |  |
|  | from 10 min. average | 18 |  | 11 |  | 14 |  | 14 |  | 14 |  | 12 |  | 12 |  | 12 |  | 12 |  | 11 |  | 15 |  | 16 |  | 18 |  | IV 1975–XII 2003 |  |
| Maximum wind gust |  |  | 34 |  | 23 |  | 24 |  | 20 |  | 20 |  | 24 |  | 24 |  | 29 |  | 24 |  | 24 |  | 25 |  | 34 |  | 34 | VII 1959–XII 2003 | VII 1959–XII 2003 |

* Maximum wind speed observed during a time period when the observations of wind gusts were not yet carried out.

3. Bauska

| Wind characteristics |  | I |  | II |  | III |  | IV |  | V |  | VI |  | VII |  | VIII |  | IX |  | X |  | XI |  | XII |  | Year |  | Period of observations |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts |  |  |
| Maximum wind speed | from 2 min. average | 28 |  | 17* |  | 17* |  | 20 |  | 17* |  | 17* |  | 17* |  | 14 |  | 18 |  | 28 |  | 17* |  | 17* |  | 28 |  | I 1946–III 1975 |  |
|  | from 10 min. average | 14 |  | 11 |  | 12 |  | 11 |  | 11 |  | 10 |  | 14 |  | 9 |  | 11 |  | 16 |  | 16 |  | 14 |  | 16 |  | IV 1975–XII 2003 |  |
| Maximum wind gust |  |  | 40 |  | 21 |  | 24 |  | 26 |  | 20 |  | 28 |  | 27 |  | 20 |  | 24 |  | 40 |  | 28 |  | 25 |  | 40 | VII 1959–XII 2003 | VII 1959–XII 2003 |

* Maximum wind speed observed during a time period when the observations of the wind gusts were not yet carried out.

4. Daugavpils

| Wind characteristics |  | I |  | II |  | III |  | IV |  | V |  | VI |  | VII |  | VIII |  | IX |  | X |  | XI |  | XII |  | Year |  | Period of observations |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts |  |  |
| Maximum wind speed | from 2 min. average | 20* |  | 20 |  | 20 |  | 20* |  | 20* |  | 17* |  | 16 |  | 17* |  | 17* |  | 25 |  | 28 |  | 20 |  | 28 |  | II 1947–IV 1973 |  |
|  | from 10 min. average | 18 |  | 12 |  | 14 |  | 14 |  | 10 |  | 15 |  | 10 |  | 10 |  | 14 |  | 15 |  | 16 |  | 20 |  | 20 |  | V 1973–XII 2003 |  |
| Maximum wind gust |  |  | 24 |  | 28 |  | 28 |  | 24 |  | 24 |  | 23 |  | 24 |  | 20 |  | 24 |  | 34 |  | 44 |  | 34 |  | 44 | VII 1959–XII 2003 | VII 1959–XII 2003 |

* Maximum wind blast observed during a time period when the observations of wind gusts were not yet carried out.

5. Dobele

| Wind characteristics |  | I |  | II |  | III |  | IV |  | V |  | VI |  | VII |  | VIII |  | IX |  | X |  | XI |  | XII |  | Year |  | Period of observations |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts |  |  |
| Maximum wind speed | from 2 min. average | 17 |  | 18* |  | 17* |  | 20* |  | 20* |  | 17* |  | 17* |  | 17* |  | 18 |  | 18 |  | 25 |  | 20 |  | 25 |  | X 1949–III 1976 |  |
|  | from 10 min. average | 22 |  | 16 |  | 13 |  | 14 |  | 13 |  | 12 |  | 13 |  | 11 |  | 14 |  | 14 |  | 15 |  | 15 |  | 22 |  | IV 1976–XII 2003 |  |
| Maximum wind gust |  |  | 31 |  | 24 |  | 24 |  | 26 |  | 23 |  | 24 |  | 33 |  | 22 |  | 28 |  | 28 |  | 34 |  | 24 |  | 34 | VII 1959–XII 2003 | VII 1959–XII 2003 |

* Maximum wind speed observed during a time period when the observations of wind gusts were not yet carried out.

6. Jelgava

| Wind characteristics |  | I |  | II |  | III |  | IV |  | V |  | VI |  | VII |  | VIII |  | IX |  | X |  | XI |  | XII |  | Year |  | Period of observations |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts |  |  |
| Maximum wind speed | from 2 min. average | 20* |  | 20* |  | 20* |  | 20* |  | 17* |  | 17* |  | 17* |  | 17* |  | 20* |  | 20 |  | 20* |  | 20* |  | 20 |  | I 1945–II 1975 |  |
|  | from 10 min. average | 17 |  | 13 |  | 12 |  | 15 |  | 11 |  | 10 |  | 10 |  | 9 |  | 13 |  | 12 |  | 14 |  | 15 |  | 17 |  | III 1975–XII 2003 |  |
| Maximum wind gust |  |  | 26 |  | 24 |  | 28 |  | 21 |  | 20 |  | 19 |  | 20 |  | 18 |  | 28 |  | 28 |  | 34 |  | 24 |  | 34 | VII 1959–XII 2003 | VII 1959–XII 2003 |

* Maximum wind speed observed during a time period when the observations of wind gusts were not yet carried out.

7. Gulbene

| Wind characteristics |  | I |  | II |  | III |  | IV |  | V |  | VI |  | VII |  | VIII |  | IX |  | X |  | XI |  | XII |  | Year |  | Period of observations |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts |  |  |
| Maximum wind speed | from 2 min. average | 14 |  | 16 |  | 14 |  | 17* |  | 12* |  | 11* |  | 10* |  | 12* |  | 14* |  | 18 |  | 12 |  | 18 |  | 18 |  | I 1945–II 1975 |  |
|  | from 10 min. average | 14 |  | 13 |  | 12 |  | 13 |  | 10 |  | 10 |  | 11 |  | 11 |  | 13 |  | 12 |  | 13 |  | 14 |  | 14 |  | III 1975–XII 2003 |  |
| Maximum wind gust |  |  | 24 |  | 21 |  | 24 |  | 24 |  | 19 |  | 20 |  | 19 |  | 19 |  | 24 |  | 34 |  | 26 |  | 27 |  | 34 | VII 1959–XII 2003 | VIII 1959–XII 2003 |

* Maximum wind speed observed during a time period when the observations of wind gust were not yet carried out.

8. Kolka

| Wind characteristics |  | I |  | II |  | III |  | IV |  | V |  | VI |  | VII |  | VIII |  | IX |  | X |  | XI |  | XII |  | Year |  | Period pf observations |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts |  |  |
| Maximum wind speed | from 2 min. average | 18 |  | 20* |  | 20 |  | 20* |  | 20* |  | 20* |  | 20* |  | 17* |  | 18 |  | 20* |  | 20 |  | 22 |  | 22 |  | VII 1945–XI 1974 |  |
|  | from 10 min. average | 15 |  | 14 |  | 17 |  | 14 |  | 10 |  | 10 |  | 12 |  | 13 |  | 16 |  | 18 |  | 18 |  | 17 |  | 18 |  | XII 1974–XII 2003 |  |
| Maximum wind gust |  |  | 28 |  | 27 |  | 28 |  | 27 |  | 23 |  | 27 |  | 22 |  | 23 |  | 24 |  | 27 |  | 34 |  | 27 |  | 34 | VII 1959–XII 2003 | VII 1959–XII 2003 |

* Maximum wind speed observed during a time period when the observations of wind gusts were not yet carried out.

9. Liepāja

| Wind characteristics |  | I |  | II |  | III |  | IV |  | V |  | VI |  | VII |  | VIII |  | IX |  | X |  | XI |  | XII |  | Year |  | Period of observations |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts |  |  |
| Maximum wind speed | from 2 min. average | 34* |  | 24 |  | 34* |  | 24* |  | 20* |  | 20* |  | 20* |  | 24* |  | 28* |  | 28* |  | 28* |  | 34* |  | 34 |  | VI 1945–IX 1965 |  |
|  | from 10 min. average | 20 |  | 18 |  | 20 |  | 18 |  | 17 |  | 14 |  | 20 |  | 17 |  | 20 |  | 30 |  | 24 |  | 24 |  | 30 |  | X 1965–XII 2003 |  |
| Maximum wind gust |  |  | 31 |  | 28 |  | 28 |  | 29 |  | 24 |  | 22 |  | 30 |  | 27 |  | 28 |  | 48 |  | 33 |  | 29 |  | 48 | VII 1959–XII 2003 | VIII 1959–XII 2003 |

* Maximum wind speed observed during a time period when the observations of wind gusts were not yet carried out.

10 Mērsrags

| Wind characteristics |  | I |  | II |  | III |  | IV |  | V |  | VI |  | VII |  | VIII |  | IX |  | X |  | XI |  | XII |  | Year |  | Period of observations |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts |  |  |
| Maximum wind speed | from 2 min. average | 20* |  | 20* |  | 20* |  | 20* |  | 17* |  | 20* |  | 20* |  | 20* |  | 20* |  | 24* |  | 24* |  | 24* |  | 24 |  | VI 1945–XI 1974 |  |
|  | from 10 min. average | 15 |  | 13 |  | 13 |  | 13 |  | 12 |  | 10 |  | 10 |  | 11 |  | 12 |  | 13 |  | 13 |  | 18 |  | 18 |  | XII 1974–XII 2003 |  |
| Maximum wind gust |  |  | 26 |  | 23 |  | 28 |  | 24 |  | 21 |  | 20 |  | 20 |  | 20 |  | 23 |  | 24 |  | 28 |  | 28 |  | 28 | VII 1959–XII 2003 | VII 1959–XII 2003 |

* Maximum wind speed observed during a time period when the observations of wind gusts were not yet carried out.

11. Pāvilosta

| Wind characteristics |  | I |  | II |  | III |  | IV |  | V |  | VI |  | VII |  | VIII |  | IX |  | X |  | XI |  | XII |  | Year |  | Period of observations |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts |  |  |
| Maximum wind speed | from 2 min. average | 24* |  | 28* |  | 24* |  | 28* |  | 20* |  | 17* |  | 18* |  | 28* |  | 24* |  | 28* |  | 28* |  | 28* |  | 28 |  | VII 1945–XI 1974 |  |
|  | from 10 min. average | 22 |  | 13 |  | 12 |  | 13 |  | 11 |  | 10 |  | 10 |  | 10 |  | 12 |  | 14 |  | 18 |  | 16 |  | 22 |  | XII 1974–XII 2003 |  |
| Maximum wind gust |  |  | 30 |  | 25 |  | 24 |  | 28 |  | 19 |  | 20 |  | 24 |  | 24 |  | 24 |  | 28 |  | 28 |  | 28 |  | 30 | VII 1959–XII 2003 | VII 1959–XII 2003 |

* Maximum wind speed observed during a time period when the observations of wind gusts were not yet carried out.

12. Priekuļi

| Wind characteristics |  | I |  | II |  | III |  | IV |  | V |  | VI |  | VII |  | VIII |  | IX |  | X |  | XI |  | XII |  | Year |  | Period of observations |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts |  |  |
| Maximum wind speed | from 2 min. average | 20* |  | 18 |  | 20* |  | 17* |  | 17* |  | 17* |  | 17* |  | 17* |  | 20 |  | 20 |  | 24 |  | 20* |  | 24 |  | I 1945–I 1976 |  |
|  | from 10 min. average | 15 |  | 14 |  | 14 |  | 14 |  | 14 |  | 12 |  | 13 |  | 11 |  | 16 |  | 12 |  | 14 |  | 15 |  | 16 |  | II 1976–XII 2003 |  |
| Maximum wind gust |  |  | 28 |  | 26 |  | 28 |  | 26 |  | 25 |  | 22 |  | 28 |  | 19 |  | 27 |  | 28 |  | 34 |  | 29 |  | 34 | VII 1959–XII 2003 | VII 1959–XII 2003 |

* Maximum wind speed observed during a time period when the observations of wind gusts were not yet carried out.

13. Rēzekne

| Wind characteristics |  | I |  | II |  | III |  | IV |  | V |  | VI |  | VII |  | VIII |  | IX |  | X |  | XI |  | XII |  | Year |  | Period of observations |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts |  |  |
| Maximum wind speed | from 2 min. average | 20* |  | 20* |  | 20* |  | 17* |  | 21* |  | 17* |  | 17* |  | 17* |  | 17* |  | 24 |  | 20 |  | 18 |  | 24 |  | X 1947–IX 1976 |  |
|  | from 10 min. average | 17 |  | 12 |  | 13 |  | 12 |  | 12 |  | 12 |  | 10 |  | 11 |  | 13 |  | 14 |  | 15 |  | 12 |  | 17 |  | X 1976–XII 2003 |  |
| Maximum wind gust |  |  | 24 |  | 24 |  | 20 |  | 22 |  | 21 |  | 28 |  | 19 |  | 19 |  | 22 |  | 34 |  | 24 |  | 24 |  | 34 | VII 1959–XII 2003 | VIII 1959–XII 2003 |

* Maximum wind speed observed during a time period when the observations of wind gusts were not yet carried out.

14. Rīga

| Wind characteristics |  | I |  | II |  | III |  | IV |  | V |  | VI |  | VII |  | VIII |  | IX |  | X |  | XI |  | XII |  | Year |  | Period of observations |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts |  |  |
| Maximum wind speed | from 2 min. average | 20* |  | 17* |  | 18 |  | 17* |  | 17* |  | 20* |  | 14* |  | 12* |  | 17* |  | 24* |  | 18 |  | 17* |  | 24 |  | I 1945–IV 1973 |  |
|  | from 10 min. average | 22 |  | 16 |  | 19 |  | 15 |  | 13 |  | 15 |  | 15 |  | 15 |  | 16 |  | 17 |  | 20 |  | 20 |  | 22 |  | V 1973–XII 2003 |  |
| Maximum wind gust |  |  | 28 |  | 24 |  | 25 |  | 24 |  | 22 |  | 20 |  | 23 |  | 23 |  | 25 |  | 23 |  | 31 |  | 27 |  | 31 | VII 1959–XII 2003 | VIII 1959–XII 2003 |

* Maximum wind speed observed during a time period when the observations of wind gusts were not yet carried out.

15. Rūjiena

| Wind characteristics |  | I |  | II |  | III |  | IV |  | V |  | VI |  | VII |  | VIII |  | IX |  | X |  | XI |  | XII |  | Year |  | Period of observations |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts |  |  |
| Maximum wind speed | from 2 min. average | 20* |  | 17* |  | 20* |  | 17* |  | 17* |  | 17* |  | 17* |  | 17* |  | 20 |  | 20 |  | 17* |  | 17* |  | 20 |  | I 1945–V 1976 |  |
|  | from 10 min. average | 13 |  | 16 |  | 13 |  | 13 |  | 13 |  | 12 |  | 13 |  | 10 |  | 15 |  | 13 |  | 17 |  | 13 |  | 17 |  | VI 1976–XII 2003 |  |
| Maximum wind gust |  |  | 26 |  | 24 |  | 22 |  | 20 |  | 24 |  | 22 |  | 24 |  | 20 |  | 24 |  | 24 |  | 29 |  | 24 |  | 29 | VII 1959–XII 2003 | VII 1959–XII 2003 |

* Maximum wind speed observed during a time period when the observations of wind gusts were not yet carried out.

16. Saldus

| Wind characteristics |  | I |  | II |  | III |  | IV |  | V |  | VI |  | VII |  | VIII |  | IX |  | X |  | XI |  | XII |  | Year |  | Period of observations |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts |  |  |
| Maximum wind speed | from 2 min. average | 17* |  | 18 |  | 17* |  | 17* |  | 17* |  | 12 |  | 12* |  | 20* |  | 17* |  | 20* |  | 14 |  | 17* |  | 20 |  | I 1946–III 1967 |  |
|  | from 10 min. average | 15 |  | 18 |  | 18 |  | 16 |  | 14 |  | 14 |  | 14 |  | 14 |  | 20 |  | 16 |  | 20 |  | 16 |  | 20 |  | IV 1967–XII 2003 |  |
| Maximum wind gust |  |  | 30 |  | 25 |  | 25 |  | 28 |  | 24 |  | 25 |  | 24 |  | 28 |  | 34 |  | 30 |  | 34 |  | 28 |  | 34 | VII 1959–XII 2003 | VII 1959–XII 2003 |

* Maximum wind speed observed during a time period when the observations of wind gusts were not yet carried out.

17. Skrīveri

| Wind characteristics |  | I |  | II |  | III |  | IV |  | V |  | VI |  | VII |  | VIII |  | IX |  | X |  | XI |  | XII |  | Year |  | Period of observations |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts |  |  |
| Maximum wind speed | from 2 min. average | 17* |  | 17* |  | 17* |  | 17* |  | 17* |  | 14* |  | 17* |  | 20* |  | 17* |  | 17* |  | 18 |  | 16 |  | 20 |  | I 1945–I 1976 |  |
|  | from 10 min. average | 11 |  | 12 |  | 10 |  | 12 |  | 10 |  | 9 |  | 13 |  | 9 |  | 9 |  | 10 |  | 11 |  | 12 |  | 13 |  | II 1976–XII 2003 |  |
| Maximum wind gust |  |  | 23 |  | 24 |  | 24 |  | 20 |  | 25 |  | 18 |  | 22 |  | 18 |  | 28 |  | 24 |  | 34 |  | 21 |  | 34 | VII 1959–XII 2003 | VII 1959–XII 2003 |

* Maximum wind speed observed during a time period when the observations of wind gusts were not yet carried out.

18. Skulte

| Wind characteristics |  | I |  | II |  | III |  | IV |  | V |  | VI |  | VII |  | VIII |  | IX |  | X |  | XI |  | XII |  | Year |  | Period of observations |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts |  |  |
| Maximum wind speed | from 2 min. average | 20* |  | 20 |  | 18 |  | 17* |  | 16* |  | 17* |  | 20* |  | 20* |  | 20* |  | 28* |  | 20 |  | 28* |  | 28 |  | III 1945–XI 1974 |  |
|  | from 10 min. average | 12 |  | 11 |  | 14 |  | 10 |  | 9 |  | 9 |  | 10 |  | 11 |  | 12 |  | 14 |  | 13 |  | 12 |  | 14 |  | XII 1974–XII 2003 |  |
| Maximum wind gust |  |  | 22 |  | 34 |  | 28 |  | 20 |  | 23 |  | 20 |  | 21 |  | 24 |  | 24 |  | 34 |  | 34 |  | 24 |  | 34 | VII 1959–XII 2003 | VII 1959–XII 2003 |

* Maximum wind speed observed during a time period when the observations of wind gusts were not yet carried out.

19. Stende

| Wind characteristics |  | I |  | II |  | III |  | IV |  | V |  | VI |  | VII |  | VIII |  | IX |  | X |  | XI |  | XII |  | Year |  | Period of observations |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts |  |  |
| Maximum wind speed | from 2 min. average | 16 |  | 17* |  | 17* |  | 17* |  | 14* |  | 12* |  | 17* |  | 17* |  | 17* |  | 17* |  | 17* |  | 17* |  | 17 |  | VI 1945–X 1965 |  |
|  | from 10 min. average | 18 |  | 15 |  | 15 |  | 14 |  | 13 |  | 13 |  | 11 |  | 11 |  | 13 |  | 15 |  | 20 |  | 15 |  | 20 |  | XI 1965–XII 2003 |  |
| Maximum wind gust |  |  | 28 |  | 23 |  | 25 |  | 22 |  | 24 |  | 20 |  | 19 |  | 20 |  | 24 |  | 28 |  | 34 |  | 25 |  | 34 | VII 1959–XII 2003 | VII 1959–XII 2003 |

* Maximum wind speed observed during a time period when the observations of wind gusts were not yet carried out.

20. Ventspils

| Wind characteristics |  | I |  | II |  | III |  | IV |  | V |  | VI |  | VII |  | VIII |  | IX |  | X |  | XI |  | XII |  | Year |  | Period of observations |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts |  |  |
| Maximum wind speed | from 2 min. average | 28* |  | 28 |  | 34 |  | 28* |  | 20* |  | 28 |  | 20 |  | 28 |  | 28* |  | 34 |  | 34 |  | 34 |  | 34 |  | VIII 1945–X 1965 |  |
|  | from 10 min. average | 20 |  | 17 |  | 20 |  | 22 |  | 20 |  | 20 |  | 20 |  | 20 |  | 26 |  | 30 |  | 26 |  | 22 |  | 30 |  | XI 1965–XII 2003 |  |
| Maximum wind gust |  |  | 40 |  | 40 |  | 40 |  | 31 |  | 25 |  | 34 |  | 28 |  | 34 |  | 34 |  | 40 |  | 40 |  | 40 |  | 40 | VII 1959–XII 2003 | IX 1959–XII 2003 |

* Maximum wind speed observed during a time period when the observations of wind gusts were not yet carried out.

21. Zīlāni

| Wind characteristics |  | I |  | II |  | III |  | IV |  | V |  | VI |  | VII |  | VIII |  | IX |  | X |  | XI |  | XII |  | Year |  | Period of observations |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts |  |  |
| Maximum wind speed | from 2 min. average | 15* |  | 16 |  | 17* |  | 20 |  | 14* |  | 12* |  | 17* |  | 12 |  | 14 |  | 24 |  | 18 |  | 17* |  | 24 |  | III 1947–VIII 1975 |  |
|  | from 10 min. average | 18 |  | 14 |  | 13 |  | 14 |  | 12 |  | 11 |  | 12 |  | 11 |  | 14 |  | 17 |  | 14 |  | 16 |  | 18 |  | IX 1975–XII 2003 |  |
| Maximum wind gust |  |  | 35 |  | 22 |  | 25 |  | 28 |  | 21 |  | 23 |  | 21 |  | 20 |  | 24 |  | 34 |  | 24 |  | 24 |  | 35 | VII 1959–XII 2003 | VII 1959–XII 2003 |

* Maximum wind speed observed during a time period when the observations of wind gusts were not yet carried out.

22. Zosēni

| Wind characteristics |  | I |  | II |  | III |  | IV |  | V |  | VI |  | VII |  | VIII |  | IX |  | X |  | XI |  | XII |  | Year |  | Period of observations |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts | speed | gusts |  |  |
| Maximum wind speed | from 2 min. average | 20* |  | 12 |  | 14* |  | 17 |  | 12 |  | 17* |  | 12* |  | 12 |  | 17* |  | 18 |  | 17* |  | 17* |  | 20 |  | V 1945–I 1976 |  |
|  | from 10 min. average | 11 |  | 10 |  | 10 |  | 10 |  | 11 |  | 10 |  | 10 |  | 8 |  | 10 |  | 11 |  | 10 |  | 10 |  | 11 |  | II 1976–XII 2003 |  |
| Maximum wind gust |  |  | 21 |  | 21 |  | 24 |  | 20 |  | 20 |  | 23 |  | 18 |  | 20 |  | 22 |  | 24 |  | 23 |  | 21 |  | 24 | VII 1959–XII 2003 | VII 1959–XII 2003 |

* Maximum wind speed observed during a time period when the observations of wind gusts were not yet carried out.

THE BALTIC SEA

THE GULF OF RIGA

Liepāja

Stende

Dobele

Riga

Daugavpils

Zīlāni (Jēkabpils)

Priekuļi (Cēsis)

Alūksne

Ainaži

Figure 1. Division of the Territory of Latvia According to the Wind Pressure that is Possible Once per Five Years (kg/m2)

Zone I

Zone II

Zone III

Zone IV

Figure 11. Fundamental Basic Wind Speed with a Probability 0.02 (once per 50 years). M/s

Figure 12. Basic Wind Pressure Value. KN/m2

Table 16

Thickness and Weight of the Sheet of Snow

| No. | Place | Average maximum thickness of the sheet of snow (cm) | Maximum weight of the sheet of snow (kg/m2) that is possible once per |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 years | 10 years | 20 years | 50 years |
| 1. | Ainaži | 18 | 63 | 80 | 96 | 112 |
| 2. | Alūksne | 43 | 134 | 154 | 171 | 191 |
| 3. | Bauska | 22 | 77 | 99 | 121 | 154 |
| 4. | Daugavpils | 25 | 79 | 98 | 120 | 154 |
| 5. | Dobele | 20 | 69 | 86 | 102 | 129 |
| 6. | Liepāja | 15 | 59 | 79 | 96 | 114 |
| 7. | Mērsrags | 15 | 65 | 78 | 101 | 156 |
| 8. | Priekuļi | 25 | 86 | 107 | 125 | 158 |
| 9. | Rēzekne | 28 | 102 | 119 | 131 | 147 |
| 10. | Rūjiena | 28 | 96 | 118 | 132 | 146 |
| 11. | Stende | 24 | 82 | 99 | 114 | 135 |
| 12. | Zīlāni | 26 | 89 | 110 | 136 | 181 |
| 13. | Gaiziņkalns | 49 | 178 | 205 | 229 | 262 |

Figure 2. Average Maximum Thickness of the Sheet of Snow (cm)

Figure 3. Maximum Weight of the Sheet of Snow that is Possible Once per Five Years (kg/m2)

Table 16.1

Characteristic Value sk of the Snow Load on the Ground
with a Probability of 0.02 (once per 50 Years), kN.m2

| No. | Place | Value | No. | Place | Value |
| --- | --- | --- | --- | --- | --- |
| 1. | Ainaži | 1.1 | 17. | Praviņi | 1.2 |
| 2. | Alūksne | 2.1 | 18. | Priekuļi | 1.6 |
| 3. | Bauska | 1.4 | 19. | Rēzekne | 1.7 |
| 4. | Carnikava | 1.2 | 20. | Rucava | 1.1 |
| 5. | Dagda | 1.7 | 21. | Rūjiena | 1.6 |
| 6. | Daugavpils | 1.5 | 22. | Saldus | 1.4 |
| 7. | Dobele | 1.2 | 23. | Sigulda | 1.7 |
| 8. | Gaiziņkalns | 2.8 | 24. | Sīļi | 1.5 |
| 9. | Gulbene | 1.6 | 25. | Skrīveri | 1.6 |
| 10. | Kuldīga | 1.3 | 26. | Smiltene | 1.6 |
| 11. | Liepāja | 1.1 | 27. | Stende | 1.5 |
| 12. | Lubāna | 1.7 | 28. | Valmiera | 1.5 |
| 13. | Mērsrags | 1.1 | 29. | Vārdava | 1.3 |
| 14. | Nereta | 1.6 | 30. | Viļaka | 1.6 |
| 15. | Pāvilosta | 1.1 | 31. | Zīlāni | 1.7 |
| 16. | Pļaviņas | 1.7 | 32. | Zosēni | 1.8 |

Figure 21. Characteristic Value of the Snow Load on the Surface of the Ground with a Probability of Once per 50 years kN/m2

Table 16.2

Value of the Emergency Snow Load on the Surface of the Ground sAd, kN/m2
(Value of the Emergency Snow Load Coefficient in Cesls is 2.0)

| No. | Place | Value | No. | Place | Value |
| --- | --- | --- | --- | --- | --- |
| 1. | Ainaži | 2.2 | 17. | Praviņi | 2.4 |
| 2. | Alūksne | 4.2 | 18. | Priekuļi | 3.2 |
| 3. | Bauska | 2.8 | 19. | Rēzekne | 3.4 |
| 4. | Carnikava | 2.4 | 20. | Rucava | 2.2 |
| 5. | Dagda | 3.4 | 21. | Rūjiena | 3.2 |
| 6. | Daugavpils | 3.0 | 22. | Saldus | 2.8 |
| 7. | Dobele | 2.4 | 23. | Sigulda | 3.4 |
| 8. | Gaiziņkalns | 5.6 | 24. | Sīļi | 3.0 |
| 9. | Gulbene | 3.2 | 25. | Skrīveri | 3.2 |
| 10. | Kuldīga | 2.6 | 26. | Smiltene | 3.2 |
| 11. | Liepāja | 2.2 | 27. | Stende | 3.0 |
| 12. | Lubāna | 3.4 | 28. | Valmiera | 3.0 |
| 13. | Mērsrags | 2.2 | 29. | Vārdava | 2.6 |
| 14. | Nereta | 3.2 | 30. | Viļaka | 3.2 |
| 15. | Pāvilosta | 2.2 | 31. | Zīlāni | 3.4 |
| 16. | Pļaviņas | 3.4 | 32. | Zosēni | 3.6 |

Figure 22. Value of the emergency snow load on the surface of the ground kN/m2 (value of the emergency snow load coefficient is 2.0)

Table 16.3

Maximum Snow Load Variation Coefficient V of the Winter Period

| No. | Place | Value | No. | Place | Value |
| --- | --- | --- | --- | --- | --- |
| 1. | Ainaži | 0.54 | 17. | Praviņi | 0.46 |
| 2. | Alūksne | 0.35 | 18. | Priekuļi | 0.55 |
| 3. | Bauska | 0.60 | 19. | Rēzekne | 0.51 |
| 4. | Carnikava | 0.61 | 20. | Rucava | 0.69 |
| 5. | Dagda | 0.43 | 21. | Rūjiena | 0.58 |
| 6. | Daugavpils | 0.57 | 22. | Saldus | 0.59 |
| 7. | Dobele | 0.53 | 23. | Sigulda | 0.45 |
| 8. | Gaiziņkalns | 0.40 | 24. | Sīļi | 0.61 |
| 9. | Gulbene | 0.47 | 25. | Skrīveri | 0.50 |
| 10. | Kuldīga | 0.62 | 26. | Smiltene | 0.47 |
| 11. | Liepāja | 0.69 | 27. | Stende | 0.57 |
| 12. | Lubāna | 0.56 | 28. | Valmiera | 0.54 |
| 13. | Mērsrags | 0.66 | 29. | Vārdava | 0.64 |
| 14. | Nereta | 0.53 | 30. | Viļaka | 0.54 |
| 15. | Pāvilosta | 0.60 | 31. | Zīlāni | 0.54 |
| 16. | Pļaviņas | 0.60 | 32. | Zosēni | 0.47 |

Table 16.4

Characteristic Value of the Snow Load on the Surface of the Ground with a Probability once per 150 Years, kN/m2

| No. | Place | Value | No. | Place | Value |
| --- | --- | --- | --- | --- | --- |
| 1. | Ainaži | 1.3 | 17. | Praviņi | 1.4 |
| 2. | Alūksne | 2.4 | 18. | Priekuļi | 1.9 |
| 3. | Bauska | 1.7 | 19. | Rēzekne | 2.0 |
| 4. | Carnikava | 1.4 | 20. | Rucava | 1.4 |
| 5. | Dagda | 2.0 | 21. | Rūjiena | 2.0 |
| 6. | Daugavpils | 1.8 | 22. | Saldus | 1.7 |
| 7. | Dobele | 1.4 | 23. | Sigulda | 2.0 |
| 8. | Gaiziņkalns | 3.2 | 24. | Sīļi | 1.8 |
| 9. | Gulbene | 1.9 | 25. | Skrīveri | 1.9 |
| 10. | Kuldīga | 1.6 | 26. | Smiltene | 1.9 |
| 11. | Liepāja | 1.4 | 27. | Stende | 1.8 |
| 12. | Lubāna | 2.1 | 28. | Valmiera | 1.8 |
| 13. | Mērsrags | 1.3 | 29. | Vārdava | 1.6 |
| 14. | Nereta | 1.9 | 30. | Viļaka | 1.9 |
| 15. | Pāvilosta | 1.3 | 31. | Zīlāni | 2.0 |
| 16. | Pļaviņas | 2.0 | 32. | Zosēni | 2.1 |

Table 17

Weight of the Ice Cover - Frost Sediment on the Wires with a Diameter of 10 mm at the Height of 10 m with Various Probabilities

| No. | Place | Weight (g/m) that is possible once per |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2 years | 5 years | 10 years | 15 years | 20 years | 25 years | 30 years | 50 years |
| 1. | Ainaži | 110 | 180 | 240 | 290 | 330 | 360 | 400 | 490 |
| 2. | Alūksne | 160 | 220 | 280 | 310 | 340 | 360 | 390 | 450 |
| 3. | Liepāja | 40 | 80 | 140 | 190 | 230 | 270 | 320 | 430 |
| 4. | Mērsrags | 30 | 60 | 100 | 140 | 170 | 200 | 240 | 330 |
| 5. | Priekuļi | 150 | 210 | 270 | 310 | 340 | 360 | 390 | 450 |
| 6. | Rēzekne | 150 | 280 | 420 | 520 | 620 | 690 | 790 | 1020 |
| 7. | Rīga | 80 | 130 | 190 | 240 | 280 | 310 | 350 | 440 |
| 8. | Stende | 120 | 200 | 280 | 340 | 390 | 430 | 480 | 590 |
| 9. | Zīlāni | 110 | 180 | 260 | 320 | 360 | 400 | 450 | 560 |
| 10. | Zosēni | 190 | 320 | 450 | 540 | 630 | 690 | 770 | 950 |

Table 18

Normative Layer of the Ice Cover on the Wires with a Diameter of 10 mm at the Height of 10 m with Various Probabilities

| No. | Place | Thickness of the ice cover (mm) that is possible once per |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2 years | 5 years | 10 years | 15 years | 20 years | 25 years | 30 years | 50 years |
| 1. | Ainaži | 3.0 | 4.5 | 5.5 | 6.0 | 6.5 | 7.0 | 7.5 | 8.5 |
| 2. | Alūksne | 4.0 | 5.0 | 6.0 | 6.5 | 7.0 | 7.5 | 7.5 | 8.0 |
| 3. | Liepāja | 1.5 | 2.5 | 3.5 | 4.5 | 5.0 | 5.5 | 6.0 | 7.0 |
| 4. | Mērsrags | 1.0 | 2.0 | 2.5 | 3.5 | 4.0 | 4.5 | 5.0 | 6.5 |
| 5. | Priekuļi | 4.0 | 5.0 | 6.0 | 6.5 | 7.0 | 7.5 | 8.0 | 8.5 |
| 6. | Rēzekne | 4.0 | 6.0 | 8.0 | 9.5 | 11.0 | 12.0 | 13.0 | 15.0 |
| 7. | Rīga | 2.0 | 3.5 | 4.5 | 5.0 | 5.5 | 6.0 | 6.5 | 7.5 |
| 8. | Stende | 3.0 | 4.5 | 6.0 | 7.0 | 7.5 | 8.5 | 9.0 | 10.5 |
| 9. | Zīlāni | 3.0 | 4.5 | 6.0 | 7.0 | 7.5 | 8.5 | 9.0 | 10.5 |
| 10. | Zosēni | 4.5 | 6.5 | 8.5 | 9.5 | 10.5 | 11.5 | 12.0 | 13.5 |

Figure 4. Division of the Territory of Latvia According to the Thickness of the Normative Ice Cover Layer

District I

District II

Table 19

Depth of the Ground Frigidity in Natural Circumstances on the Last Day of the Month

| No. | Place | Average depth of frigidity |  |  |  |  |  |  | Maximum depth of frigidity |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | X | XI | XII | I | II | III | IV | average | greatest |
| 1. | Ainaži |  | * | 18 | 32 | 45 | 38 |  | 53 | 113 |
| 2. | Alūksne | * | 9 | 22 | 38 | 42 | 36 |  | 48 | 95 |
| 3. | Daugavpils | * | 12 | 35 | 62 | 70 | 51 |  | 78 | 134 |
| 4. | Dobele | * | * | 8 | 17 | 22 | 17 |  | 24 | 88 |
| 5. | Mērsrags | * | 2 | 14 | 28 | 31 | 23 |  | 41 | 116 |
| 6. | Liepāja |  | * | 9 | 23 | 26 | * |  | 34 | 87 |
| 7. | Priekuļi | * | * | 11 | 20 | 22 | 18 | * | 26 | 61 |
| 8. | Rīga | * | * | 7 | 15 | 18 | 13 |  | 24 | 47 |
| 9. | Stende | * | * | 12 | 24 | 27 | 22 | * | 36 | 128 |
| 10. | Zīlāni | * | 7 | 21 | 38 | 43 | 40 | * | 48 | 112 |

Figure 5. Normative Depth of the Frigidity of Loamy Ground that is Possible Once per Two Years (cm)

Figure 6. Normative Depth of the Frigidity of Loamy Ground that is Possible Once per 10 Years (cm)

Figure 7. Normative Depth of the Frigidity of Loamy Ground that is Possible Once per 100 Years (cm)

Table 20

Solar Radiation on Variously Oriented Surfaces in Clear Weather in July (MJ/m2)

| No. | Station | Type of radiation | Time interval (in hours) (Solar time) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Daily total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3-4 | 4-5 | 5-6 | 6-7 | 7-8 | 8-9 | 9-10 | 10-11 | 11-12 | 12-13 | 13-14 | 14-15 | 15-16 | 16-17 | 17-18 | 18-19 | 19-20 | 20-21 |  |
|  |  |  | Horizontal surface |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. | Rīga | direct | 0.02 | 0.21 | 0.52 | 0.94 | 1.34 | 1.73 | 2.09 | 2.39 | 2.53 | 2.53 | 2.39 | 2.09 | 1.73 | 1.34 | 0.94 | 0.52 | 0.21 | 0.02 | 23.54 |
|  |  | total | 0.04 | 0.34 | 0.77 | 1.20 | 1.65 | 2.02 | 2.45 | 2.75 | 2.89 | 2.89 | 2.75 | 2.45 | 2.02 | 1.60 | 1.14 | 0.72 | 0.34 | 0.04 | 28.06 |
| 2. | Zosēni | direct | 0.02 | 0.18 | 0.47 | 0.86 | 1.26 | 1.66 | 2.05 | 2.45 | 2.63 | 2.63 | 2.48 | 2.20 | 1.80 | 1.33 | 0.86 | 0.43 | 0.18 | 0.04 | 23.53 |
|  |  | total | 0.03 | 0.32 | 0.70 | 1.19 | 1.66 | 2.16 | 2.52 | 2.88 | 3.06 | 3.06 | 2.88 | 2.59 | 2.20 | 1.69 | 1.19 | 0.68 | 0.32 | 0.05 | 29.18 |
|  |  |  | Vertical surface directed southwards |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. | Rīga | direct | - | - | - | - | 0.27 | 0.80 | 1.27 | 1.63 | 1.84 | 1.84 | 1.65 | 1.29 | 0.81 | 0.27 | - | - | - | - | 11.67 |
|  |  | total | 0.01 | 0.10 | 0.20 | 0.25 | 0.59 | 1.16 | 1.70 | 2.09 | 2.31 | 2.31 | 2.10 | 1.72 | 1.16 | 0.56 | 0.21 | 0.17 | 0.10 | 0.01 | 16.75 |
| 4. | Zosēni | direct | - | - | - | - | 0.25 | 0.76 | 1.22 | 1.60 | 1.80 | 1.80 | 1.60 | 1.24 | 0.77 | 0.25 | - | - | - | - | 11.30 |
|  |  | total | 0.01 | 0.11 | 0.19 | 0.29 | 0.63 | 1.25 | 1.74 | 2.13 | 2.35 | 2.35 | 2.11 | 1.72 | 1.21 | 0.62 | 0.29 | 0.20 | 0.11 | 0.01 | 17.33 |
|  |  |  | Vertical surface directed westwards |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5. | Rīga | direct | - | - | - | - | - | - | - | - | - | 0.39 | 1.13 | 1.75 | 2.18 | 2.39 | 2.30 | 1.91 | 0.99 | 0.24 | 13.28 |
|  |  | total | 0.01 | 0.10 | 0.20 | 0.25 | 0.32 | 0.37 | 0.43 | 0.46 | 0.47 | 0.86 | 1.58 | 2.17 | 2.52 | 2.68 | 2.52 | 2.08 | 1.09 | 0.25 | 18.36 |
| 6. | Zosēni | direct | - | - | - | - | - | - | - | - | - | 0.38 | 1.09 | 1.67 | 2.07 | 2.20 | 2.09 | 1.69 | 0.99 | 0.13 | 12.31 |
|  |  | total | 0.01 | 0.11 | 0.19 | 0.29 | 0.38 | 0.49 | 0.51 | 0.53 | 0.55 | 0.93 | 1.60 | 2.15 | 2.51 | 2.56 | 2.38 | 1.89 | 1.10 | 0.14 | 18.35 |
|  |  |  | Vertical surface directed eastwards |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7. | Rīga | direct | 0.24 | 0.99 | 1.91 | 2.30 | 2.39 | 2.18 | 1.75 | 1.13 | 0.39 | - | - | - | - | - | - | - | - | - | 13.28 |
|  |  | total | 0.25 | 1.09 | 2.08 | 2.52 | 2.68 | 2.52 | 2.17 | 1.58 | 0.86 | 0.47 | 0.46 | 0.43 | 0.37 | 0.32 | 0.25 | 0.20 | 0.10 | 0.01 | 18.36 |
| 8. | Zosēni | direct | 0.13 | 0.99 | 1.69 | 2.09 | 2.20 | 2.07 | 1.67 | 1.09 | 0.38 | - | - | - | - | - | - | - | - | - | 12.31 |
|  |  | total | 0.14 | 1.10 | 1.89 | 2.38 | 2.58 | 2,56 | 2.18 | 1.62 | 0.93 | 0.55 | 0.51 | 0.48 | 0.44 | 0.37 | 0.29 | 0.20 | 0.11 | 0.01 | 18.35 |

Table 21

Average and the Highest  0°C Temperature Depth in Soil

| Depth (cm) | Month |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | XI | XII | I | II | III | IV |
| Daugavpils |  |  |  |  |  |  |
| Average | * | 36 | 67 | 79 | 78 | 27 |
| Highest | 56 | 104 | 133 | 144 | 142 | 96 |
| Gulbene |  |  |  |  |  |  |
| Average | * | 12 | 23 | 27 | 29 | 17 |
| Highest | 44 | 48 | 65 | 73 | 80 | 80 |
| Riga |  |  |  |  |  |  |
| Average | * | * | 11 | 14 | 19 | * |
| Highest | 20 | 38 | 48 | 63 | 80 | 80 |
| Stende |  |  |  |  |  |  |
| Average | * | * | 28 | 36 | 35 | * |
| Highest | 26 | 56 | 103 | 118 | 138 | 135 |
| Zosēni |  |  |  |  |  |  |
| Average | * | 13 | 22 | 23 | 20 | * |
| Highest | 39 | 42 | 73 | 73 | 73 | 58 |

* Soil temperature is less than 0°C in less than 50% of the years in the relevant month.

Minister for Environmental Protection

and Regional Development 			V. Makarovs

[7 June 2005]

Annex 2

to Latvian Building Code LBN 003-01

Construction Climatology

(approved by Cabinet

Regulation No. 376

23 August 2001)

Explanations to the Climatic Data Tables

1. The information regarding the average monthly and annual air temperature has been provided in Table 1 Average Air Temperature (°C), which is the arithmetical average of a period of 30 years (1961 – 1990) of the World Meteorological Organisation (hereinafter – WMO). The probability of the referred to temperatures is 0.5 on average, i.e. they may repeat once per two years on average.

2. Table 2 Absolute Minimum of Air Temperature and the Probabilities thereof (°C). The absolute minimum of the average air temperature of each month shall be the lowest air temperature registered in the station during all of the time period of the observations in the specific month. The absolute minimum of the annual air temperature shall be the lowest air temperature of the year.

The probability of the absolute minimum temperature of the year has been characterised by the temperatures, the excess of which is possible once per 50 years (in such case, the probability that the annual absolute minimum temperature shall not exceed this value, is 0.98) and once per 10 years (probability – 0.90).

3. Table 3 Absolute Maximum of Air Temperature and the Probabilities thereof (°C). The absolute maximum of the air temperature of each month shall be the highest air temperature registered in the station during all of the time period of the observations in the specific month. The absolute maximum of the annual air temperature shall be the highest air temperature of the year.

The probability of the absolute maximum temperature of the year has been characterised by the temperatures, the excess of which is possible once per 50 years (in such case, the probability that the annual absolute minimum temperature shall not exceed this value, is 0.98) and once per 10 years (probability - 0.90).

4. Table 4 Average Maximum Air Temperature (°C) of the Hottest Month and the Probabilities thereof. The average maximum air temperature of the hottest month of each year shall be calculated as the arithmetical average of the absolute maximum temperature of all of the days of the hottest month. The long-term average maximum air temperature shall be the value of the arithmetical average of the station’s observation period. The probability thereof is 0.5 on average, i.e. it may repeat once per two years on average.

The probability of the average maximum temperature of the hottest month has been characterised in addition by the temperatures, the excess of which is possible once per 50 years (in such case, the probability that the annual average maximum temperature shall not exceed this value, is 0.98) and once per 10 years (probability – 0.90).

5. Table 5 Average Minimum Air Temperature (°C) of the Coldest Month and the Probabilities thereof. The average minimum air temperature of the coldest month of each year shall be calculated as the arithmetical average of the absolute minimum temperature of all of the days of the coldest month. The long-term average minimum air temperature shall be the value of the arithmetical average of the station’s observation period. The probability thereof is 0.5 on average, i.e. it may repeat once per two years on average.

The probability of the average minimum temperature of the hottest month has been characterised in addition by the temperatures, the excess of which is possible once per 50 years (in such case, the probability that the average maximum temperature shall not exceed this value, is 0.98) and once per 10 years (probability – 0.90).

6. Table 6 Average Air Temperature (°C) of the Five Coldest Days and the Probabilities thereof. The coldest period of five days, following each other, for the last approximately 50 years of the operation of each station in each cold season has been found and the average temperature has been calculated. The long-term average air temperature of the coldest five days has been calculated as the arithmetical average, taking into account the air temperature of the coldest five days of eight periods (16% from the number of all seasons existent in the data line).

The values of the average air temperature of the coldest five days have been calculated for the probability levels 0.98 and 0.92. For example, the probability that the average air temperature of the five coldest days in the vicinity of Ainaži shall not exceed -26.8 (°C) is 0.98, once per 50 years (probability – 0.02) even a lower temperature is possible.

7. Table 7 Duration of the Heating Period and the Average Air Temperature (°C). Such time period shall be considered as the heating period, when the average daily air temperature is constantly equal to 8.0°C or is lower. The characteristics of the heating period have been calculated for the WMO data period of 30 years (1961 – 1990).

Dates have been found for each cold season of the data line, when the average daily air temperature in the autumn becomes constantly equal to or lower than 8.0°C and in the spring constantly higher than 8.0°C. The number of days between these dates has been calculated (the duration of the heating period of the specific cold season), as well as the sum of the average daily air temperatures has been calculated. The length of the average heating period shall be the arithmetical average of the duration of heating periods of all data line seasons. The average air temperature of the heating period shall be the division of the average daily air temperatures of the whole data line by the number of years and days.

8. Table 8 Average Amplitude of Air Temperature (°C). The average amplitude of the air temperature of each month has been calculated as the difference of the average maximum and average minimum air temperature of the month. The data have been calculated for the WMO period of 30 years (1961 – 1990).

9. Table 9 Daily Average Partial Pressure of Water Vapour in the Air (hPa). Characteristics of air humidity – the partial pressure of the water vapour – is a pressure that water vapour would have, if it alone took such capacity at a specific temperature and atmosphere that is taken by wet air. The average data have been calculated for the WMO period of 30 years (1961 – 1990).

10. Table 10 Daily Average Relative Air Humidity (%). The relative air humidity shall be the ratio of the water vapour partial pressure in the air existent at a specific temperature and in the atmosphere expressed in percentage to the partial pressure of saturated water vapour at the same temperature and in the atmosphere. The relative air humidity shall characterise the saturation of air with water vapour. The average data have been calculated for the WMO period of 30 years (1961 – 1990).

11. Table 11 Amplitude of Relative Air Humidity. During the day, the relative air humidity usually is the greatest before the sunrise and the lowest – in the afternoon. Therefore, for a more precise calculation of the amplitude, a data period of 30 years has been chosen, starting from 1966 when the observations of eight terms have been started in the meteorological stations, in the place of four terms. The relative air humidity amplitude has been calculated as the difference of the values of the average monthly relative air humidity at 3 o’clock and at 12 o’clock according to the average Greenwich time (at 5 (6) o’clock and at 14 (15) according to the Latvian winter (summer) time).

12. Table 12 Monthly and Annual Sum of Precipitation (mm). The term “precipitation” includes the liquid, solid and mixed atmospheric precipitation, as well as precipitation caused by a fierce dew, white frost, frost, and fog. The amount of precipitation in meteorology shall be measured in millimetres. A 1 mm thick water layer arises by pouring out 1 litre of water on a horizontal surface of 1m2, without flowing away, soaking in and vaporising. The average data have been calculated for the WMO period of 30 years (1961 – 1990).

13.Tables 13, 14 and 15 include the horizontal component of the wind, characterising it with a direction, speed and turbulence. The wind direction shall be such cardinal point, from which the wind blows. The wind speed shall be the average wind speed during some small time period (usually, during 2 or 10 minutes). The maximum gusts shall be the greatest momentary wind speed.

The data of Tables 13 and 14 have been calculated for the time period of 30 years (1961 - 1990) specified by the WMO, the data of Table 15 - for the whole period, during which the data represented by each station have been obtained.

The wind observations at the meteorological stations shall be carried out at the height of 10-12m above the ground. The monthly and annual maximum wind speed and the maximum wind gusts referred to in Table 15 for the whole relevant observation period (until 2003 inclusive) have been determined at the actual location and height of each station and each wind speed measuring instrument.

14. Figure 1 Division of the Territory of Latvia According to the Wind Pressure that is Possible Once per Five Years (kg/m2). Wind speed, which is possible once per five years (v), has been used for the calculation of wind pressure. Wind pressure P (kgf/m2) has been calculated by using the following formula: P = v2/16. Wind zones have been characterised with the maximum wind speed in the zone that is possible once a year, once per 5, 10, 15, 20, 25 and 50 years.

| Wind zone | Maximum wind speed that is possible once per |  |  |  |  |  |  | Wind pressure (kgf/m2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | within a year | 5 years | 10 years | 15 years | 20 years | 25 years | 50 years |  |
| I | 17 | 20 | 21 | 22 | 22 | 23 | 24 | < 28 |
| II | 20 | 23 | 24 | 25 | 25 | 25 | 27 | 28- 35 |
| III | 24 | 27 | 29 | 30 | 31 | 31 | 33 | 36- 52 |
| IV | 26 | 30 | 32 | 33 | 33 | 33 | 35 | 53- 67 |

14.1 Figure 11. The value of the fundamental basic wind speed has been determined in each point of observations by using the information regarding the maximum average wind speed of each year during a time period of 10 minutes irrespective of the wind direction and season. The value of the maximum wind speed of each year has been recalculated for the height of 10 metres above flat, open rural areas with the length of territory undulation zo 0.05 m. The recalculation has been carried out in conformity with the methodology recommended by the World Meteorology Organisation. The basic wind speed with a probability 0.02 (once per 50 years) has been calculated by using the Gumbel probability distribution.

14.2 Figure 12. The basic values of the wind pressure have been determined in accordance with formulas 4.1 and 4.10 referred to in the standard LVS EN 1991-1-4. The value of direction factor cdir, season factor cseason in accordance with the requirements of the standard LVS EN 1991-1-4 is 1.0. The value of the air density in accordance with the requirements of the referred to standard is 1.25 kg/m3.

15. Table 16 Thickness and Weight of the Sheet of Snow. The results of the special measurements of the sheet of snow have been used in rural routes in an extended number of stations and, as well as in separate meteorological offices during the whole period of the operation thereof in order to characterise the sheet of snow completely territorially.

The long-term average maximum thickness of the sheet of snow in winter has been calculated as the arithmetical average of the highest average thickness of the sheet of snow of all winters. Figure 2 Average Maximum Thickness of the Sheet of Snow (cm) has been developed on the basis of these data.

The maximum weight of the sheet of snow, which is possible once per 5, 10, 20 and 50 years, has been calculated by using the results of the observations regarding the values of the maximum weight of the sheet of snow of all winters. It is accepted to carry out the division of territory according to the weight of the sheet of snow in the climatological practice by using the data of the maximum weight of the sheet of snow that is possible once per five years. Figure 3 Maximum Weight of the Sheet of Snow that is Possible Once per Five Years (kg/m2) has been developed on the basis of these data.

15.1 The calculations of the snow load on the surface of ground and statistical information related thereto, referred to in Tables 16.1, 16.2, 16.3 and 16.4 and in Figure 2.2, have been carried out in 32 points of observation in all climatic zones of Latvia by using the data of the time periods of 54 winters (from the winter of 1950 – 1951 until the winter of 2003 – 2004). The data regarding the maximum amount of water in the sheet of snow have been used for the calculation of snow load in each point of observation, which have been obtained when measuring the sheet of snow in the rural route. The calculation has been carried out by using the Gumbel probability distribution.

15.2 The values of the possible emergency snow load on the surface of the ground in all 32 points of observation, referred to in Table 16.2 and Figure 2.2, have been calculated as the loads that would develop to the thickest sheet of snow up to now, determined in daily observations, with the biggest unit weight of the sheet of snow that is characteristic of wet snow (Table E.1. to Annex E “Eurocode 1 – Actions on Structures. – Part 1-3: General Actions – Snow Loads” of the standard LVS EN 1991-1-3). Snow loads, calculated in such way practically in all climatic regions of Latvia are 1.8-2 times greater than the characteristic values of the snow load on the surface of ground (with a probability of once per 50 years). Thereby, the emergency snow load coefficient value 2.0, recommended in Sub-paragraph 4.3 of the standard LVS EN 1991-1-3, may be used in the conditions of Latvia in all its climatic regions.

15.3 In accordance with Annex D to the standard LVS EN 1991-1-3 by using the formula D.1., the snow loads on the surface of the ground may be corrected so that they conform with the characteristic values at any period of repetition that differ from the characteristic value of the snow load with a probability 0.02 (once per 50 years). Corresponding calculations may be carried out by using the information referred to in Table 16.3 regarding the maximum snow load variation coefficient of the winter period for each point of observation.

15.4 Information regarding the characteristic values of the snow load on the surface of ground with a probability once per 150 years has been provided in Table 16.4. Such time period conforms with the biggest average lifetime of the residential houses, the construction elements thereof, finish (also of the carrying elements of the roof). The calculation has been carried out by using the Gumbel probability distribution.

16. Table 17 Weight of the Ice Cover - Frost Sediment on the Wires with the Diameter of 10 mm at the Height of 10 m with Various Probabilities, Table 18 Normative Layer of the Ice Cover on the Wires with the Diameter of 10 mm at the Height of 10 m with Various Probabilities. The results of the observations of maximum dimensions and maximum weight of the ice cover – frost sediment on the wires with the diameter of 10 mm at the height of 2 m, carried out in meteorological stations, each winter during a time period from 1951 until 1998 have been used.

The weight of sediment has been recalculated according to a conforming methodology on the wires of the diameter of 10 mm at the height of 10 m, the ice cover layer – on the wires of the diameter of 10 mm at the height of 10 m, if the maximum possible density of sediment is 0.9 g/cm3. The ice cover layer, calculated in such way, shall be referred to as a normative in the construction climatology.

17. Figure 4 Division of the Territory of Latvia According to Thickness of the Normative Ice Cover Layer. The data regarding the thickness of the ice cover layer on the wires of the diameter of 10 mm at the height of 10 m that is possible once per 10 years have been used for the division.

Ice cover regions encountered in Latvia may be characterised with the following parameters:

| No. | Characteristics that are possible once per 10 years | Ice cover region |  |
| --- | --- | --- | --- |
|  |  | I | II |
| 1. | Scope of changes of the normative ice cover layer thickness (mm) | 2.5- 7.4 | 7.5- 12.4 |
| 2. | Average normative ice cover layer thickness (mm) | 5 | 10 |
| 3. | Scope of changes of the ice cover – frost sediment mass (g/m) on the wires the diameter of which is 10 mm at the height of 10 m | 90- 370 | 370- 800 |
| 4. | Average ice cover – frost sediment mass (g/m) on the wires the diameter of which is 10 mm at the height of 10 m | 210 | 570 |

18. Table 19 Depth of the Ground Frigidity in Natural Circumstances on the Last Day of the Month. The average depth of the ground frigidity has been calculated for the WMO period of 30 years (1961 – 1990) on the last day of each month under the natural, soil-covered layer at the actual thickness of the sheet of snow. The maximum frigidity depth has been found for each winter and the average and the greatest value thereof has been determined.

19. Figure 5 Normative Depth of the Frigidity of Loamy Ground that is Possible Once per Two Years (cm), Figure 6 Normative Depth of the Frigidity of Loamy Ground that is Possible Once per 10 years (cm) and Figure 7 Normative Depth of the Frigidity of Loamy Ground that is Possible Once per 100 Years (cm).

The normative ground frigidity depth shall be used for the characterisation of the maximum frigidity depth of the ground. The normative frigidity depth of the ground shall be the arithmetical average of the maximum frigidity depth of a snowless ground of the cold seasons. Special ground frigidity observations were carried out in the fields free from snow and the quantitative interconnection thereof with the constantly negative air temperature sum was determined in order to determine the normative ground frigidity depth. The values of the normative ground frigidity depth were calculated by using the found interconnections and constantly negative air temperature sums for each winter during a time period from 1923 – until 1998, the excess of which is possible once per two years (the probability that the normative ground frigidity depth shall not exceed this value is 0.50), once per 10 years (probability – 0.90), once per 100 years (probability - 0.99).

The referred to characteristics have been calculated to loamy soils. There are 56% of loamy soil, 36% - sandy and approximately 8% - peaty soil in Latvia, according to the soil diversity assessment. Sandy soils freeze deeper than loamy. The characteristics of loamy soils may be used for the determination of the normative ground frigidity depth in sandy soils by using the coefficient 1.2. The results of the special observations of an uncovered sandy ground frigidity depth confirm the correctness of this coefficient.

20. Table 20 Solar Radiation on Variously Oriented Surfaces in Clear Weather in July (MJ/m2). Information regarding the direct and total (direct and dispersed radiation sum) sums of radiation hours and days (MJ/m2) on a horizontal surface and on vertical surfaces oriented southwards, westwards and eastwards during the hottest month of the year – July – has been provided in the Table. The average energetic lighting (radiation intensity, kW/m2) may be calculated by dividing the radiation hour sum by 3.6.

The observations of solar radiation have been carried out in various places in Latvia over the years. The observations of Riga characterise the cities of Latvia, while Zosēni – rural areas in heightened relief.

21. Table 21 Average and the Highest 0°C Temperature Depth in Soil. Zero temperature depth in the soil has been calculated by using the results of the observations regarding the temperature depth until 3.2 m in the WMO period of 30 years (1961 - 1990). The referred to thermometers have been located under the natural ground covering surface (in summer – grass, in winter – sheet of snow). The Interpolation Method has been used for the calculation of temperature.

Minister for Environmental Protection

and Regional Development 			V. Makarovs

* Ground frigidity has been marked as less than 50% of the years in the specific month.