Republic of Latvia

Cabinet

Regulation No. 483

Adopted 20 June 2006

# Regulations regarding Electromagnetic Compatibility of Equipment

Issued pursuant to

Section 7

of the Law on Conformity Assessment

1. General Provisions

1. These Regulations prescribe the requirements for electromagnetic compatibility, which shall be applied to all electrical and electronic equipment (hereinafter – equipment) placed on the market, which may create electromagnetic disturbances or the operation of which may be affected by such disturbances, as well as prescribe the procedures for conformity assessment of the equipment and for market supervision, and market supervision authorities.

2. Equipment shall be any of the following apparatus or fixed installation:

2.1. apparatus - any finished appliance or combination thereof made commercially available as a single functional unit, intended for the user and liable to generate electromagnetic disturbance, or the performance of which is liable to be affected by such disturbance; and,

2.2. fixed installation - a particular combination of several types of apparatus and other devices which are assembled, installed and intended to be used permanently at a predefined location.

3. Electromagnetic compatibility shall be the ability of equipment to function satisfactorily in its electromagnetic environment, without introducing intolerable electromagnetic disturbances to other equipment in that environment.

4. Electromagnetic disturbances shall be any electromagnetic phenomenon that may degrade the performance of equipment. Electromagnetic disturbances may be electromagnetic noise, an unwanted signal or changes in the electromagnetic propagation medium itself.

5. These Regulations shall not apply to:

5.1. radio equipment, electronic communication terminal equipment and aeronautical equipment, for which special requirements for electromagnetic compatibility have been prescribed by regulatory enactments;

5.2. radio equipment used by radio amateurs, unless such radio equipment is placed on the market; or,

5.3. equipment, the inherent nature of the physical characteristics of which is incapable of generating or contributing to electromagnetic emissions which exceed a level allowing the operation of radio equipment, electronic communication equipment or other equipment, and it is not possible to noticeably degrade its operation if, when using such equipment, electromagnetic disturbances are created.

6. Equipment shall be designed and manufactured in order to ensure that:

6.1. their generated electromagnetic disturbances shall not disturb the steady operation of radio equipment, electronic communication equipment and other equipment, as defined by the manufacturers thereof; and,

6.2. they are able to operate without noticeable degradation in the presence of the electromagnetic disturbance that is normally consequent upon its intended use.

7. The Ministry of Economics in co-operation with the relevant technical committee for standardisation shall recommend to the limited liability company Latvijas standarts [Latvian Standard] a list of standards to be adapted.

8. The limited liability company Latvijas standarts shall submit for publication in the newspaper Latvijas Vēstnesis [the official Gazette of the Government of Latvia] a list of those harmonised standards of the European Union adapted in the status of Latvian national standards, which may be applied for the implementation of the requirements prescribed by these Regulations (hereinafter – applicable standards).

9. It shall only be permitted to place on the market and put into service such equipment that conforms to the requirements prescribed by these Regulations, if these have been installed and are utilised according to the intended purposes.

10. It shall be only permitted to exhibit equipment which does not conform to the requirements of these Regulations, at trade fairs, exhibitions and similar events if there is a clearly visible sign clearly indicating that the equipment shall not be placed on the market or put into service until conformity to the requirements of these Regulations has been ensured. Equipment may only be demonstrated if appropriate measures have been performed to eliminate electromagnetic disturbances.

11. The Consumer Rights Protection Centre shall perform the functions of market supervision (hereinafter – market supervision authority).

12. Conformity of the electromagnetic compatibility of equipment shall be assessed by certifying bodies that are accredited by the State Agency “Latvijas Nacionālais akreditācijas birojs’’ [Latvian National Accreditation Bureau] (hereinafter – accreditation bureau) according to the standard LVS EN 45011:2004 “General requirements for bodies operating product certification systems”, or other notified bodies of European Union or European Economic Area countries, (hereinafter – notified bodies), regarding the accreditation of which the Ministry of Economics has published a notification in the newspaper “Latvijas Vēstnesis”.

13. A notified body shall conform to the following minimum requirements:

13.1. it shall have personnel and technical facilities at its disposal for the performance of technical and administrative tasks, that are associated with the procedures for testing electromagnetic compatibility;

13.2. the personnel thereof shall have the relevant professional and technical qualifications for the performance of testing procedures, as well as experience in the preparation of reports, notifications and certificates;

13.3. the head and personnel thereof that are responsible for the conformity assessment of electromagnetic compatibility and procedures for the certification of conformity, shall not be the designer, manufacturer, supplier or installer of the equipment to be tested, or an authorised representative of the referred to persons. This prohibition does not exclude the opportunity of exchanging technical information between the manufacturer and the notified body;

13.4. the remuneration of its personnel shall not depend on the quantity and results of inspections or tests;

13.5. the personnel thereof shall observe confidentiality in relation to information acquired by implementing tasks of employment, except information that following a substantiated request is provided to the market supervision authority; and,

13.6. it shall ensure the civil liability insurance thereof in relation to activities that it is authorised to perform.

14. The accreditation bureau, which has assessed the competence of the notified body, shall inspect the implementation of the requirements prescribed in Paragraph 13 of these Regulations.

2. Requirements for Apparatus and Conformity Assessment Thereof

15. Apparatus shall be presumed as compliant with the requirements of electromagnetic compatibility if its manufacturer or the authorised representative thereof has proved that the apparatus is designed and manufactured according to the requirements prescribed in Paragraph 6 of these Regulations.

16. The manufacturer shall append to the apparatus the instructions for use of the apparatus, in which the following information shall be included:

16.1. the intended purpose of use of the apparatus;

16.2. the given name and surname of the manufacturer or the merchant firm and address, or the given name and surname of the authorised representative of the manufacturer, or the merchant firm and address;

16.3. apparatus identification (type, batch, serial number or any other information allowing for the identification of the apparatus);

16.4. all specific precautions that shall be observed when the apparatus is assembled, installed and used or when performing maintenance thereof, in order to ensure the conformity of the apparatus with the requirements prescribed in Paragraph 6 of these Regulations; and,

16.5. a clear indication regarding restriction of use, if the apparatus conformity with the requirements prescribed in Paragraph 6 of these Regulations is not ensured in residential areas. In such case, such indication shall also be necessary on the apparatus and packaging thereof.

2.1. Internal Control of Manufacture

17. The manufacturer, when manufacturing apparatus, shall assess its conformity with electromagnetic compatibility according to the requirements prescribed in Paragraph 6 of these Regulations. Correct application of the relevant applicable standards shall ensure apparatus conformity with the requirements prescribed in Paragraph 6 of these Regulations and shall be equivalent to an assessment of the electromagnetic compatibility of the apparatus.

18. If the manufacturer does not utilise the relevant applicable standards or utilises these incompletely, a conformity assessment of the apparatus shall be necessary for all its intended configurations, taking into account the intended operating conditions of the apparatus.

19. In order to certify conformity of the apparatus with the requirements of these Regulations, the manufacturer shall develop technical documentation according to the requirements prescribed in Paragraph 20 of these Regulations.

20. The technical documentation shall include:

20.1. a general description of the apparatus;

20.2. attestation by the manufacturer regarding conformity of the apparatus with the applicable standards and a list thereof if these are utilised completely or partially;

20.3. descriptions and accounts of all measures that have been performed in order to comply with the requirements prescribed in Paragraph 6 of these Regulations (including a description of the assessment of electromagnetic compatibility, results of design calculations, information regarding tests performed), if the manufacturer has not utilised the applicable standards or utilised these partially; and,

20.4. the certificate issued by the notified body, if conformity of the apparatus has been assessed by a notified body.

21. In order to certify conformity of the apparatus with the requirements of these Regulations, the manufacturer or its authorised representative shall draw up an EC declaration of conformity according to the requirements prescribed in Paragraph 22 of these Regulations.

22. The EC declaration of conformity shall include the following information:

22.1. a reference to these Regulations and the regulatory enactments with whose requirements the apparatus is in conformity with;

22.2. apparatus identification (type, batch, serial number or any other information allowing for the identification of the apparatus);

22.3. the given name and surname of the manufacturer or the merchant firm and address, or the given name and surname of the authorised representative of the manufacturer, or the merchant firm and address;

22.4. a dated reference to the characteristics in accordance with which the manufacturer has declared conformity of the apparatus with the requirements of these Regulations;

22.5. the date of drawing up of the EC declaration of conformity; and,

22.6. the given name, surname and signature of the manufacturer or authorised representative thereof.

23. The manufacturer or its authorised representative shall keep the technical documentation and the EC declaration of conformity for 10 years following the manufacture of the apparatus and, if necessary, present these to the market supervision authority. If the manufacturer or its authorised representative does not perform commercial activities in the European Union Member States and European Economic Area states, the technical documentation and EC declaration of conformity shall be kept and presented to the market supervision authority by the person that has placed the apparatus on the market.

2.2. Conformity Assessment of Apparatus by a Notified Body

24. If the manufacturer or its authorised representative prior to the apparatus being placed on the market wishes a notified body to perform a conformity assessment of the apparatus, the manufacturer or its authorised representative shall hand over the apparatus and its technical documentation to the disposal of the notified body. In order to test the electromagnetic compatibility of the apparatus, it shall be indicated which essential requirements are to be assessed by the notified body.

25. The notified body according to the essential requirements indicated by the manufacturer or its authorised representative shall:

25.1. verify the technical documentation of the apparatus and determine whether the apparatus has been manufactured in accordance with the documentation submitted and the requirements of these Regulations, identify components in the design of which applicable standards have been used, and the components in the design of which the applicable standards have not been used;

25.2. if the applicable standards have not been used, the electromagnetic compatibility shall be assessed or its assessment shall be ensured, in order to establish whether the apparatus conforms to the requirements prescribed in Paragraph 6 of these Regulations; and,

25.3. perform the inspections and tests or ensure the performance thereof, in order to determine whether the manufacturer has used the applicable standards, the utilisation of which has been notified by the manufacturer.

26. If the apparatus conforms to the requirements of these Regulations, the notified body shall issue to the manufacturer or its authorised representative a certificate of apparatus conformity according to the assessment.

27. If the notified body takes the decision not to issue a certificate of conformity, it shall provide a written substantiated refusal to the manufacturer or its authorised representative. The decision of a notified body may be disputed at the Ministry of Economics according to the procedures prescribed by the Administrative Procedure Law.

3. Marking of Apparatus

28. Apparatus which conforms to the requirements of these Regulations, shall be marked by the manufacturer with the CE marking (Annex) according to the requirements prescribed in Paragraphs 29 and 30 of these Regulations.

29. If the apparatus shall also conform to other requirements prescribed by regulatory enactments in relation to the CE marking, the apparatus may be marked with the CE marking only if it also conforms to these requirements. The CE marking shall indicate the conformity only to those regulatory enactments that have been applied by the manufacturer of the apparatus.

30. The CE marking shall be affixed to the apparatus or its descriptive plate, but if this is not possible, on the packaging and with its accompanying documents.

31. Any other marking may be affixed to the apparatus, its packaging or accompanying documents that does not degrade the visibility and legibility of the CE marking, but markings which are identical to the CE marking and which could be misleading as to the meaning and form of the CE marking may not be placed thereto.

4. Installation of Fixed Installations

32. Fixed installations shall be installed utilising good engineering practice which shall be documented utilising the components of fixed installations that conform to the requirements of these Regulations, and observing precisely the manufacturer instructions for installation components (apparatus and devices).

33. All the requirements prescribed by these Regulations relating to apparatus shall be applied to apparatus which are placed on the market and which may be incorporated into a fixed installation.

34. If apparatus intended for incorporation into a fixed installation is not otherwise commercially available, the requirements prescribed in Paragraph 6, Sub-chapter 2.1 and Chapter 3 of these Regulations in relation to the apparatus shall not be compulsory. In such cases the accompanying documentation shall identify the fixed installation and its electromagnetic compatibility characteristics, as well as:

34.1. identification of each apparatus (type, batch, serial number or other information allowing for the identification of the apparatus);

34.2. the accompanying documents of the apparatus shall indicate the given name and surname of the manufacturer or the merchant firm and address, or the given name and surname of the authorised representative of the manufacturer, or the merchant firm and address; and,

34.3. the precautionary measures that shall be performed in order to incorporate the apparatus into the fixed installation, without compromising the conformity of that installation.

35. Fixed installations shall be presumed as conforming to the requirements of electromagnetic compatibility if they conform to the requirements prescribed in Paragraphs 6 and 32 of these Regulations.

36. In order to certify the conformity of the fixed installation with the requirements of these Regulations, its installer or the authorised representative thereof shall draw up the technical documentation for the fixed installation according to the requirements prescribed in Paragraph 38 of these Regulations.

37. The technical documentation shall include:

37.1. a general description of the fixed installation in which information is indicated regarding the surrounding environment of the fixed installation, requirements for additional auxiliary devices (for example, protection devices, filters), the prescribed length of cabling for ensuring external communications, the length of use of the fixed installation and special precautionary measures for electromagnetic compatibility (for example, safety earthing);

37.2. a description and account of all measures that have been performed in order to observe the requirements prescribed in Paragraph 6 of these Regulations, including a description of the electromagnetic compatibility assessment, results of design calculations, information regarding testing performed;

37.3. documentation of the good engineering practices utilised; and,

37.4. the accompanying documents specified in Paragraph 34 of these Regulations, if the apparatus specified in Paragraph 34 of these Regulations is utilised in a fixed installation.

38. A person that has a fixed installation in his or her possession (hereinafter – possessor) shall keep the technical documentation for as long as the fixed installation is in operation and, if necessary, shall present it to the market supervision authority.

5. Market Supervision

39. If the market supervision authority determines that the apparatus does not comply with the requirements prescribed by these Regulations, the market supervision authority shall perform all the measures provided for in the regulatory enactments regulating market supervision, in order to withdraw the apparatus from the market, prohibit its trade or restrict the circulation thereof, and shall inform the Ministry of Economics regarding the determination of non-compliant apparatus, indicating the reasons for non-compliance:

39.1. non-compliance with the requirements included in Paragraph 6 of these Regulations because the requirements of the applicable standards are not being observed;

39.2. incorrect utilisation of the applicable standards;

39.3. shortcomings in the requirements prescribed by the applicable standards; or

39.4. other reasons for non-compliance, if the applicable standards are not used.

40. The Ministry of Economics shall inform the European Commission and European Union Member States regarding determined cases of non-compliant apparatus and the measures performed, attaching the report of the market supervision authority regarding the reasons for non-compliance.

41. The market supervision authority shall inform the manufacturer or its authorised representative regarding the withdrawal of the non-compliant apparatus from the market and the prohibition of its free circulation, indicating the reasons for the taking of the decision and the procedures for disputing and appealing the decision.

42. The manufacturer or its authorised representative has the right to express its opinion regarding the non-compliance of apparatus with the requirements of these Regulations, prior to the market supervision authority taking a decision regarding the withdrawal of the apparatus from the market and prohibition of the free circulation thereof, except for the cases if the decision is to be taken urgently for reasons of consumer protection.

43. The market supervision authority may request evidence regarding the conformity of a fixed installation and initiate conformity assessment by the notified body, if the fixed installation features indicate that it does not conform with the requirements of these Regulations, or if complaints have been received regarding disturbances in the operation of the fixed installation.

44. If the market supervision authority determines non-compliance of a fixed installation, it shall perform measures according to the procedures prescribed by the regulatory enactments regulating market supervision, in order to ensure the compliance of the fixed installation with the requirements prescribed in Paragraph 6 of these Regulations.

6. Closing Provisions

45. Cabinet Regulation No. 188 of 30 May 2000, Regulations Regarding Electromagnetic Compatibility of Equipment (Latvijas Vēstnesis, 2000, No. 201/204) is repealed.

46. After coming into force of these Regulations it shall be permitted to put into service and trade freely the equipment that conforms with Cabinet Regulation No.188 of 30 May 2000, Regulations Regarding Electromagnetic Compatibility of Equipment and that have been placed on the market until 20 July 2009.

47. These Regulations shall come into force on 20 July 2007.

Informative Reference to European Union Directive

These Regulations contain legal norms arising from Directive 2004/108/EC of the European Parliament and of the Council of 15 December 2004 on the approximation of the Laws of the Member States relating to electromagnetic compatibility and repealing Directive 89/336/EEC.

Prime Minister 				A. Kalvītis

Acting for the Minister for Economics,

Minister for Finance 				O. Spurdziņš

Annex

Cabinet Regulation No. 483

20 June 2006

.

## The CE Mark of Conformity

1. The CE marking which certifies the conformity of the equipment with the requirements of electromagnetic compatibility shall consist of the capital letters “CE”.

2. If the size of the CE marking is being reduced or enlarged, the proportions given by the sample shall be observed.

3. The vertical size of the marking shall not be less than 5 mm.

Acting for the Minister for Economics,

Minister for Finance 				O. Spurdziņš