Republic of Latvia

Cabinet

Regulation No. 826

Adopted 3 October 2006

Procedures for Ensuring Operation of the Coastal Communications Network of the Automatic Identification System (AIS) within the Scope of the Vessel Traffic Monitoring and Information Data Exchange System

Issued pursuant to

Section 45, Paragraph three of the

Maritime Administration and

Maritime Safety Law

I. General Provisions

1. These Regulations prescribe the procedures, by which the operation of the coastal communications network of the Automatic Identification System (AIS) is ensured within the scope of the vessel traffic monitoring and information data exchange system.

2. The vessel traffic monitoring and information data exchange system shall be interoperable with the European Community maritime information exchange system (established in order to increase the maritime safety and efficiency by improving the readiness of institutions to react to incidents and accidents at sea (including search and rescue operations) and to promote more efficient prevention and detection of the pollution caused by vessels).

II. Operation of the Coastal Communications Network of the Automatic Identification System

3. Operation of the coastal communications network of the Automatic Identification System shall be ensured by the Maritime Rescue Co-ordination Centre (MRCC) of the Coast Guard Service of the Navy of the National Armed Forces (hereinafter – Co-ordination Centre). The Co-ordination Centre shall be the centre for the receipt, storage, exchange and distribution of information of the Automatic Identification System of Latvia.

4. In utilisation of data of the Automatic Identification System the Co-ordination Centre shall comply with the requirements, which have been specified in the regulatory enactments in the field of transport and communications, the requirements of the International Maritime Organisation Resolution A.851(20), General principles for ship reporting systems and ship reporting requirements, including guidelines for reporting incidents involving dangerous goods, harmful substances and/or marine pollutants, Resolution A.917(22), Guidelines for the onboard operational use of shipborne automatic identification systems (AIS) (including amendments), Resolution A.949(23), Guidelines on places of refuge for ships in need of assistance, and Resolution A.950(23), Maritime assistance services (MAS), as well as recommendations of the International Association of Lighthouse Authorities and the requirements of regulatory enactments issued by the International Hydrographic Organization.

5. The Automatic Identification System of Latvia shall ensure exchange of data with automatic identification systems of other states in accordance with these Regulations and international agreements binding to Latvia.

III. Information of the Automatic Identification System

6. Information of the Automatic Identification System shall consist of online data and statistical data:

6.1. online data – data of the Automatic Identification System of the Helsinki Commission established for implementation of the requirements specified in the 1992 Convention on the Protection of the Marine Environment of the Baltic Sea Area (hereinafter – Helsinki Commission) and data of the Automatic Identification System of Latvia in accordance with the regulatory enactments referred to in Paragraph 4 of these Regulations. Data of the Automatic Identification System of Latvia shall be data, which reflect the movement of ships in the detection zone of the coastal network of the Automatic Identification System; and

6.2. statistical data – accounting of the movement of ships in the Baltic Sea according to the ship types pursuant to the contract On Access to AIS Information entered between and by the Navy of Latvia and the Royal Danish Administration of Navigation and Hydrography.

7. The Co-ordination Centre, in accordance with the competence thereof, shall use online data of the Automatic Identification System and ensure free availability thereof to the following institutions:

7.1. the State Environmental Service;

7.2. the State joint stock company "Latvijas Jūras administrācija" [Latvian Maritime Administration] (hereinafter – Latvian Maritime Administration);

7.3. the service of the harbour master;

7.4. the State Border Guard;

7.5. the customs authorities of the State Revenue Service; and

7.6. the State Fire-Fighting and Rescue Service.

8. The institutions referred to in Paragraph 7 of these Regulations shall use online data of the Automatic Identification System only according to the competence thereof. It is prohibited to distribute the referred to data to third persons.

9. The institutions referred to in Paragraph 7 of these Regulations shall be allowed to send reports to an individual ship, a group of ships or ships in a specific geographic area, using a direct connection to the server of the Automatic Identification System.

10. The Co-ordination Centre shall determine limited access to online data of the Automatic Identification System (filtering of data) and allow the usage of the data of the Automatic Identification System of the Helsinki Commission and the data of the Automatic Identification System of Latvia to the following persons:

10.1. the port authority – regarding vessels entering and leaving the relevant port;

10.2. the operator of a ship, as well as any person who is authorised to provide information in the name of the operator of the ship (hereinafter – agent) – regarding ships in the Baltic Sea area, which are serviced by the particular operator of the ship or agent; and

10.3. the owner of a ship – regarding ships of an owner in the detection zone of the coastal networks of the Automatic Identification Systems of European Union Member States.

11. The persons referred to in Paragraph 10 of these Regulations are prohibited to send reports to ships, using the Automatic Identification System.

IV. Exchange of Data of the Automatic Identification System among Member States of the Helsinki Commission

12. The Co-ordination Centre shall perform the exchange of data among the member states of the Helsinki Commission in accordance with the contract On Access to AIS Information entered into and by the Navy of Latvia and the Royal Danish Administration of Navigation and Hydrography.

13. The Ministry of Defence, the Ministry of Transport, the Ministry of Environment, the Ministry of the Interior, the Ministry of Finance and the Ministry of Economics, in applying as users, shall receive monthly or weekly statistical information of the Automatic Identification System from the Royal Danish Administration of Navigation and Hydrography.

V. Exchange of Data in the European Community Maritime Information Exchange System

14. A vessel traffic monitoring and information data exchange system (hereinafter – national system) has been established in Latvia for the processing of maritime information and it shall be interoperable with the European Community maritime information exchange system (hereinafter – SafeSeaNet system).

15. Within the meaning of these Regulations, maritime information shall be reports, warnings and information in accordance with the Annex to these Regulations.

16. The national competent authority and local competent authorities shall be the institutions of Latvia involved in the SafeSeaNet system.

17. The Co-ordination Centre shall be the national competent authority of Latvia. The Co-ordination Centre shall:

17.1. ensure the compliance of the national system and the information entered therein with the requirements specified in the latest revision of the Interface Control Document (hereinafter – Interface Control Document) drawn up in accordance with the conditions of Clause 2 of Annex III to Directive 2002/59/EC of the European Parliament and of the Council of 27 June 2002 establishing a Community vessel traffic monitoring and information system and repealing Council Directive 93/75/EEC; and

17.2. ensure and maintain the access rights of local competent authorities to the network of the national system in accordance with the procedures specified in Paragraph 28 and the Annex to these Regulations.

18. The Co-ordination Centre shall establish and maintain an infrastructure in order to ensure forwarding, detection and conversion of data among systems, which use XML or EDIFACT syntax and are based on the Internet or X.400 communication equipment.

19. The national system shall ensure an opportunity to use the collected information for operational purposes, and it shall meet the following conditions:

19.1. data exchange takes place electronically, allowing to receive and process maritime information;

19.2. exchange of information is ensured round the clock; and

19.3. it is possible to send maritime information to a competent authority of another European Union Member State upon the request without delay.

20. The Co-ordination Centre shall ensure interoperability of the national system with the SafeSeaNet system.

21. The Ministry of Transport shall be the institution responsible for co-ordination of the development of the national system.

22. Within the meaning of the SafeSeaNet system, local competent authorities shall be:

22.1. the port authority (including service of the harbour master);

22.2. the Latvian Maritime Administration; and

22.3. the State Environmental Service.

23. Users of the national system and the SafeSeaNet system with a limited access right shall be:

23.1. the master, owner or operator of a ship or – pursuant to authorisation – agent;

23.2. the State Border Guard;

23.3. the customs authorities of the State Revenue Service;

23.4. the State Fire-fighting and Rescue Service; and

23.5. the Safety Police.

24. Access rights to the data of the national system and the SafeSeaNet system shall be determined to the persons referred to in Paragraphs 22 and 23 of these Regulations in accordance with the Annex to these Regulations.

25. In using the access to the national system provided by the national competent authority, a master, an owner or an operator of a ship shall provide defence information in electronic form in accordance with Chapter XI-2, Regulation 9, Sub-paragraph 2.1 of the International Convention for the Safety of Life at Sea, 1974, and protocols thereof of 1978 and 1988, as well as the following information pursuant to the requirements, which have been specified in the following regulatory enactments:

25.1. regarding formalities, which are related to ships entering and leaving a port – only general information regarding the vessel;

25.2. regarding reports on dangerous and polluting cargoes of vessels – general information and information regarding cargo; and

25.3. regarding acceptance of waste created by and water polluted by ships and plans for the management of waste created by ships – information, which is included in the waste declaration of the ship.

26. A master, owner or operator of a ship may authorise an agent for entering of information in the national system. Persons who enter information in the national system in accordance with Paragraphs 25 and 26 of these Regulations shall ensure the veracity of the information.

27. Warnings shall be entered in the national system by:

27.1. the Co-ordination Centre – warnings regarding incidents (SITREP), warnings regarding pollution of the maritime environment (POLREP), as well as warnings regarding drifting objects (for example, containers), which cause a threat to maritime safety; and

27.2. a port authority (including service of the harbour master) – warnings regarding the fact that a ship has not transferred waste in the port, and other warnings according to the competence.

28. Local competent authorities, as well as persons who enter information in the national system in accordance with Paragraphs 25 and 26 of these Regulations shall request the national competent authority to provide the necessary access rights for entering and supplementing of data.

29. If sending of information in electronic form fails five times, a sender of the report shall inform a recipient of the report, using any other available means (telephone, fax).

30. Local competent authorities shall ensure forwarding of information to the national competent authority in accordance with the requirements of the Interface Control Document.

VI. Closing Provisions

31. Paragraph 10 of these Regulations shall come into force on 1 September 2007.

32. Paragraphs 24, 25, 26, 27, 28, 29 and 30 of these Regulations shall come into force on 1 January 2008.

Informative Reference to European Union Directive

These Regulations contain legal norms arising from Directive 2002/59/EC of the European Parliament and of the Council of 27 June 2002 establishing a Community vessel traffic monitoring and information system and repealing Council Directive 93/75/EEC.

Prime Minister				A. Kalvītis

Minister for Transport

K. K. Peters

Annex

Cabinet Regulation No. 826

3 October 2006

Access Rights of Users of the National System for the Management of Maritime Information and the SeaSafeNet System

| Users of the system | Reports (Internet/XML) |  |  |  |  |  |  |  |  | Requests for information (Internet/XML) |  |  |  |  |  |  |  |  |  | Requests for information (Internet only) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | details of reports |  |  |  |  |  |  |  |  |  | search of vessels |  |  |  | search or ports |
|  | port | ship | dangerous cargoes | ISPS safety | warnings |  |  |  |  | port | ship |  | dangerous cargoes | ISPS safety | warnings |  |  |  |  | last report | voyage | cargo manifest | last incidents |  |
|  |  |  |  |  | regarding incidents (SITREP) | regarding pollution of the maritime environment (POLREP) | regarding the fact that the ship has not transferred waste at the port | regarding drifting objects (for example, containers, which may cause threats to the maritime safety) | other |  | mandatory ship reports (MRS) | AIS |  |  | regarding incidents (SITREP) | regarding pollution of the maritime environment (POLREP) | regarding the fact that the ship has not transferred waste at the port | regarding drifting objects (for example, containers, which may cause threats to the maritime safety) | other |  |  |  |  |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 |
| Port authority* | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y1 | Y1 | Y1 | Y1 | Y | N | N | Y | N | N | Y1 | Y1 | Y1 | Y | Y1 |
| Latvian Maritime Administration | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y |
| State Environmental Service | N | N | N | N | Y | Y | Y | Y | Y | Y | Y | Y | Y | N | N | Y | Y | N | Y | Y | Y | Y | Y | Y |
| Maritime Rescue Co-ordination Centre | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y |
| Customs authorities of the State Revenue Service | N2 | N2 | N2 | N2 | N2 | N2 | N2 | N2 | N2 | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y | N | Y | Y |
| State Border Guard | N2 | N2 | N2 | N2 | N2 | N2 | N2 | N2 | N2 | Y | N | Y | N | N | N | N | N | N | N | Y | Y | N | Y | Y |
| State Fire-fighting and Rescue Service | N2 | N2 | N2 | N2 | N2 | N2 | N2 | N2 | N2 | Y | N | Y | N | Y | Y | Y | Y | Y | Y | Y | Y | N | Y | Y |
| Security Police | N | N | N | N | N | N | N | N | N | N | N | Y | Y | Y | N | N | N | N | N | N | N | N | N | N |
| Agent** | N | N | N | N | N | N | N | N | N | Y | N | Y/N3 | N | N | N | N | N | N | N | N | N | N | N | N |

Notes.

1. Y – access rights to the data of the national system for the management of maritime information and the SeaSafeNet system have been granted.

2. N – access rights to the data of the national system for the management of maritime information and the SeaSafeNet system have not been granted.

3. 1 If the port is the next port of entry.

4. 2 The national competent authority (MRCC) shall take a case-by-case decision regarding access, taking into account the requirements specified in the Interface Control Document.

5. 3 Limited data of the Automatic Identification System are available in accordance with Paragraph 10 of these Regulations.

6. * Port authority (including service of the harbour master).

7. ** The master, owner or operator of the ship or – pursuant to authorisation – agent.

Minister for Transport				 K. Peters