Republic of Latvia

Cabinet

Regulation No. 40

Adopted 13 January 2009

Regulations Regarding Energy Certification of Buildings

Issued pursuant to

Section 8, Section 10, Paragraph two,

Section 15 and 19 of

Law on the Energy Performance of Buildings

I. General Provisions

1. These Regulations prescribe:

1.1. the minimum requirements of the energy performance for buildings to be reconstructed, the total area of which is more than 1 000 square metres and the total reconstruction costs of which exceed 25% from the cadastral value of the relevant buildings or the reconstruction works of which affect 25% from the scale of construction (hereinafter – a building to be reconstructed), as well as for new buildings;

1.2. the procedures by which the energy certification of buildings shall be carried out;

1.3. the type, sample, content, the procedures for the issue and registration of the energy certificate;

1.4. the procedures for supervision of the energy certification;

1.5. the procedures, by which the information regarding the issued energy certificates of buildings shall be compiled, updated and used; and

1.6. the procedures for the inspection of boilers, the rated output of which is more than 20 kW (hereinafter – boilers), and of air-conditioning systems, the rated output of which is more than 12 kW (hereinafter – air-conditioning systems).

2. The minimum requirements of the energy performance for a building determined in regulatory enactments in the field of construction and in the relevant standards shall be applied for:

2.1. overall thermotechnical parameters for the architectural elements of external envelopes of buildings to be reconstructed and new buildings; and

2.2. technical systems (for example, heating, cooling, water supply, ventilation, air-conditioning, lighting systems) of buildings to be reconstructed and new buildings.

3. Energy consumptive technical systems the power of which exceeds the need intended in the design of the relevant building may not be installed or added in the building for the provision of the functions thereof, except cases when the installation of the relevant systems is technically and economically justified.

4. The minimum requirements of the energy performance for buildings to be reconstructed specified in regulatory enactments regulating the field of construction and in the relevant standards shall be applied so far as it is technically, functionally and economically possible.

II. Energy Certification of Buildings

5. In order to perform energy certification of a building, an energy auditor shall:

5.1. inspect the output data quality necessary for the calculations and determine the applicability thereof;

5.2. perform measurements and evaluation of measurements;

5.3. calculate the energy performance indicators using the methodology of calculation for the energy performance which has been developed in accordance with Section 9, Paragraph two of the Law On the Energy Performance of Buildings;

5.4. issue a certificate in which the energy performance of the building is evaluated; and

5.5. prepare recommendations for improvement of the energy performance of a building

6. Energy certification of present buildings shall be performed in accordance with the methodology of calculation for the energy performance of buildings evaluating the calculated energy performance and measured energy performance.

7. Energy certification of buildings to be designed (a new building and a building to be reconstructed) shall be performed evaluating the calculated energy performance.

8. Energy performance of a building shall be expressed by total annual indicators:

8.1. for energy consumption – kilowatt-hours per square meter per year; and

8.2. for evaluation of carbon dioxide emission – kilograms of carbon dioxide per square meter per year.

9. A mutual agreement shall determine the regulation of the relationship between an energy auditor and the customer of an energy audit.

10. In order that an energy auditor could evaluate energy performance of a building, the customer of an energy audit of a building shall ensure access to the following documents regarding the building:

10.1. a building design or inventory plan;

10.2. designs of technical systems or layout drawings of the building;

10.3. instructions for technical systems of the building;

10.4. exploitation and maintenance log-books or reports of technical systems of the building;

10.5. actual purpose of use of premises and temperature in the premises;

10.6. bills of providers of public utilities regarding the amount of consumed energy in the building or technical systems of the building;

10.7. data regarding putting into service of the building;

10.8. data regarding installation or change of technical systems of a building during the exploitation of the building;

10.9. energy calculations, if any;

10.10. opinions of previous inspections, if any;

10.11. entry in the premises and access to technical systems of the building; and

10.12. other necessary information related with the successful process of the energy audit of the building and inspection of a boiler or air-conditioning system.

11. If a customer can not ensure the documents or information referred to Paragraph of these Regulations for an energy audit, which is necessary for evaluation of the energy performance of a building, the customer shall ensure alternative possibilities for the energy auditor for obtaining of information or agree on another type for obtaining of necessary information.

III. Energy Certificate of a Building

12. The energy certificate of a building shall be prepared for existing buildings in accordance with Annex 1 to these Regulations, but a temporary energy certificate for buildings to be designed (new buildings and buildings to be reconstructed) – in accordance with Annex 2 to these Regulations.

13. Energy certificate of a building shall be appended by:

13.1. an annex in which the values of the input date used for calculations are indicated specifying the type for acquiring of the data and data source:

13.1.1. areas of premises and zones and temperature therein;

13.1.2. areas of building envelopes and heat transmission coefficients;

13.1.3. length of thermal bridges and heat transmission coefficients;

13.1.4. consumption of accounted energy and energy carriers indicating periods and distribution by technical systems;

13.1.5. the values used in the evaluation of the technical systems utilised in the building;

13.1.6. the values accepted in order to evaluate the factors affecting the evaluation of energy performance of the building;

13.1.7. the coefficients used for correction of weather conditions and adjustment of calculations;

13.1.8. plausibility interval of evaluation of energy performance, if any specified;

13.2. inspection reports of the boiler and air-conditioning system; and

13.3. recommendations for improvement of energy performance of the building (Annex 3).

14. The Annex shall be appended to the temporary energy certificate of a building. The input data values used for the calculations and specified in Sub-paragraphs 13.1.1, 13.1.2, 13.1.3, 13.1.5 and 13.1.6 of these Regulations shall be indicated in the Annex.

15. An energy certificate of a building or a temporary energy certificate of a building shall be issued in one of the following forms:

15.1. in the form of a printed document;

15.2. in the form of an electronic document; or

15.3. drawing up a document on the construction information system.

16. An energy certificate of a building issued in the form of a printed document or electronic document, shall be submitted by an energy auditor for registration in the construction information system within five days after the issuance thereof.

IV. Supervision of the Energy Certification and Circulation of Information

17. The Ministry of Economics shall:

17.1. analyse the information regarding energy certification;

17.2. assess the operation of the energy certification system of buildings and affect to the public and national economy;

17.3. include in the programmes and plans of the Ministry the measures for improvement of the energy certification system of buildings;

17.4. develop proposals for improvement of regulatory enactments regulating the energy performance of buildings; and

17.5. ensure the supervision of the activities of the State Agency “Construction, Energy and Housing State Agency” (hereinafter – Agency) in the field of energy certification of buildings.

18. The Agency shall compile and technically maintain the information regarding energy certification of buildings pursuant to the regulatory enactments in the field of State information systems, construction and personal data protection.

19. The following data necessary for supervision of energy certification of buildings shall be compiled in the construction information system:

19.1. the data which have been included in the documents issued in accordance with these Regulations

19.1.1. the data in the energy certificate of a building;

19.1.2. the data in the temporary energy certificate of a building;

19.1.3. the data in boiler inspection report;

19.1.4. the data in the inspection report of an air-conditioning system; and

19.2. the data which personnel certification institutions issue to the Agency pursuant to the regulations issued in accordance with Section 20, Paragraph two of the Law on the Energy Performance of Buildings.

20. The Agency shall ensure:

20.1. the authorised access to a register of energy certificates of buildings in order that the energy auditors could prepare for issuance the energy certificates of buildings and the documents to be appended thereto;

20.2. compilation and updating of information regarding energy certificates of buildings which have been registered in the system for ensuring the supervision of energy certification of buildings;

20.3. public accessibility to the energy certificates of buildings free of charge on the Internet home page of the Agency;

20.4. publication of samples of documents to be prepared by the energy auditors of buildings on the Internet home page of the Agency; and

20.5. the supervision of energy auditors in accordance with the procedures specified in the regulatory enactments in the field of operation of energy auditors.

V. Inspection of Boilers and Air-conditioning Systems

21. A boiler and air-conditioning system shall be inspected during the energy certification of a building. The inspection may be ordered on an optional basis at any other time.

22. The customer of an inspection of a boiler or air-conditioning system shall ensure for an energy auditor the documentation of the system to be inspected (a design or survey scheme), previous inspection reports, notes or summaries made during the exploitation, as well as shall ensure access to the system to be inspected and the components thereof and the possibility to perform measurements.

23. During an inspection an energy auditor shall evaluate the performance of a boiler or air-conditioning system and provide recommendations for further exploitation of the boiler or air-conditioning system.

24. Boilers shall be inspected in accordance with the standard LVS EN 15378:2007 ”Heating systems in buildings - Inspection of boilers and heating systems”. An energy auditor shall draw up a report regarding inspection of a boiler (Annex 4).

25. Air-conditioning systems shall be inspected in accordance with the standard LVS EN 15240:2007 ”Ventilation for buildings - Energy performance of buildings - Guidelines for inspection of air-conditioning systems”. An energy auditor shall draw up a report regarding inspection of an air-conditioning system (Annex 5).

VI. Closing Provisions

26. Chapter V of these Regulations shall come into force on 1 March 2009.

27. Paragraph 19 and Sub-paragraphs 20.1, 20.2 and 20.3 of these Regulations shall come into force on 1 October 2009.

28. An energy auditor shall submit the energy certificate of a building and temporary energy certificate of a building, which has been issued in the form of a printed document or in the form of an electronic document by 1 October 2009, to the Agency for registration within five working days after the issuance thereof.

Prime Minister 			I. Godmanis

Minister for Economics 		K. Gerhards

Annex 1

Cabinet Regulation

No. 40

13 January 2009

Sample for an Energy Certificate of a Building

Page 1

| Energy certificate of a building Valid for 10 years |  |  | Depiction of the building (optional) |  |
| --- | --- | --- | --- | --- |
| building classification | In accordance with Section 9, Paragraph one of the Law on the Energy Performance of Buildings |  |  |  |
| ADDRESS of the BUILDING | Street, house, county, city, postal code |  |  |  |
| cadastre number of the building |  |  |  |  |
| part of the building | Indicate, if the part of the building has been assessed |  |  |  |
| Characterisation of the building |  |  |  |  |
| Year of acceptance for service |  | year of reconstruction |  |  |
| Number of Storeys | [ ] cellar, [ ] attic or garret-floor |  |  |  |
| Area |  |  |  |  |

| Purpose for energy certification of the building |  | [ ] sale, [ ] lease, [ ] on an optional basis |  |
| --- | --- | --- | --- |
| Evaluation of energy performance for the building |  |  |  |
| very good |  |  | very bad |

within a year

kgCO2/m2

kWh/m2 per year

| Issuer of energy certificate of the building |  |
| --- | --- |
| Energy auditor | Given name and surname |
| Certificate | Certificate registration number |
| Company | Name, registration number and address, if the energy auditor represents a company |
| Date* | Signature* |

Page 2

| Energy certificate of a building |  |  |
| --- | --- | --- |
| ADDRESS of the BUILDING | Street, house, county, city, postal code |  |
| cadastre number of the building |  |  |
| Owner of the building | Given name, surname and personal identity number of a natural person, or name and registration number of a legal person |  |
| Contact details | Telephone number and e-mail address |  |
| Address |  |  |
| energy audit CUSTOMER | Given name, surname and personal identity number of a natural person, or name and registration number of a legal person |  |
| Contact details | Telephone number and e-mail address |  |
| Address |  |  |
| Average heat transmission coefficient of THE building envelopes HT | _________ [W/m2 K] present _________ [W/m2 K] standard calculated in accordance with the Latvian Construction Standard LBN 002-01 Thermotechnics of Building Envelopes, (approved by Cabinet Regulation No 495 of 27 November 2001) |  |
| Distribution of energy consumption | total [MWh per year] | specific [kWh/m2 per year] |
| for heating |  |  |
| for cooling |  |  |
| for preparation of hot water |  |  |
| for lighting |  |  |
| for ventilation |  |  |
| _____________ |  |  |
| A list of the annexes and documents attached 1. _____________ 2. _____________ 3. _____________ 4. ... |  |  |

Explanations and designations

Evaluation of energy performance calculated for the building – evaluation of energy performance which is performed on the basis of the calculations of energy consumption for heating, cooling, ventilation, preparation of hot water and lighting of the building.

Evaluation of energy performance measured for the building – evaluation of energy performance which is performed on the basis of the measured amounts of the supplied and exported energy.

Evaluation of the calculated carbon dioxide (CO2) emission

Evaluation of the measured carbon dioxide (CO2) emission.

Carbon dioxide (CO2) originates in the combustion process of fossil fuel for the production of energy, including for heating, cooling, preparation of hot water and lighting of the building.

Minister for Economics 		K. Gerhards

Annex 2

Cabinet Regulation

No. 40

13 January 2009

Sample for a Temporary Energy Certificate of a Building

Page 1

| Sample for temporary energy certificate of a building Valid for 2 years |  |  | Depiction of the building (optional) |  |
| --- | --- | --- | --- | --- |
| building classification | In accordance with Section 9, Paragraph one of the Law on the Energy Performance of Buildings |  |  |  |
| ADDRESS of the BUILDING | Street, house, county, city, postal code |  |  |  |
| cadastre number of the building |  |  |  |  |
| part of the building | Indicate, if the part of the building has been assessed |  |  |  |
| Characterisation of the building |  |  |  |  |
| Year of acceptance for service |  | year of reconstruction |  |  |
| Number of Storeys | [ ] cellar, [ ] attic or garret-floor |  |  |  |
| Area |  |  |  |  |

| Purpose for energy certification of the building |  | [ ] new building, [ ] reconstruction |  |
| --- | --- | --- | --- |
| Evaluation of energy performance for the building |  |  |  |
| very good |  |  | very bad |

within a year

kgCO2/m2

kWh/m2 per year

| Issuer of energy certificate of the building |  |
| --- | --- |
| Energy auditor | Given name and surname |
| Certificate | Certificate registration number |
| Company | Name, registration number and address, if the energy auditor represents a company |
| Date* | Signature* |

Page 2

| for temporary energy certificate of a building |  |  |
| --- | --- | --- |
| ADDRESS of the BUILDING | Street, house, county, city, postal code |  |
| cadastre number of the building |  |  |
| Average heat transmission coefficient of building envelopes HT | _________ [W/m2 K] present _________ [W/m2 K] standard calculated in accordance with the Latvian Construction Standard LBN 002-01 Thermotechnics of Building Envelopes, (approved by Cabinet Regulation No 495 of 27 November 2001) |  |
| Distribution of energy consumption | total [MWh per year] | specific [kWh/m2 per year] |
| for heating |  |  |
| for cooling |  |  |
| for preparation of hot water |  |  |
| for lighting |  |  |
| for ventilation |  |  |
| _____________ |  |  |
| A list of the annexes and documents attached 1. _____________ 2. _____________ 3. _____________ 4. ... |  |  |

Explanations and designations

Evaluation of energy performance calculated for the building – evaluation of energy performance which is performed on the basis of the calculations of energy consumption for heating, cooling, ventilation, preparation of hot water and lighting of the building.

Evaluation of the calculated carbon dioxide (CO2) emission Carbon dioxide (CO2) originates in the combustion process of fossil fuel for the production of energy, including for heating, cooling, preparation of hot water and lighting of the building.

Minister for Economics 		K. Gerhards

Annex 3

Cabinet Regulation

No. 40

13 January 2009

Recommendations for Improvement of Energy Performance of a Building

| building classification | In accordance with Section 9, Paragraph one of the Law on the Energy Performance of Buildings |
| --- | --- |
| ADDRESS of the BUILDING | Street, house, county, city, postal code |
| cadastre number of the building |  |
| part of the building | Indicate, if applies for a part of the building |

| Measure | Saving of supplied energy [kWh/m2 per year] | Saving of primary energy [kWh/m2 per year] | Reduction of carbon dioxide emissions [kg/m2 per year] |
| --- | --- | --- | --- |

| issuer of recommendations for improvement of energy performance of the building |  |
| --- | --- |
| Energy auditor | Given name and surname |
| Certificate | Certificate registration number |
| Company | Name, registration number and address, if the energy auditor represents a company |
| Date* | Signature* |

Minister for Economics 		K. Gerhards

Annex 4

Cabinet Regulation

No. 40

13 January 2009

Boiler Inspection Report

| Address of the building |  | Street, apartment, house, county, city, postal code |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Cadastre number of the building |  |  |  |  |  |
| Boiler |  |  |  |  |  |
| Model |  |  | Batch number |  | Year |
| Type of heating fuel |  |  |  |  |  |
| Function | [ ] for heating (heating area _____ m2 and capacity _____ m3), [ ] for hot water heating, [ ] other ____________ |  |  |  |  |
| Output power | maximum _________ kW, minimum __________ kW |  |  |  |  |
| Power to be supplied | maximum _________ kW, minimum __________ kW |  |  |  |  |
| [ ] Condensing [ ] Non-condensing |  |  |  | CE efficiency mark ___________ |  |
| Evaluation of visual inspection |  |  |  |  |  |
| Explanations regarding inspections and tests performed |  |  |  |  |  |
| Evaluation of boiler efficiency |  |  |  |  |  |
| Recommendations for further exploitation of the boiler |  |  |  |  |  |
| A list of the documents attached: 1. _____________ 2. _____________ 3. _____________ 4. ... (schemes, settings and notes or printouts of measurements, calculations of combustion efficiency and other explanatory documents) |  |  |  |  |  |

| Issuer of THE BOILER Inspection Report |  |
| --- | --- |
| Energy auditor | Given name and surname |
| Certificate | Certificate registration number |
| Company | Name, registration number and address, if the energy auditor represents a company |
| Date* | Signature* |

Minister for Economics 		K. Gerhards

Annex 5

Cabinet Regulation

No. 40

13 January 2009

Inspection Report of an Air-conditioning System

| Address of the building |  | Street, apartment, house, county, city, postal code |  |  |
| --- | --- | --- | --- | --- |
| Cadastre number of the building |  |  |  |  |
| AIR-CONDITIONING SYSTEM |  |  |  |  |
| Model | Batch number |  |  | Year |
| Type of energy or heating fuel |  |  |  |  |
| Cooling area _____ m2 and capacity _____ m3 |  |  |  |  |
| Output power |  |  | maximum ________ kW, minimum _________ kW |  |
| Power to be supplied |  |  | maximum ________ kW, minimum _________ kW |  |
| Evaluation of visual inspection |  |  |  |  |
| Explanations regarding inspections and tests performed |  |  |  |  |
| Evaluation of the system efficiency |  |  |  |  |
| Recommendations for further exploitation of the system |  |  |  |  |
| A list of the documents attached: 1. _____________ 2. _____________ 3. _____________ 4. ... (schemes, settings and notes or printouts of measurements, calculations of efficiency and other explanatory documents) |  |  |  |  |

| Issuer of the inspection report of THE air-conditioning system |  |
| --- | --- |
| Energy auditor | Given name and surname |
| Certificate | Certificate registration number |
| Company | Name, registration number and address, if the energy auditor represents a company |
| Date* | Signature* |

Minister for Economics 		K. Gerhards

* ”Date” and ”signature” boxes need not be completed if the electronic document is completed in accordance with the regulatory enactments regarding the completion of electronic documents.

* ”Date” and ”signature” boxes need not be completed if the electronic document is completed in accordance with the regulatory enactments regarding the completion of electronic documents.

* ”Date” and ”signature” boxes need not be completed if the electronic document is completed in accordance with the regulatory enactments regarding the completion of electronic documents.

* ”Date” and ”signature” boxes need not be completed if the electronic document is completed in accordance with the regulatory enactments regarding the completion of electronic documents.

* ”Date” and ”signature” boxes need not be completed if the electronic document is completed in accordance with the regulatory enactments regarding the completion of electronic documents.