Text consolidated by Valsts valodas centrs (State Language Centre) with amending regulations of:

10 December 2019 [shall come into force on 23 December 2021];

27 April 2021 [shall come into force on 1 May 2021].

If a whole or part of a paragraph has been amended, the date of the amending regulation appears in square brackets at the end of the paragraph. If a whole paragraph or sub-paragraph has been deleted, the date of the deletion appears in square brackets beside the deleted paragraph or sub-paragraph.

Republic of Latvia

Cabinet

Regulation No. 432

Adopted 17 September 2019

Regulations Regarding the Latvian Building Code LBN 003-19, Construction Climatology

Issued pursuant to

Section 5, Paragraph one, Clause 3 of the Construction Law

1. The Regulation approves the Latvian Building Code LBN 003-19, Construction Climatology (hereinafter – the Building Code).

2. Cabinet Regulation No. 338 of 30 June 2015, Regulations Regarding the Latvian Building Code LBN 003-15, Construction Climatology (Latvijas Vēstnesis, 2015, No. 125), is repealed.

3. Until the day of coming into force of this Regulation, the construction intention documentation which has been developed or submitted for agreement thereupon to the building authority or the authority performing the functions of the building authority need not be revised in accordance with the requirements laid down in the Building Code.

4. The building designs which have been developed or submitted for agreement thereupon to the building authority from 21 September 2019 to 22 December 2019 shall be considered valid.

[10 December 2019]

Prime Minister	A. K. Kariņš

Minister for Economics	R. Nemiro

Approved by

Cabinet Regulation No. 432

17 September 2019

Latvian Building Code LBN 003-19, Construction Climatology

1. The Latvian Building Code LBN 003-19, Construction Climatology (hereinafter – the Building Code), prescribes the climatic indicators applicable to structures and their elements in construction.

2. The climatic indicators included in the tables of the Annex to the Building Code shall be applied in engineering research, the designing of structures, and the performance of construction work.

3. The climatic indicators of any geographic point shall be determined for the needs of construction within the territory of Latvia according to the climatic indicators of the nearest geographical point included in the tables of the Annex to the Building Code.

4. The characteristics of wind and snow loads are included in the national annexes to the Eurocode standards LVS EN 1991-1-3:2003/NA:2019, Eurocode 1 – Actions on structures – Part 1–3: General actions – Snow loads – National Annex, and LVS EN 1991-1-4:2005/NA:2011, Eurocode 1 – Actions on structures – Part 1–4: General actions – Wind actions – National Annex.

5. Climatic information not included in the Building Code shall be prepared by valsts sabiedrība ar ierobežotu atbildību “Latvijas Vides, ģeoloģijas un meteoroloģijas centrs” [State limited liability company Latvian Environment, Geology and Meteorology Centre] in accordance with the technical task of the commissioning party and on the basis of a relevant contract.

6. Annual hourly temperature data for the calculation of the energy performance of a building are available on the website https://www.em.gov.lv/lv/tehniskas-prasibas-buvem.

[27 April 2021]

Minister for Economics	R. Nemiro

Annex

to Latvian Building Code LBN 003-19,

Construction Climatology

(approved by Cabinet Regulation No. 432 of 17 September 2019)

[10 December 2019; 27 April 2021]

Table 1

Average Air Temperature (°C)

| No. | Station | Month |  |  |  |  |  |  |  |  |  |  |  | On average per year |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII |  |
| 1. | Ainaži | –2.6 | –3.3 | –0.3 | 5.2 | 10.5 | 14.6 | 17.7 | 17.0 | 12.4 | 7.1 | 2.5 | –0.6 | 6.7 |
| 2. | Alūksne | –4.9 | –5.0 | –1.0 | 5.6 | 11.3 | 14.8 | 17.5 | 16.0 | 11.1 | 5.2 | 0.2 | –3.2 | 5.6 |
| 3. | Bauska | –2.9 | –2.8 | 0.7 | 7.0 | 12.3 | 15.6 | 18.3 | 17.3 | 12.4 | 6.8 | 2.1 | –1.5 | 7.1 |
| 4. | Daugavpils | –4.0 | –4.1 | 0.0 | 6.8 | 12.3 | 15.6 | 18.1 | 16.7 | 11.8 | 6.1 | 1.3 | –2.4 | 6.5 |
| 5. | Dobele | –2.4 | –2.6 | 0.7 | 6.6 | 12.1 | 15.4 | 18.1 | 17.3 | 12.4 | 6.8 | 2.4 | –1.0 | 7.2 |
| 6. | Gulbene | –4.5 | –4.7 | –0.5 | 6.2 | 11.8 | 15.2 | 17.8 | 16.4 | 11.3 | 5.5 | 0.5 | –2.8 | 6.0 |
| 7. | Jelgava | –2.5 | –2.8 | 0.7 | 6.7 | 11.9 | 15.3 | 17.9 | 16.9 | 12.1 | 6.7 | 2.3 | –1.2 | 7.0 |
| 8. | Kolka | –0.7 | –1.5 | 0.5 | 4.4 | 9.7 | 14.1 | 17.5 | 17.1 | 13.1 | 8.0 | 3.8 | 0.8 | 7.2 |
| 9. | Liepāja | –0.8 | –1.1 | 1.3 | 6.2 | 11.2 | 14.6 | 17.9 | 17.6 | 13.5 | 8.3 | 4.0 | 1.0 | 7.8 |
| 10. | Mērsrags | –1.5 | –2.0 | 0.5 | 5.2 | 10.5 | 14.6 | 17.6 | 16.9 | 12.6 | 7.2 | 3.0 | –0.2 | 7.0 |
| 11. | Pāvilosta | –0.9 | –1.2 | 1.2 | 5.6 | 10.3 | 14.2 | 17.2 | 17.1 | 13.1 | 8.1 | 3.8 | 0.8 | 7.4 |
| 12. | Priekuļi | –3.7 | –3.8 | –0.1 | 6.2 | 11.8 | 15.1 | 17.8 | 16.7 | 11.8 | 6.1 | 1.2 | –2.1 | 6.4 |
| 13. | Rēzekne | –4.6 | –4.6 | –0.4 | 6.5 | 12.0 | 15.3 | 17.8 | 16.4 | 11.5 | 5.6 | 0.6 | –3.0 | 6.1 |
| 14. | Rīga | –2.2 | –2.2 | 1.2 | 6.9 | 12.4 | 16.0 | 18.9 | 17.9 | 13.1 | 7.3 | 2.6 | –0.8 | 7.6 |
| 15. | Rūjiena | –3.7 | –4.1 | –0.4 | 5.9 | 11.5 | 15.1 | 17.8 | 16.5 | 11.6 | 5.9 | 1.1 | –2.3 | 6.2 |
| 16. | Saldus | –2.5 | –2.5 | 0.4 | 6.2 | 11.6 | 14.8 | 17.6 | 16.6 | 12.0 | 6.6 | 2.2 | –1.1 | 6.8 |
| 17. | Skrīveri | –3.5 | –3.5 | 0.3 | 6.7 | 12.1 | 15.4 | 18.0 | 16.8 | 11.9 | 6.2 | 1.5 | –2.1 | 6.6 |
| 18. | Skulte | –2.6 | –2.9 | 0.3 | 5.8 | 11.0 | 15.0 | 18.1 | 17.4 | 12.7 | 7.2 | 2.4 | –1.0 | 7.0 |
| 19. | Stende | –2.4 | –2.6 | 0.2 | 5.8 | 11.1 | 14.5 | 17.2 | 16.5 | 12.0 | 6.6 | 2.3 | –0.9 | 6.7 |
| 20. | Ventspils | –0.5 | –1.0 | 1.3 | 5.6 | 10.4 | 14.3 | 17.7 | 17.8 | 13.7 | 8.5 | 4.2 | 1.2 | 7.8 |
| 21. | Zīlāni | –3.9 | –3.9 | 0.0 | 6.6 | 12.1 | 15.5 | 18.1 | 16.8 | 11.8 | 6.1 | 1.1 | –2.5 | 6.5 |
| 22. | Zosēni | –4.4 | –4.7 | –0.9 | 5.5 | 11.0 | 14.5 | 17.2 | 15.8 | 10.9 | 5.3 | 0.6 | –2.8 | 5.7 |

Note. Table 1 shows the average monthly air temperature in the period from 1989 to 2018. The calculations are based on actual air temperature measurements recorded by the Latvian Environment, Geology and Meteorology Centre eight times per day. The arithmetic average has been calculated in accordance with the guidelines of the World Meteorological Organization (WMO, 2017a).

Table 2

Absolute Maximum of Air Temperature and the Probabilities Thereof (°C)

| No. | Station | Month |  |  |  |  |  |  |  |  |  |  |  | Year | Annual maximum air temperature with the following exceedance probability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII |  | 0.02 | 0.1 |
| 1. | Ainaži | 8.1 | 10.8 | 18.3 | 25.3 | 29.8 | 33.2 | 33.7 | 33.2 | 30.0 | 22.0 | 15.4 | 10.5 | 33.7 | 33.0 | 31.8 |
| 2. | Alūksne | 8.8 | 10.9 | 18.2 | 26.5 | 30.2 | 30.8 | 33.3 | 32.9 | 29.4 | 21.1 | 13.1 | 10.8 | 33.3 | 32.7 | 31.4 |
| 3. | Bauska | 10.5 | 12.8 | 19.2 | 27.2 | 29.9 | 32.1 | 34.5 | 34.3 | 30.5 | 23.7 | 17.2 | 11.4 | 34.5 | 34.5 | 33.1 |
| 4. | Daugavpils | 10.2 | 13.1 | 18.4 | 27.4 | 31.8 | 31.8 | 35.1 | 33.5 | 31.3 | 23.6 | 16.3 | 10.6 | 35.1 | 34.2 | 33.0 |
| 5. | Dobele | 10.4 | 13.8 | 20.0 | 26.5 | 30.6 | 33.1 | 35.8 | 35.1 | 30.7 | 23.5 | 16.8 | 11.5 | 35.8 | 35.0 | 33.7 |
| 6. | Gulbene | 9.7 | 11.0 | 17.3 | 25.7 | 30.1 | 31.4 | 33.6 | 33.8 | 29.8 | 22.5 | 14.3 | 11.3 | 33.8 | 33.5 | 31.9 |
| 7. | Jelgava | 10.7 | 13.5 | 19.7 | 27.4 | 30.0 | 32.8 | 36.0 | 33.7 | 30.1 | 23.4 | 17.0 | 11.6 | 36.0 | 34.5 | 32.9 |
| 8. | Kolka | 10.0 | 13.4 | 19.1 | 23.5 | 28.0 | 31.2 | 32.0 | 32.1 | 28.4 | 21.5 | 15.3 | 12.2 | 32.1 | 31.5 | 30.2 |
| 9. | Liepāja | 9.0 | 15.5 | 18.6 | 25.4 | 30.2 | 33.0 | 33.7 | 35.6 | 30.7 | 23.0 | 15.4 | 10.5 | 35.6 | 33.7 | 32.0 |
| 10. | Mērsrags | 10.7 | 13.6 | 20.8 | 27.0 | 31.6 | 34.6 | 34.4 | 33.8 | 29.1 | 24.9 | 15.5 | 12.4 | 34.6 | 34.2 | 32.7 |
| 11. | Pāvilosta | 9.4 | 14.7 | 18.7 | 27.3 | 30.2 | 32.9 | 33.4 | 35.9 | 30.5 | 22.4 | 15.9 | 11.7 | 35.9 | 34.5 | 32.8 |
| 12. | Priekuļi | 9.7 | 10.9 | 17.4 | 26.6 | 30.3 | 32.0 | 34.0 | 32.7 | 29.5 | 21.5 | 14.3 | 11.0 | 34.0 | 33.2 | 32.1 |
| 13. | Rēzekne | 10.3 | 10.5 | 17.2 | 25.8 | 30.0 | 32.1 | 34.1 | 34.7 | 30.1 | 22.6 | 15.2 | 10.5 | 34.7 | 33.9 | 32.3 |
| 14. | Rīga | 10.2 | 13.6 | 20.5 | 27.9 | 30.5 | 34.0 | 34.5 | 33.9 | 29.4 | 23.4 | 17.2 | 11.8 | 34.5 | 33.6 | 32.6 |
| 15. | Rūjiena | 9.7 | 10.7 | 18.1 | 26.4 | 34.4 | 32.0 | 34.4 | 35.6 | 29.0 | 21.3 | 14.0 | 11.6 | 35.6 | 34.2 | 32.4 |
| 16. | Saldus | 9.4 | 12.0 | 18.8 | 27.1 | 29.6 | 32.4 | 34.3 | 33.1 | 29.4 | 23.8 | 16.8 | 11.6 | 34.3 | 33.4 | 32.1 |
| 17. | Skrīveri | 10.6 | 13.5 | 19.8 | 26.7 | 29.9 | 32.2 | 35.3 | 33.8 | 31.6 | 22.2 | 16.2 | 10.7 | 35.3 | 34.2 | 32.6 |
| 18. | Skulte | 8.4 | 11.0 | 19.0 | 26.1 | 30.1 | 33.0 | 33.2 | 35.0 | 30.2 | 21.9 | 15.0 | 11.2 | 35.0 | 33.9 | 32.7 |
| 19. | Stende | 9.5 | 12.0 | 18.3 | 27.5 | 33.4 | 32.6 | 33.6 | 34.3 | 29.7 | 23.5 | 16.1 | 11.6 | 34.3 | 33.5 | 32.1 |
| 20. | Ventspils | 8.9 | 14.2 | 18.6 | 25.0 | 31.7 | 33.1 | 34.8 | 37.8 | 30.2 | 22.6 | 15.1 | 12.5 | 37.8 | 35.0 | 32.8 |
| 21. | Zīlāni | 10.0 | 11.8 | 18.0 | 26.3 | 30.0 | 31.9 | 34.7 | 34.3 | 30.5 | 22.6 | 16.6 | 10.3 | 34.7 | 34.2 | 32.9 |
| 22. | Zosēni | 8.7 | 10.6 | 18.5 | 26.5 | 29.1 | 31.3 | 34.1 | 33.6 | 28.8 | 21.2 | 14.5 | 10.6 | 34.1 | 33.3 | 31.8 |

Notes.

1. Table 2 shows the value of the daily maximum air temperature for each month throughout the observation period. The monthly and annual absolute maximum of air temperature is the highest air temperature recorded during the observation period.

2. The probability of the annual absolute maximum air temperature is characterised by temperature values with the exceedance probabilities of 0.02 and 0.1 (such probabilities indicate the likelihood of occurrence of the specific air temperature value once in 50 years and once in 10 years, respectively).

Table 3

Absolute Minimum of Air Temperature and the Probabilities Thereof (°C)

| No. | Station | Month |  |  |  |  |  |  |  |  |  |  |  | Year | Annual minimum air temperature with the following exceedance probability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII |  | 0.02 | 0.1 |
| 1. | Ainaži | –35.7 | –35.3 | –34.4 | –18.8 | –7.5 | –1.3 | 2.0 | 0.2 | –4.8 | –13.8 | –18.7 | –37.1 | –37.1 | –35.6 | –32.1 |
| 2. | Alūksne | –37.4 | –36.8 | –28.8 | –15.9 | –5.8 | –0.3 | 2.9 | 0.7 | –5.1 | –10.6 | –20.6 | –36.8 | –37.4 | –35.5 | –31.5 |
| 3. | Bauska | –34.6 | –33.9 | –25.1 | –13.0 | –4.0 | 0.4 | 3.5 | 1.1 | –4.6 | –10.8 | –20.4 | –33.9 | –34.6 | –33.9 | –30.7 |
| 4. | Daugavpils | –42.7 | –43.2 | –32.0 | –18.6 | –5.5 | –1.3 | 2.1 | –1.5 | –5.0 | –14.7 | –24.1 | –38.7 | –43.2 | –39.7 | –35.5 |
| 5. | Dobele | –34.1 | –35.9 | –25.7 | –13.2 | –3.7 | 0.3 | 4.7 | 1.9 | –3.7 | –9.2 | –21.7 | –31.9 | –35.9 | –34.2 | –30.7 |
| 6. | Gulbene | –39.0 | –37.6 | –26.5 | –15.4 | –6.0 | –1.0 | 3.6 | 0.5 | –5.6 | –13.5 | –22.8 | –38.4 | –39.0 | –36.4 | –32.2 |
| 7. | Jelgava | –34.5 | –34.9 | –30.1 | –14.2 | –5.3 | –1.1 | 2.8 | 0.3 | –6.4 | –10.1 | –22.4 | –32.2 | –34.9 | –34.3 | –31.3 |
| 8. | Kolka | –29.3 | –31.5 | –22.5 | –15.1 | –4.6 | –1.6 | 4.2 | 1.0 | –3.2 | –5.8 | –15.5 | –22.3 | –31.5 | –29.3 | –25.3 |
| 9. | Liepāja | –32.9 | –31.6 | –23.8 | –10.1 | –4.3 | 0.5 | 4.6 | 4.6 | –1.7 | –7.3 | –17.5 | –25.8 | –32.9 | –30.3 | –26.3 |
| 10. | Mērsrags | –33.7 | –36.2 | –28.9 | –16.2 | –6.1 | –2.2 | 3.3 | 1.4 | –3.9 | –10.0 | –18.1 | –25.6 | –36.2 | –34.1 | –29.6 |
| 11. | Pāvilosta | –32.0 | –34.1 | –23.1 | –12.2 | –4.6 | –1.2 | 2.1 | 1.7 | –4.0 | –9.5 | –16.1 | –25.7 | –34.1 | –31.1 | –27.0 |
| 12. | Priekuļi | –36.5 | –38.0 | –26.4 | –12.6 | –5.4 | –1.4 | 3.5 | –0.1 | –4.5 | –12.9 | –20.6 | –39.0 | –39.0 | –36.2 | –31.2 |
| 13. | Rēzekne | –38.6 | –38.8 | –29.0 | –18.4 | –5.0 | –0.8 | 2.0 | –0.4 | –5.0 | –12.5 | –24.2 | –38.5 | –38.8 | –36.9 | –32.5 |
| 14. | Rīga | –33.7 | –34.9 | –30.3 | –13.1 | –5.5 | –2.3 | 4.0 | 0.0 | –4.1 | –9.5 | –20.5 | –31.9 | –34.9 | –33.8 | –29.4 |
| 15. | Rūjiena | –42.2 | –38.7 | –30.2 | –18.5 | –6.5 | –1.1 | 2.8 | 1.0 | –5.8 | –13.9 | –21.3 | –39.2 | –42.2 | –38.5 | –33.4 |
| 16. | Saldus | –34.2 | –36.2 | –26.9 | –12.6 | –4.0 | –1.4 | 3.8 | 1.8 | –4.2 | –9.4 | –22.5 | –30.5 | –36.2 | –33.4 | –29.2 |
| 17. | Skrīveri | –37.3 | –38.4 | –30.7 | –18.0 | –4.9 | –2.2 | 3.5 | –0.6 | –4.6 | –12.5 | –23.6 | –32.8 | –38.4 | –36.2 | –32.6 |
| 18. | Skulte | –41.8 | –34.9 | –27.6 | –18.4 | –5.8 | –2.3 | 3.8 | 0.3 | –5.2 | –12.2 | –17.7 | –32.7 | –41.8 | –37.0 | –32.0 |
| 19. | Stende | –34.5 | –36.1 | –27.9 | –15.9 | –5.4 | –3.1 | 2.0 | –0.2 | –4.5 | –11.8 | –16.7 | –26.0 | –36.1 | –33.3 | –29.2 |
| 20. | Ventspils | –31.6 | –29.5 | –26.7 | –12.7 | –4.3 | 0.5 | 4.2 | 2.6 | –2.9 | –8.5 | –15.0 | –25.0 | –31.6 | –28.9 | –24.5 |
| 21. | Zīlāni | –35.4 | –36.7 | –24.6 | –14.4 | –4.6 | –0.4 | 4.1 | 0.3 | –4.2 | –10.9 | –22.0 | –35.2 | –36.7 | –34.9 | –31.6 |
| 22. | Zosēni | –40.6 | –42.2 | –30.5 | –21.8 | –10.5 | –2.0 | 1.1 | –1.7 | –7.3 | –16.3 | –24.6 | –42.9 | –42.9 | –40.3 | –35.9 |

Notes.

1. Table 3 shows the value of the daily minimum air temperature for each month throughout the observation period. The monthly and annual absolute minimum of air temperature is the lowest air temperature recorded during the observation period.

2. The probability of the annual absolute minimum air temperature is characterised by temperature values with the exceedance probabilities of 0.02 and 0.1 (such probabilities indicate the likelihood of occurrence of the specific air temperature value once in 50 years and once in 10 years, respectively).

Table 4

Average Maximum Air Temperature of the Hottest Month and Probabilities Thereof (°C)

| No. | Station | Average maximum air temperature of the hottest month | Average maximum air temperature of the hottest month with the following exceedance probability |  |
| --- | --- | --- | --- | --- |
|  |  |  | 0.02 | 0.1 |
| 1. | Ainaži | 21.8 | 25.8 | 24.2 |
| 2. | Alūksne | 22.2 | 25.8 | 24.3 |
| 3. | Bauska | 23.5 | 26.9 | 25.7 |
| 4. | Daugavpils | 23.5 | 26.8 | 25.7 |
| 5. | Dobele | 23.4 | 27.1 | 25.7 |
| 6. | Gulbene | 22.7 | 26.6 | 25.1 |
| 7. | Jelgava | 23.3 | 27.0 | 26.0 |
| 8. | Kolka | 20.7 | 24.1 | 23.0 |
| 9. | Liepāja | 21.4 | 25.1 | 23.7 |
| 10. | Mērsrags | 21.7 | 25.0 | 23.8 |
| 11. | Pāvilosta | 21.3 | 24.8 | 23.5 |
| 12. | Priekuļi | 22.6 | 26.3 | 24.9 |
| 13. | Rēzekne | 22.9 | 26.5 | 25.1 |
| 14. | Rīga | 23.0 | 28.2 | 25.7 |
| 15. | Rūjiena | 22.8 | 26.9 | 25.2 |
| 16. | Saldus | 22.4 | 26.2 | 24.9 |
| 17. | Skrīveri | 23.1 | 26.8 | 25.4 |
| 18. | Skulte | 22.2 | 25.7 | 24.5 |
| 19. | Stende | 22.2 | 25.7 | 24.7 |
| 20. | Ventspils | 20.8 | 24.8 | 23.4 |
| 21. | Zīlāni | 23.1 | 27.0 | 25.6 |
| 22. | Zosēni | 22.3 | 26.0 | 24.5 |

Notes.

1. Table 4 shows the value of the average maximum air temperature of the hottest month obtained as the average value of the daily maximum air temperature of the hottest month of each year throughout the observation period. The average maximum air temperature of the hottest month is obtained by calculating the arithmetic average of the values obtained for each year.

2. The average maximum air temperature of the hottest month is characterised by temperature values with the exceedance probabilities of 0.02 and 0.1 (such probabilities indicate the likelihood of occurrence of the specific air temperature value once in 50 years and once in 10 years, respectively).

Table 5

Average Minimum Air Temperature of the Coldest Month and Probabilities Thereof (°C)

| No. | Station | Average minimum air temperature of the coldest month | Average minimum air temperature of the coldest month with the following exceedance probability |  |
| --- | --- | --- | --- | --- |
|  |  |  | 0.02 | 0.1 |
| 1. | Ainaži | –10.7 | –19.6 | –16.3 |
| 2. | Alūksne | –12.0 | –19.7 | –17.1 |
| 3. | Bauska | –10.4 | –19.0 | –15.8 |
| 4. | Daugavpils | –12.1 | –21.1 | –17.9 |
| 5. | Dobele | –9.6 | –17.8 | –15.1 |
| 6. | Gulbene | –12.2 | –20.4 | –17.4 |
| 7. | Jelgava | –10.3 | –19.6 | –15.7 |
| 8. | Kolka | –7.4 | –17.0 | –12.6 |
| 9. | Liepāja | –7.3 | –15.2 | –12.1 |
| 10. | Mērsrags | –8.8 | –18.3 | –14.1 |
| 11. | Pāvilosta | –7.6 | –15.1 | –12.3 |
| 12. | Priekuļi | –11.0 | –19.3 | –16.1 |
| 13. | Rēzekne | –11.9 | –20.2 | –17.2 |
| 14. | Rīga | –9.5 | –18.8 | –14.9 |
| 15. | Rūjiena | –11.6 | –20.0 | –16.9 |
| 16. | Saldus | –9.4 | –17.2 | –14.3 |
| 17. | Skrīveri | –11.1 | –20.0 | –16.5 |
| 18. | Skulte | –9.9 | –19.0 | –15.6 |
| 19. | Stende | –9.7 | –18.2 | –14.7 |
| 20. | Ventspils | –6.9 | –14.9 | –11.5 |
| 21. | Zīlāni | –11.1 | –19.5 | –16.5 |
| 22. | Zosēni | –12.7 | –21.6 | –18.4 |

Notes.

1. Table 5 shows the value of the average minimum air temperature of the coldest month obtained as the average value of the daily minimum air temperature of the coldest month of each year throughout the observation period. The average minimum air temperature of the coldest month is obtained by calculating the arithmetic average of the values obtained for each year.

2. The average minimum air temperature of the coldest month is characterised by temperature values with the exceedance probabilities of 0.02 and 0.1 (such probabilities indicate the likelihood of occurrence of the specific air temperature value once in 50 years and once in 10 years, respectively).

Table 6

Average Air Temperature of the Five Coldest Days (°C)

| No. | Station | Average air temperature of the five coldest days | Average air temperature of the five coldest days with the following exceedance probability |  |
| --- | --- | --- | --- | --- |
|  |  |  | 0.02 | 0.1 |
| 1. | Ainaži | –22.1 | –26.0 | –20.9 |
| 2. | Alūksne | –23.3 | –27.3 | –22.4 |
| 3. | Bauska | –21.8 | –24.6 | –20.5 |
| 4. | Daugavpils | –23.3 | –26.4 | –22.3 |
| 5. | Dobele | –21.5 | –24.9 | –20.1 |
| 6. | Gulbene | –23.4 | –27.3 | –22.3 |
| 7. | Jelgava | –21.7 | –24.9 | –20.2 |
| 8. | Kolka | –16.6 | –22.3 | –15.6 |
| 9. | Liepāja | –17.8 | –21.2 | –16.5 |
| 10. | Mērsrags | –18.7 | –24.1 | –17.4 |
| 11. | Pāvilosta | –17.7 | –21.2 | –16.4 |
| 12. | Priekuļi | –22.3 | –26.7 | –21.1 |
| 13. | Rēzekne | –22.8 | –26.8 | –22.0 |
| 14. | Rīga | –20.0 | –23.9 | –18.6 |
| 15. | Rūjiena | –23.2 | –27.3 | –22.1 |
| 16. | Saldus | –19.9 | –23.1 | –18.7 |
| 17. | Skrīveri | –22.0 | –25.0 | –20.8 |
| 18. | Skulte | –21.2 | –25.1 | –19.8 |
| 19. | Stende | –19.3 | –22.4 | –18.0 |
| 20. | Ventspils | –16.4 | –19.8 | –15.2 |
| 21. | Zīlāni | –22.9 | –26.5 | –21.7 |
| 22. | Zosēni | –24.6 | –28.6 | –23.1 |

Notes.

1. Table 6 shows the value of the average air temperature of the five coldest consecutive days over the last 50 years (1969–2018) obtained as the average value of 20 % (10 values over 50 years) of the average values of the coldest of the five coldest consecutive days of each year.

2. The average air temperature of the five coldest days is characterised by temperature values with the exceedance probabilities of 0.02 and 0.1 (such probabilities indicate the likelihood of occurrence of the specific air temperature value once in 50 years and once in 10 years, respectively).

Table 7

Duration of the Heating Period and Average Air Temperature (°C)

| No. | Station | Heating period |  |
| --- | --- | --- | --- |
|  |  | duration of the period (days) | average temperature (°C) |
| 1. | Ainaži | 201 | 0.7 |
| 2. | Alūksne | 204 | –0.9 |
| 3. | Bauska | 195 | 0.6 |
| 4. | Daugavpils | 197 | –0.2 |
| 5. | Dobele | 196 | 0.9 |
| 6. | Gulbene | 204 | –0.5 |
| 7. | Jelgava | 197 | 0.7 |
| 8. | Kolka | 200 | 1.7 |
| 9. | Liepāja | 193 | 2.0 |
| 10. | Mērsrags | 201 | 1.4 |
| 11. | Pāvilosta | 196 | 1.9 |
| 12. | Priekuļi | 197 | –0.2 |
| 13. | Rēzekne | 202 | –0.5 |
| 14. | Rīga | 192 | 1.1 |
| 15. | Rūjiena | 204 | –0.1 |
| 16. | Saldus | 195 | 0.7 |
| 17. | Skrīveri | 197 | 0.1 |
| 18. | Skulte | 200 | 0.8 |
| 19. | Stende | 199 | 0.8 |
| 20. | Ventspils | 196 | 2.1 |
| 21. | Zīlāni | 198 | –0.3 |
| 22. | Zosēni | 206 | –0.5 |

Note. Table 7 shows the average duration of the heating period and the average daily air temperature in the period from 1989 to 2018. The duration of the heating period is the number of days in a year between the periods of the first and last time when the daily average air temperature of 8 °C or lower is recorded for at least three consecutive days. The calculations are based on actual air temperature measurements recorded by the Latvian Environment, Geology and Meteorology Centre eight times per day. The arithmetic average of the number of days and air temperature is calculated in accordance with the guidelines of the World Meteorological Organization (WMO, 2017a).

Table 8

Average Air Temperature Amplitude (°C)

| No. | Station | Month |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII |
| 1. | Ainaži | 4.9 | 5.5 | 6.7 | 8.9 | 10.1 | 8.9 | 9.0 | 8.9 | 8.1 | 6.2 | 4.7 | 4.7 |
| 2. | Alūksne | 4.6 | 5.7 | 7.0 | 9.3 | 10.6 | 9.8 | 9.6 | 9.0 | 7.6 | 5.5 | 3.9 | 4.0 |
| 3. | Bauska | 4.7 | 5.6 | 7.4 | 10.1 | 11.7 | 11.2 | 11.2 | 11.2 | 9.4 | 6.9 | 4.5 | 4.4 |
| 4. | Daugavpils | 5.2 | 6.5 | 8.1 | 10.8 | 12.2 | 11.3 | 11.3 | 11.0 | 9.6 | 7.1 | 4.5 | 4.4 |
| 5. | Dobele | 4.9 | 5.6 | 7.1 | 10.0 | 11.5 | 10.7 | 10.8 | 10.6 | 9.1 | 6.9 | 4.5 | 4.3 |
| 6. | Gulbene | 4.8 | 5.8 | 7.3 | 9.9 | 11.4 | 10.6 | 10.5 | 10.2 | 8.8 | 6.3 | 4.3 | 4.3 |
| 7. | Jelgava | 5.1 | 6.1 | 8.2 | 11.0 | 12.7 | 12.0 | 12.0 | 11.8 | 10.4 | 7.6 | 4.9 | 4.8 |
| 8. | Kolka | 4.1 | 4.7 | 5.9 | 7.2 | 8.4 | 7.8 | 7.6 | 7.4 | 6.5 | 5.4 | 4.1 | 4.0 |
| 9. | Liepāja | 4.5 | 4.9 | 5.9 | 7.8 | 8.7 | 7.7 | 7.5 | 7.6 | 7.0 | 5.8 | 4.3 | 4.4 |
| 10. | Mērsrags | 4.6 | 5.4 | 7.2 | 9.1 | 10.4 | 9.9 | 9.7 | 9.5 | 8.4 | 6.5 | 4.5 | 4.4 |
| 11. | Priekuļi | 4.6 | 5.3 | 6.7 | 9.0 | 10.8 | 9.9 | 9.9 | 9.3 | 7.9 | 5.7 | 4.0 | 4.1 |
| 12. | Pāvilosta | 4.5 | 5.0 | 6.5 | 8.7 | 9.5 | 8.6 | 8.4 | 8.8 | 8.1 | 6.3 | 4.6 | 4.4 |
| 13. | Rēzekne | 4.7 | 5.7 | 7.2 | 9.9 | 11.3 | 10.7 | 10.6 | 10.6 | 8.8 | 6.4 | 4.3 | 4.2 |
| 14. | Rīga | 4.3 | 5.0 | 6.5 | 8.8 | 9.7 | 9.0 | 8.8 | 8.4 | 7.4 | 5.7 | 3.9 | 4.0 |
| 15. | Rūjiena | 4.9 | 5.8 | 7.3 | 10.0 | 11.7 | 10.7 | 10.7 | 10.3 | 9.0 | 6.5 | 4.5 | 4.5 |
| 16. | Saldus | 4.6 | 5.4 | 7.2 | 9.9 | 11.5 | 10.6 | 10.4 | 9.8 | 8.6 | 6.5 | 4.2 | 4.2 |
| 17. | Skrīveri | 4.9 | 6.0 | 7.5 | 10.2 | 11.8 | 11.0 | 10.9 | 10.5 | 9.0 | 6.7 | 4.4 | 4.3 |
| 18. | Skulte | 4.7 | 5.2 | 6.5 | 8.6 | 9.7 | 8.7 | 8.5 | 8.4 | 7.8 | 6.0 | 4.5 | 4.4 |
| 19. | Stende | 4.8 | 5.4 | 7.4 | 9.9 | 11.7 | 11.0 | 10.9 | 10.2 | 9.0 | 6.6 | 4.5 | 4.6 |
| 20. | Ventspils | 3.7 | 4.1 | 4.7 | 6.2 | 6.7 | 6.1 | 6.1 | 6.4 | 5.9 | 4.9 | 3.9 | 3.9 |
| 21. | Zosēni | 5.0 | 6.0 | 7.8 | 10.4 | 12.5 | 11.3 | 11.1 | 10.6 | 9.0 | 6.4 | 4.3 | 4.4 |
| 22. | Zīlāni | 4.6 | 5.8 | 7.5 | 10.0 | 11.5 | 10.7 | 10.7 | 10.5 | 8.7 | 6.4 | 4.2 | 4.2 |
| Note. Table 8 shows the annual average air temperature amplitude of each month calculated as the difference of the average maximum and average minimum air temperature of the month. The calculation is based on the data for the period from 1989 to 2018. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 9

Daily Average Partial Pressure of Water Vapour in the Air (hPa)

| No. | Station | Month |  |  |  |  |  |  |  |  |  |  |  | On average per year |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII |  |
| 1. | Ainaži | 4.8 | 4.6 | 5.1 | 6.7 | 9.4 | 13.0 | 15.9 | 15.2 | 12.0 | 8.8 | 6.7 | 5.5 | 9.0 |
| 2. | Alūksne | 4.3 | 4.1 | 4.6 | 6.4 | 8.9 | 12.2 | 14.9 | 14.3 | 11.1 | 8.0 | 6.0 | 4.8 | 8.3 |
| 3. | Bauska | 4.7 | 4.6 | 5.3 | 7.1 | 9.8 | 12.8 | 15.3 | 14.6 | 11.7 | 8.7 | 6.7 | 5.2 | 8.9 |
| 4. | Daugavpils | 4.4 | 4.3 | 4.9 | 6.8 | 9.7 | 12.7 | 15.2 | 14.5 | 11.4 | 8.3 | 6.3 | 4.9 | 8.6 |
| 5. | Dobele | 4.9 | 4.8 | 5.3 | 7.0 | 9.6 | 12.6 | 15.3 | 14.7 | 11.8 | 8.8 | 6.8 | 5.4 | 8.9 |
| 6. | Gulbene | 4.3 | 4.2 | 4.7 | 6.4 | 9.2 | 12.4 | 15.1 | 14.3 | 11.2 | 8.1 | 6.1 | 4.9 | 8.4 |
| 7. | Jelgava | 4.8 | 4.7 | 5.2 | 7.0 | 9.7 | 12.7 | 15.5 | 14.8 | 11.8 | 8.6 | 6.7 | 5.3 | 8.9 |
| 8. | Kolka | 5.2 | 5.0 | 5.4 | 6.8 | 9.3 | 12.6 | 15.9 | 15.7 | 12.3 | 9.0 | 7.1 | 5.8 | 9.2 |
| 9. | Liepāja | 5.4 | 5.2 | 5.7 | 7.3 | 9.8 | 12.9 | 15.8 | 15.6 | 12.5 | 9.3 | 7.3 | 6.0 | 9.4 |
| 10. | Mērsrags | 5.0 | 4.8 | 5.2 | 6.8 | 9.3 | 12.4 | 15.5 | 15.2 | 11.9 | 8.7 | 6.8 | 5.5 | 8.9 |
| 11. | Pāvilosta | 5.3 | 5.1 | 5.5 | 7.0 | 9.6 | 12.7 | 15.6 | 15.4 | 12.3 | 9.1 | 7.2 | 5.9 | 9.2 |
| 12. | Priekuļi | 4.5 | 4.4 | 4.9 | 6.6 | 9.0 | 12.2 | 14.9 | 14.3 | 11.3 | 8.3 | 6.3 | 5.0 | 8.5 |
| 13. | Rēzekne | 4.2 | 4.1 | 4.7 | 6.5 | 9.4 | 12.5 | 15.1 | 14.3 | 11.2 | 8.0 | 6.1 | 4.8 | 8.4 |
| 14. | Rīga | 4.8 | 4.7 | 5.2 | 6.9 | 9.6 | 12.8 | 15.6 | 15.1 | 12.0 | 8.8 | 6.7 | 5.4 | 9.0 |
| 15. | Rūjiena | 4.5 | 4.4 | 4.9 | 6.6 | 9.1 | 12.4 | 15.2 | 14.6 | 11.5 | 8.4 | 6.3 | 5.0 | 8.6 |
| 16. | Saldus | 4.9 | 4.8 | 5.2 | 6.8 | 9.3 | 12.2 | 14.9 | 14.6 | 11.8 | 8.7 | 6.8 | 5.5 | 8.8 |
| 17. | Skrīveri | 4.6 | 4.5 | 5.0 | 6.8 | 9.5 | 12.6 | 15.2 | 14.7 | 11.6 | 8.4 | 6.5 | 5.1 | 8.7 |
| 18. | Skulte | 4.8 | 4.6 | 5.2 | 6.9 | 9.7 | 13.0 | 15.9 | 15.2 | 12.0 | 8.7 | 6.7 | 5.3 | 9.0 |
| 19. | Stende | 4.9 | 4.7 | 5.1 | 6.6 | 9.0 | 11.9 | 14.6 | 14.4 | 11.6 | 8.6 | 6.7 | 5.4 | 8.6 |
| 20. | Ventspils | 5.3 | 5.1 | 5.6 | 7.0 | 9.7 | 12.9 | 16.1 | 15.8 | 12.6 | 9.2 | 7.2 | 5.9 | 9.4 |
| 21. | Zīlāni | 4.4 | 4.4 | 4.9 | 6.7 | 9.5 | 12.6 | 15.2 | 14.5 | 11.4 | 8.2 | 6.2 | 4.9 | 8.6 |
| 22. | Zosēni | 4.3 | 4.2 | 4.7 | 6.3 | 8.8 | 12.0 | 14.6 | 14.0 | 11.0 | 8.0 | 6.1 | 4.9 | 8.2 |

Note. Table 9 shows the monthly value of the daily average partial pressure of water vapour in the period from 1989 to 2018. The partial pressure of water vapour is a characteristic of air humidity, indicating the pressure that water vapour would exert if, at a specific temperature and in the atmosphere, it occupied the same volume of the moist air. The parameter is calculated by using the measurements of actual air temperature and relative humidity at ground level recorded by the Latvian Environment, Geology and Meteorology Centre and in accordance with the guidelines of the World Meteorological Organization (WMO, 2017a; WMO, 2017b).

Table 10

Daily Average Relative Air Humidity (%)

| No. | Station | Month |  |  |  |  |  |  |  |  |  |  |  | On average per year |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII |  |
| 1. | Ainaži | 88 | 87 | 82 | 76 | 74 | 78 | 79 | 79 | 82 | 84 | 88 | 88 | 82 |
| 2. | Alūksne | 91 | 88 | 78 | 70 | 67 | 73 | 76 | 79 | 84 | 88 | 92 | 92 | 82 |
| 3. | Bauska | 87 | 85 | 79 | 71 | 69 | 73 | 74 | 76 | 81 | 86 | 89 | 89 | 80 |
| 4. | Daugavpils | 87 | 85 | 77 | 69 | 69 | 73 | 75 | 78 | 82 | 85 | 89 | 89 | 80 |
| 5. | Dobele | 88 | 86 | 80 | 72 | 68 | 73 | 75 | 76 | 82 | 86 | 90 | 90 | 80 |
| 6. | Gulbene | 89 | 86 | 77 | 68 | 67 | 73 | 75 | 78 | 83 | 87 | 91 | 91 | 80 |
| 7. | Jelgava | 87 | 85 | 79 | 72 | 71 | 74 | 77 | 79 | 83 | 86 | 89 | 90 | 81 |
| 8. | Kolka | 86 | 86 | 83 | 80 | 77 | 79 | 80 | 81 | 81 | 82 | 86 | 87 | 82 |
| 9. | Liepāja | 88 | 86 | 83 | 77 | 75 | 78 | 78 | 78 | 81 | 83 | 87 | 87 | 82 |
| 10. | Mērsrags | 86 | 84 | 80 | 77 | 73 | 75 | 77 | 79 | 82 | 83 | 87 | 87 | 81 |
| 11. | Pāvilosta | 87 | 85 | 81 | 77 | 76 | 79 | 80 | 79 | 81 | 83 | 87 | 87 | 82 |
| 12. | Priekuļi | 88 | 86 | 78 | 70 | 66 | 72 | 74 | 76 | 81 | 86 | 90 | 90 | 80 |
| 13. | Rēzekne | 88 | 85 | 77 | 68 | 68 | 73 | 76 | 78 | 83 | 86 | 90 | 90 | 80 |
| 14. | Rīga | 85 | 82 | 75 | 68 | 65 | 69 | 70 | 72 | 78 | 82 | 86 | 86 | 76 |
| 15. | Rūjiena | 89 | 87 | 79 | 71 | 68 | 73 | 76 | 79 | 84 | 87 | 90 | 91 | 81 |
| 16. | Saldus | 90 | 87 | 80 | 73 | 69 | 74 | 76 | 79 | 84 | 87 | 91 | 91 | 82 |
| 17. | Skrīveri | 88 | 86 | 78 | 70 | 68 | 73 | 75 | 78 | 83 | 87 | 90 | 90 | 80 |
| 18. | Skulte | 87 | 85 | 80 | 74 | 74 | 76 | 77 | 77 | 81 | 83 | 87 | 88 | 81 |
| 19. | Stende | 89 | 86 | 80 | 72 | 69 | 74 | 76 | 78 | 83 | 86 | 90 | 90 | 81 |
| 20. | Ventspils | 86 | 85 | 82 | 78 | 77 | 79 | 79 | 78 | 80 | 81 | 85 | 85 | 81 |
| 21. | Zīlāni | 88 | 86 | 78 | 70 | 68 | 73 | 75 | 77 | 82 | 86 | 89 | 89 | 80 |
| 22. | Zosēni | 89 | 86 | 78 | 71 | 69 | 74 | 76 | 79 | 84 | 87 | 91 | 91 | 81 |

Note. Table 10 shows the daily average relative air humidity of each month for the period from 1989 to 2018. The relative air humidity characterises the saturation of air with water vapour and is expressed as a percentage of the ratio of the partial pressure of water vapour in the air to the maximum partial pressure of saturated water vapour in the air at a specific temperature and atmospheric conditions. The parameter is calculated by using the measurements of relative humidity at ground level recorded by the Latvian Environment, Geology and Meteorology Centre and in accordance with the guidelines for statistical calculations of the World Meteorological Organization (WMO, 2017a).

Table 11

Amplitude of Relative Air Humidity (%)

| No. | Station | Month |  |  |  |  |  |  |  |  |  |  |  | On average per year |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII |  |
| 1. | Ainaži | 13 | 16 | 24 | 33 | 35 | 31 | 31 | 31 | 27 | 22 | 15 | 14 | 24 |
| 2. | Alūksne | 11 | 17 | 30 | 39 | 42 | 39 | 38 | 35 | 30 | 21 | 11 | 10 | 27 |
| 3. | Bauska | 12 | 17 | 28 | 39 | 42 | 40 | 41 | 40 | 33 | 23 | 13 | 11 | 28 |
| 4. | Daugavpils | 14 | 18 | 32 | 43 | 46 | 42 | 42 | 40 | 34 | 25 | 14 | 12 | 30 |
| 5. | Dobele | 13 | 17 | 28 | 38 | 42 | 39 | 40 | 41 | 33 | 23 | 13 | 11 | 28 |
| 6. | Gulbene | 12 | 18 | 30 | 40 | 42 | 39 | 39 | 38 | 32 | 22 | 13 | 10 | 28 |
| 7. | Jelgava | 15 | 20 | 32 | 44 | 47 | 43 | 42 | 41 | 34 | 25 | 15 | 12 | 31 |
| 8. | Kolka | 15 | 17 | 23 | 28 | 31 | 30 | 28 | 27 | 24 | 21 | 16 | 15 | 23 |
| 9. | Liepāja | 14 | 17 | 24 | 31 | 33 | 29 | 29 | 29 | 27 | 22 | 16 | 15 | 24 |
| 10. | Mērsrags | 15 | 18 | 27 | 34 | 38 | 35 | 34 | 32 | 29 | 23 | 16 | 14 | 26 |
| 11. | Pāvilosta | 15 | 18 | 26 | 33 | 34 | 31 | 30 | 31 | 28 | 23 | 17 | 16 | 25 |
| 12. | Priekuļi | 12 | 16 | 28 | 37 | 41 | 38 | 38 | 36 | 30 | 20 | 12 | 11 | 27 |
| 13. | Rēzekne | 12 | 17 | 29 | 39 | 41 | 39 | 39 | 38 | 31 | 23 | 13 | 11 | 28 |
| 14. | Rīga | 14 | 18 | 28 | 37 | 39 | 37 | 36 | 34 | 30 | 22 | 14 | 13 | 27 |
| 15. | Rūjiena | 12 | 17 | 30 | 40 | 43 | 40 | 40 | 37 | 31 | 22 | 12 | 11 | 28 |
| 16. | Saldus | 12 | 17 | 29 | 39 | 43 | 40 | 39 | 38 | 31 | 22 | 13 | 10 | 28 |
| 17. | Skrīveri | 13 | 18 | 31 | 41 | 44 | 41 | 41 | 39 | 32 | 23 | 13 | 11 | 29 |
| 18. | Skulte | 14 | 17 | 25 | 32 | 34 | 32 | 32 | 30 | 26 | 22 | 15 | 14 | 24 |
| 19. | Stende | 13 | 17 | 29 | 40 | 44 | 41 | 40 | 38 | 32 | 22 | 13 | 11 | 28 |
| 20. | Ventspils | 14 | 16 | 21 | 25 | 25 | 23 | 24 | 25 | 23 | 20 | 16 | 15 | 21 |
| 21. | Zīlāni | 13 | 17 | 29 | 40 | 43 | 40 | 40 | 40 | 32 | 23 | 13 | 11 | 28 |
| 22. | Zosēni | 12 | 17 | 31 | 43 | 46 | 42 | 41 | 38 | 31 | 22 | 12 | 10 | 29 |

Note. Table 11 shows the average monthly difference between the highest and lowest daily relative humidity measurements in the period from 1989 to 2018. The calculation is based on the measurements of relative humidity at ground level recorded by the Latvian Environment, Geology and Meteorology Centre eight times per day at 00:00, 03:00, 06:00, 09:00, 12:00, 15:00, 18:00, and 21:00 UTC. The long-term average arithmetic value is calculated in accordance with the guidelines of the World Meteorological Organization (WMO, 2017a).

Table 12

Monthly and Annual Sum of Precipitation (mm)

| No. | Station | Month |  |  |  |  |  |  |  |  |  |  |  | Total per year |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | I | II | III | IV | V | VI | VII | VIII | IX | X | XI | XII |  |
| 1. | Ainaži | 46 | 34 | 32 | 34 | 39 | 63 | 49 | 79 | 59 | 80 | 62 | 47 | 624 |
| 2. | Alūksne | 58 | 45 | 43 | 41 | 61 | 93 | 82 | 87 | 63 | 75 | 64 | 53 | 765 |
| 3. | Bauska | 38 | 31 | 29 | 38 | 47 | 62 | 71 | 61 | 54 | 62 | 48 | 42 | 583 |
| 4. | Daugavpils | 42 | 39 | 37 | 36 | 58 | 77 | 78 | 75 | 59 | 60 | 49 | 45 | 655 |
| 5. | Dobele | 39 | 30 | 30 | 35 | 43 | 66 | 78 | 66 | 56 | 65 | 47 | 40 | 595 |
| 6. | Kolka | 46 | 38 | 35 | 34 | 33 | 62 | 72 | 85 | 57 | 69 | 60 | 51 | 642 |
| 7. | Liepāja | 58 | 42 | 37 | 30 | 37 | 49 | 68 | 79 | 74 | 87 | 78 | 71 | 710 |
| 8. | Mērsrags | 44 | 34 | 32 | 38 | 44 | 70 | 78 | 78 | 53 | 69 | 55 | 45 | 640 |
| 9. | Priekuļi | 52 | 40 | 38 | 42 | 58 | 82 | 82 | 87 | 56 | 76 | 58 | 49 | 720 |
| 10. | Rēzekne | 43 | 36 | 32 | 31 | 61 | 82 | 65 | 66 | 53 | 62 | 48 | 41 | 620 |
| 11. | Rīga | 42 | 37 | 33 | 36 | 47 | 66 | 81 | 81 | 66 | 77 | 57 | 48 | 671 |
| 12. | Rūjiena | 52 | 40 | 39 | 38 | 48 | 81 | 72 | 85 | 55 | 79 | 60 | 51 | 700 |
| 13. | Saldus | 51 | 39 | 37 | 38 | 46 | 63 | 76 | 72 | 63 | 70 | 53 | 53 | 661 |
| 14. | Skrīveri | 57 | 43 | 42 | 43 | 59 | 79 | 83 | 84 | 62 | 78 | 64 | 55 | 749 |
| 15. | Skulte | 52 | 43 | 37 | 38 | 46 | 70 | 78 | 88 | 57 | 82 | 61 | 52 | 704 |
| 16. | Stende | 53 | 40 | 39 | 40 | 45 | 74 | 78 | 87 | 67 | 78 | 58 | 53 | 712 |
| 17. | Ventspils | 60 | 45 | 37 | 33 | 37 | 51 | 63 | 78 | 65 | 83 | 74 | 73 | 699 |
| 18. | Zosēni | 60 | 45 | 42 | 42 | 57 | 80 | 80 | 80 | 57 | 79 | 59 | 56 | 737 |

Notes.

1. Table 12 shows the value of long-term monthly and annual sum of precipitation in the period from 1989 to 2018. The term “precipitation” includes the liquid, solid, and mixed atmospheric precipitation and also precipitation caused by a fierce dew, white frost, frost, and fog. The amount of precipitation in meteorology is measured in millimetres: a 1 mm thick water layer results from pouring 1 litre of water on a horizontal surface of 1 m2, without the water flowing away, infiltrating, and vaporising. The total value is calculated by using the measurements of precipitation at ground level recorded by the Latvian Environment, Geology and Meteorology Centre and in accordance with the guidelines for calculating long-term climate parameters of the World Meteorological Organization (WMO, 2017a).

2. Due to the lack of data, the sum of precipitation was not calculated at four meteorological observation stations, i.e., Jelgava, Gulbene, Pāvilosta, and Zīlāni.

Table 13

Weight of Glaze Icing and Rime Icing on Wires with a Diameter of 10 mm at the Height of 10 m with Various Probabilities

| No. | Place | Weight (g/m) possible once every |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2 years | 5 years | 10 years | 15 years | 20 years | 25 years | 30 years | 50 years |
| 1. | Alūksne | 181 | 316 | 406 | 456 | 492 | 519 | 541 | 603 |
| 2. | Daugavpils | 49 | 85 | 109 | 123 | 132 | 139 | 145 | 162 |
| 3. | Liepāja | 32 | 61 | 80 | 91 | 99 | 105 | 109 | 123 |
| 4. | Rīga | 46 | 97 | 130 | 149 | 162 | 172 | 180 | 203 |
| 5. | Skrīveri | 123 | 238 | 315 | 358 | 388 | 412 | 431 | 484 |
| 6. | Ventspils | 60 | 119 | 158 | 180 | 196 | 208 | 218 | 245 |
| Note. Table 13 shows the results of observations at meteorological stations (every winter in the period from 1989 to 2018) of the maximum weight of glaze icing and rime icing on wires with the diameter of 5 mm at the height of 2 m, recalculated in accordance with appropriate methodology for wires with the diameter of 10 mm at the height of 10 m. |  |  |  |  |  |  |  |  |  |

Table 14

Normative Ice Layer on Wires with the Diameter of 10 mm at the Height of 10 m with Various Probabilities

| No. | Place | Thickness of the ice layer (mm) possible once every |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2 years | 5 years | 10 years | 15 years | 20 years | 25 years | 30 years | 50 years |
| 1. | Alūksne | 4.4 | 6.7 | 8.0 | 8.6 | 9.1 | 9.4 | 9.7 | 10.4 |
| 2. | Daugavpils | 1.5 | 2.4 | 3.0 | 3.3 | 3.5 | 3.6 | 3.7 | 4.1 |
| 3. | Liepāja | 1.0 | 1.8 | 2.3 | 2.6 | 2.7 | 2.9 | 3.0 | 3.3 |
| 4. | Rīga | 1.4 | 2.7 | 3.4 | 3.8 | 4.1 | 4.3 | 4.4 | 4.8 |
| 5. | Skrīveri | 3.3 | 5.5 | 6.7 | 7.3 | 7.7 | 8.1 | 8.3 | 9.0 |
| 6. | Ventspils | 1.8 | 3.2 | 4.0 | 4.4 | 4.7 | 4.9 | 5.1 | 5.6 |
| Note. Table 14 shows the results of observations at meteorological stations (every winter in the period from 1989 to 2018) of the maximum size of the ice layer on wires with the diameter of 5 mm at the height of 2 m, recalculated in accordance with appropriate methodology on wires with the diameter of 10 mm at the height of 10 m. |  |  |  |  |  |  |  |  |  |

Table 15

Normative Depth of the Frigidity of Loamy Ground (cm) for Various Repeatability Periods in Years

| No. | Place | Normative depth of the frigidity of loamy ground (cm) possible once every |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2 years | 5 years | 10 years | 20 years | 50 years | 100 years |
| 1. | Ainaži | 60 | 82 | 94 | 105 | 116 | 125 |
| 2. | Alūksne | 88 | 108 | 119 | 129 | 141 | 149 |
| 3. | Bauska | 62 | 82 | 93 | 103 | 114 | 121 |
| 4. | Daugavpils | 78 | 98 | 110 | 120 | 131 | 140 |
| 5. | Dobele | 62 | 85 | 97 | 107 | 119 | 127 |
| 6. | Gulbene | 79 | 98 | 109 | 119 | 130 | 138 |
| 7. | Jelgava | 59 | 79 | 91 | 100 | 111 | 119 |
| 8. | Kolka | 47 | 67 | 78 | 87 | 98 | 105 |
| 9. | Liepāja | 46 | 67 | 78 | 87 | 97 | 104 |
| 10. | Mērsrags | 49 | 70 | 81 | 90 | 101 | 108 |
| 11. | Pāvilosta | 42 | 62 | 72 | 80 | 90 | 96 |
| 12. | Priekuļi | 70 | 90 | 101 | 110 | 121 | 129 |
| 13. | Rēzekne | 80 | 99 | 109 | 119 | 130 | 137 |
| 14. | Rīga | 59 | 80 | 91 | 101 | 112 | 120 |
| 15. | Rūjiena | 72 | 93 | 104 | 114 | 126 | 134 |
| 16. | Saldus | 62 | 84 | 96 | 105 | 117 | 125 |
| 17. | Skrīveri | 73 | 94 | 106 | 116 | 128 | 136 |
| 18. | Skulte | 58 | 80 | 91 | 101 | 113 | 121 |
| 19. | Stende | 57 | 78 | 89 | 99 | 110 | 117 |
| 20. | Ventspils | 39 | 56 | 66 | 73 | 82 | 89 |
| 21. | Zīlāni | 74 | 94 | 104 | 114 | 125 | 133 |
| 22. | Zosēni | 80 | 99 | 111 | 120 | 132 | 140 |
| Notes. 1. Table 15 (in the period from 1989 to 2018) shows the normative depth of the frigidity of loamy ground. 2. The normative depth of the frigidity of ground is the arithmetic average of the maximum frigidity depth of a snowless ground in the cold seasons. According to the ground diversity assessment, there is 56 % of loamy ground, 36 % of sandy ground, and 8 % of peaty ground in Latvia. Sandy grounds freeze deeper than loamy. The characteristics of loamy ground may be used for the determination of the normative depth of the frigidity of ground for sandy grounds by applying the coefficient 1.2 (the results of the special observations of the depth of the frigidity of an uncovered sandy ground confirm the correctness of this coefficient). |  |  |  |  |  |  |  |

Table 16

Data on the Average Daily Sum of Solar Radiation on Diversely Oriented Surfaces in January (kWh/m2)

| No. | Station | Direct solar radiation |  |  |  |  | Diffuse solar radiation |  | Global solar radiation |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | orientation of the vertical wall |  |  |  | horizontal surface | orientation of the vertical wall | horizontal surface | orientation of the vertical wall |  |  |  | horizontal surface |
|  |  | East | North | South | West |  |  |  | East | North | South | West |  |
| 1. | Ainaži | 0.16 | 0.00 | 0.51 | 0.13 | 0.10 | 0.10 | 0.21 | 0.27 | 0.10 | 0.62 | 0.23 | 0.31 |
| 2. | Alūksne | 0.18 | 0.00 | 0.54 | 0.13 | 0.11 | 0.11 | 0.22 | 0.29 | 0.11 | 0.65 | 0.24 | 0.32 |
| 3. | Bauska | 0.17 | 0.00 | 0.47 | 0.12 | 0.10 | 0.13 | 0.25 | 0.29 | 0.13 | 0.60 | 0.25 | 0.35 |
| 4. | Daugavpils | 0.21 | 0.00 | 0.60 | 0.16 | 0.12 | 0.14 | 0.28 | 0.34 | 0.14 | 0.74 | 0.30 | 0.40 |
| 5. | Dobele | 0.16 | 0.00 | 0.47 | 0.12 | 0.09 | 0.12 | 0.25 | 0.28 | 0.12 | 0.59 | 0.25 | 0.34 |
| 6. | Gulbene | 0.18 | 0.00 | 0.54 | 0.13 | 0.10 | 0.12 | 0.23 | 0.30 | 0.12 | 0.65 | 0.24 | 0.34 |
| 7. | Jelgava | 0.16 | 0.00 | 0.45 | 0.11 | 0.09 | 0.12 | 0.25 | 0.28 | 0.12 | 0.57 | 0.23 | 0.34 |
| 8. | Kolka | 0.14 | 0.00 | 0.46 | 0.12 | 0.09 | 0.10 | 0.21 | 0.25 | 0.10 | 0.56 | 0.22 | 0.29 |
| 9. | Liepāja | 0.18 | 0.00 | 0.56 | 0.15 | 0.11 | 0.13 | 0.26 | 0.31 | 0.13 | 0.69 | 0.28 | 0.37 |
| 10. | Mērsrags | 0.16 | 0.00 | 0.50 | 0.13 | 0.10 | 0.11 | 0.22 | 0.27 | 0.11 | 0.61 | 0.24 | 0.32 |
| 11. | Pāvilosta | 0.18 | 0.00 | 0.58 | 0.15 | 0.12 | 0.12 | 0.25 | 0.31 | 0.12 | 0.70 | 0.28 | 0.36 |
| 12. | Priekuļi | 0.18 | 0.00 | 0.53 | 0.13 | 0.10 | 0.11 | 0.22 | 0.29 | 0.11 | 0.64 | 0.24 | 0.33 |
| 13. | Rēzekne | 0.20 | 0.00 | 0.61 | 0.16 | 0.12 | 0.12 | 0.25 | 0.32 | 0.12 | 0.73 | 0.28 | 0.37 |
| 14. | Rīga | 0.17 | 0.00 | 0.55 | 0.14 | 0.11 | 0.12 | 0.24 | 0.29 | 0.12 | 0.67 | 0.26 | 0.35 |
| 15. | Rūjiena | 0.15 | 0.00 | 0.47 | 0.11 | 0.09 | 0.10 | 0.21 | 0.25 | 0.10 | 0.58 | 0.22 | 0.30 |
| 16. | Saldus | 0.18 | 0.00 | 0.52 | 0.13 | 0.10 | 0.12 | 0.25 | 0.30 | 0.12 | 0.65 | 0.26 | 0.35 |
| 17. | Skrīveri | 0.18 | 0.00 | 0.57 | 0.15 | 0.11 | 0.12 | 0.25 | 0.31 | 0.12 | 0.69 | 0.27 | 0.36 |
| 18. | Skulte | 0.17 | 0.00 | 0.54 | 0.14 | 0.11 | 0.11 | 0.22 | 0.28 | 0.11 | 0.65 | 0.25 | 0.33 |
| 19. | Stende | 0.17 | 0.00 | 0.52 | 0.13 | 0.10 | 0.12 | 0.23 | 0.28 | 0.12 | 0.63 | 0.25 | 0.34 |
| 20. | Ventspils | 0.16 | 0.00 | 0.51 | 0.13 | 0.10 | 0.11 | 0.22 | 0.27 | 0.11 | 0.62 | 0.24 | 0.32 |
| 21. | Zīlāni | 0.20 | 0.00 | 0.59 | 0.15 | 0.12 | 0.13 | 0.25 | 0.33 | 0.13 | 0.71 | 0.28 | 0.37 |
| 22. | Zosēni | 0.18 | 0.00 | 0.54 | 0.13 | 0.11 | 0.11 | 0.23 | 0.29 | 0.11 | 0.66 | 0.25 | 0.34 |

Note. Table 16 provides information on the long-term monthly average daily sum of solar radiation on diversely oriented surfaces. Solar radiation data were obtained from long-term satellite measurements of the CMSAF SARAH 2.1 product at the locations of meteorological stations (CMSAF, 2019). The satellite data also take into account the physical state of the atmosphere, including cloudiness, which affects the amount of the measured solar radiation. The analysis is based on the solar radiation data in the period from 1988 to 2017. The global (or summed) solar radiation value is obtained by adding the direct and diffuse solar radiation values.

Table 17

Data on the Average Daily Sum of Solar Radiation on Diversely Oriented Surfaces in February (kWh/m2)

| No. | Station | Direct solar radiation |  |  |  |  | Diffuse solar radiation |  | Global solar radiation |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | orientation of the vertical wall |  |  |  | horizontal surface | orientation of the vertical wall | horizontal surface | orientation of the vertical wall |  |  |  | horizontal surface |
|  |  | East | North | South | West |  |  |  | East | North | South | West |  |
| 1. | Ainaži | 0.36 | 0.00 | 0.97 | 0.32 | 0.33 | 0.26 | 0.52 | 0.62 | 0.26 | 1.23 | 0.58 | 0.85 |
| 2. | Alūksne | 0.38 | 0.00 | 0.98 | 0.30 | 0.34 | 0.27 | 0.54 | 0.65 | 0.27 | 1.25 | 0.57 | 0.88 |
| 3. | Bauska | 0.32 | 0.00 | 0.85 | 0.27 | 0.30 | 0.29 | 0.57 | 0.60 | 0.29 | 1.14 | 0.55 | 0.87 |
| 4. | Daugavpils | 0.39 | 0.00 | 1.01 | 0.32 | 0.36 | 0.30 | 0.61 | 0.69 | 0.30 | 1.31 | 0.63 | 0.97 |
| 5. | Dobele | 0.31 | 0.00 | 0.83 | 0.28 | 0.29 | 0.29 | 0.57 | 0.60 | 0.29 | 1.12 | 0.56 | 0.86 |
| 6. | Gulbene | 0.39 | 0.00 | 0.99 | 0.30 | 0.35 | 0.28 | 0.56 | 0.67 | 0.28 | 1.27 | 0.58 | 0.91 |
| 7. | Jelgava | 0.29 | 0.00 | 0.76 | 0.25 | 0.27 | 0.28 | 0.57 | 0.58 | 0.28 | 1.05 | 0.53 | 0.83 |
| 8. | Kolka | 0.31 | 0.00 | 0.77 | 0.25 | 0.26 | 0.25 | 0.51 | 0.56 | 0.25 | 1.03 | 0.51 | 0.77 |
| 9. | Liepāja | 0.35 | 0.00 | 0.90 | 0.30 | 0.31 | 0.30 | 0.60 | 0.65 | 0.30 | 1.20 | 0.60 | 0.91 |
| 10. | Mērsrags | 0.32 | 0.00 | 0.79 | 0.26 | 0.27 | 0.27 | 0.53 | 0.59 | 0.27 | 1.05 | 0.52 | 0.80 |
| 11. | Pāvilosta | 0.39 | 0.00 | 1.02 | 0.33 | 0.35 | 0.28 | 0.55 | 0.66 | 0.28 | 1.30 | 0.61 | 0.91 |
| 12. | Priekuļi | 0.39 | 0.00 | 0.98 | 0.30 | 0.33 | 0.26 | 0.53 | 0.65 | 0.26 | 1.24 | 0.56 | 0.86 |
| 13. | Rēzekne | 0.39 | 0.00 | 1.05 | 0.34 | 0.37 | 0.29 | 0.58 | 0.68 | 0.29 | 1.33 | 0.62 | 0.95 |
| 14. | Rīga | 0.35 | 0.00 | 0.94 | 0.32 | 0.33 | 0.28 | 0.56 | 0.63 | 0.28 | 1.22 | 0.60 | 0.88 |
| 15. | Rūjiena | 0.36 | 0.00 | 0.94 | 0.30 | 0.32 | 0.26 | 0.51 | 0.62 | 0.26 | 1.19 | 0.55 | 0.83 |
| 16. | Saldus | 0.35 | 0.00 | 0.90 | 0.29 | 0.32 | 0.28 | 0.56 | 0.63 | 0.28 | 1.18 | 0.57 | 0.88 |
| 17. | Skrīveri | 0.37 | 0.00 | 0.95 | 0.30 | 0.33 | 0.28 | 0.57 | 0.66 | 0.28 | 1.24 | 0.59 | 0.90 |
| 18. | Skulte | 0.39 | 0.00 | 1.01 | 0.34 | 0.35 | 0.27 | 0.54 | 0.65 | 0.27 | 1.28 | 0.61 | 0.88 |
| 19. | Stende | 0.34 | 0.00 | 0.87 | 0.28 | 0.30 | 0.27 | 0.55 | 0.61 | 0.27 | 1.14 | 0.55 | 0.84 |
| 20. | Ventspils | 0.33 | 0.00 | 0.85 | 0.28 | 0.29 | 0.27 | 0.55 | 0.61 | 0.27 | 1.13 | 0.56 | 0.84 |
| 21. | Zīlāni | 0.37 | 0.00 | 0.95 | 0.30 | 0.33 | 0.29 | 0.58 | 0.66 | 0.29 | 1.24 | 0.59 | 0.91 |
| 22. | Zosēni | 0.38 | 0.00 | 0.97 | 0.31 | 0.33 | 0.27 | 0.54 | 0.65 | 0.27 | 1.24 | 0.58 | 0.88 |

Note. Table 17 provides information on the long-term monthly average daily sum of solar radiation on diversely oriented surfaces. Solar radiation data were obtained from long-term satellite measurements of the CMSAF SARAH 2.1 product at the locations of meteorological stations (CMSAF, 2019). The satellite data also take into account the physical state of the atmosphere, including cloudiness, which affects the amount of the measured solar radiation. The analysis is based on the solar radiation data in the period from 1988 to 2017. The global (or summed) solar radiation value is obtained by adding the direct and diffuse solar radiation values.

Table 18

Data on the Average Daily Sum of Solar Radiation on Diversely Oriented Surfaces in March (kWh/m2)

| No. | Station | Direct solar radiation |  |  |  |  | Diffuse solar radiation |  | Global solar radiation |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | orientation of the vertical wall |  |  |  | horizontal surface | orientation of the vertical wall | horizontal surface | orientation of the vertical wall |  |  |  | horizontal surface |
|  |  | East | North | South | West |  |  |  | East | North | South | West |  |
| 1. | Ainaži | 0.67 | 0.00 | 1.41 | 0.70 | 0.94 | 0.56 | 1.13 | 1.24 | 0.56 | 1.98 | 1.26 | 2.07 |
| 2. | Alūksne | 0.64 | 0.00 | 1.28 | 0.59 | 0.86 | 0.58 | 1.15 | 1.21 | 0.58 | 1.86 | 1.16 | 2.01 |
| 3. | Bauska | 0.60 | 0.00 | 1.22 | 0.58 | 0.84 | 0.58 | 1.17 | 1.18 | 0.58 | 1.80 | 1.17 | 2.00 |
| 4. | Daugavpils | 0.57 | 0.00 | 1.17 | 0.58 | 0.81 | 0.61 | 1.22 | 1.17 | 0.61 | 1.77 | 1.19 | 2.03 |
| 5. | Dobele | 0.56 | 0.00 | 1.20 | 0.60 | 0.83 | 0.58 | 1.16 | 1.14 | 0.58 | 1.78 | 1.18 | 1.99 |
| 6. | Gulbene | 0.61 | 0.00 | 1.21 | 0.57 | 0.82 | 0.59 | 1.17 | 1.20 | 0.59 | 1.80 | 1.15 | 1.99 |
| 7. | Jelgava | 0.59 | 0.00 | 1.17 | 0.57 | 0.81 | 0.58 | 1.16 | 1.17 | 0.58 | 1.75 | 1.15 | 1.97 |
| 8. | Kolka | 0.63 | 0.00 | 1.31 | 0.66 | 0.88 | 0.56 | 1.12 | 1.19 | 0.56 | 1.87 | 1.22 | 1.99 |
| 9. | Liepāja | 0.69 | 0.00 | 1.46 | 0.72 | 0.99 | 0.60 | 1.20 | 1.29 | 0.60 | 2.06 | 1.32 | 2.19 |
| 10. | Mērsrags | 0.65 | 0.00 | 1.31 | 0.67 | 0.88 | 0.57 | 1.15 | 1.22 | 0.57 | 1.88 | 1.24 | 2.03 |
| 11. | Pāvilosta | 0.74 | 0.00 | 1.53 | 0.75 | 1.02 | 0.58 | 1.16 | 1.32 | 0.58 | 2.11 | 1.33 | 2.18 |
| 12. | Priekuļi | 0.65 | 0.00 | 1.28 | 0.60 | 0.86 | 0.57 | 1.13 | 1.21 | 0.57 | 1.85 | 1.17 | 1.99 |
| 13. | Rēzekne | 0.59 | 0.00 | 1.18 | 0.55 | 0.81 | 0.59 | 1.18 | 1.18 | 0.59 | 1.77 | 1.14 | 1.99 |
| 14. | Rīga | 0.64 | 0.00 | 1.28 | 0.64 | 0.87 | 0.59 | 1.17 | 1.22 | 0.59 | 1.87 | 1.22 | 2.04 |
| 15. | Rūjiena | 0.63 | 0.00 | 1.22 | 0.58 | 0.82 | 0.56 | 1.12 | 1.19 | 0.56 | 1.78 | 1.15 | 1.94 |
| 16. | Saldus | 0.61 | 0.00 | 1.22 | 0.61 | 0.83 | 0.59 | 1.17 | 1.19 | 0.59 | 1.80 | 1.20 | 2.00 |
| 17. | Skrīveri | 0.61 | 0.00 | 1.25 | 0.59 | 0.85 | 0.58 | 1.17 | 1.20 | 0.58 | 1.83 | 1.18 | 2.02 |
| 18. | Skulte | 0.69 | 0.00 | 1.45 | 0.72 | 0.97 | 0.56 | 1.13 | 1.25 | 0.56 | 2.01 | 1.28 | 2.10 |
| 19. | Stende | 0.64 | 0.00 | 1.31 | 0.67 | 0.88 | 0.58 | 1.15 | 1.22 | 0.58 | 1.89 | 1.25 | 2.04 |
| 20. | Ventspils | 0.68 | 0.00 | 1.44 | 0.72 | 0.97 | 0.57 | 1.15 | 1.26 | 0.57 | 2.02 | 1.29 | 2.12 |
| 21. | Zīlāni | 0.62 | 0.00 | 1.24 | 0.58 | 0.85 | 0.59 | 1.17 | 1.20 | 0.59 | 1.82 | 1.17 | 2.02 |
| 22. | Zosēni | 0.64 | 0.00 | 1.29 | 0.60 | 0.87 | 0.57 | 1.15 | 1.21 | 0.57 | 1.86 | 1.17 | 2.01 |

Note. Table 18 provides information on the long-term monthly average daily sum of solar radiation on diversely oriented surfaces. Solar radiation data were obtained from long-term satellite measurements of the CMSAF SARAH 2.1 product at the locations of meteorological stations (CMSAF, 2019). The satellite data also take into account the physical state of the atmosphere, including cloudiness, which affects the amount of the measured solar radiation. The analysis is based on the solar radiation data in the period from 1988 to 2017. The global (or summed) solar radiation value is obtained by adding the direct and diffuse solar radiation values.

Table 19

Data on the Average Daily Sum of Solar Radiation on Diversely Oriented Surfaces in April (kWh/m2)

| No. | Station | Direct solar radiation |  |  |  |  | Diffuse solar radiation |  | Global solar radiation |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | orientation of the vertical wall |  |  |  | horizontal surface | orientation of the vertical wall | horizontal surface | orientation of the vertical wall |  |  |  | horizontal surface |
|  |  | East | North | South | West |  |  |  | East | North | South | West |  |
| 1. | Ainaži | 1.12 | 0.03 | 1.73 | 1.28 | 2.01 | 0.87 | 1.74 | 1.99 | 0.90 | 2.60 | 2.15 | 3.75 |
| 2. | Alūksne | 1.00 | 0.02 | 1.41 | 0.99 | 1.65 | 0.92 | 1.84 | 1.92 | 0.95 | 2.33 | 1.91 | 3.49 |
| 3. | Bauska | 1.10 | 0.03 | 1.60 | 1.15 | 1.89 | 0.90 | 1.80 | 2.00 | 0.93 | 2.50 | 2.05 | 3.69 |
| 4. | Daugavpils | 1.00 | 0.02 | 1.50 | 1.02 | 1.78 | 0.92 | 1.85 | 1.93 | 0.95 | 2.43 | 1.95 | 3.62 |
| 5. | Dobele | 1.13 | 0.03 | 1.63 | 1.15 | 1.92 | 0.90 | 1.79 | 2.02 | 0.92 | 2.52 | 2.05 | 3.71 |
| 6. | Gulbene | 1.03 | 0.03 | 1.49 | 1.04 | 1.73 | 0.92 | 1.83 | 1.95 | 0.94 | 2.41 | 1.96 | 3.57 |
| 7. | Jelgava | 1.10 | 0.03 | 1.61 | 1.16 | 1.90 | 0.89 | 1.79 | 2.00 | 0.92 | 2.50 | 2.05 | 3.68 |
| 8. | Kolka | 1.14 | 0.03 | 1.72 | 1.30 | 2.00 | 0.87 | 1.75 | 2.02 | 0.91 | 2.60 | 2.18 | 3.75 |
| 9. | Liepāja | 1.25 | 0.03 | 1.91 | 1.35 | 2.24 | 0.88 | 1.76 | 2.13 | 0.91 | 2.79 | 2.23 | 4.00 |
| 10. | Mērsrags | 1.08 | 0.03 | 1.62 | 1.22 | 1.90 | 0.87 | 1.74 | 1.95 | 0.90 | 2.49 | 2.09 | 3.64 |
| 11. | Pāvilosta | 1.22 | 0.03 | 1.83 | 1.35 | 2.14 | 0.88 | 1.75 | 2.10 | 0.91 | 2.70 | 2.23 | 3.89 |
| 12. | Priekuļi | 1.04 | 0.03 | 1.46 | 1.02 | 1.70 | 0.91 | 1.81 | 1.95 | 0.93 | 2.36 | 1.93 | 3.51 |
| 13. | Rēzekne | 1.01 | 0.02 | 1.48 | 1.01 | 1.73 | 0.92 | 1.85 | 1.93 | 0.95 | 2.40 | 1.93 | 3.58 |
| 14. | Rīga | 1.09 | 0.03 | 1.60 | 1.16 | 1.87 | 0.90 | 1.79 | 1.98 | 0.92 | 2.49 | 2.06 | 3.67 |
| 15. | Rūjiena | 1.04 | 0.03 | 1.50 | 1.11 | 1.74 | 0.90 | 1.80 | 1.94 | 0.93 | 2.40 | 2.00 | 3.54 |
| 16. | Saldus | 1.13 | 0.03 | 1.58 | 1.13 | 1.86 | 0.90 | 1.81 | 2.04 | 0.93 | 2.48 | 2.03 | 3.67 |
| 17. | Skrīveri | 1.05 | 0.03 | 1.48 | 1.04 | 1.74 | 0.91 | 1.82 | 1.95 | 0.93 | 2.39 | 1.95 | 3.56 |
| 18. | Skulte | 1.09 | 0.03 | 1.69 | 1.23 | 1.97 | 0.88 | 1.76 | 1.97 | 0.91 | 2.57 | 2.11 | 3.72 |
| 19. | Stende | 1.13 | 0.03 | 1.61 | 1.21 | 1.90 | 0.89 | 1.79 | 2.03 | 0.92 | 2.50 | 2.10 | 3.68 |
| 20. | Ventspils | 1.23 | 0.03 | 1.87 | 1.41 | 2.18 | 0.86 | 1.73 | 2.09 | 0.89 | 2.73 | 2.27 | 3.90 |
| 21. | Zīlāni | 1.04 | 0.02 | 1.50 | 1.05 | 1.76 | 0.91 | 1.83 | 1.95 | 0.94 | 2.41 | 1.96 | 3.59 |
| 22. | Zosēni | 1.01 | 0.03 | 1.43 | 1.03 | 1.68 | 0.92 | 1.83 | 1.93 | 0.94 | 2.35 | 1.94 | 3.51 |

Note. Table 19 provides information on the long-term monthly average daily sum of solar radiation on diversely oriented surfaces. Solar radiation data were obtained from long-term satellite measurements of the CMSAF SARAH 2.1 product at the locations of meteorological stations (CMSAF, 2019). The satellite data also take into account the physical state of the atmosphere, including cloudiness, which affects the amount of the measured solar radiation. The analysis is based on the solar radiation data in the period from 1988 to 2017. The global (or summed) solar radiation value is obtained by adding the direct and diffuse solar radiation values.

Table 20

Data on the Average Daily Sum of Solar Radiation on Diversely Oriented Surfaces in May (kWh/m2)

| No. | Station | Direct solar radiation |  |  |  |  | Diffuse solar radiation |  | Global solar radiation |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | orientation of the vertical wall |  |  |  | horizontal surface | orientation of the vertical wall | horizontal surface | orientation of the vertical wall |  |  |  | horizontal surface |
|  |  | East | North | South | West |  |  |  | East | North | South | West |  |
| 1. | Ainaži | 1.77 | 0.20 | 1.84 | 2.07 | 3.32 | 1.05 | 2.11 | 2.82 | 1.26 | 2.89 | 3.12 | 5.43 |
| 2. | Alūksne | 1.59 | 0.17 | 1.40 | 1.51 | 2.61 | 1.15 | 2.29 | 2.74 | 1.32 | 2.55 | 2.66 | 4.90 |
| 3. | Bauska | 1.62 | 0.17 | 1.57 | 1.67 | 2.91 | 1.13 | 2.27 | 2.76 | 1.31 | 2.70 | 2.81 | 5.18 |
| 4. | Daugavpils | 1.52 | 0.15 | 1.51 | 1.50 | 2.79 | 1.14 | 2.29 | 2.66 | 1.29 | 2.65 | 2.64 | 5.08 |
| 5. | Dobele | 1.66 | 0.18 | 1.58 | 1.70 | 2.94 | 1.13 | 2.27 | 2.79 | 1.31 | 2.72 | 2.84 | 5.21 |
| 6. | Gulbene | 1.63 | 0.17 | 1.49 | 1.56 | 2.75 | 1.14 | 2.28 | 2.77 | 1.31 | 2.63 | 2.70 | 5.03 |
| 7. | Jelgava | 1.63 | 0.17 | 1.58 | 1.71 | 2.94 | 1.13 | 2.26 | 2.76 | 1.30 | 2.71 | 2.84 | 5.20 |
| 8. | Kolka | 1.70 | 0.20 | 1.75 | 2.01 | 3.19 | 1.07 | 2.13 | 2.77 | 1.27 | 2.82 | 3.07 | 5.32 |
| 9. | Liepāja | 1.70 | 0.19 | 1.83 | 2.00 | 3.34 | 1.07 | 2.15 | 2.78 | 1.26 | 2.90 | 3.07 | 5.49 |
| 10. | Mērsrags | 1.67 | 0.19 | 1.61 | 1.81 | 2.97 | 1.07 | 2.13 | 2.74 | 1.26 | 2.68 | 2.88 | 5.10 |
| 11. | Pāvilosta | 1.67 | 0.19 | 1.76 | 1.96 | 3.23 | 1.08 | 2.16 | 2.75 | 1.27 | 2.84 | 3.04 | 5.39 |
| 12. | Priekuļi | 1.64 | 0.18 | 1.44 | 1.55 | 2.68 | 1.14 | 2.27 | 2.77 | 1.32 | 2.58 | 2.69 | 4.95 |
| 13. | Rēzekne | 1.53 | 0.16 | 1.47 | 1.52 | 2.71 | 1.14 | 2.28 | 2.67 | 1.30 | 2.61 | 2.66 | 4.99 |
| 14. | Rīga | 1.64 | 0.18 | 1.58 | 1.73 | 2.92 | 1.12 | 2.24 | 2.75 | 1.30 | 2.70 | 2.85 | 5.16 |
| 15. | Rūjiena | 1.69 | 0.20 | 1.52 | 1.71 | 2.81 | 1.12 | 2.24 | 2.81 | 1.32 | 2.64 | 2.83 | 5.05 |
| 16. | Saldus | 1.63 | 0.18 | 1.48 | 1.64 | 2.78 | 1.15 | 2.30 | 2.77 | 1.33 | 2.63 | 2.79 | 5.07 |
| 17. | Skrīveri | 1.58 | 0.17 | 1.44 | 1.52 | 2.69 | 1.15 | 2.29 | 2.72 | 1.31 | 2.59 | 2.67 | 4.98 |
| 18. | Skulte | 1.69 | 0.19 | 1.73 | 1.91 | 3.16 | 1.08 | 2.15 | 2.77 | 1.26 | 2.81 | 2.99 | 5.31 |
| 19. | Stende | 1.66 | 0.19 | 1.52 | 1.73 | 2.83 | 1.12 | 2.24 | 2.78 | 1.31 | 2.64 | 2.85 | 5.07 |
| 20. | Ventspils | 1.74 | 0.20 | 1.84 | 2.04 | 3.34 | 1.05 | 2.11 | 2.79 | 1.25 | 2.90 | 3.10 | 5.45 |
| 21. | Zīlāni | 1.57 | 0.16 | 1.49 | 1.49 | 2.74 | 1.14 | 2.27 | 2.70 | 1.29 | 2.62 | 2.63 | 5.01 |
| 22. | Zosēni | 1.60 | 0.17 | 1.43 | 1.50 | 2.65 | 1.15 | 2.29 | 2.74 | 1.32 | 2.57 | 2.65 | 4.94 |

Note. Table 20 provides information on the long-term monthly average daily sum of solar radiation on diversely oriented surfaces. Solar radiation data were obtained from long-term satellite measurements of the CMSAF SARAH 2.1 product at the locations of meteorological stations (CMSAF, 2019). The satellite data also take into account the physical state of the atmosphere, including cloudiness, which affects the amount of the measured solar radiation. The analysis is based on the solar radiation data in the period from 1988 to 2017. The global (or summed) solar radiation value is obtained by adding the direct and diffuse solar radiation values.

Table 21

Data on the Average Daily Sum of Solar Radiation on Diversely Oriented Surfaces in June (kWh/m2)

| No. | Station | Direct solar radiation |  |  |  |  | Diffuse solar radiation |  | Global solar radiation |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | orientation of the vertical wall |  |  |  | horizontal surface | orientation of the vertical wall | horizontal surface | orientation of the vertical wall |  |  |  | horizontal surface |
|  |  | East | North | South | West |  |  |  | East | North | South | West |  |
| 1. | Ainaži | 1.70 | 0.31 | 1.52 | 2.10 | 3.41 | 1.19 | 2.39 | 2.89 | 1.50 | 2.71 | 3.29 | 5.79 |
| 2. | Alūksne | 1.59 | 0.30 | 1.13 | 1.61 | 2.68 | 1.30 | 2.60 | 2.89 | 1.60 | 2.44 | 2.91 | 5.28 |
| 3. | Bauska | 1.68 | 0.30 | 1.29 | 1.76 | 3.05 | 1.26 | 2.53 | 2.94 | 1.56 | 2.55 | 3.02 | 5.57 |
| 4. | Daugavpils | 1.55 | 0.26 | 1.21 | 1.59 | 2.86 | 1.28 | 2.57 | 2.83 | 1.55 | 2.49 | 2.88 | 5.42 |
| 5. | Dobele | 1.71 | 0.30 | 1.29 | 1.76 | 3.04 | 1.27 | 2.54 | 2.98 | 1.57 | 2.56 | 3.03 | 5.59 |
| 6. | Gulbene | 1.62 | 0.30 | 1.19 | 1.67 | 2.81 | 1.30 | 2.59 | 2.92 | 1.60 | 2.49 | 2.96 | 5.40 |
| 7. | Jelgava | 1.67 | 0.29 | 1.29 | 1.75 | 3.04 | 1.26 | 2.52 | 2.93 | 1.55 | 2.55 | 3.01 | 5.56 |
| 8. | Kolka | 1.79 | 0.33 | 1.46 | 2.05 | 3.34 | 1.19 | 2.38 | 2.98 | 1.52 | 2.65 | 3.24 | 5.73 |
| 9. | Liepāja | 1.73 | 0.31 | 1.56 | 2.12 | 3.57 | 1.20 | 2.39 | 2.93 | 1.51 | 2.75 | 3.32 | 5.96 |
| 10. | Mērsrags | 1.67 | 0.31 | 1.29 | 1.80 | 3.00 | 1.21 | 2.42 | 2.88 | 1.52 | 2.50 | 3.01 | 5.42 |
| 11. | Pāvilosta | 1.74 | 0.32 | 1.47 | 2.08 | 3.41 | 1.20 | 2.41 | 2.94 | 1.52 | 2.68 | 3.28 | 5.82 |
| 12. | Priekuļi | 1.55 | 0.28 | 1.16 | 1.61 | 2.72 | 1.29 | 2.58 | 2.84 | 1.57 | 2.45 | 2.91 | 5.30 |
| 13. | Rēzekne | 1.59 | 0.30 | 1.18 | 1.61 | 2.79 | 1.29 | 2.58 | 2.88 | 1.59 | 2.47 | 2.90 | 5.37 |
| 14. | Rīga | 1.64 | 0.29 | 1.25 | 1.74 | 2.93 | 1.27 | 2.54 | 2.90 | 1.56 | 2.52 | 3.01 | 5.47 |
| 15. | Rūjiena | 1.64 | 0.31 | 1.24 | 1.76 | 2.87 | 1.28 | 2.56 | 2.92 | 1.59 | 2.52 | 3.05 | 5.44 |
| 16. | Saldus | 1.63 | 0.30 | 1.19 | 1.68 | 2.84 | 1.28 | 2.56 | 2.91 | 1.58 | 2.47 | 2.96 | 5.40 |
| 17. | Skrīveri | 1.57 | 0.28 | 1.17 | 1.62 | 2.78 | 1.29 | 2.58 | 2.86 | 1.57 | 2.46 | 2.91 | 5.36 |
| 18. | Skulte | 1.65 | 0.29 | 1.43 | 1.95 | 3.25 | 1.20 | 2.40 | 2.85 | 1.49 | 2.63 | 3.14 | 5.65 |
| 19. | Stende | 1.67 | 0.31 | 1.22 | 1.72 | 2.88 | 1.27 | 2.55 | 2.94 | 1.58 | 2.49 | 3.00 | 5.43 |
| 20. | Ventspils | 1.78 | 0.33 | 1.55 | 2.18 | 3.53 | 1.18 | 2.36 | 2.96 | 1.51 | 2.73 | 3.36 | 5.89 |
| 21. | Zīlāni | 1.56 | 0.27 | 1.19 | 1.60 | 2.80 | 1.28 | 2.57 | 2.85 | 1.56 | 2.47 | 2.88 | 5.37 |
| 22. | Zosēni | 1.56 | 0.29 | 1.15 | 1.61 | 2.70 | 1.30 | 2.60 | 2.85 | 1.59 | 2.44 | 2.91 | 5.30 |

Note. Table 21 provides information on the long-term monthly average daily sum of solar radiation on diversely oriented surfaces. Solar radiation data were obtained from long-term satellite measurements of the CMSAF SARAH 2.1 product at the locations of meteorological stations (CMSAF, 2019). The satellite data also take into account the physical state of the atmosphere, including cloudiness, which affects the amount of the measured solar radiation. The analysis is based on the solar radiation data in the period from 1988 to 2017. The global (or summed) solar radiation value is obtained by adding the direct and diffuse solar radiation values.

Table 22

Data on the Average Daily Sum of Solar Radiation on Diversely Oriented Surfaces in July (kWh/m2)

| No. | Station | Direct solar radiation |  |  |  |  | Diffuse solar radiation |  | Global solar radiation |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | orientation of the vertical wall |  |  |  | horizontal surface | orientation of the vertical wall | horizontal surface | orientation of the vertical wall |  |  |  | horizontal surface |
|  |  | East | North | South | West |  |  |  | East | North | South | West |  |
| 1. | Ainaži | 1.60 | 0.22 | 1.64 | 1.99 | 3.27 | 1.19 | 2.38 | 2.79 | 1.41 | 2.83 | 3.18 | 5.65 |
| 2. | Alūksne | 1.52 | 0.22 | 1.28 | 1.58 | 2.67 | 1.27 | 2.55 | 2.80 | 1.49 | 2.55 | 2.85 | 5.21 |
| 3. | Bauska | 1.55 | 0.21 | 1.42 | 1.69 | 2.95 | 1.25 | 2.50 | 2.80 | 1.46 | 2.67 | 2.94 | 5.45 |
| 4. | Daugavpils | 1.45 | 0.19 | 1.33 | 1.58 | 2.80 | 1.26 | 2.51 | 2.71 | 1.45 | 2.59 | 2.84 | 5.32 |
| 5. | Dobele | 1.62 | 0.22 | 1.42 | 1.73 | 2.98 | 1.25 | 2.51 | 2.87 | 1.48 | 2.67 | 2.98 | 5.49 |
| 6. | Gulbene | 1.55 | 0.21 | 1.33 | 1.61 | 2.77 | 1.27 | 2.53 | 2.82 | 1.48 | 2.60 | 2.87 | 5.31 |
| 7. | Jelgava | 1.54 | 0.21 | 1.41 | 1.71 | 2.94 | 1.25 | 2.50 | 2.79 | 1.46 | 2.66 | 2.97 | 5.44 |
| 8. | Kolka | 1.66 | 0.24 | 1.49 | 1.87 | 3.07 | 1.20 | 2.40 | 2.86 | 1.44 | 2.68 | 3.07 | 5.47 |
| 9. | Liepāja | 1.59 | 0.21 | 1.57 | 1.84 | 3.21 | 1.22 | 2.45 | 2.82 | 1.44 | 2.79 | 3.06 | 5.66 |
| 10. | Mērsrags | 1.61 | 0.23 | 1.38 | 1.71 | 2.88 | 1.20 | 2.41 | 2.81 | 1.43 | 2.58 | 2.91 | 5.28 |
| 11. | Pāvilosta | 1.58 | 0.22 | 1.48 | 1.78 | 3.05 | 1.24 | 2.47 | 2.81 | 1.45 | 2.71 | 3.02 | 5.52 |
| 12. | Priekuļi | 1.50 | 0.21 | 1.26 | 1.56 | 2.65 | 1.27 | 2.54 | 2.78 | 1.48 | 2.54 | 2.84 | 5.19 |
| 13. | Rēzekne | 1.49 | 0.20 | 1.31 | 1.58 | 2.74 | 1.27 | 2.53 | 2.76 | 1.47 | 2.57 | 2.84 | 5.27 |
| 14. | Rīga | 1.51 | 0.22 | 1.35 | 1.66 | 2.81 | 1.27 | 2.54 | 2.78 | 1.49 | 2.63 | 2.93 | 5.36 |
| 15. | Rūjiena | 1.54 | 0.22 | 1.34 | 1.73 | 2.80 | 1.26 | 2.52 | 2.80 | 1.48 | 2.60 | 2.99 | 5.32 |
| 16. | Saldus | 1.53 | 0.22 | 1.28 | 1.58 | 2.71 | 1.28 | 2.56 | 2.81 | 1.50 | 2.56 | 2.86 | 5.27 |
| 17. | Skrīveri | 1.47 | 0.20 | 1.29 | 1.59 | 2.71 | 1.27 | 2.54 | 2.74 | 1.47 | 2.56 | 2.86 | 5.25 |
| 18. | Skulte | 1.54 | 0.21 | 1.52 | 1.82 | 3.08 | 1.22 | 2.43 | 2.75 | 1.43 | 2.74 | 3.03 | 5.51 |
| 19. | Stende | 1.55 | 0.23 | 1.27 | 1.64 | 2.71 | 1.28 | 2.56 | 2.83 | 1.50 | 2.55 | 2.91 | 5.26 |
| 20. | Ventspils | 1.64 | 0.23 | 1.57 | 1.93 | 3.21 | 1.21 | 2.41 | 2.84 | 1.44 | 2.78 | 3.13 | 5.62 |
| 21. | Zīlāni | 1.47 | 0.20 | 1.30 | 1.59 | 2.73 | 1.27 | 2.53 | 2.74 | 1.47 | 2.57 | 2.86 | 5.27 |
| 22. | Zosēni | 1.49 | 0.20 | 1.27 | 1.54 | 2.65 | 1.27 | 2.54 | 2.76 | 1.48 | 2.54 | 2.81 | 5.20 |

Note. Table 22 provides information on the long-term monthly average daily sum of solar radiation on diversely oriented surfaces. Solar radiation data were obtained from long-term satellite measurements of the CMSAF SARAH 2.1 product at the locations of meteorological stations (CMSAF, 2019). The satellite data also take into account the physical state of the atmosphere, including cloudiness, which affects the amount of the measured solar radiation. The analysis is based on the solar radiation data in the period from 1988 to 2017. The global (or summed) solar radiation value is obtained by adding the direct and diffuse solar radiation values.

Table 23

Data on the Average Daily Sum of Solar Radiation on Diversely Oriented Surfaces in August (kWh/m2)

| No. | Station | Direct solar radiation |  |  |  |  | Diffuse solar radiation (kWh/m2) |  | Global solar radiation (kWh/m2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | orientation of the vertical wall |  |  |  | horizontal surface | orientation of the vertical wall | horizontal surface | orientation of the vertical wall |  |  |  | horizontal surface |
|  |  | East | North | South | West |  |  |  | East | North | South | West |  |
| 1. | Ainaži | 1.31 | 0.07 | 1.76 | 1.55 | 2.44 | 0.99 | 1.99 | 2.30 | 1.06 | 2.75 | 2.54 | 4.43 |
| 2. | Alūksne | 1.15 | 0.06 | 1.46 | 1.30 | 2.06 | 1.03 | 2.06 | 2.19 | 1.10 | 2.49 | 2.33 | 4.12 |
| 3. | Bauska | 1.28 | 0.06 | 1.62 | 1.35 | 2.29 | 1.04 | 2.09 | 2.32 | 1.10 | 2.66 | 2.39 | 4.38 |
| 4. | Daugavpils | 1.18 | 0.06 | 1.58 | 1.33 | 2.25 | 1.05 | 2.09 | 2.22 | 1.10 | 2.62 | 2.38 | 4.34 |
| 5. | Dobele | 1.34 | 0.06 | 1.64 | 1.35 | 2.32 | 1.05 | 2.10 | 2.38 | 1.11 | 2.69 | 2.40 | 4.41 |
| 6. | Gulbene | 1.21 | 0.06 | 1.51 | 1.30 | 2.14 | 1.04 | 2.08 | 2.25 | 1.10 | 2.55 | 2.34 | 4.21 |
| 7. | Jelgava | 1.28 | 0.06 | 1.61 | 1.34 | 2.28 | 1.05 | 2.09 | 2.33 | 1.11 | 2.65 | 2.39 | 4.37 |
| 8. | Kolka | 1.32 | 0.07 | 1.65 | 1.47 | 2.32 | 1.01 | 2.02 | 2.33 | 1.08 | 2.66 | 2.48 | 4.34 |
| 9. | Liepāja | 1.31 | 0.06 | 1.73 | 1.44 | 2.43 | 1.03 | 2.07 | 2.35 | 1.10 | 2.76 | 2.47 | 4.50 |
| 10. | Mērsrags | 1.30 | 0.07 | 1.60 | 1.39 | 2.26 | 0.99 | 1.99 | 2.29 | 1.06 | 2.60 | 2.39 | 4.25 |
| 11. | Pāvilosta | 1.27 | 0.06 | 1.60 | 1.37 | 2.26 | 1.05 | 2.11 | 2.32 | 1.12 | 2.65 | 2.42 | 4.37 |
| 12. | Priekuļi | 1.21 | 0.07 | 1.47 | 1.30 | 2.09 | 1.04 | 2.08 | 2.25 | 1.10 | 2.51 | 2.34 | 4.17 |
| 13. | Rēzekne | 1.15 | 0.06 | 1.52 | 1.30 | 2.15 | 1.04 | 2.08 | 2.19 | 1.10 | 2.56 | 2.34 | 4.23 |
| 14. | Rīga | 1.25 | 0.06 | 1.53 | 1.33 | 2.18 | 1.06 | 2.11 | 2.30 | 1.12 | 2.59 | 2.38 | 4.29 |
| 15. | Rūjiena | 1.22 | 0.07 | 1.51 | 1.39 | 2.13 | 1.04 | 2.07 | 2.25 | 1.10 | 2.54 | 2.42 | 4.20 |
| 16. | Saldus | 1.25 | 0.06 | 1.53 | 1.31 | 2.17 | 1.06 | 2.11 | 2.31 | 1.12 | 2.58 | 2.36 | 4.28 |
| 17. | Skrīveri | 1.18 | 0.06 | 1.53 | 1.34 | 2.17 | 1.04 | 2.08 | 2.22 | 1.10 | 2.57 | 2.38 | 4.25 |
| 18. | Skulte | 1.26 | 0.06 | 1.68 | 1.43 | 2.35 | 1.01 | 2.02 | 2.27 | 1.07 | 2.69 | 2.44 | 4.36 |
| 19. | Stende | 1.26 | 0.07 | 1.50 | 1.32 | 2.14 | 1.06 | 2.12 | 2.31 | 1.13 | 2.56 | 2.38 | 4.26 |
| 20. | Ventspils | 1.31 | 0.06 | 1.69 | 1.47 | 2.37 | 1.03 | 2.07 | 2.34 | 1.10 | 2.72 | 2.50 | 4.44 |
| 21. | Zīlāni | 1.17 | 0.06 | 1.52 | 1.32 | 2.16 | 1.04 | 2.09 | 2.21 | 1.10 | 2.57 | 2.36 | 4.25 |
| 22. | Zosēni | 1.18 | 0.06 | 1.45 | 1.28 | 2.07 | 1.04 | 2.08 | 2.22 | 1.10 | 2.49 | 2.32 | 4.15 |

Note. Table 23 provides information on the long-term monthly average daily sum of solar radiation on diversely oriented surfaces. Solar radiation data were obtained from long-term satellite measurements of the CMSAF SARAH 2.1 product at the locations of meteorological stations (CMSAF, 2019). The satellite data also take into account the physical state of the atmosphere, including cloudiness, which affects the amount of the measured solar radiation. The analysis is based on the solar radiation data in the period from 1988 to 2017. The global (or summed) solar radiation value is obtained by adding the direct and diffuse solar radiation values.

Table 24

Data on the Average Daily Sum of Solar Radiation on Diversely Oriented Surfaces in September (kWh/m2)

| No. | Station | Direct solar radiation (kWh/m2) |  |  |  |  | Diffuse solar radiation |  | Global solar radiation |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | orientation of the vertical wall |  |  |  | horizontal surface | orientation of the vertical wall | horizontal surface | orientation of the vertical wall |  |  |  | horizontal surface |
|  |  | East | North | South | West |  |  |  | East | North | South | West |  |
| 1. | Ainaži | 0.88 | 0.00 | 1.62 | 0.88 | 1.33 | 0.70 | 1.39 | 1.58 | 0.70 | 2.31 | 1.58 | 2.72 |
| 2. | Alūksne | 0.75 | 0.00 | 1.35 | 0.74 | 1.11 | 0.70 | 1.40 | 1.45 | 0.70 | 2.05 | 1.44 | 2.51 |
| 3. | Bauska | 0.91 | 0.00 | 1.61 | 0.86 | 1.34 | 0.73 | 1.45 | 1.64 | 0.73 | 2.34 | 1.59 | 2.79 |
| 4. | Daugavpils | 0.80 | 0.00 | 1.53 | 0.80 | 1.27 | 0.73 | 1.46 | 1.53 | 0.73 | 2.26 | 1.53 | 2.72 |
| 5. | Dobele | 0.95 | 0.00 | 1.66 | 0.88 | 1.37 | 0.73 | 1.46 | 1.68 | 0.73 | 2.39 | 1.61 | 2.83 |
| 6. | Gulbene | 0.78 | 0.00 | 1.41 | 0.78 | 1.16 | 0.71 | 1.42 | 1.49 | 0.71 | 2.12 | 1.49 | 2.58 |
| 7. | Jelgava | 0.91 | 0.00 | 1.61 | 0.85 | 1.33 | 0.73 | 1.45 | 1.64 | 0.73 | 2.34 | 1.58 | 2.79 |
| 8. | Kolka | 0.87 | 0.00 | 1.53 | 0.84 | 1.26 | 0.70 | 1.40 | 1.57 | 0.70 | 2.23 | 1.54 | 2.66 |
| 9. | Liepāja | 0.95 | 0.00 | 1.75 | 0.92 | 1.45 | 0.73 | 1.46 | 1.69 | 0.73 | 2.48 | 1.65 | 2.91 |
| 10. | Mērsrags | 0.88 | 0.00 | 1.53 | 0.82 | 1.26 | 0.69 | 1.39 | 1.57 | 0.69 | 2.22 | 1.52 | 2.65 |
| 11. | Pāvilosta | 0.91 | 0.00 | 1.59 | 0.86 | 1.31 | 0.74 | 1.47 | 1.65 | 0.74 | 2.33 | 1.60 | 2.79 |
| 12. | Priekuļi | 0.80 | 0.00 | 1.40 | 0.77 | 1.16 | 0.71 | 1.41 | 1.51 | 0.71 | 2.10 | 1.48 | 2.57 |
| 13. | Rēzekne | 0.76 | 0.00 | 1.44 | 0.78 | 1.19 | 0.71 | 1.42 | 1.47 | 0.71 | 2.15 | 1.49 | 2.62 |
| 14. | Rīga | 0.85 | 0.00 | 1.49 | 0.82 | 1.24 | 0.73 | 1.46 | 1.58 | 0.73 | 2.22 | 1.55 | 2.69 |
| 15. | Rūjiena | 0.81 | 0.00 | 1.40 | 0.78 | 1.15 | 0.71 | 1.42 | 1.51 | 0.71 | 2.11 | 1.49 | 2.57 |
| 16. | Saldus | 0.91 | 0.00 | 1.56 | 0.83 | 1.29 | 0.73 | 1.47 | 1.64 | 0.73 | 2.29 | 1.56 | 2.75 |
| 17. | Skrīveri | 0.78 | 0.00 | 1.42 | 0.79 | 1.18 | 0.72 | 1.44 | 1.49 | 0.72 | 2.14 | 1.51 | 2.62 |
| 18. | Skulte | 0.86 | 0.00 | 1.58 | 0.85 | 1.30 | 0.71 | 1.42 | 1.57 | 0.71 | 2.29 | 1.56 | 2.72 |
| 19. | Stende | 0.89 | 0.00 | 1.49 | 0.82 | 1.23 | 0.73 | 1.45 | 1.62 | 0.73 | 2.22 | 1.55 | 2.69 |
| 20. | Ventspils | 0.90 | 0.00 | 1.63 | 0.91 | 1.34 | 0.72 | 1.44 | 1.62 | 0.72 | 2.35 | 1.63 | 2.78 |
| 21. | Zīlāni | 0.77 | 0.00 | 1.43 | 0.79 | 1.19 | 0.72 | 1.44 | 1.49 | 0.72 | 2.15 | 1.50 | 2.62 |
| 22. | Zosēni | 0.77 | 0.00 | 1.38 | 0.77 | 1.14 | 0.71 | 1.41 | 1.47 | 0.71 | 2.08 | 1.47 | 2.55 |

Note. Table 24 provides information on the long-term monthly average daily sum of solar radiation on diversely oriented surfaces. Solar radiation data were obtained from long-term satellite measurements of the CMSAF SARAH 2.1 product at the locations of meteorological stations (CMSAF, 2019). The satellite data also take into account the physical state of the atmosphere, including cloudiness, which affects the amount of the measured solar radiation. The analysis is based on the solar radiation data in the period from 1988 to 2017. The global (or summed) solar radiation value is obtained by adding the direct and diffuse solar radiation values.

Table 25

Data on the Average Daily Sum of Solar Radiation on Diversely Oriented Surfaces in October (kWh/m2)

| No. | Station | Direct solar radiation |  |  |  |  | Diffuse solar radiation |  | Global solar radiation |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | orientation of the vertical wall |  |  |  | horizontal surface | orientation of the vertical wall | horizontal surface | orientation of the vertical wall |  |  |  | horizontal surface |
|  |  | East | North | South | West |  |  |  | East | North | South | West |  |
| 1. | Ainaži | 0.47 | 0.00 | 1.10 | 0.40 | 0.50 | 0.35 | 0.70 | 0.82 | 0.35 | 1.45 | 0.75 | 1.20 |
| 2. | Alūksne | 0.38 | 0.00 | 0.91 | 0.36 | 0.41 | 0.35 | 0.70 | 0.73 | 0.35 | 1.26 | 0.71 | 1.12 |
| 3. | Bauska | 0.56 | 0.00 | 1.35 | 0.51 | 0.62 | 0.39 | 0.78 | 0.94 | 0.39 | 1.73 | 0.89 | 1.40 |
| 4. | Daugavpils | 0.46 | 0.00 | 1.16 | 0.45 | 0.54 | 0.39 | 0.78 | 0.84 | 0.39 | 1.55 | 0.84 | 1.32 |
| 5. | Dobele | 0.55 | 0.00 | 1.36 | 0.51 | 0.62 | 0.39 | 0.78 | 0.95 | 0.39 | 1.75 | 0.90 | 1.41 |
| 6. | Gulbene | 0.41 | 0.00 | 1.00 | 0.40 | 0.46 | 0.36 | 0.72 | 0.77 | 0.36 | 1.37 | 0.76 | 1.18 |
| 7. | Jelgava | 0.55 | 0.00 | 1.33 | 0.50 | 0.61 | 0.39 | 0.78 | 0.94 | 0.39 | 1.72 | 0.88 | 1.39 |
| 8. | Kolka | 0.47 | 0.00 | 1.10 | 0.41 | 0.50 | 0.36 | 0.72 | 0.83 | 0.36 | 1.46 | 0.77 | 1.23 |
| 9. | Liepāja | 0.53 | 0.00 | 1.28 | 0.46 | 0.59 | 0.40 | 0.79 | 0.93 | 0.40 | 1.67 | 0.86 | 1.38 |
| 10. | Mērsrags | 0.50 | 0.00 | 1.15 | 0.43 | 0.53 | 0.37 | 0.73 | 0.86 | 0.37 | 1.52 | 0.79 | 1.26 |
| 11. | Pāvilosta | 0.51 | 0.00 | 1.20 | 0.44 | 0.55 | 0.38 | 0.77 | 0.89 | 0.38 | 1.59 | 0.82 | 1.32 |
| 12. | Priekuļi | 0.44 | 0.00 | 1.05 | 0.40 | 0.48 | 0.35 | 0.71 | 0.79 | 0.35 | 1.41 | 0.76 | 1.19 |
| 13. | Rēzekne | 0.42 | 0.00 | 1.05 | 0.40 | 0.49 | 0.37 | 0.75 | 0.80 | 0.37 | 1.42 | 0.78 | 1.23 |
| 14. | Rīga | 0.52 | 0.00 | 1.21 | 0.45 | 0.56 | 0.38 | 0.76 | 0.89 | 0.38 | 1.59 | 0.83 | 1.32 |
| 15. | Rūjiena | 0.44 | 0.00 | 1.02 | 0.38 | 0.46 | 0.35 | 0.70 | 0.79 | 0.35 | 1.37 | 0.73 | 1.16 |
| 16. | Saldus | 0.52 | 0.00 | 1.24 | 0.46 | 0.57 | 0.39 | 0.78 | 0.91 | 0.39 | 1.63 | 0.85 | 1.35 |
| 17. | Skrīveri | 0.46 | 0.00 | 1.14 | 0.44 | 0.52 | 0.37 | 0.75 | 0.83 | 0.37 | 1.51 | 0.82 | 1.27 |
| 18. | Skulte | 0.49 | 0.00 | 1.17 | 0.44 | 0.54 | 0.37 | 0.73 | 0.85 | 0.37 | 1.54 | 0.81 | 1.27 |
| 19. | Stende | 0.52 | 0.00 | 1.20 | 0.45 | 0.55 | 0.38 | 0.75 | 0.89 | 0.38 | 1.57 | 0.82 | 1.30 |
| 20. | Ventspils | 0.51 | 0.00 | 1.21 | 0.45 | 0.56 | 0.37 | 0.74 | 0.88 | 0.37 | 1.59 | 0.82 | 1.30 |
| 21. | Zīlāni | 0.44 | 0.00 | 1.11 | 0.44 | 0.51 | 0.38 | 0.76 | 0.82 | 0.38 | 1.49 | 0.82 | 1.27 |
| 22. | Zosēni | 0.40 | 0.00 | 1.00 | 0.40 | 0.46 | 0.35 | 0.71 | 0.75 | 0.35 | 1.35 | 0.75 | 1.17 |

Note. Table 25 provides information on the long-term monthly average daily sum of solar radiation on diversely oriented surfaces. Solar radiation data were obtained from long-term satellite measurements of the CMSAF SARAH 2.1 product at the locations of meteorological stations (CMSAF, 2019). The satellite data also take into account the physical state of the atmosphere, including cloudiness, which affects the amount of the measured solar radiation. The analysis is based on the solar radiation data in the period from 1988 to 2017. The global (or summed) solar radiation value is obtained by adding the direct and diffuse solar radiation values.

Table 26

Data on the Average Daily Sum of Solar Radiation on Diversely Oriented Surfaces in November (kWh/m2)

| No. | Station | Direct solar radiation |  |  |  |  | Diffuse solar radiation |  | Global solar radiation |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | orientation of the vertical wall |  |  |  | horizontal surface | orientation of the vertical wall | horizontal surface | orientation of the vertical wall |  |  |  | horizontal surface |
|  |  | East | North | South | West |  |  |  | East | North | South | West |  |
| 1. | Ainaži | 0.17 | 0.00 | 0.49 | 0.14 | 0.12 | 0.14 | 0.28 | 0.30 | 0.14 | 0.63 | 0.28 | 0.39 |
| 2. | Alūksne | 0.16 | 0.00 | 0.47 | 0.14 | 0.11 | 0.14 | 0.29 | 0.30 | 0.14 | 0.61 | 0.28 | 0.40 |
| 3. | Bauska | 0.20 | 0.00 | 0.59 | 0.18 | 0.15 | 0.16 | 0.33 | 0.36 | 0.16 | 0.75 | 0.34 | 0.48 |
| 4. | Daugavpils | 0.19 | 0.00 | 0.55 | 0.17 | 0.14 | 0.17 | 0.34 | 0.37 | 0.17 | 0.72 | 0.34 | 0.49 |
| 5. | Dobele | 0.20 | 0.00 | 0.58 | 0.17 | 0.15 | 0.16 | 0.33 | 0.36 | 0.16 | 0.74 | 0.34 | 0.47 |
| 6. | Gulbene | 0.17 | 0.00 | 0.50 | 0.15 | 0.12 | 0.15 | 0.30 | 0.32 | 0.15 | 0.65 | 0.30 | 0.43 |
| 7. | Jelgava | 0.19 | 0.00 | 0.57 | 0.17 | 0.15 | 0.16 | 0.32 | 0.35 | 0.16 | 0.73 | 0.33 | 0.47 |
| 8. | Kolka | 0.17 | 0.00 | 0.51 | 0.15 | 0.12 | 0.14 | 0.29 | 0.31 | 0.14 | 0.65 | 0.29 | 0.41 |
| 9. | Liepāja | 0.21 | 0.00 | 0.63 | 0.19 | 0.16 | 0.17 | 0.34 | 0.38 | 0.17 | 0.80 | 0.36 | 0.50 |
| 10. | Mērsrags | 0.18 | 0.00 | 0.57 | 0.16 | 0.14 | 0.15 | 0.30 | 0.33 | 0.15 | 0.72 | 0.31 | 0.44 |
| 11. | Pāvilosta | 0.20 | 0.00 | 0.59 | 0.18 | 0.14 | 0.16 | 0.32 | 0.36 | 0.16 | 0.75 | 0.34 | 0.47 |
| 12. | Priekuļi | 0.17 | 0.00 | 0.51 | 0.15 | 0.12 | 0.15 | 0.29 | 0.32 | 0.15 | 0.65 | 0.29 | 0.42 |
| 13. | Rēzekne | 0.17 | 0.00 | 0.52 | 0.16 | 0.13 | 0.16 | 0.32 | 0.33 | 0.16 | 0.68 | 0.32 | 0.45 |
| 14. | Rīga | 0.18 | 0.00 | 0.55 | 0.17 | 0.14 | 0.16 | 0.32 | 0.33 | 0.16 | 0.70 | 0.33 | 0.45 |
| 15. | Rūjiena | 0.15 | 0.00 | 0.45 | 0.13 | 0.11 | 0.14 | 0.28 | 0.29 | 0.14 | 0.59 | 0.27 | 0.38 |
| 16. | Saldus | 0.19 | 0.00 | 0.55 | 0.17 | 0.14 | 0.16 | 0.32 | 0.35 | 0.16 | 0.72 | 0.33 | 0.46 |
| 17. | Skrīveri | 0.18 | 0.00 | 0.52 | 0.16 | 0.13 | 0.16 | 0.32 | 0.34 | 0.16 | 0.68 | 0.32 | 0.45 |
| 18. | Skulte | 0.17 | 0.00 | 0.54 | 0.17 | 0.13 | 0.15 | 0.30 | 0.32 | 0.15 | 0.69 | 0.31 | 0.43 |
| 19. | Stende | 0.18 | 0.00 | 0.54 | 0.16 | 0.13 | 0.15 | 0.31 | 0.33 | 0.15 | 0.70 | 0.32 | 0.44 |
| 20. | Ventspils | 0.19 | 0.00 | 0.56 | 0.16 | 0.14 | 0.15 | 0.31 | 0.34 | 0.15 | 0.71 | 0.31 | 0.44 |
| 21. | Zīlāni | 0.18 | 0.00 | 0.53 | 0.16 | 0.13 | 0.16 | 0.32 | 0.34 | 0.16 | 0.69 | 0.32 | 0.46 |
| 22. | Zosēni | 0.16 | 0.00 | 0.46 | 0.14 | 0.11 | 0.15 | 0.30 | 0.31 | 0.15 | 0.61 | 0.28 | 0.41 |

Note. Table 26 provides information on the long-term monthly average daily sum of solar radiation on diversely oriented surfaces. Solar radiation data were obtained from long-term satellite measurements of the CMSAF SARAH 2.1 product at the locations of meteorological stations (CMSAF, 2019). The satellite data also take into account the physical state of the atmosphere, including cloudiness, which affects the amount of the measured solar radiation. The analysis is based on the solar radiation data in the period from 1988 to 2017. The global (or summed) solar radiation value is obtained by adding the direct and diffuse solar radiation values.

Table 27

Data on the Average Daily Sum of Solar Radiation on Diversely Oriented Surfaces in December (kWh/m2)

| No. | Station | Direct solar radiation |  |  |  |  | Diffuse solar radiation |  | Global solar radiation |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | orientation of the vertical wall |  |  |  | horizontal surface | orientation of the vertical wall | horizontal surface | orientation of the vertical wall |  |  |  | horizontal surface |
|  |  | East | North | South | West |  |  |  | East | North | South | West |  |
| 1. | Ainaži | 0.11 | 0.00 | 0.37 | 0.08 | 0.05 | 0.07 | 0.13 | 0.18 | 0.07 | 0.43 | 0.15 | 0.19 |
| 2. | Alūksne | 0.10 | 0.00 | 0.35 | 0.08 | 0.05 | 0.07 | 0.14 | 0.17 | 0.07 | 0.42 | 0.15 | 0.19 |
| 3. | Bauska | 0.13 | 0.00 | 0.40 | 0.10 | 0.06 | 0.09 | 0.17 | 0.21 | 0.09 | 0.48 | 0.18 | 0.23 |
| 4. | Daugavpils | 0.14 | 0.00 | 0.42 | 0.10 | 0.07 | 0.10 | 0.19 | 0.24 | 0.10 | 0.51 | 0.20 | 0.26 |
| 5. | Dobele | 0.13 | 0.00 | 0.39 | 0.10 | 0.06 | 0.08 | 0.17 | 0.21 | 0.08 | 0.47 | 0.18 | 0.23 |
| 6. | Gulbene | 0.12 | 0.00 | 0.38 | 0.09 | 0.06 | 0.08 | 0.15 | 0.19 | 0.08 | 0.45 | 0.16 | 0.21 |
| 7. | Jelgava | 0.12 | 0.00 | 0.39 | 0.10 | 0.06 | 0.08 | 0.17 | 0.21 | 0.08 | 0.47 | 0.18 | 0.23 |
| 8. | Kolka | 0.11 | 0.00 | 0.38 | 0.10 | 0.06 | 0.07 | 0.14 | 0.18 | 0.07 | 0.45 | 0.17 | 0.19 |
| 9. | Liepāja | 0.13 | 0.00 | 0.42 | 0.11 | 0.07 | 0.09 | 0.18 | 0.22 | 0.09 | 0.51 | 0.20 | 0.25 |
| 10. | Mērsrags | 0.12 | 0.00 | 0.42 | 0.10 | 0.06 | 0.08 | 0.15 | 0.20 | 0.08 | 0.49 | 0.18 | 0.22 |
| 11. | Pāvilosta | 0.13 | 0.00 | 0.40 | 0.10 | 0.06 | 0.08 | 0.17 | 0.21 | 0.08 | 0.49 | 0.18 | 0.23 |
| 12. | Priekuļi | 0.11 | 0.00 | 0.35 | 0.08 | 0.05 | 0.07 | 0.15 | 0.19 | 0.07 | 0.42 | 0.15 | 0.20 |
| 13. | Rēzekne | 0.12 | 0.00 | 0.38 | 0.10 | 0.06 | 0.08 | 0.17 | 0.21 | 0.08 | 0.46 | 0.18 | 0.23 |
| 14. | Rīga | 0.13 | 0.00 | 0.40 | 0.10 | 0.06 | 0.08 | 0.16 | 0.21 | 0.08 | 0.48 | 0.18 | 0.23 |
| 15. | Rūjiena | 0.11 | 0.00 | 0.35 | 0.08 | 0.05 | 0.07 | 0.13 | 0.17 | 0.07 | 0.42 | 0.15 | 0.18 |
| 16. | Saldus | 0.13 | 0.00 | 0.41 | 0.10 | 0.06 | 0.09 | 0.17 | 0.21 | 0.09 | 0.49 | 0.19 | 0.24 |
| 17. | Skrīveri | 0.13 | 0.00 | 0.40 | 0.09 | 0.06 | 0.08 | 0.17 | 0.21 | 0.08 | 0.48 | 0.18 | 0.23 |
| 18. | Skulte | 0.12 | 0.00 | 0.38 | 0.09 | 0.06 | 0.08 | 0.15 | 0.19 | 0.08 | 0.45 | 0.17 | 0.21 |
| 19. | Stende | 0.12 | 0.00 | 0.38 | 0.09 | 0.06 | 0.08 | 0.16 | 0.20 | 0.08 | 0.46 | 0.17 | 0.22 |
| 20. | Ventspils | 0.11 | 0.00 | 0.35 | 0.08 | 0.05 | 0.07 | 0.15 | 0.18 | 0.07 | 0.43 | 0.16 | 0.20 |
| 21. | Zīlāni | 0.13 | 0.00 | 0.39 | 0.10 | 0.06 | 0.09 | 0.17 | 0.21 | 0.09 | 0.48 | 0.19 | 0.24 |
| 22. | Zosēni | 0.11 | 0.00 | 0.36 | 0.09 | 0.06 | 0.08 | 0.15 | 0.19 | 0.08 | 0.44 | 0.16 | 0.21 |

Note. Table 27 provides information on the long-term monthly average daily sum of solar radiation on diversely oriented surfaces. Solar radiation data were obtained from long-term satellite measurements of the CMSAF SARAH 2.1 product at the locations of meteorological stations (CMSAF, 2019). The satellite data also take into account the physical state of the atmosphere, including cloudiness, which affects the amount of the measured solar radiation. The analysis is based on the solar radiation data in the period from 1988 to 2017. The global (or summed) solar radiation value is obtained by adding the direct and diffuse solar radiation values.

Table 28

Average and Highest 0 °C Temperature Depth in Soil (cm)

| No. | Place |  | Month |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | XI | XII | I | II | III | IV |
| 1. | Daugavpils | highest | 67 | 71 | 74 | 87 | 84 | 80 |
|  |  | frost years | 7 % | 23 % | 57 % | 53 % | 40 % | 7 % |
|  |  | average | 3 | 12 | 25 | 25 | 18 | 4 |
| 2. | Dobele | highest | 28 | 33 | 44 | 51 | 43 | 42 |
|  |  | frost years | 3 % | 13 % | 33 % | 43 % | 30 % | 13 % |
|  |  | average | 1 | 4 | 10 | 13 | 11 | 4 |
| 3. | Liepāja | highest | 37 | 47 | 66 | 71 | 70 | 30 |
|  |  | frost years | 7 % | 21 % | 33 % | 50 % | 33 % | 3 % |
|  |  | average | 2 | 7 | 13 | 20 | 14 | 1 |
| 4. | Stende | highest | 44 | 50 | 53 | 69 | 73 | 47 |
|  |  | frost years | 8 % | 21 % | 42 % | 38 % | 29 % | 12 % |
|  |  | average | 3 | 8 | 15 | 17 | 13 | 4 |
| 5. | Zosēni | highest | 68 | 73 | 69 | 53 | 58 | 50 |
|  |  | frost years | 8 % | 35 % | 52 % | 56 % | 56 % | 19 % |
|  |  | average | 4 | 15 | 21 | 22 | 20 | 7 |

Notes. 1. In Table 28, 0 °C temperature depth in soil (in the period from 1989 to 2018) is determined on the basis of soil temperature measurements at depths of 0.2 m, 0.4 m, 0.8 m, 1.6 m, and 3.2 m. 2. The average 0 °C temperature depth in soil is determined as the average value of the maximum 0 °C temperature depths in soil for each specific month throughout the year. The interpolation method is applied to the calculation of temperature. 3. For each month, the relative number of frost years is also determined, specifically, the number of years in which at least one day with frost occurred in the respective month is divided by the total number of years during which observations were made.

Minister for Economics	R. Nemiro