Text consolidated by Tulkošanas un terminoloģijas centrs (Translation and Terminology Centre) with amending regulations of:

20 April 2004 (No. 358).

If a whole or part of a paragraph has been amended, the date of the amending regulation appears in square brackets at the end of the paragraph. If a whole paragraph or sub-paragraph has been deleted, the date of the deletion appears in square brackets beside the deleted paragraph or sub-paragraph.

Republic of Latvia

Cabinet

Regulation No 144

Adopted 1 April 2003

Regulations regarding the Construction, Conformity Assessment and Placing on the Market of Recreational Craft

Section 7, Paragraphs one and two of

the Law On Conformity Assessment

1. General Provisions

1. These Regulations prescribe the safety requirements for the design and construction of recreational craft, which requirements shall be applicable upon the placing thereof on the market, as well as the procedures for the conformity assessment thereof.

2. A recreational craft is any boat intended for the purposes of sports and leisure (regardless of the means of propulsion), the hull length of which is from 2.5 to 24 meters, measured in accordance with the adapted standards, except for:

2.1. craft intended solely for racing, including rowing racing boats and training rowing boats labelled accordingly by the manufacturer;

2.2. canoes, kayaks, gondolas and hydrocycles;

2.3. sailing surfboards;

2.4. powered surfboards and other similar powered vehicles;

2.5. original and individual replicas of historical craft designed before 1950 and built predominantly with the original materials, and labelled as such by the builder;

2.6. experimental floating craft, if they are not subsequently placed on the market;

2.7. floating craft built for own use, if they are not subsequently placed on the market within a period of five years;

2.8. craft specifically intended to be crewed and to carry passengers for commercial purposes, regardless of the number of passengers;

2.9. submersibles;

2.10. air cushion vehicles; and

2.11. hydrofoils.

3. All the requirements of these Regulations apply to a recreational craft that is chartered or utilised for recreational boating training if the referred to craft is placed on the market for recreation.

4. These Regulations apply to recreational craft, partly completed boats and components of the constructions thereof (both installed and separate) which are referred to in Paragraph 5 of these Regulations.

5. The components of the constructions of recreational craft shall be as follows:

5.1. ignition-protected equipment for all inboard and stern drive engines;

5.2. start-in-gear protection devices for all outboard engines;

5.3. steering wheels, steering mechanisms and cable assemblies;

5.4. fuel tanks and fuel hoses; and

5.5. industrially manufactured hatches and port-lights.

6. The products referred to in Paragraph 4 of these Regulations shall comply with the safety, health and consumer protection requirements specified in Chapter 2 of these Regulations.

7. The Ministry of Transport, in co-operation with the relevant technical committee for standardisation, shall recommend to the limited liability company Latvijas standarts [Latvian Standard] a list of adaptable standards.

[20 April 2004]

8. The limited liability company Latvijas standarts shall submit for publication in the newspaper Latvijas Vēstnesis [the official Gazette of the Government of the Republic of Latvia] a list of those adapted European standards and Latvian national standards which may be applied, at the builder’s option, to fulfil the requirements of these Regulations (hereinafter – adapted standards).

[20 April 2004]

2. Safety Requirements for the Design and Construction of Recreational Craft

9. Depending on the design type, recreational craft shall be divided into four categories:

9.1. Category A “Ocean” – designed for extended voyages where the wind force may exceed magnitude 8 (Beaufort scale), wave heights – 4 meters;

9.2. Category B “Offshore”- designed for offshore voyages where the wind force may reach magnitude 8 (inclusive), the height of waves up to 4 meters;

9.3. Category C “Inshore” – designed for voyages in coastal waters, large bays, estuaries, lakes and rivers where the wind force may reach magnitude 6 (inclusive), the wave heights – up to 2 meters; and

9.4. Category D “Sheltered waters” – designed for voyages on lakes, rivers, and canals where the wind force may reach magnitude 4 (inclusive), wave heights – up to 0.5 meters.

10. Recreational craft of the relevant category shall be designed and built so that they comply with the criteria of the categories referred to in Paragraph 9 of these Regulations as regards stability, buoyancy, and other requirements specified in this Chapter, and shall have good handling characteristics.

11. The following shall be displayed on the hull of the recreational craft:

11.1. an identification number of the recreational craft in compliance with the adopted standards including the following information:

11.1.1. the builder’s code;

11.1.2. the country of construction;

11.1.3. the serial number;

11.1.4. the year of construction;

11.1.5. the release year of the model;

11.2. a builder’s plate containing the following information:

11.2.1. the builder’s name;

11.2.2. the CE marking in conformity with the conditions of Chapter 8 of these Regulations;

11.2.3. the category of the craft in conformity with the criteria specified in Paragraph 9 of these Regulations;

11.2.4. the maximum recommended load specified by the builder in compliance with the requirements specified in Paragraph 21 of these Regulations; and

11.2.5. the number of persons specified by the builder for the carriage of which the boat is intended.

12. Depending on the design category of the recreational craft, it shall be designed to minimise the risks of falling overboard and to facilitate reboarding.

13. The person operating the recreational craft from the main steering position under normal conditions of use (taking into consideration speed and load) shall have good all-round visibility.

14. Recreational craft shall be provided with an owner’s manual in the official language. This manual shall specify, in particular, the risks of fire and flooding and shall include the information listed in Sub-paragraph 11.2 and Paragraph 21 of these Regulations.

15. The choice and combination of materials and the design of a recreational craft shall ensure that the recreational craft is strong enough and conforms to the category thereof in accordance with Paragraph 9 of these Regulations and to the maximum load recommended by the builder in accordance with Paragraph 21 of these Regulations.

16. A recreational craft shall have sufficient stability and freeboard height with regard to the category thereof in accordance with Paragraph 9 of these Regulations and the maximum load recommended by the builder in accordance with Paragraph 21 of these Regulations.

17. In building the hull of a recreational craft, it shall be ensured that the craft has buoyancy characteristics appropriate to the category thereof in accordance with Paragraph 9 of these Regulations, and the maximum load recommended by the builder in accordance with Paragraph 21 of these Regulations.

18. All habitable multihull recreational craft shall be so constructed as to have sufficient buoyancy for keeping the craft afloat when capsized.  A recreational craft the length of which does not exceed six meters and which may rapidly be overwhelmed with water shall be provided with appropriate means of floatation.

19. Openings in the hull, deck and superstructure must not affect the structural integrity of a recreational craft or the watertight integrity thereof when such openings are closed. Windows, portholes, doors and hatchcovers shall withstand such water pressure which is possible with regard to the position thereof, as well as any pressure applied by the weight of persons moving on deck.  Openings through the hull which allow water passage into the hull and out of the hull, and are located below the waterline, corresponding to the maximum load recommended by the builder in accordance with Paragraph 21 of these Regulations, shall be fitted with readily accessible shutoff means.

20. Recreational craft shall be designed and built so as to minimise the risk of sinking, paying particular attention to:

20.1. cockpits and drainage sumps, which should be self-draining or have other means that prevent water from penetrating into the interior;

20.2. ventilation fittings; and

20.3. evacuation of water by utilising pumps or other means.

21. The maximum load recommended by the builder (including fuel, water, provisions, miscellaneous equipment and people (in kilograms)) for which the recreational craft was designed and which is specified on the builder’s plate, shall be determined according to the design category in accordance with Paragraph 9 of these Regulations, the stability and height of the freeboard in accordance with Paragraph 16 of these Regulations and the buoyancy and flotation in accordance with Paragraph 17 of these Regulations.

22. All craft of Categories A and B, and craft of Categories C and D which are longer than six meters shall be equipped with one or more readily available liferafts. The size of the liferafts shall conform to the number of persons the boat may carry in conformity with the construction thereof and the recommendations of a builder.

23. Habitable multihull craft over 12 metres long shall be provided with rescue facilities in the event of a capsize or fire.

24. Recreational craft, taking into consideration the category and characteristics thereof shall be fitted with one or several rope fixing points or other means capable of safely accepting appropriate anchoring, mooring and towing loads.

25. The builder shall ensure that the handling characteristics of the craft are satisfactory for the usage of the most powerful engine for which the boat is designed and constructed. The maximum rated engine power of a recreational craft shall be specified in the owner’s manual in accordance with the recognised standards of the state in which the craft was constructed.

26. The inboard mounted engines shall be placed within an enclosure separated from living quarters so as to minimise the risk of fires or the spread of fires, as well as to minimise hazards from toxic fumes, heat, noise or vibration in the living quarters.

27. Engine parts and accessories that require frequent inspection and servicing shall be readily accessible.

28. The insulating materials inside engine compartments must not be flammable.

29. The engine compartment shall be ventilated and prevent the ingress of water through the inlets of the compartment.

30. Exposed hot or moving parts of the engine that may injure people shall be effectively shielded, except for cases where the engine is protected by a cover or the hull.

31. Recreational craft, which have an outboard engine, require a device that prevents the starting of the engine if the gear-box is not in the neutral position, except for the following cases:

31.1. the engine produces less than 500 newtons (N) of static thrust; and

31.2. the engine is equipped with a throttle-limiting device, which does not permit thrust exceeding 500 N at the time of the starting of the engine.

32. The filling, storage, venting and fuel-supply equipment shall be designed and installed so as to minimise the risk of fire and explosion.

33. Fuel tanks, lines and hoses shall be separated or protected from any significant source of heat.  The material from which the tanks are made and the construction design thereof shall comply with the capacity thereof, as well as the type of fuel. All tank compartments shall be ventilated.

34. Liquid fuel the flash point of which is below 55 °C shall be kept in tanks which do not form part of the hull of the craft and are:

34.1. insulated from the engine compartment and from all other sources of ignition; and

34.2. separated from living quarters.

35. Liquid fuel the flash point of which is equal to or above 55 °C may be kept in tanks that are integral with the hull.

36. The electrical system shall be designed and installed so as to ensure proper operation of the recreational craft under normal conditions of use and to minimise the risk of fire and electric shock.

37. The electrical system shall be designed and installed so as to provide overload and short-circuit protection for all circuits, except for engine ignition circuits in which a battery is utilised.

38. The electrical system shall have ventilation in order to prevent the accumulation of gases, which can be emitted from batteries.

39. Batteries shall be firmly fixed and protected from the ingress of water.

40. The steering system shall be designed, constructed and installed so as to ensure the transmission of control commands under foreseeable operating conditions.

41. Recreation sail boats and single-engine inboard powered recreational craft with a remote-control steering system shall be provided with emergency means of craft steering to allow steering of the craft at a reduced speed.

42. Gas systems intended for use on recreational craft shall ensure the outflow of vapour. They shall be designed and installed so as to prevent any leaks and the risk of explosion and be accessible for being tested for leaks. Materials and components of the gas system shall be appropriate for the gas and resistant to the stresses and exposures caused by the marine environment.

43. Each gas appliance shall be equipped with an anti-igniting device, which shall be effective on all types of burners.  Each gas-consuming appliance shall be supplied by a separate branch of the distribution system, and closed by a separate closing device. Adequate ventilation shall be ensured in order to prevent any harm caused by the leakage of gas and products of combustion.

44. Craft with a permanently installed gas system shall be fitted with an enclosed compartment for the placing of gas cylinders. The compartments shall be separated from the living quarters, they shall be accessible only from the outside and ventilated to the outside so that gas shall drain overboard. A permanently installed gas system shall be tested after the installation thereof.

45. When choosing the type and layout of equipment on the craft, the risk and possibility of the spread of fire shall be taken into consideration. Special attention shall be paid to open flame devices (it shall be ensured that extremely flammable objects are not kept in their vicinity), engines, auxiliary mechanisms and hot zones of mechanisms (electrical wiring shall not be installed thereabove), as well as to uncovered oil and fuel pipes (the possibility of oil and fuel overflows shall be prevented).

46. Recreational craft shall be supplied with appropriate fire-fighting equipment. Petrol engine enclosures shall be protected by a fire extinguishing system that avoids the need to open the enclosure in the event of fire. Portable fire extinguishers shall be installed at readily accessible places, one of them shall be so positioned that it can easily be reached from the main steering position of the craft.

47. Navigation lights of recreational craft shall comply with the requirements of the Convention of 20 October 1972 on the International Regulations for Preventing Collisions at Sea (COLREG – 72) or the European Code for Inland Waterways (CEVNI).

48. Recreational craft shall be constructed so as to prevent the accidental discharge of pollutants (for example, oil, fuel) overboard.

49. Recreational craft fitted with toilets require a toilet evacuation system (mutually joined sanitary equipment, including the toilet, hoses, pipes, container and joint of the deck which shall be suitable for the collection, storage and drainage of waste water) which is emptied into the reception equipment of a port. A portable toilet and portable holding tank may also be utilised if such may be emptied into coastal reception equipment.  The toilet evacuation system shall comply with the following conditions:

49.1. the toilet evacuation system installed on the craft shall be designed in compliance with the international standard ISO 8099;

49.2. technical requirements for the containers and drainage hoses shall be as follows:

49.2.1. the diameter of a hose, depending on the volume of the tank in accordance with the international standard ISO 8099 is sufficient for the hose to withstand the pumping pressure.  If necessary, the hose shall be equipped in such a manner as to make the utilisation of the air-supply method possible through the opening of the tank which is open during evacuation; and

49.2.2. a special type of hose shall be used which contains odours if the wastewater remains in the hose;

49.3. the technical requirements for the deck joint shall be as follows:

49.3.1. a joint with an opening of 38 mm shall be used for draining off the deck, but if it is threaded from the inside, the thread G11/2 shall be used in accordance with the international standard ISO 228-1. A joint with an opening of 45 mm and 50 mm may also be used;

49.3.2. the deck joint shall be labelled with the appropriate symbol of the International Organisation for Standardisation (ISO);

49.3.3. the hose of the deck joint shall extend at least 300 mm directly from the deck joint; and

49.3.4. the deck joint has been installed in accordance with the manufacturer’s instructions; and

49.4. only chemical substances, which are friendly to the marine environment, shall be used in fixed or portable containers.

50.  Pipes going through the hull intended for the draining of human waste directly into the water shall be shut in accordance with the regulations for drainage.

3. Placing on the Market of Recreational Craft and Components of the Design thereof

51. A notified body shall take the necessary measures to ensure, as specified in Paragraph 4 of these Regulations, that those products which have been properly constructed and maintained may be placed on the market and put into service in accordance with the intended purpose only if they do not threaten the safety and health of people, property or the surrounding environment.

[20 April 2004]

52. The placing on the market and putting into service of the products specified in Paragraph 4 of these Regulations shall not be prohibited, restricted or hindered in Latvia, if such are labelled with the CE marking granted by a notified body and if they comply with the requirements of these Regulations, including the conformity procedures which are prescribed in Chapter 6 of these Regulations.

[20 April 2004]

53. The placing on the market and putting into service of partly completed recreational craft shall not be prohibited, restricted or hindered in Latvia, if the builder, an authorised representative thereof (located in Latvia) or a person who is responsible for the placing on the market declares, in accordance with Paragraph 54 of these Regulations, that the construction of the recreational craft will be completed.

54. The following information shall be included in the declaration of the builder referred to in Paragraph 53 of these Regulations, an authorised representative thereof (located in Latvia) or the person who is responsible for the placing on the market:

54.1. the name and address of the builder, representative of the builder (located in Latvia) or person who is responsible for placing on the market;

54.2. a description of the partly completed craft; and

54.3. a statement that the construction of the partly completed craft will be completed and that the craft complies with the requirements that apply at this stage of construction.

55. The placing on the market and putting into service of the components of the design specified in Paragraph 4 of these Regulations shall not be prohibited, restricted or hindered in Latvia, if they are marked with the CE marking granted by a notified body and these components are intended to be installed in recreational craft in accordance with the declaration specified in Paragraph 56 of these Regulations submitted by the builder, an authorised representative thereof (located in Latvia) or a person who places such components on the Latvian market.

[20 April 2004]

56. The declaration of the builder, an authorised representative thereof (located in Latvia) or a person who places the components on the Latvian market shall include the following information:

56.1. the name and address of the builder, the representative thereof (located in Latvia) or a person who places the components on the Latvian market;

56.2. a description of the components; and

56.3. a statement that the component complies with the relevant requirements.

57. The products referred to in Paragraph 4 of these Regulations which do not comply with the requirements of these Regulations may be displayed during exhibitions, fairs and other events, if a visible sign clearly indicates that such products may be marketed or put into service only after the compliance thereof has been ensured in accordance with these Regulations.

4. Guarantee Conditions

58. If components of the construction referred to in Paragraph 5 or recreational craft which have the CE marking and which have been properly built, installed, maintained and used in accordance with the intended purpose might threaten the safety and health of people, property or the surrounding environment, the market supervision authority shall take measures in conformity with its competence to withdraw such from the market, or prohibit or limit the placing on the market or the putting into service thereof. The market supervision authority shall immediately inform the notified body regarding the implementation of the referred to measures, specifying the reasons for the decision, especially, if the non-conformity is a result of the failure to comply with the requirements referred to in Paragraph 6 of these Regulations.

[20 April 2004]

59. If the design component referred to in Paragraph 5 of these Regulations or the craft is labelled with the CE marking but does not comply with the requirements specified in these Regulations, the notified body which controls the person who has labelled it, shall take appropriate measures and inform the Consumer Rights Protection Centre thereof.

[20 April 2004]

5. Procedures for Conformity Assessment

60. Before the manufacturer commences the production and placing on the market of the craft and components thereof referred to in Paragraph 4 of these Regulations, the manufacturer or an authorised representative thereof (located in Latvia) shall take into consideration the requirements specified in Chapters 5 and 6 of these Regulations appropriate to the recreational craft categories specified in Paragraph 9 of these Regulations.

61. The internal production control with a test (module Aa) shall be performed for recreational craft of Categories A and B the hull length of which is less than 12 meters, in conformity with the conditions of Sub-chapter 6.2 of these Regulations.

62. The CE type-examination (module B), in conformity with the conditions of Sub-chapter 6.3 of these Regulations, supplemented by the type conformity (module C) or any of the modules B+D, or B+F, or G or H shall be performed for recreational craft of Categories A and B the hull length of which is from 12 to 24 meters.

63. For Category C recreational craft the hull length of which is from 2,5 to 12 meters:

63.1. if the standards referred to in Paragraphs 16 and 17 of these Regulations have been complied with as regards stability, length of the free board and buoyancy, the internal production control (module A) shall be performed in conformity with the conditions of Sub-chapter 6.1 of these Regulations; or

63.2. if the standards referred to in Paragraphs 16 and 17 of these Regulations have not been complied with as regards stability, length of the free side panel and buoyancy, the internal production control with a test (module Aa) shall be performed in compliance with the conditions of Sub-chapter 6.2 of these Regulations;

64. The CE type-examination (module B) supplemented by the type conformity (module C) or any of the modules B+D, or B+F, or G or H shall be performed in conformity with the conditions of Sub-chapter 6.3 of these Regulations for recreational craft of Category C the hull length of which is from 12 to 24 meters.

65. The internal production control (module A) shall be performed for Category D recreational craft the hull length of which is from 2,5 to 24 meters,  in compliance with the conditions of Sub-chapter 6.1 of these Regulations.

66. One of the modules B+C, or B+D, or B+F, or G, or H shall be utilised for the components of the design of recreational craft.

6. Conformity Assessment Procedures

6.1. Internal Production Control (Module A)

67. The builder or an authorised representative thereof (located in Latvia) shall ensure the conformity of the respective products to the requirements of these Regulations. The builder the product of whom is labelled with the CE marking or an authorised representative thereof (located in Latvia) shall draw up a written declaration of conformity in accordance with the requirements of Paragraph 9 of these Regulations.

68. The technical documentation prepared by the builder shall be kept by the builder or the authorised representative thereof (located in Latvia) for a period ending at least 10 years after the last product has been manufactured. These documents shall be available to control institutions for the performing of examinations.

69. If neither the builder nor the authorised representative thereof is located in Latvia, the technical documentation shall be kept by the person who distributes the product on the Latvian market.

70. The conformity of a product with the requirements of these Regulations shall be assessed according to the technical documentation, which shall include a description of the design, production and operation of the product in conformity with the requirements of Chapter 7 of these Regulations.

71. The builder or the authorised representative thereof (located in Latvia) shall keep a copy of the declaration of conformity together with the technical documentation.

6.2. Internal Production Control with a Test (Module Aa, Option 1)

72. For internal production control with a test, in addition to module A, a requirement has been specified that one or more tests of stability as well as tests of buoyancy characteristics shall be performed in conformity with the requirements of these Regulations, or equivalent calculations or control.

73. The notified body selected by the builder shall be responsible for the tests referred to in Paragraph 72 of these Regulations or for equivalent calculations.

[20 April 2004]

6.3. CE Type-Examination (Module B)

74. A notified body shall ascertain and attest that the type representing the production envisaged conforms to the requirements of these Requirements.

[20 April 2004]

75. The application for the CE type-examination shall be submitted by the builder or an authorised representative thereof (located in Latvia) to a notified body chosen thereby . The application shall include the following information:

75.1. the name and address of the builder (if the application is submitted by the authorised representative, also the name and address thereof;

75.2. a written declaration that the application has not been submitted to any other notified body; and

75.3. the technical documentation referred to in Paragraph 79 of these Regulations.

[20 April 2004]

76. The applicant shall place at the disposal of the notified body a sample of the product (hereinafter – sample).

[20 April 2004]

77. A sample may relate to several versions of a product, if the differences between those versions do not affect the level of safety and other requirements that relate to the product.

78. If necessary, a notified body may request additional samples in order to carry out the test programme.

[20 April 2004]

79. The technical documentation shall provide the possibility to assess the conformity of the product with the requirements of these Regulations.  It shall include (to the extent necessary for such assessment) a description of the design, production and operation of the product in accordance with Chapter 7 of these Regulations.

80. A notified body shall:

80.1. examine the technical documentation, ascertain that the sample has been manufactured in accordance with the technical documentation and identify which elements have and which have not been designed in accordance with the adopted standards regarding which official references have been published;

80.2. perform an appropriate examination and the necessary tests in order to ascertain that the solutions adopted by the builder conform to the requirements of the Regulations, if the adopted standards have not been applied;

80.3. perform an appropriate examination and the necessary tests in order to ascertain that the standards chosen by the builder have been applied; and

80.4. agree with the applicant for the examination upon a location for the performance of the examinations and necessary tests.

[20 April 2004]

81. If the sample conforms to the requirements of these Regulations, a notified body shall issue the CE type-examination certificate to the applicant. The certificate shall include the name and address of the builder, the conclusions of the examination, the conditions of validity and the necessary information for the identification of the approved sample.

[20 April 2004]

82. The relevant parts of the technical documentation shall be annexed to the certificate. A copy of the certificate shall be kept by the notified body, which has issued the certificate.

[20 April 2004]

83. If a sample certification is refused to a builder, the notified body shall provide a detailed justification for the refusal.

[20 April 2004]

84. The applicant shall inform the notified body which has the technical documentation concerning the CE type-examination certificate at its disposal of all modifications to the approved product which require additional approval if such changes may affect the conformity with the requirements or the prescribed conditions for the use of the product. An additional approval shall be given in the form of an addendum to the original CE type-examination certificate.

[20 April 2004]

85. Each notified body shall communicate to other authorised bodies the relevant information concerning the CE type-examination certificates as well as any addenda issued or withdrawn.

[20 April 2004]

86. The other authorised bodies may receive the CE type-examination certificate and copies of the additions thereof. Addenda to the certificates shall be available to other authorised bodies.

87. The builder or an authorised representative thereof shall keep the CE type-examination certificate and copies of the addenda thereto together with the technical documentation for a period of at least 10 years after the last product has been manufactured.

88. If the builder or the authorised representative thereof is not located in Latvia, the technical documentation shall be kept by the person who markets the product in Latvia.

6.4. Conformity to Type (Module C)

89. The builder or the authorised representative thereof (located in Latvia) shall ensure and declare that the relevant products conform to the sample as described in the CE type-examination certificate and satisfy the requirements of these Regulations. The builder shall label each product with the CE marking and draw up a written declaration of conformity.

90. The builder shall take the necessary measures in order to ensure that the manufacturing process guarantees the compliance of the manufactured products with the type as described in the CE type-examination certificate and with the requirements of these Regulations.

91. The builder or the authorised representative thereof shall keep a copy of the declaration of conformity for a period of at least ten years after the last product has been manufactured.

92. If the builder or the authorised representative is not located in Latvia, the technical documentation shall be kept by the person marketing the product in Latvia.

6.5. Production Quality Assurance (Module D)

93. The builder shall ensure and declare that the respective products comply with the sample as described in the CE type-examination certificate and observe the requirements of these Regulations. The builder or the authorised representative thereof (located in Latvia) shall label each product with the CE marking and draw up a written declaration of conformity. The CE marking shall be accompanied by the distinguishing number of the notified body, which has performed the examinations specified in Paragraph 104 of these Regulations.

[20 April 2004]

94. The builder shall ensure the operation of the quality system specified in Sub-chapter 6.6 of these Regulations and shall be subject to the examinations specified in Paragraph 104 of these Regulations.

6.6. Quality System

95. In order to have a quality system evaluated the builder shall submit to the notified body an application which shall include:

95.1. information regarding the product category envisaged;

95.2. the documentation concerning the quality system; and

95.3. if necessary, the technical documentation of the approved type in conformity with the requirements of Chapter 7 of these Regulations and a copy of the CE type-examination certificate.

[20 April 2004]

96. The quality system shall ensure compliance of the products with the type as described in the CE type-examination certificate and with the requirements of these Regulations.

97. The elements, requirements and provisions of the quality system adopted by the builder shall be documented in a systematic manner in the form of written plans, procedures and instructions. The quality system documentation shall ensure a consistent interpretation of the quality programmes, plans, manuals and records.

98. The quality system documentation shall include:

98.1. the objectives of the quality system and the organisational structure, responsibilities and powers of the management with regard to product quality;

98.2. the techniques and processes of manufacturing, quality control and quality assurance;

98.3. the examinations and tests which are carried out before, during and after building, as well as the frequency with which they are to be carried out;

98.4. quality records (for example, inspection reports and test data, calibration data, qualification reports for the personnel concerned); and

98.5. the means for monitoring the required product quality and for the effective operation of the quality system.

99. A notified body shall assess the conformity of a quality system with the requirements of these Regulations. The notified body shall presume that the quality systems, which implement the adapted standards, conform to these requirements.

[20 April 2004]

100. The team assessing the conformity of the quality system shall have at least one person with experience in evaluation of the product technology concerned. The evaluation procedure shall include a visit to the manufacturer’s premises.

101. The decision shall be notified to the builder. The notification shall contain the conclusions of the examination and the basis for the assessment.

102. The builder shall undertake to fulfil the duties arising out of the quality system as approved and to maintain an adequate and efficient quality system.

103. The builder or an authorised representative thereof shall notify the notified body regarding any intended updating of the quality system. The notified body shall evaluate the proposed modifications and decide whether the supplemented quality system would comply with the requirements of these Regulations or whether a reassessment shall be required. The notified body shall notify its decision to the builder. The notification shall include the conclusions of the examination and the justification for the decision.

[20 April 2004]

104. A notified body shall:

104.1. perform examinations to make sure that the builder is duly fulfilling the duties arising out of the approved quality system;

104.2. periodically carry out audits to ascertain how the builder is maintaining and applying the quality system. The notified body shall submit an inspection report to the builder; and

104.3. perform extraordinary examinations without warning. During such examinations the notified body may carry out, or cause to be carried out, tests in order to inspect the operation of the quality system. The notified body shall submit to the builder an inspection report and, if the testing has been performed, also the test results.

[20 April 2004]

105. For a notified body to perform an inspection, the builder shall ensure that the notified body has free access to the places of construction, inspection, testing and storage, as well as provide all necessary information including:

105.1. the quality system documentation; and

105.2. quality records (inspection reports, test and calibration data, qualification reports for the personnel concerned).

[20 April 2004]

106. The builder shall, for a period of ten years after the last product has been manufactured, keep at the disposal of the supervisory institutions the following documents:

106.1. the quality system documentation;

106.2. the documents regarding improvements to the quality system; and

106.3. the decisions and reports of the notified body regarding the conformity of the quality system with the adapted standards, regarding the periodical audits and regarding the additional examinations of the builder.

[20 April 2004]

6.7. Product Verification (Module F)

107. This module describes the procedure in accordance with which the manufacturer or the authorised representative thereof (located in Latvia) checks and attests that the products to which the requirements of these Regulations apply are in conformity with the type as described in the CE type-examination certificate and with the requirements of these Regulations.

108. The manufacturer shall take necessary measures so that the manufacturing process ensures the conformity of the products with the type as described in the CE type-examination certificate and with the requirements of these Regulations.  The manufacturer or the authorised representative thereof (located in Latvia) shall label each product with the CE marking and shall draw up a declaration of conformity.

109. A notified body shall perform the appropriate examination and tests in order to check the conformity of the product with the requirements of these Regulations.  The examination shall be performed at the manufacturer’s choice by:

109.1. examining and testing each product in accordance with Paragraph 111 of these Regulations; or

109.2. examining and testing the products using the statistical method in accordance with Paragraph 112 of these Regulations.

[20 April 2004]

110. The manufacturer or the authorised representative thereof shall keep a copy of the declaration of conformity for a period of at least 10 years after the last product has been manufactured.

111. Examination and testing of products:

111.1. the products shall be individually examined and tests as specified in the adopted standards or tests equivalent thereto shall be performed in order to verify the conformity of the product with the type as specified in the CE type examination certificate and with the requirements of these Regulations;

111.2. the notified body shall affix its identification number upon each approved product and draw up a written certificate of conformity for the tests performed; and

111.3. the manufacturer or the authorised representative thereof shall ensure that the certificates of conformity issued by the authorised representative are available upon request.

[20 April 2004]

112. Statistical verification:

112.1. the manufacturer shall present the products as a homogeneous group and take measures to ensure that the manufacturing process ensures the homogeneity of each group produced;

112.2. all products shall be available for verification in a homogeneous group. A sample shall be chosen from each group. The sample of the product shall be individually examined and tests as set out in the adapted standards or tests equivalent thereto shall be performed in order to verify the conformity of the product with the requirements of these Regulations and to determine whether the group is to be approved or rejected;

112.3. the following elements shall be utilised in the statistical verification:

112.3.1. the statistical method; and

112.3.2. the sampling plan with  characterisations for the implementation thereof;

112.4. if a group of products has been accepted, the notified body shall affix its identification number upon each product and shall draw up a written certificate of conformity that relates to the tests performed. All products in the group may be placed on the market, except those products, which were found to be nonconforming;

112.5. if a group of products is rejected, the notified body or market supervision authority shall take appropriate measures in order to prevent any sales of that group of products on the market;

112.6. if a group of products has been rejected several times, the notified body may suspend the results of the statistical verification;

112.7. if the notified body undertakes the responsibility, the manufacturer may affix the identification number of the institution during the manufacturing process; and

112.8. the manufacturer or the authorised representative thereof shall, upon request, present the conformity certificates issued by the notified body.

[20 April 2004]

6.8. Unit Verification (Module G)

113. This module describes the procedures in accordance with which the builder shall verify and declare that the relevant product for which the certificate referred to in Paragraph 115 has been issued conforms with the requirements of these Regulations. The builder or an authorised representative thereof (located in Latvia) shall affix the CE marking to the product and draw up a declaration of conformity.

114. The notified body shall examine each product and perform the tests specified in the adapted standards, or tests equivalent thereto, in order to verify the conformity of the products with the requirements of these Regulations.

[20 April 2004]

115. The notified body shall affix its identification number upon the approved product and shall draw up a certificate of conformity relating to the tests performed.

[20 April 2004]

116. The technical documentation shall provide an opportunity to assess the conformity of the product with the requirements of these Regulations and to provide clarification regarding the design, manufacture and utilisation of the product.

6.9. Full Quality Assurance (Module H)

117. This module describes the procedure in accordance with which the builder who performs the duties referred to in the Regulations verifies and declares that the relevant products conform with the requirements of these Regulations, which apply thereto.  The builder or the authorised representative thereof (located in Latvia) shall affix the CE marking upon each product and draw up a written declaration of conformity.

118. The CE marking shall be accompanied by the identification number of the notified body responsible for the CE examination specified in Paragraph 130 of these Regulations.

[20 April 2004]

119. The builder shall ensure the operation of the quality system referred to in Sub-chapter 6.10 of these Regulations with regard to the design, manufacture and final inspection and testing of the product, and the builder shall be subject to the examination specified in Paragraph 130 of these Regulations in accordance with the procedures for supervision.

6.10. Quality system

120. In order for the quality system to undergo evaluation, the manufacturer shall submit to the notified body an application, which shall include information regarding the product category envisaged and the documentation of the quality system.

[20 April 2004]

121. The quality system shall ensure conformity of the products with the requirements of these Regulations.

122. The elements, requirements and provisions adopted by the manufacturer shall be documented in a systematic manner, drawing them up as written plans, procedures and instructions.

123. The quality system documentation shall ensure a common understanding of the quality policy, procedures, quality programmes, plans, manuals and records.

124. The quality system documentation shall include:

124.1. the objectives and structure of the quality system, responsibilities and powers of the management with regard to the design and quality of the products;

124.2. technical instructions, including the standards which will be applied (if the standards will not be applied in full, the means that are to be utilised to ensure compliance with the requirements of these Regulations);

124.3. the design control and design verification techniques, processes and actions which pertain to the product category;

124.4. the corresponding manufacturing, quality control and quality assurance techniques which pertain to the processes and systematic actions;

124.5. the examinations and tests that will be carried out before, during and after manufacture, as well as the frequency for the performance of the examinations and tests;

124.6. the quality records (for example, inspection reports and test data, calibration data, qualification reports for the personnel concerned); and

124.7. the means for monitoring the required design and product quality and for the effective operation of the quality system.

125. The notified body shall assess the quality system in order to determine the conformity thereof with the requirements specified in Paragraphs 121, 122, 123, and 124 of these Regulations and shall presume that the quality system implementing the adopted standard conforms with these requirements.

[20 April 2004]

126. The team of experts assessing conformity shall have at least one person experienced as an assessor in the product technology concerned. The evaluation procedure shall include a visit to the manufacturer’s premises.

127. The decision shall be notified to the manufacturer. The notification shall include the conclusions of the examination and the justification for the evaluation.

128. The manufacturer shall undertake to fulfil the duties arising from out of the quality system as approved and to maintain an adequate and efficient quality system.

129. The manufacturer or the authorised representative thereof shall inform the notified body regarding any intended improvement of the quality system. The notified body shall evaluate the planned modifications and decide whether the supplemented quality system complies with the requirements specified in Paragraphs 121, 122, 123, and 124 of these Regulations or whether a reassessment shall be required. The notified body shall notify its decision to the manufacturer. The notification shall include the conclusions of the examination and the justification for the decision.

[20 April 2004]

130. A notified body shall:

130.1. perform examinations to make sure that the manufacturer duly fulfils the duties arising from the approved quality system;

130.2. periodically carry out audits to make sure that the manufacturer is maintaining and applying the quality system. The notified body shall submit the audit report to the manufacturer; and

130.3. have the right to perform examinations of the manufacturer without warning. During such visits, the notified body may carry out tests or have them carried out in order to check the functioning of the quality system. The notified body shall submit to the manufacturer the examination report and, if testing has been performed, also the test results.

[20 April 2004]

131. For a notified body to perform an examination, the manufacturer shall ensure that the notified body has free access to the places of the design, manufacture, inspection, testing and storage, as well as shall provide all necessary information including:

131.1. the quality system documentation;

131.2. the quality records with respect to the design (results of analyses, calculations, tests); and

131.3. the quality records with respect to the manufacture (inspection reports, test and calibration data, and qualification reports of the personnel concerned).

[20 April 2004]

132. The manufacturer shall, for a period of ten years after the last product has been manufactured, keep the following documents at the disposal of the supervisory authorities:

132.1. the quality system documentation;

132.2. documents regarding improvements to the quality system; and

132.3. the decisions and reports from the notified body regarding the conformity of the quality system with the adapted standards, regarding the periodic audits and regarding additional examinations of the manufacturer.

[20 April 2004]

133. The duty of each notified body shall be to forward to other notified bodies the information regarding quality system approvals issued and cancelled.

[20 April 2004]

7. Technical Documentation Submitted by the Manufacturer

134. The technical documentation referred to in Modules A, B, C and D shall contain the information and means used by the manufacturer to ensure the conformity of craft and the structural components thereof with the specified requirements.

135. The technical documentation shall provide information regarding the design, manufacture and operation of the product and shall enable assessment of conformity with the requirements of these Regulations.

136. In order to perform an evaluation, the technical documentation shall include:

136.1. a general description of the sample;

136.2. drawings and schemes of conceptual designs and components, subassemblies, circuits and other elements, as well as explanatory descriptions and product operation instructions;

136.3. the list of adapted standards applied in full or in part, and descriptions of the solutions adopted to fulfil the requirements of these Regulations if the adapted standards are not being applied;

136.4. results of design calculations and examinations; and

136.5. test results or calculations with regard to stability in accordance with the conditions of Paragraph 16 of these Regulations and to buoyancy in accordance with the conditions of Paragraph 17 of these Regulations.

8. CE Marking

137. Recreational craft and components of the designs referred to in Paragraph 5 of these Regulations which conform with the requirements specified in Paragraph 6 of these Regulations shall be labelled with the CE marking when they are placed on the market.

138. The CE marking (Annex) shall appear on the plate of the builder of the craft and on the components of the design which are specified in Paragraph 5 of these Regulations and on the packaging thereof. The CE marking shall be easily visible, clearly legible and indelible. The CE marking shall be accompanied by the identification number of the notified body, which has implemented the procedures specified in Modules Aa, D, F, G, and H.

[20 April 2004]

139. The affixing of markings or inscriptions upon craft, which are likely to mislead third parties with regard to the meaning or the form of the CE marking, shall be prohibited. Other markings may be affixed to the recreational craft and the components of the design which are specified in Paragraph 5 of these Regulations, if the visibility and legibility of the CE marking is not thereby reduced.

140. If the notified body determines that the CE marking has been displayed incorrectly, the duty of the manufacturer or the authorised representative thereof (located in Latvia) is to rectify this violation.

[20 April 2004]

141. If the requirements specified in these Regulations for the labelling of products are also henceforth not being complied with, the notified body shall take measures in order to restrict or prohibit the placing on the market of such products, or to ensure that they are withdrawn from the market in conformity with the requirements specified in Chapter 4 of these Regulations.

[20 April 2004]

9. Written Declaration of Conformity

142. The written declaration of conformity for recreational craft and the components specified in Paragraph 5 of these Regulations with the conditions of these Regulations shall be present on the recreational craft and attached to the owner’s manual.

143. The written declaration of conformity shall include the following information:

143.1. the name and address of the manufacturer or the authorised representative thereof (located in Latvia);

143.2. a description of the components of the recreational craft or construction;

143.3. the adapted standards or references to the instructions in accordance with which conformity has been declared;

143.4. if necessary, information regarding the CE type-examination certificate which has been issued by the notified body;

143.5. the name and address of the notified body;

143.6. identification data of the person who has signatory rights and who is empowered to represent the manufacturer in Latvia;

143.7. the undertaking of the authorised person of the manufacturer (name and address). The authorised representative shall also provide the name and address of the undertaking of the manufacturer; and

143.8. if necessary, a description of the product manufacture, type and serial number.

[20 April 2004]

144. The written declaration of conformity shall be drawn up in the official language.

10. Notified Body

[20 April 2004]

145. The Ministry of Economics shall publish in the newspaper Latvijas Vēstnesis a list of those bodies which are accredited to perform the conformity assessment procedures referred to in Chapter 5 of these Regulations.

[20 April 2004]

146. A notified body, its director and the staff carrying out the verification tests upon recreational craft or components of the design thereof shall not be the designer, manufacturer, supplier or installer of the relevant recreational craft or components of the design thereof, nor their authorised representatives.  They may not be involved directly or as authorised representatives in the design, manufacture, marketing or maintenance of these products.

[20 April 2004]

147. The conditions referred to in Paragraph 146 of these Regulations do not preclude the possibility of exchanges of technical information between a manufacturer and a notified body.

[20 April 2004]

148. A notified body shall have at its disposal the necessary staff, financial resources and equipment to enable it to perform effectively the administrative and technical tasks connected with verification.

[20 April 2004]

149. A notified body shall be registered in the Commercial Register of the Republic of Latvia.

[20 April 2004]

150. The staff responsible for inspection shall comply with the following requirements:

150.1. relevant technical and professional education and training;

150.2. knowledge of the tests to be carried out and experience in the conduct of such tests; and

150.3. the ability to draw up the certificates, records and reports required to certify the performance of the tests.

151. A notified body shall guarantee the impartiality of inspection staff. The remuneration of the staff shall not depend on the number of tests carried out or on the results of the tests.

[20 April 2004]

152. A notified body shall insure its liability.

[20 April 2004]

153. The duty of the staff of a notified body is not to disclose the information obtained in the carrying out of professional activities.

[20 April 2004]

11. Market Supervision

154. The Consumer Rights Protection Centre shall perform the functions of the market supervision authority in conformity with the funds allocated from the State budget. The Consumer Rights Protection Centre shall take the measures specified in the relevant regulatory enactments in order to prevent any distribution on the market of such recreational craft and the equipment thereof, which do not comply with the requirements of these Regulations.

155. If the manufacturer of recreational craft and the equipment thereof or the authorised representative thereof in Latvia have failed to comply with the requirements provided for in these Regulations, no trade in the relevant products shall be permitted.

12. Closing Provisions

156. Recreational craft which are not equipped with the toilet evacuation system, to be emptied into the reception equipment of a port, referred to in Paragraph 49 of these Regulations, must be equipped with the relevant system before 1 January 2005.

157. These Regulations shall come into force with special Cabinet Regulation.

Informative Reference to European Union Directives

These Regulations contain legal norms arising from Directive 94/25/EC.

Prime Minister 	E. Repše

Minister for Transport 	R. Zīle

## Annex

Cabinet Regulation No. 144

9 April 2002

CE Marking

1. The CE marking shall consist of the capital letters “CE” of the following form.

2. If the size of the CE marking is reduced or enlarged, the proportions given by the sample shall be observed.

3. The CE marking shall be supplemented by the identification number of the notified body if it participates in building control, as well as by the last two digits of the year in which the CE marking was affixed.

[20 April 2004]

4. Both components of the CE marking shall be of the same vertical size, and it may not be less than 5 millimetres.

# Minister for Transport 	R. Zīle