Republic of Latvia

Cabinet

Regulation No. 262

Adopted 21 July 1998

Regulations Regarding Indication of Composition for Textile Fibres on the Labelling of Textile Products

Issued pursuant to Section 14, Clause 3

of the Law on the Structure of the Cabinet

I. General Provisions

1. Terms used in these Regulations:

1.1. textile product - a raw, semi-worked, worked, semi-manufactured or manufactured (semi-made-up or made-up) product which is composed of textile fibres only, irrespective of the mixing or assembly method;

1.2. textile fibre - a flexible and fine unit of matter the length of which is substantially greater than the transverse dimension, and which may be utilised for the manufacture of textile products, as well as flexible strips or tubes, of which the apparent width (the width of the strip or tube when folded, flattened, compressed or twisted), or the average width (if the width is not uniform throughout the strip or tube) does not exceed 5 mm, (including strips cut from wider strips or films, which are produced by extrusion from polymers used for the manufacture of the textile fibres listed under items 19 to 41 in Annex 1 of these Regulations, and which are suitable for use in textile products; and

1.3. tolerance – the maximum deviation from the percentage amount indicated on the labelling.

2. These Regulations prescribe the requirements for the indication of composition for textile fibres on the labelling of textile products.

3. These Regulations apply to textile products that are sold or intended for sale in Latvia, as well as to:

3.1. products in which there is at least 80% textile fibres of the total product mass;

3.2. furniture, umbrellas and sunshades in which there is at least 80% textile components of the total product mass;

3.3. multi-layer floor coverings, as well as mattresses, camping goods, warm linings of footwear, gloves and mittens if their parts or linings are at least 80% textile components of the total product mass; and

3.4. textile products incorporated in other products if they are an integral part of the relevant product and if their textile fibre composition is indicated.

4. These Regulations do not apply to textile products which:

4.1. are intended for exportation;

4.2. are imported under customs control for transit;

4.3. are imported for processing; and

4.4. are imported in accordance with a work-performance contract or a contract entered into by a person who performs individual work in the field of home handicraft.

5. The information specified in these Regulations may be indicated in the accompanying documents in the following cases only:

5.1. if a textile product is supplied pursuant to a procurement by state administrative institutions; and

5.2. if the textile product is not intended for sale to consumers.

II. Information on the Labelling of Textile Products

6. The textile fibre composition of a textile product shall be clearly indicated in the relevant accompanying documents. It is not permitted to use abbreviations in contracts, bills and invoices, except the mechanised processing code if it is explained in the same document.

7. If a textile product is intended for sale to consumers, the textile fibre composition thereof shall be indicated on the labelling in a clearly legible and uniform print, as well as in catalogues and trade literature.

8. Information on the labelling of textile products regarding the textile fibre composition shall be completely separated from all other types of information, except the trade mark or the name of the undertaking (company) which may be located directly before or after the indication regarding the textile fibre composition of the textile product.

9. If the trade mark or the name of the undertaking (company) indicated on the labelling of a textile product intended for sale to consumers has the name of a textile fibre referred to in Annex 1 of these Regulations, or if it is possible to confuse the name of the relevant undertaking (company) with these textile fibres, the textile fibre composition of the textile product shall be indicated, in clearly legible and uniform print, before or after the trade mark or the name of the undertaking (company).

10 . Information regarding the textile fibre composition of textile products manufactured in Latvia shall be in the official language. Imported textile products, the labelling of which provides information in a foreign language, shall be accompanied by a translation of the relevant information in the official language. If small units of sowing yarn, mending yarn or embroidery yarn are sold (for example, bobbins, reels, skeins, balls), a translation of the information in the official language shall be provided only on the labelling of the general outer packaging or at the trading location.

III. Indication of Textile Fibre Composition on the Labelling of Textile Products

11. To indicate the textile fibre composition on the labelling of textile products, the names of textile fibres prescribed in Annex 1 of these Regulations shall be used.

12. The names of textile fibres prescribed in Annex 1 of these Regulations may not be used for the description of any other fibre, neither in the name, nor in form of word roots or as an adjective.

13. It is not permitted to use the word “zīds" [silk] for the description of chemical textile fibres.

14. It is not permitted to use designation “100%”, “tīrs" [pure], “vienīgi" [solely] or similar designations if the textile product is not composed only of the relevant textile fibre. A textile product may contain up to 2% other textile fibres of the total product mass if it is necessary for production due to technical reasons. Textile products, which have undergone a carding process, may contain up to 5% other textile fibres of the total product mass if it is necessary for production due to technical reasons.

15. The labelling of wool textile products may contain such designations as “pirmlietojuma vilna" [virgin wool] or “cirptā vilna" [fleece wool], indicating the complete percentage textile fibre composition of the textile product, in the following cases:

15.1. if a textile product is composed exclusively of wool fibre which has not previously been incorporated in a finished product, has not been subjected to any spinning or felting process (other than those required in the manufacture of the relevant product) and has not been damaged by treatment or use thereof;

15.2. the wool fibre referred to in Sub-paragraph 15.1 of these Regulations accounts for not less than 25% of the total product mass; and

15.3. in the case of a scribbled mixture, the wool fibre referred to in Sub-paragraph 15.1 of these Regulations is mixed with only one other type of textile fibre.

16. For the textile products referred to in Paragraph 15 of these Regulations (also textile products which have undergone a carding process), the tolerance justified on technical grounds associated with manufacture may not be higher than 0.3%.

17. For textile products which are composed of two or more fibres, one of which accounts for at least 85% of the total product mass, the textile fibre composition on the labelling shall be indicated in one of the following ways:

17.1. by indicating the name of the textile fibre that accounts for at least 85% of the total product mass, and the percentage amount thereof;

17.2. by indicating the name of the textile fibre which accounts for at least 85% of the total product mass, and the words “vismaz 85%" [at least 85%], or

17.3. by indicating the full percentage composition of the textile product.

18. Textile products which are composed of two or more textile fibres, none of which accounts for 85% of the total product mass, shall be indicated by the names of at least the two main fibres and the percentage mass, as well as the names of other textile fibres in descending order of mass (with or without an indication of the percentage mass thereof).

19. Textile fibres that separately account for less than 10% of the total textile product mass may be designated by the words “citas tekstilšķiedras” [other textile fibres] and indicating the total percentage mass thereof. If a textile fibre which accounts for less than 10% of the total textile product mass is referred to on the labelling, the full percentage composition of the textile product shall be indicated.

20. Textile products having a pure cotton warp and a pure flax weft containing at least 40% flax of the total unsized fabric mass may be designated by the words “kokvilna un lins" [cotton and flax] and the specification “tīri kokvilnas šķēri – tīri linu audi" [pure cotton warp – pure flax weft].

21. The designation “dažādas tekstilšķiedras" [mixed textile fibres] or “neprecizēts tekstilšķiedru sastāvs" [unspecified textile fibre composition] shall be used for textile products the composition of which cannot easily be specified in the manufacturing process.

22. If a textile product is intended for sale to consumers, in the cases referred to in Paragraphs 17 to 21 of these Regulations the following tolerances may be allowed:

22.1. a quantity of extraneous textile fibres – up to 2% (up to 5% in the case of products which have undergone a carding process) of the total textile product mass if it is technically necessary; and

22.2.  up to 3% – a manufacturing tolerance between the stated fibre percentage amounts and the percentage amounts obtained from analysis in relation to the total fibre mass indicated on the label. The condition referred to shall also apply to the fibres which, in accordance with Paragraphs 18 and 19 of these Regulations, are indicated in descending order of mass with no indication of the percentage amount thereof, and to the fibres referred to in Sub-paragraph 15.2 of these Regulations.

23. In analysis, the tolerances shall be calculated separately. The total mass to be taken into account in calculating the tolerance referred to in Sub-paragraph 22.2 of these Regulations shall be that of the finished product textile fibre mass except for extraneous fibre mass, which is specified in observing the tolerance referred to in Sub-paragraph 22.1 of these Regulations.

24.  Increase of the tolerances referred to in Sub-paragraphs 22.1 and 22.2 shall be permitted only if it is proven that any of the extraneous fibres found by analysis, taking into account the tolerance referred to in Sub-paragraph 22.1, is of the same chemical composition as one or more of the textile fibres indicated on the label.

25. In exceptional cases, if it is required by the manufacturing process and if the conformity of the product is examined in accordance with Paragraph 43 of these Regulations and adequate justification is provided by the manufacturer, textile products may have greater tolerances than specified in Sub-paragraphs 22.1 and 22.2 of these Regulations.

26. The tolerances specified in Paragraphs 14, 16, 22, 23, 24 and 25 of these Regulations do not apply to decorative, visible, isolated textile fibres which do not exceed 7% of the finished product mass, as well as to textile fibres (for example, metallic textile fibres) which are incorporated in order to obtain an antistatic effect and which are not more than 2% of the finished product mass. The textile fibres referred to do not have to be indicated on the labelling. In the cases referred to in Paragraph 20 of these Regulations, the percentage amount shall be calculated utilising not the fabric mass, but the warp mass and the weft mass separately.

27. If a textile product consists of two or more components, which have a different textile fibre composition, the labelling shall indicate the textile fibre composition for each component. The composition does not have to be indicated for components (except main linings) representing less than 30% of the total product mass.

28. If two or more components of a textile product have an equal textile fibre composition and they normally form a single whole unit, only one label shall be necessary for such textile products.

29. The textile fibre composition of corsetry products shall be indicated either on the whole of the product or separately on the components of such products:

29.1. for brassieres: the outside and inside fabric of the cups and back;

29.2. for corsets: the front, rear and side stiffening panels; and

29.3. for corselets: the outside and inside fabric of the cups, the front and rear stiffening panels and the side panels; and

29.4. for other corsetry articles – for such components the mass of which is more than 10% of the total product mass.

30. The textile fibre composition of separate corsetry product component parts shall be indicated on the labelling so that the consumer can easily understand to which component part of the product the information refers to.

31. The textile fibre composition of etched textile products shall be indicated on the labelling of the product as a whole, and may be indicated separately for the base fabric and the etched parts, indicating the names of the relevant components.

32. The textile fibre composition of embroidered textile products shall be indicated on the labelling of the product as a whole, and may be indicated separately for the base fabric and the embroidery yarn, indicating the names of the relevant components. If the embroidered part is less than 10% of the surface area of the textile product, only the textile fibre composition of the base fabric needs to be indicated.

33. The textile fibre composition of reinforced yarns, consisting of different textile fibre core and cover, and offered for sale to the consumer, shall be indicated for the product as a whole, or may be indicated separately for the core and the cover, indicating the names of the relevant components.

34. The textile fibre composition of velvet and plush textile products, or of textile products, which are similar to velvet or plush, shall be indicated on the labelling of the textile product as a whole. If the base and surface of the textile product is composed of different fibres, the composition may be indicated separately for each of the component parts, indicating the names of the relevant components.

35.  For floor coverings and carpets, the base and surface of which are composed of different fibres, the textile fibre composition may be indicated only for the surface, indicating the names of the relevant components.

36. The textile fibre composition of the textile products referred to in Annex 2 of these Regulations does not have to be indicated, except in the case referred to in Paragraph 9 of these Regulations.

37. On the labelling of the textile products of the one type and uniform composition referred to in Annex 3 of these Regulations, only the common textile fibre composition shall be indicated if these products are to be offered for sale to consumers.

38. The textile fibre composition of textile products that are sold by the metre shall be indicated on the length or the roll (bale).

39. The textile fibre composition of the textile products referred in Paragraphs 37 and 38 of these Regulations shall be indicated in such a way that the consumer may become fully acquainted with the relevant information.

IV. Specification of the Textile Fibre Composition

40. In determining the textile fibre composition percentage, the following component parts of the textile product shall not be taken into account:

40.1. for all textile products:

40.1.1. non-textile parts, selvedges, labels, badges, emblems, edgings and trimmings not forming an integral part of the textile product;

40.1.2. buttons and buckles covered with textile materials;

40.1.3. accessories, decorations, non-elastic ribbons, elastic threads and bands which are attached at specific points of the textile product;

40.1.4. visible, isolated textile fibres that are decorative; and

40.1.5. textile fibres having an antistatic effect;

40.2. for floor coverings and carpets: all parts except the surface;

40.3. for furniture fabrics: binding and filling warps and wefts that do not form part of the use-surface;

40.4. for hangings and curtains: binding filling warps, binding and filling wefts, which do not form part of the right side of the fabric;

40.5. for other textile products:

40.5.1. base fabrics, as well as stiffenings and reinforcements (yarns or materials which are attached at specific points of the textile products to give them strength or thickness);

40.5.2. inter-linings and the backings thereof;

40.5.3. stitching threads if they do not replace the warp or weft of the fabric;

40.5.4. fillings which do not have heat insulating functions; and

40.5.5. linings (in compliance with the conditions referred to in Section 27 of these Regulations); and

40.6. lubricators, binders, weightings, sizings, dressings, impregnating substances, dyeing substances, printing substances and other textile processing substances. The substances referred to may not be present in textile products in such quantities that the consumer may be misled.

41. The underlying material of textile products which is utilised in double fabrics or multilayer fabrics and as bases of velvet or plush fabrics and similar products, shall not be considered as base fabrics which are to be removed prior to the specification of fibre composition.

V. Control of Labelling of Textile Products

42. The Consumer Rights Protection Centre shall control compliance with these Regulations.

43. The Consumer Rights Protection Centre shall assess the conformity of the composition of a textile product to the information indicated on the labelling, utilising the methods of analysis specified in the standards of the National Standardisation and Metrology Centre.

44. After removal of the components referred to in Paragraph 40 of these Regulations, the textile fibre percentage composition of a textile product shall be determined by applying the appropriate tolerance which is indicated in Annex 4 of these Regulations to the dehydrated mass of each textile fibre.

45. If the information provided on the labelling does not conform to the textile fibre composition specified by relevant analyses, the manufacturer or the seller shall cover the expenditures for the carrying out the analysis.

VI. Closing Provisions

46. These Regulations shall come into force on 1 January 1999

Prime Minister								G. Krasts

Minister for Economics							L. Strujevičs

Annex 1

Cabinet Regulation No. 262

21 July 1998

Names of Textile Fibres Used on the Labelling of Textile Products

| No. | Name of the textile fibre |  |
| --- | --- | --- |
|  | (the word “tekstilšķiedra” [textile fibre] does not have to be indicated on the labelling) | Fibre description |
| 1. | Wool* | fibre from sheep's or lamb's (Ovis aries) fleece |
| 2. | Wool or hair of alpaca, angora, beaver, guanaco, yak, camel, cashgora, kashmir, llama, otter, vicuna (the words “vilna" [wool] or “mati" [hair] do not have to be indicated on the labelling)* | hair of the following animals: hair of alpaca, angora goat, angora rabbit, beaver, guanaco, yak, camel, cashgora goat, kashmir goat, llama, vicuna, otter |
| 3. | Horsehair or hair of other animals (the species of animal does not have to be indicated on the labelling) | horsehair and hair of other animals not referred to in Paragraphs 1 or 2 of this Annex |
| 4. | Silk | fibre obtained from silk-worm cocoons |
| 5. | Cotton | fibre obtained from cotton (Gossypium) boll |
| 6. | Kapok | fibre obtained from kapok (Ceiba pentandra) |
| 7. | Flax | fibre obtained from the bast of flax (Linum usitatissimum) |
| 8. | True hemp | fibre obtained from the bast of hemp (Cannabis sativa) |
| 9. | Jute | fibre obtained from the bast of Corchorus olitorius and Corchorus capsularis, as well as from Hibiscus cannabinus, Hibiscus sabdariffa, Abutilon avcennae, Urena lobata, Urena sinuata |
| 10. | Abaca (Manila hemp) | fibre obtained from the leaves of Musa textilis |
| 11. | Alfa | fibre obtained from the leaves of Stipa tenacissima |
| 12. | Coir (coconut) | fibre obtained from the covering of the fruit of Cocos nucifera |
| 13. | Broom | fibre obtained from the bast of Cytisus scoparius or Spartium junceum |
| 14. | Ramie | fibre obtained from the bast of Boehmeria nivea and Boehmeria tenacissima |
| 15. | Sisal | fibre obtained from the leaves of Agave sisalana |
| 16. | Sunn | fibre obtained from the bast of Crotalaria juncea |
| 17. | Henequen | fibre obtained from the leaves of Agave fourcroydes |
| 18. | Maguey | fibre obtained from the leaves of Agave cantala |
| 19. | Acetate | cellulose acetate fibre in which 74-92% of the hydroxyl groups are acetylated |
| 20. | Alginate | fibre which obtained from the metallic salts of alginic acid |
| 21. | Cupro (cuprammonium rayon) | regenerated cellulose fibre obtained by the cupramonnium reagent method |
| 22. | Modal | regenerated cellulose fibre obtained by a modified viscose process having a high breaking force and high wet modulus. The breaking force in a conditioned state (Bc) and the force required to produce an elongation of 5% in a wet state (Bm) are: Bc (CN) 1.3 ÷ T + 2T; Bm (CN) 0.5 ÷ T, where T is the mean linear density (decitex) |
| 23. | Protein | fibre obtained from natural protein that is regenerated and stabilised through the action upon them of chemical substances |
| 24. | Triacetate | cellulose acetate fibre wherein at least 92% of the hydroxil groups are acetylated |
| 25. | Viscose | regenerated cellulose fibre obtained by the viscose process for the acquisition of filament fibre and discontinuous fibre |
| 26. | Acrylic | fibre formed of linear macromolecules having in their chain at least 85% by mass of acrylonitrilic monomers |
| 27. | Chlorofibre | fibre formed of linear macromolecules having in their chain more than 50% by mass of chlorinated vinyl or chlorinated vinylidene monomers |
| 28. | Fluorofibre | fibre formed of linear macromolecules having in the composition of their chain tetrafluoroethylene monomers |
| 29. | Modacrylic | fibre formed of linear macromolecules having in their chain 50-85% by mass acrylonitrilic monomers |
| 30. | Polyamide or nylon | fibre formed of synthetic linear macromolecules having in their chain recurring amide linkages of which at least 85% are joined to aliphatic or cycloaliphatic units |
| 31. | Aramid | fibre formed of synthetic linear macromolecules made up of aromatic groups joined by amide or imide linkages of which at least 85% are joined directly to two aromatic rings and with a number of imide linkages, if the latter do not exceed the number of amide linkages |
| 32. | Polyimide | fibre formed of synthetic linear macromolecules having in their chain periodically recurring imide functional groups |
| 33. | Lyocell | regenerated cellulose fibre obtained by dissolution, and an organic solvent spinning process |
| 34. | Polyester | fibre formed of linear macromolecules having in their chain at least 85% by mass diol and terephthalic acid ester monomers |
| 35. | Polyethylene | fibre formed of unsubstituted aliphatic saturated hydrocarbon linear macromolecules |
| 36. | Polypropylene | fibre formed of an aliphatic saturated hydrocarbon linear macromolecule where one carbon atom in two carries a methyl side chain in an isotactic disposition and without further substitution |
| 37. | Polycarbamide | fibre formed of linear macromolecules having in the chain the recurring ureylene (NH-CO-NH) functional group |
| 38. | Polyurethane | fibre formed of linear macromolecules having in their chain recurring urethane functional groups |
| 39. | Vinylal | fibre formed of linear macromolecules the chain of which is formed by polyvinyl alcohol with differing levels of acetalisation |
| 40. | Trivinyl | fibre formed of acrylonitrile terpolymer, a chlorinated vinyl monomer and a third vinyl monomer, none of which represents more than 50% of the total mass |
| 41. | Elastodiene | elastofibre composed of natural or synthetic polyisoprene, or composed of one or more dienes polymerised with or without one or more vinyl monomers, and which, when stretched to three times its original length and released, recovers rapidly and substantially to its initial length |
| 42. | Elastane | elastofibre composed of at least 85% (by mass) of a segmented polyurethane, and which, when stretched to three times its original length and released, recovers rapidly and substantially to its initial length |
| 43. | Glass fibre | fibre made of glass |
| 44. | Name corresponding to the material of which the fibres are composed, for example, metal (metallic, metallized) asbestos, paper, followed or not by the word “šķiedra" [fibre] or “pavediens" [yarn] | fibres obtained from miscellaneous or new materials not listed above |

*The name “vilna" [wool] relates also to a mixture of fibres from sheep’s or lambs’ fleeces and the hairs of the animals listed in column 3 of item 2, as well as to textile products referred to in Paragraphs 14-25 of these Regulations, if partly composed of the fibres listed in items 1 and 2 of this Annex.

Minister for Economics								L.Strujevičs

Annex 2

Cabinet Regulation No. 262

21 July 1998

Textile Products for Which the Fibre Composition need not be Indicated

1. Sleeve-supporting armbands

2. Watch straps

3. Labels, badges and emblems

4. Stuffed pan-holders

5. Coffee cosy covers

6. Tea cosy covers

7. Sleeve protectors

8. Muffs other than in pile fabric

9. Artificial flowers

10. Pin cushions

11. Painted canvas

12. Textile products used as stiffenings or for base or underlying fabrics

13. Felt and thick felt

14. Used sown textile products, where explicitly stated to be such

15. Gaiters

16. Packaging, not new and sold as such

17. Felt hats

18. Bags and saddlery of textile materials

19. Travel goods

20. Hand-embroidered tapestries (finished or unfinished) and materials for the production thereof, including embroidery yarns, sold separately from the canvas and specially presented for use in such tapestries

21. Zippers

22. Buttons and buckles covered with textile materials

23. Book covers

24. Toys

25. Textile material parts of footwear, except warm linings

26. Table mats having several components and a surface area of not more than 500 cm2

27. Oven gloves and cloths

28. Egg cosies

29. Make-up bags

30. Tobacco pouches

31. Spectacle, comb, cigarette, cigar and lighter boxes and cases

32. Protective requisites for sports (except gloves)

33. Toilet cases

34. Shoe-cleaning cases

35. Funeral items

36. Disposable articles, with the exception of wading (textile articles designed to be used once only or for a limited time period, and the normal use of which precludes any restoring for subsequent use for the same or a similar purpose)

37. Textile products which are subject to the provisions of the European Pharmacopoeia and covered by a reference to those provisions, non-disposable bandages for medical and orthopaedic use, and orthopaedic textile articles in general

38. Textile articles (including cordage, ropes and strings), intended:

38.1. for use as equipment components in the manufacture or processing of goods; and

38.2. for incorporation in machines, installations (e.g. for heating, air conditioning or lighting), domestic and other appliances, means of transport or for the operation thereof, maintenance or equipment, other than tarpaulin covers and textile motor vehicle accessories sold separately from the vehicle

39. Textile articles for safety and protection: safety belts, parachutes, life-jackets, emergency chutes, fire-fighting devices, bullet-proof waistcoats and special protective garments (e.g. protection against fire, chemical substances or other safety hazards)

40. Air-supported structures (e.g. sports halls, exhibition stands, or storage facilities), provided that the particulars of performances and technical specifications are indicated.

41. Sails.

42. Animal clothing.

43. Flags and banners.

Minister for Economics								L.Strujevičs

Annex 3

Cabinet Regulation No. 262

21 July 1998

Textile Products for Which Only Inclusive Labelling of the Textile Fibre Composition is Necessary

1. Floorcloths

2. Cleaning cloths

3. Edgings and trimmings

4. Passementerie

5. Belts

6. Braces

7. Suspenders and garters

8. Shoe and boot laces

9. Ribbons

10. Elastic

11. New packaging sold as such

12. Packing string and agricultural twine, cordage and ropes used for mountaineering and water sports, as well as cordage, ropes and strings other than those falling within item 38 of Annex 2 of these Regulations. For the textile products referred to, if sold by the metre, the inclusive labelling shall be on the reel.

13. Table mats

14. Handkerchiefs

15. Bun nets and hair nets

16. Ties and bow ties for children

17. Bibs, washgloves and face flannels

18. Sewing, mending and embroidery yarns presented for retail sale in small quantities with a net weight of 1 gram or less

19. Tape for curtains, blinds and shutters

Minister for Economics								L.Strujevičs

Annex 4

Cabinet Regulation No. 262

21 July 1998

Agreed Tolerances to be Used for Calculation of Textile Fibre Mass Contained in a Textile Product

| No. (in conformity with | Fibre name | Percentages |
| --- | --- | --- |
| Annex 1) |  |  |
| 1 | 2 | 3 |
| 1.-2. | Wool and animal hair: |  |
|  | combed (worsted) fibres | 18.25 |
|  | carded (including machine yarn) fibres (also if it is impossible to ascertain whether the textile product containing wool and/or animal hair is combed or carded) | 17.00 |
| 3. | Animal hair: |  |
| 3.1. | combed (worsted) fibres | 18.25 |
| 3.2. | carded (including machine yarn) fibres (also if it is impossible to ascertain whether the textile product containing wool and/or animal hair is combed or carded) | 17.00 |
| 3.3. | horsehair: |  |
| 3.3.1. | combed (worsted) fibres | 16.00 |
| 3.3.2. | carded (including machine yarn) fibres | 15.00 |
| 4. | Silk | 11.00 |
| 5. | Cotton: |  |
| 5.1. | normal fibres | 8.50 |
| 5.2. | mercerized fibres | 10.50 |
| 6. | Kapok | 10.90 |
| 7. | Flax | 12.00 |
| 8. | True hemp | 12.00 |
| 9. | Jute | 17.00 |
| 10. | Abaca | 14.00 |
| 11. | Alfa | 14.00 |
| 12. | Coir (coconut) | 13.00 |
| 13. | Broom | 14.00 |
| 14. | Ramie (bleached fibre) | 8.50 |
| 15. | Sisal | 14.00 |
| 16. | Sunn | 12.00 |
| 17. | Henequen | 14.00 |
| 18. | Maguey | 14.00 |
| 19. | Acetate | 19.00 |
| 20. | Alginate | 20.00 |
| 21. | Cupro | 13.00 |
| 22. | Modal | 13.00 |
| 23. | Protein | 17.00 |
| 24. | Triacetate | 7.00 |
| 25. | Viscose | 13.00 |
| 26. | Acrylic | 2.00 |
| 27. | Chlorofibre | 2.00 |
| 28. | Fluorofibre | 0.00 |
| 29. | Modacrylic | 2.00 |
| 30. | Polyamide (nylon): |  |
| 30.1. | discontinuous fibre | 6.25 |
| 30.2. | filament | 5.75 |
| 31. | Aramid | 8.00 |
| 32. | Polyimide | 3.50 |
| 33. | Lyocell | 13.00 |
| 34. | Polyester: |  |
| 34.1. | discontinous fibre | 1.50 |
| 34.2. | filament | 1.50 |
| 35. | Polyethylene | 1.50 |
| 36. | Polypropylene | 2.00 |
| 37. | Polycarbamide | 2.00 |
| 38. | Polyurethane: |  |
| 38.1. | discontinous fibre | 3.50 |
| 38.2. | filament | 3.00 |
| 39. | Vinylal | 5.00 |
| 40. | Trivinyl | 3.00 |
| 41. | Elastodiene | 1.00 |
| 42. | Elastane | 1.50 |
| 43. | Glass fibre: |  |
| 43.1. | with a diameter of over 5 mm | 2.00 |
| 43.2. | with a diameter of 5 mm or less | 3.00 |
| 44. | Fibres of other material: |  |
| 44.1. | metal fibres | 2.00 |
| 44.2. | metallized fibres | 2.00 |
| 44.3. | asbestos | 2.00 |
| 44.4. | paper yarn | 13.75 |

Minister for Economics								L.Strujevičs