Republic of Latvia

Cabinet

Regulation No. 288

Adopted 3 July 2001

Regulations Regarding Activities with Sources of Ionising Radiation for which a Special Permit (Licence) or Permit is not Required

Issued pursuant to Section 3, Paragraph one, Clause 5 and Section 11, Paragraph two, Clause 3 of the Law On Radiation Protection and Nuclear Safety

I. General Provisions

1. These Regulations determine cases where activities with sources of ionising radiation a special permit (licence) or a permit (hereinafter – activities with sources of ionising radiation) is not required, because it is not possible to influence such activities by human actions or the potential dose of ionising radiation and the harmful effects of the radiation are so small that from the viewpoint of radiation safety it need not be taken into account.

2. Persons performing activities with sources of ionising radiation shall ensure that limit values of the sources of ionising radiation specified in Paragraph 4 of these Regulations are complied with, and shall be liable for radiation safety and nuclear safety in the zone under the control of such persons.

3. If changes in the activities with sources of ionising radiation need to be made and such changes could be significant for radiation safety and nuclear safety, or when making such changes the limit values of the sources of ionising radiation specified in Paragraph 4 of these Regulations may be exceeded, the person performing the relevant activities shall inform the Radiation Safety Centre regarding the necessary changes and, if necessary, prior to making such changes shall apply for a special permit (licence) or a permit.

II. Radioactivity Limit Values, Strength of Dose

and Dose Amount of Sources of Ionising Radiation

4. A special permit (licence) or a permit is not required for activities with sources of ionising radiation if the following sources of ionising radiation are utilised:

4.1. radioactive substances. In the utilisation of radioactive substances, one of the following conditions shall be observed:

4.1.1. the total radioactivity of the relevant radio-nuclide does not exceed the level specified in Annex 1 of these Regulations; or

4.1.2. the specific radioactivity per unit weight of the relevant radio-nuclide does not exceed the limit value specified in Annex 1 of these Regulations if the total amount of the radioactive substance does not exceed 1 000 kg;

4.2. a cathode-ray tube intended for recording visual images. In utilising a cathode-ray tube one of the following conditions shall be observed:

4.2.1. under operating conditions that conform to the conditions prescribed by the manufacturer, the strength of ionising radiation dose created by a cathode-ray tube is less than 1 μSv per hour at a distance of 0.1 m from any accessible part of the surface of the apparatus; or

4.2.2. the voltage is less than 5 kV;

4.3. electric apparatus. In utilising electric apparatus, one of the following conditions shall be observed:

4.3.1. under operating conditions that conform to the conditions prescribed by the manufacturer, the strength of ionising radiation dose created by such apparatus is less than 1 μSv per hour at a distance of 0.1 m from any accessible part of the surface of the apparatus; or

4.3.2. the voltage is less than 5 kV;

4.4. equipment or apparatus that contain a sealed radiation source and the total radioactivity, specific radioactivity or both of these radioactive levels for the relevant radionuclide present in the radioactive source exceed the levels specified in Annex 1 of these Regulations. In utilisation of such equipment or apparatus one of the following conditions shall be observed:

4.4.1. under operating conditions that conform to the conditions prescribed by the manufacturer, the strength of ionising radiation dose created by such apparatus is less than 1 μSv per hour at a distance of 0.1 m from any accessible part of the surface of the apparatus; or

4.4.2. the expected effective radiation dose for a person is less than 10 μSv per year;

4.5. smoke detectors in which the americium isotope 241Am is utilised if the total radioactivity of a single detector does not exceed 74 kBq and the total number of such detectors is 100 units;

4.6. thorium alloys in electrodes of luminescent lamps, discharge tubes and in picture tubes, as well as in filaments of heated gas-filled lamps, if the number of units of the relevant products to be stored in a single undertaking does not exceed 10 000 units per year;

4.7. thorium in special alloys in aircraft engines or refractory laboratory equipment if the total weight of such alloys does not exceed 1 000 kg;

4.8. thorium in electrodes for welding if the concentration of thorium is less than 5% and the total quantity of such electrodes utilised per year does not exceed 1 000 kg; and

4.9. natural radioactive materials the specific radioactivity of which exceeds the levels specified in Annex 1 of these Regulations if such material is not specially treated in order to increase its specific radioactivity. In utilising natural radioactive materials, the condition shall be observed that during performance of activities with such materials the total dose of ionising radiation received by an employees does not exceed 1 mSv per year.

5. In a mixture of radionuclides the quotients of the total or specific radioactivity of each nuclide divided by the value of the relevant radionuclide specified in Annex 1 of these Regulations shall be summed up and multiplied by the percentage concentration of the relevant nuclide in the mixture. If the sum is less than 1 or equal to 1, a special permit (licence) or a permit is not necessary.

6. If Annex 1 of these Regulations specifies that in the process of degradation of a radio-nuclide (parent nuclide) other radio-nuclides (daughter nuclides) referred to in Annex 2 of these Regulations are generated, the values of radioactivity specified in Annex 1 of these Regulations apply not only to the parent nuclide but also to the relevant daughter nuclides. In such case, in order to determine whether activities with sources of ionising radiation require a special permit (licence) or a permit, the value specified for the parent nuclide shall be utilised.

III. Suspension of Activities with Sources of Ionising Radiation

7. The Radiation Safety Centre may suspend activities with sources of ionising radiation in cases prescribed in Section 6, Paragraph 2 of the Law On Radiation Protection and Nuclear Safety as well as in cases where it has been established that the limit values of ionising radiation specified in Paragraph 4 of these Regulations have been exceeded.

8. If a decision to suspend activities with sources of ionising radiation has been taken, the Radiation Safety Centre not later than within 10 days of the date the decision was taken shall inform the person performing the relevant activities thereof in writing. The person referred to shall suspend the activities with sources of ionising radiation set out in the decision on the day of receipt of such decision.

Prime Minister								A. Bērziņš

Minister for Environmental Protection and Regional Development		V. Makarovs

Annex 1

Cabinet Regulation No. 288

3 July 2001

Limit Values of Radioactivity up to which for Activities with Sources of Ionising Radiation a Special Permit (Licence) or Permit is not required

| No. | Radio-nuclide 1 | Total radioactivity Bq | Specific radioactivity Bq/g |
| --- | --- | --- | --- |
| 1 | 3H | 1 x 109 | 1 x 106 |
| 2 | 7Be | 1 x 107 | 1 x 103 |
| 3 | 14C | 1x 107 | 1 x 104 |
| 4 | 15O | 1 x 109 | 1 x 102 |
| 5 | 18F | 1 x 106 | 1 x 101 |
| 6 | 22Na | 1 x 106 | 1 x 101 |
| 7 | 24Na | 1 x 105 | 1 x 101 |
| 8 | 31Si | 1 x 106 | 1 x 103 |
| 9 | 32P | 1 x 105 | 1 x 103 |
| 10 | 33P | 1 x 108 | 1 x 105 |
| 11 | 35S | 1 x 108 | 1 x 105 |
| 12 | 36Cl | 1 x 106 | 1 x 104 |
| 13 | 38Cl | 1 x 105 | 1 x 101 |
| 14 | 37Ar | 1 x 108 | 1 x 106 |
| 15 | 41Ar | 1 x 109 | 1 x 102 |
| 16 | 40K | 1 x 106 | 1 x 102 |
| 17 | 42K | 1 x 106 | 1 x 102 |
| 18 | 43K | 1 x 106 | 1 x 101 |
| 19 | 45Ca | 1 x 107 | 1 x 104 |
| 20 | 47Ca | 1 x 106 | 1 x 101 |
| 21 | 46Sc | 1 x 106 | 1 x 101 |
| 22 | 47Sc | 1 x 106 | 1 x 102 |
| 23 | 48Sc | 1 x 105 | 1 x 101 |
| 24 | 48V | 1 x 105 | 1 x 101 |
| 25 | 51Cr | 1 x 107 | 1 x 103 |
| 26 | 51Mn | 1 x 105 | 1 x 101 |
| 27 | 52Mn | 1 x 105 | 1 x 101 |
| 28 | 52mMn | 1 x 105 | 1 x 101 |
| 29 | 53Mn | 1 x 109 | 1 x 104 |
| 30 | 54Mn | 1 x 106 | 1 x 101 |
| 31 | 56Mn | 1 x 105 | 1 x 101 |
| 32 | 52Fe | 1 x 106 | 1 x 101 |
| 33 | 55Fe | 1 x 106 | 1 x 104 |
| 34 | 59Fe | 1 x 106 | 1 x 101 |
| 35 | 55Co | 1 x 106 | 1 x 101 |
| 36 | 56Co | 1 x 105 | 1 x 101 |
| 37 | 57Co | 1 x 106 | 1 x 102 |
| 38 | 58Co | 1 x 106 | 1 x 101 |
| 39 | 58mCo | 1 x 107 | 1 x 104 |
| 40 | 60Co | 1 x 105 | 1 x 101 |
| 41 | 60mCo | 1 x 106 | 1 x 103 |
| 42 | 61Co | 1 x 106 | 1 x 102 |
| 43 | 62mCo | 1 x 105 | 1 x 101 |
| 44 | 59Ni | 1 x 108 | 1 x 104 |
| 45 | 63Ni | 1 x 108 | 1 x 105 |
| 46 | 65Ni | 1 x 106 | 1 x 101 |
| 47 | 64Cu | 1 x 106 | 1 x 102 |
| 48 | 65Zn | 1 x 106 | 1 x 101 |
| 49 | 69Zn | 1 x 106 | 1 x 104 |
| 50 | 69mZn | 1 x 106 | 1 x 102 |
| 51 | 72Ga | 1 x 105 | 1 x 101 |
| 52 | 71Ge | 1 x 108 | 1 x 104 |
| 53 | 73As | 1 x 107 | 1 x 103 |
| 54 | 74As | 1 x 106 | 1 x 101 |
| 55 | 76As | 1 x 105 | 1 x 102 |
| 56 | 77As | 1 x 106 | 1 x 103 |
| 57 | 75Se | 1 x 106 | 1 x 102 |
| 58 | 82Br | 1 x 106 | 1 x 101 |
| 59 | 74Kr | 1 x 109 | 1 x 102 |
| 60 | 76Kr | 1 x 109 | 1 x 102 |
| 61 | 77Kr | 1 x 109 | 1 x 102 |
| 62 | 79Kr | 1 x 109 | 1 x 103 |
| 63 | 81Kr | 1 x 107 | 1 x 104 |
| 64 | 83mKr | 1 x 1012 | 1 x 105 |
| 65 | 85Kr | 1 x 104 | 1 x 105 |
| 66 | 85mKr | 1 x 1010 | 1 x 103 |
| 67 | 87Kr | 1 x 109 | 1 x 102 |
| 68 | 88Kr | 1 x 109 | 1 x 102 |
| 69 | 86Rb | 1 x 105 | 1 x 102 |
| 70 | 85Sr | 1 x 106 | 1 x 102 |
| 71 | 85mSr | 1 x 107 | 1 x 102 |
| 72 | 87mSr | 1 x 106 | 1 x 102 |
| 73 | 89Sr | 1 x 106 | 1 x 103 |
| 74 | 90Sr+ | 1 x 104 | 1 x 102 |
| 75 | 91Sr | 1 x 105 | 1 x 101 |
| 76 | 92Sr | 1 x 106 | 1 x 101 |
| 77 | 90Y | 1 x 105 | 1 x 103 |
| 78 | 91Y | 1 x 106 | 1 x 103 |
| 79 | 91mY | 1 x 106 | 1 x 102 |
| 80 | 92Y | 1 x 105 | 1 x 102 |
| 81 | 93Y | 1 x 105 | 1 x 102 |
| 82 | 93Zr+ | 1 x 107 | 1 x 103 |
| 83 | 95Zr | 1 x 106 | 1 x 101 |
| 84 | 97Zr+ | 1 x 105 | 1 x 101 |
| 85 | 93mNb | 1 x 107 | 1 x 104 |
| 86 | 94Nb | 1 x 106 | 1 x 101 |
| 87 | 95Nb | 1 x 106 | 1 x 101 |
| 88 | 97Nb | 1 x 106 | 1 x 101 |
| 89 | 98Nb | 1 x 105 | 1 x 101 |
| 90 | 90Mo | 1 x 106 | 1 x 101 |
| 91 | 93Mo | 1 x 108 | 1 x 103 |
| 92 | 99Mo | 1 x 106 | 1 x 102 |
| 93 | 101Mo | 1 x 106 | 1 x 101 |
| 94 | 96Tc | 1 x 106 | 1 x 101 |
| 95 | 96mTc | 1 x 107 | 1 x 103 |
| 96 | 97Tc | 1 x 108 | 1 x 103 |
| 97 | 97mTc | 1 x 107 | 1 x 103 |
| 98 | 99Tc | 1 x 107 | 1 x 104 |
| 99 | 99mTc | 1 x 107 | 1 x 102 |
| 100 | 97Ru | 1 x 107 | 1 x 102 |
| 101 | 103Ru | 1 x 106 | 1 x 102 |
| 102 | 105Ru | 1 x 106 | 1 x 101 |
| 103 | 106Ru+ | 1 x 105 | 1 x 102 |
| 104 | 103mRh | 1 x 108 | 1 x 104 |
| 105 | 105Rh | 1 x 107 | 1 x 102 |
| 106 | 103Pd | 1 x 108 | 1 x 103 |
| 107 | 109Pd | 1 x 106 | 1 x 103 |
| 108 | 105Ag | 1 x 106 | 1 x 102 |
| 109 | 108mAg+ | 1 x 106 | 1 x 101 |
| 110 | 110mAg | 1 x 106 | 1 x 101 |
| 111 | 111Ag | 1 x 106 | 1 x 103 |
| 112 | 109Cd | 1 x 106 | 1 x 104 |
| 113 | 115Cd | 1 x 106 | 1 x 102 |
| 114 | 115mCd | 1 x 106 | 1 x 103 |
| 115 | 111In | 1 x 106 | 1 x 102 |
| 116 | 113mIn | 1 x 106 | 1 x 102 |
| 117 | 114mIn | 1 x 106 | 1 x 102 |
| 118 | 115mIn | 1 x 106 | 1 x 102 |
| 119 | 113Sn | 1 x 107 | 1 x 103 |
| 120 | 125Sn | 1 x 105 | 1 x 102 |
| 121 | 122Sb | 1 x 104 | 1 x 102 |
| 122 | 124Sb | 1 x 106 | 1 x 101 |
| 123 | 125Sb | 1 x 106 | 1 x 102 |
| 124 | 123mTe | 1 x 107 | 1 x 102 |
| 125 | 125mTe | 1 x 107 | 1 x 103 |
| 126 | 127Te | 1 x 106 | 1 x 103 |
| 127 | 127mTe | 1 x 107 | 1 x 103 |
| 128 | 129Te | 1 x 106 | 1 x 102 |
| 129 | 129mTe | 1 x 106 | 1 x 103 |
| 130 | 131Te | 1 x 105 | 1 x 102 |
| 131 | 131mTe | 1 x 106 | 1 x 101 |
| 132 | 132Te | 1 x 107 | 1 x 102 |
| 133 | 133Te | 1 x 105 | 1 x 101 |
| 134 | 133mTe | 1 x 105 | 1 x 101 |
| 135 | 134Te | 1 x 106 | 1 x 101 |
| 136 | 123I | 1 x 107 | 1 x 102 |
| 137 | 125I | 1 x 106 | 1 x 103 |
| 138 | 126I | 1 x 106 | 1 x 102 |
| 139 | 129I | 1 x 105 | 1 x 102 |
| 140 | 130I | 1 x 106 | 1 x 101 |
| 141 | 131I | 1 x 106 | 1 x 102 |
| 142 | 132I | 1 x 105 | 1 x 101 |
| 143 | 133I | 1 x 106 | 1 x 101 |
| 144 | 134I | 1 x 105 | 1 x 101 |
| 145 | 135I | 1 x 106 | 1 x 101 |
| 146 | 131mXe | 1 x 104 | 1 x 104 |
| 147 | 133Xe | 1 x 104 | 1 x 103 |
| 148 | 135Xe | 1 x 1010 | 1 x 103 |
| 149 | 129Cs | 1 x 105 | 1 x 102 |
| 150 | 131Cs | 1 x 106 | 1 x 103 |
| 151 | 132Cs | 1 x 105 | 1 x 101 |
| 152 | 134mCs | 1 x 105 | 1 x 103 |
| 153 | 134Cs | 1 x 104 | 1 x 101 |
| 154 | 135Cs | 1 x 107 | 1 x 104 |
| 155 | 136Cs | 1 x 105 | 1 x 101 |
| 156 | 137Cs+ | 1 x 104 | 1 x 101 |
| 157 | 138Cs | 1 x 104 | 1 x 101 |
| 158 | 131Ba | 1 x 106 | 1 x 102 |
| 159 | 140Ba+ | 1 x 105 | 1 x 101 |
| 160 | 140La | 1 x 105 | 1 x 101 |
| 161 | 139Ce | 1 x 106 | 1 x 102 |
| 162 | 141Ce | 1 x 107 | 1 x 102 |
| 163 | 143Ce | 1 x 106 | 1 x 102 |
| 164 | 144Ce+ | 1 x 105 | 1 x 102 |
| 165 | 142Pr | 1 x 105 | 1 x 102 |
| 166 | 143Pr | 1 x 106 | 1 x 104 |
| 167 | 147Nd | 1 x 106 | 1 x 102 |
| 168 | 149Nd | 1 x 106 | 1 x 102 |
| 169 | 147Pm | 1 x 107 | 1 x 104 |
| 170 | 149Pm | 1 x 106 | 1 x 103 |
| 171 | 151Sm | 1 x 108 | 1 x 104 |
| 172 | 153Sm | 1 x 106 | 1 x 102 |
| 173 | 152Eu | 1 x 106 | 1 x 101 |
| 174 | 152mEu | 1 x 106 | 1 x 102 |
| 175 | 154Eu | 1 x 106 | 1 x 101 |
| 176 | 155Eu | 1 x 107 | 1 x 102 |
| 177 | 153Gd | 1 x 107 | 1 x 102 |
| 178 | 159Gd | 1 x 106 | 1 x 103 |
| 179 | 160Tb | 1 x 106 | 1 x 101 |
| 180 | 165Dy | 1 x 106 | 1 x 103 |
| 181 | 166Dy | 1 x 106 | 1 x 103 |
| 182 | 166Ho | 1 x 105 | 1 x 103 |
| 183 | 169Er | 1 x 107 | 1 x 104 |
| 184 | 171Er | 1 x 106 | 1 x 102 |
| 185 | 170Tm | 1 x 106 | 1 x 103 |
| 186 | 171Tm | 1 x 108 | 1 x 104 |
| 187 | 175Yb | 1 x 107 | 1 x 103 |
| 188 | 177Lu | 1 x 107 | 1 x 103 |
| 189 | 181Hf | 1 x 106 | 1 x 101 |
| 190 | 182Ta | 1 x 104 | 1 x 101 |
| 191 | 181W | 1 x 107 | 1 x 103 |
| 192 | 185W | 1 x 107 | 1 x 104 |
| 193 | 187W | 1 x 106 | 1 x 102 |
| 194 | 186Re | 1 x 106 | 1 x 103 |
| 195 | 188Re | 1 x 105 | 1 x 102 |
| 196 | 185Os | 1 x 106 | 1 x 101 |
| 197 | 191Os | 1 x 107 | 1 x 102 |
| 198 | 191mOs | 1 x 107 | 1 x 103 |
| 199 | 193Os | 1 x 106 | 1 x 102 |
| 200 | 190Ir | 1 x 106 | 1 x 101 |
| 201 | 192Ir | 1 x 104 | 1 x 101 |
| 202 | 194Ir | 1 x 105 | 1 x 102 |
| 203 | 191Pt | 1 x 106 | 1 x 102 |
| 204 | 193mPt | 1 x 107 | 1 x 103 |
| 205 | 197Pt | 1 x 106 | 1 x 103 |
| 206 | 197mPt | 1 x 106 | 1 x 102 |
| 207 | 198Au | 1 x 106 | 1 x 102 |
| 208 | 199Au | 1 x 106 | 1 x 102 |
| 209 | 197Hg | 1 x 107 | 1 x 102 |
| 210 | 197mHg | 1 x 106 | 1 x 102 |
| 211 | 203Hg | 1 x 105 | 1 x 102 |
| 212 | 200Tl | 1 x 106 | 1 x 101 |
| 213 | 201Tl | 1 x 106 | 1 x 102 |
| 214 | 202Tl | 1 x 106 | 1 x 102 |
| 215 | 204Tl | 1 x 104 | 1 x 104 |
| 216 | 203Pb | 1 x 106 | 1 x 102 |
| 217 | 210Pb+ | 1 x 104 | 1 x 101 |
| 218 | 212Pb+ | 1 x 105 | 1 x 101 |
| 219 | 206Bi | 1 x 105 | 1 x 101 |
| 220 | 207Bi | 1 x 106 | 1 x 101 |
| 221 | 210Bi | 1 x 106 | 1 x 103 |
| 222 | 212Bi+ | 1 x 105 | 1 x 101 |
| 223 | 203Po | 1 x 106 | 1 x 101 |
| 224 | 205Po | 1 x 106 | 1 x 101 |
| 225 | 207Po | 1 x 106 | 1 x 101 |
| 226 | 210Po | 1 x 104 | 1 x 101 |
| 227 | 211At | 1 x 107 | 1 x 103 |
| 228 | 220Rn+ | 1 x 107 | 1 x 104 |
| 229 | 222Rn+ | 1 x 108 | 1 x 101 |
| 230 | 223Ra+ | 1 x 105 | 1 x 102 |
| 231 | 224Ra+ | 1 x 105 | 1 x 101 |
| 232 | 225Ra | 1 x 105 | 1 x 102 |
| 233 | 226Ra+ | 1 x 104 | 1 x 101 |
| 234 | 227Ra | 1 x 106 | 1 x 102 |
| 235 | 228Ra+ | 1 x 105 | 1 x 101 |
| 236 | 228Ac | 1 x 106 | 1 x 101 |
| 237 | 226Th+ | 1 x 107 | 1 x 103 |
| 238 | 227Th | 1 x 104 | 1 x 101 |
| 239 | 228Th+ | 1 x 104 | 1 x 100 |
| 240 | 229Th+ | 1 x 103 | 1 x 100 |
| 241 | 230Th | 1 x 104 | 1 x 100 |
| 242 | 231Th | 1 x 107 | 1 x 103 |
| 243 | dab.Th (232Th) | 1 x 103 | 1 x 100 |
| 244 | 234Th+ | 1 x 105 | 1 x 103 |
| 245 | 230Pa | 1 x 106 | 1 x 101 |
| 246 | 231Pa | 1 x 103 | 1 x 100 |
| 247 | 233Pa | 1 x 107 | 1 x 102 |
| 248 | 230U+ | 1 x 105 | 1 x 101 |
| 249 | 231U | 1 x 107 | 1 x 102 |
| 250 | 232U+ | 1 x 103 | 1 x 100 |
| 251 | 233U | 1 x 104 | 1 x 101 |
| 252 | 234U | 1 x 104 | 1 x 101 |
| 253 | 235U+ | 1 x 104 | 1 x 101 |
| 254 | 236U | 1 x 104 | 1 x 101 |
| 255 | 237U | 1 x 106 | 1 x 102 |
| 256 | 238U+ | 1 x 104 | 1 x 101 |
| 257 | dab.U | 1 x 103 | 1 x 100 |
| 258 | 239U | 1 x 106 | 1 x 102 |
| 259 | 240U | 1 x 107 | 1 x 103 |
| 260 | 240U+ | 1 x 106 | 1 x 101 |
| 261 | 237Np+ | 1 x 103 | 1 x 100 |
| 262 | 239Np | 1 x 107 | 1 x 102 |
| 263 | 240Np | 1 x 106 | 1 x 101 |
| 264 | 234Pu | 1 x 107 | 1 x 102 |
| 265 | 235Pu | 1 x 107 | 1 x 102 |
| 266 | 236Pu | 1 x 104 | 1 x 101 |
| 267 | 237Pu | 1 x 107 | 1 x 103 |
| 268 | 238Pu | 1 x 104 | 1 x 100 |
| 269 | 239Pu | 1 x 104 | 1 x 100 |
| 270 | 240Pu | 1 x 103 | 1 x 100 |
| 271 | 241Pu | 1 x 105 | 1 x 102 |
| 272 | 242Pu | 1 x 104 | 1 x 100 |
| 273 | 243Pu | 1 x 107 | 1 x 103 |
| 274 | 244Pu | 1 x 104 | 1 x 100 |
| 275 | 241Am | 1 x 104 | 1 x 100 |
| 276 | 242Am | 1 x 106 | 1 x 103 |
| 277 | 242mAm+ | 1 x 104 | 1 x 101 |
| 278 | 243Am+ | 1 x 103 | 1 x 100 |
| 279 | 242Cm | 1 x 105 | 1 x 102 |
| 280 | 243Cm | 1 x 104 | 1 x 100 |
| 281 | 244Cm | 1 x 104 | 1 x 101 |
| 282 | 245Cm | 1 x 103 | 1 x 100 |
| 283 | 246Cm | 1 x 103 | 1 x 100 |
| 284 | 247Cm | 1 x 103 | 1 x 100 |
| 285 | 248Cm | 1 x 103 | 1 x 100 |
| 286 | 249Bk | 1 x 106 | 1 x 103 |
| 287 | 246Cf | 1 x 106 | 1 x 103 |
| 288 | 248Cf | 1 x 104 | 1 x 101 |
| 289 | 249Cf | 1 x 103 | 1 x 100 |
| 290 | 250Cf | 1 x 104 | 1 x 101 |
| 291 | 251Cf | 1 x 103 | 1 x 100 |
| 292 | 252Cf | 1 x 104 | 1 x 101 |
| 293 | 253Cf | 1 x 105 | 1 x 102 |
| 294 | 254Cf | 1 x 103 | 1 x 100 |
| 295 | 253Es | 1 x 105 | 1 x 102 |
| 296 | 254Es | 1 x 104 | 1 x 101 |
| 297 | 254mEs | 1 x 106 | 1 x 102 |
| 298 | 254Fm | 1 x 107 | 1 x 104 |
| 299 | 255Fm | 1 x 106 | 1 x 103 |

1. A sign "+" or "dab" shall indicate parent nuclide in balance with the relevant daughter nuclides specified in Annex 2 of these Regulations.

Minister for Environmental Protection and Regional Development		V. Makarovs

Annex 2

Cabinet Regulation No. 288

3 July 2001

List of Nuclides that are in Long-term Balance

| No. | Parent nuclide | Daughter nuclides |
| --- | --- | --- |
| 1 | 90Sr+ | 90Y |
| 2 | 93Zr+ | 93mNb |
| 3 | 97Zr+ | 97Nb |
| 4 | 106Ru+ | 106Rh |
| 5 | 108mAg+ | 108Ag |
| 6 | 137Cs+ | 137mBa |
| 7 | 134Ce+ | 134La |
| 8 | 144Ce+ | 144Pr |
| 9 | 140Ba+ | 140La |
| 10 | 212Bi+ | 208Tl (0,36), 212Po (0,64) |
| 11 | 210Pb+ | 210Bi, 210Po |
| 12 | 212Pb+ | 212Bi, 208Tl (0,36), 212Po (0,64) |
| 13 | 220Rn+ | 216Po |
| 14 | 222Rn+ | 218Po, 214Pb, 214Bi, 214Po |
| 15 | 223Ra+ | 219Rn, 215Po, 211Pb, 211Bi, 207Tl |
| 16 | 224Ra+ | 220Rn, 216Po, 212Pb, 212Bi, 208Tl (0,36), 212Po (0,64) |
| 17 | 226Ra+ | 222Rn, 218Po, 214Pb, 214Bi,214Po, 210Pb, 210Bi, 210Po |
| 18 | 228Ra+ | 228Ac |
| 19 | 226Th+ | 222Ra, 218Rn, 214Po |
| 20 | 228Th+ | 224Ra, 220Rn, 216Po, 212Pb, 212Bi, 208Tl (0,36), 212Po (0,64) |
| 21 | 229Th+ | 225Ra, 225Ac, 221Fr, 217At, 213Bi, 213Po, 209Pb |
| 22 | dab.Th | 228Ra, 228Ac, 228Th, 224Ra, 220Rn, 216Po, 212Pb, 212Bi, 208Tl (0,36), 212Po (0,64) |
| 23 | 234Th+ | 234mPa |
| 24 | 230U+ | 226Th, 222Ra, 218Rn, 214Po |
| 25 | 232U+ | 228Th, 224Ra, 220Rn, 216Po, 212Pb,212Bi,208Tl (0,36), 212Po (0,64) |
| 26 | 235U+ | 231Th |
| 27 | 238U+ | 234Th, 234mPa |
| 28 | dab.U | 234Th, 234mPa, 234U, 230Th, 226Ra, 222Rn, 218Po, 214Pb, 214Bi, 214Po, 210Pb, 210Bi, 210Po |
| 29 | 240U+ | 240mNp |
| 30 | 237Np+ | 233Pa |
| 31 | 242mAm+ | 242Am |
| 32 | 243Am+ | 239Np |

Minister for Environmental Protection and Regional Development			V. Makarovs