Text consolidated by Valsts valodas centrs (State Language Centre) with amending regulations of:

8 July 2003 (No. 376);

3 February 2004 (No. 64);

22 February 2004 (No. 415);

26 July 2005 (No. 567);

25 July 2006 (No. 602);

19 December 2006 (No. 1021);

6 January 2009 (No. 11);

13 October 2009 (No. 1169).

If a whole or part of a paragraph has been amended, the date of the amending regulation appears in square brackets at the end of the paragraph. If a whole paragraph or sub-paragraph has been deleted, the date of the deletion appears in square brackets beside the deleted paragraph or sub-paragraph.

Republic of Latvia

Cabinet

Regulation No. 294

Adopted 9 July 2002

Procedures by which Polluting Activities of Category A, B and C shall be Declared and Permits for the Performance of Category A and B Polluting Activities shall be Issued

Issued pursuant to

Section 20, Paragraph five,

Section 22, Paragraphs two and three,

Section 23, Paragraph two,

Section 29, Paragraphs one, two, three and five,

Section 30, Paragraph one,

Section 32, Paragraph two, and Clause 1 of the

Transitional Provisions

of the Law On Pollution

I. General Provisions

1. These Regulations prescribe:

1.1. the conditions for the declaration of a Category A polluting activity, as well as the conditions on the basis of which a permit shall be issued for the performance of a Category A polluting activity (hereinafter – Category A permit), and the conditions for the use of the best available techniques, including:

1.1.1. a timetable in accordance with which Category A permits shall be issued for installations for which such permits are required;

1.1.2. the maximum transition period within which compliance with the conditions of a Category A permit in relation to the use of the best available techniques shall be ensured;

1.1.3. samples of submission and permit forms;

1.1.4. the procedures by which a permit shall be requested and issued;

1.1.5. the procedures by which the public may become acquainted with a submission and submit its proposals, become acquainted with the permit conditions, as well as with the monitoring and examination results; and

1.1.6. the procedures by which other states shall be informed if transboundary impact of pollution is possible, and monitoring of such pollution;

1.2. the conditions for the declaration of a Category B polluting activity, as well as the conditions on the basis of which a permit shall be issued for the performance of a Category B polluting activity (hereinafter – Category B permit), including:

1.2.1. samples of submission and permit forms;

1.2.2. the procedures by which a permit shall be requested and issued;

1.2.3. the procedures by which the public may become acquainted with the permit conditions, as well as with the monitoring and examination results;

1.2.4. cases where a submission for a permit shall be accessible to the public for the submission of proposals; and

1.2.5. a timetable by which the conditions of the Law On Pollution shall come into force in relation to existing Category B polluting activities;

1.3. Category C polluting activities, as well as the content and the procedures for presenting a submission of a polluting activity;

1.4. the time period after which a Category A or Category B polluting activity may not be commenced without a Category A or Category B permit;

1.5. the deadline by which an operator shall present a submission for the receipt of a permit for an existing Category A or Category B polluting activity;

1.6. the time period after which an existing Category A or Category B polluting activity may not be continued, if a Category A or Category B permit has not been received;

1.7. the time period within which an operator shall announce a change of activity; and

1.8. the procedures by which a Regional Environmental Board of the State Environmental Service (hereinafter – Board) shall review the conditions that are specified in a Category A or Category B permit (hereinafter – permit) and, if necessary, shall renew or supplement such conditions.

[3 February 2004]

2. Category B polluting activities are specified in Annex 1 of these Regulations.

3. Category C polluting activities are specified in Annex 2 of these Regulations.

II. Procedures for Presentation of a Submission

4. In order to receive a permit, an operator shall present a submission to the Board:

4.1. at least 180 days before the intended commencement of a Category A polluting activity or substantial changes in an existing polluting activity;

4.2. at least 90 days before the intended commencement of a Category B polluting activity or substantial changes in an existing polluting activity;

4.3. at least 90 days before the expiry of the time period of a Category A permit;

4.4. at least 60 days before the expiry of the time period of a Category B permit;

4.5. in accordance with the time period specified in Annex 3 of these Regulations for an existing Category A polluting activity, if substantial changes in the polluting activity are not intended before the expiry of the relevant time period; or

4.6. in accordance with the time period specified in Annex 1 of these Regulations for an existing Category B polluting activity, if substantial changes in the polluting activity are not intended before the expiry of the relevant time period.

[8 July 2003]

4.1 The Board, on the basis of a substantiated submission of an operator, may take a decision to extend the time period for the presentation of a submission up to 60 days for the continuation of a Category B polluting activity and up to 90 days for the continuation of a Category A polluting activity if the operator will not continue the polluting activity after the expiry of the extended time period for the presentation of a submission or will continue a polluting activity of such category for which less stringent environmental protection requirements are specified.

[22 April 2004]

5. An operator shall present a submission regarding a Category C polluting activity to the Board at least 30 days before the intended commencement of the Category C polluting activity or substantial changes in the Category C polluting activity.

6. If an operator is unable to determine whether a polluting activity corresponds to Category A, B or C, he or she shall clarify such correspondence at the Board by submitting information regarding the production capacity of an installation, the amount of manufactured products or other indicators specified in the Law On Pollution or in Annex 1 or 2 of these Regulations. The Board shall provide an opinion regarding the category of the polluting activity (Category A, B or C) not later than 14 days after the receipt of the information.

7. If several polluting activities are performed in an installation that correspond to different categories of polluting activities, an operator shall present only one submission to the Board for the performance of polluting activities of such category for which more stringent environmental protection requirements have been specified. Within the meaning of these Regulations the most stringent environmental protection requirements have been put forth for Category A polluting activities, followed by Category B and Category C polluting activities. All the polluting activities shall be described in the submission.

[26 July 2005]

7.1 If a greenhouse gas emission permit has been issued for an installation and the information presented in a submission for the receipt of a permit for Category A or B polluting activities differs from the information included in the conditions of the greenhouse gas emission permit, an operator shall present a submission simultaneously with the submission for the receipt of a permit for Category A or B polluting activities in which such operator shall indicate which amendments are required to the conditions of the greenhouse gas emission permit. The referred to submission shall be accessible to the public in accordance with the same conditions as are specified for a submission for the receipt of a permit for Category A or B polluting activity.

[26 July 2005]

7.2 If in accordance with the amendments to these Regulations an operator requires a Category B permit that was not required before the coming into force of the amendments, the operator shall present to the Board a submission for the receipt of a Category B permit within six months after the coming into force of the amendments, unless a different time period has been specified in these Regulations.

[26 July 2005]

7.3 If in accordance with the amendments to these Regulations, an operator requires a Category C confirmation that was not required before the coming into force of the amendments, the operator shall present the relevant submission to the Board within three months after the coming into force of the amendments.

[26 July 2005]

8. In presenting a submission in accordance with Paragraph 7 of these Regulations, an operator shall indicate in the submission which categories of polluting activities are included therein.

9. An operator shall prepare a submission for a Category A or Category B polluting activity in accordance with Annex 4 to these Regulations. The tables of the submission shall be numbered in accordance with Annex 4 to these Regulations. The operator shall present the submission in printed form (the relevant information shall be presented also in electronic form) or in the form of electronic document.

[6 January 2009]

10. If one of the points of a submission does not apply to a Category A or Category B polluting activity, an operator shall not complete it. If the operator is unable to determine which points of the submission do not apply to the relevant polluting activity, he or she shall clarify it in writing at the Board. The Board shall, within 14 days, provide an opinion regarding which points of the submission do not apply to the relevant polluting activity.

[19 December 2006]

11. An operator shall attach documents that substantiate the information referred to in a submission in the Annex to the submission, indicating references to such documents in the submission.

12. [26 July 2005]

13. In presenting a submission for the receipt of a permit for the commencement of a polluting activity or substantial changes in an existing polluting activity, an operator shall indicate the date of issuance of the planning and architectural task or a building permit and the number thereof.

[6 January 2009]

14. [26 July 2005]

15. [26 July 2005]

16. An operator shall attach to a submission a management balance of solvents used in an installation, if it is provided for in the regulatory enactments regulating emissions from stationary sources of pollution.

17. An operator shall include in a submission a summary of the measures referred to in a submission, which provide information to the public regarding the effect of a polluting activity on human health and the environment. In order for the summary to be comprehensible to the public, technical descriptions and detailed information shall not be used therein. Restricted access information shall not be included in the summary.

[26 July 2005]

18. Before declaring a Category A polluting activity an operator shall consult with the Board and take into account the relevant guidelines of the best available techniques. The Board shall indicate information regarding the guidelines of the best available techniques at the request of the operator.

19. If it is possible to perform a Category A polluting activity on the basis of several guidelines of the best available techniques, priority shall be given to the guidelines approved or drawn up by the European Commission, or to the guidelines developed by other states that provide for higher environmental protection requirements (the level of detail, time of issuance and suitability thereof for the relevant installation shall be taken into account, as well as the efficiency of costs in implementing the conditions of the guidelines).

[26 July 2005; 6 January 2009]

20. In presenting a submission for existing Category A polluting activities, an operator shall attach a plan to the submission, in order to ensure that the emission does not exceed the limit values specified in the guidelines of the best available techniques. The plan shall provide for the aims and the time period for the implementation thereof, the necessary modifications, modernisation and replacement of chemical substances or chemical products (in order to reduce the risk to human health and the environment) and the time period for the implementation thereof, and shall also indicate the investments and contributions in fixed capital in order to implement the planned measures.

20.1 If an existing installation that causes Category A or B pollution does not fully comply with the environmental protection requirements specified by the regulatory enactments regulating the field of environmental protection, an operator shall present to the Board a plan of how the fulfilment of the referred to requirements will be ensured.

[26 July 2005]

20.2 The best available techniques for Category A installations shall be introduced within the time period specified in the regulatory enactments regulating the environmental protection.

[6 January 2009]

21. In completing the tables of a submission, an operator shall provide detailed information regarding the emission of the substances and groups of substances referred to in Annex 5 to these Regulations. With regard to emission into water, an operator shall provide detailed information regarding the substances and groups of substances specified in Annexes 1 and 2 to Cabinet Regulation No. 34 of 22 January 2002, Regulations regarding Emission of Polluting Substances into Water, as well as regarding substances that have been specified in Annex 2 to Cabinet Regulation No. 858 of 19 October 2004, Regulations regarding Characterisation, Classification, Quality Criteria of Surface Water Bodies and Procedures for Determination of Anthropogenic Loads.

[19 December 2006]

22. The Board shall, within 20 working days after receipt of the submission, provide a written opinion regarding the acceptance of the submission, if all of the information specified in the regulatory enactments regulating the environmental protection has been indicated in the submission or attached thereto. If all of the information specified in the regulatory enactments regulating the environmental protection has not been indicated in the submission or attached thereto, the Board shall prepare a written opinion and send it to an operator with a view to provide the missing information. If the operator fails to provide all additional information within the time period specified by the Board, which is not less than 20 days from the day of sending of the referred to opinion, the Board shall take a decision regarding refusal to issue a permit. The day on which the Board provides the opinion that the information for assessment of the submission is sufficient shall be considered as the day of acceptance of the submission.

[26 July 2005; 19 December 2006; 6 January 2009]

22.1 Upon the acceptance of a submission the Board has the right to request and receive from an operator additional information which clarifies the submitted information or which is necessary for the taking of a decision. In the referred to case, the time period for the issuance of a permit shall not be extended.

[26 July 2005]

23. The Board shall electronically send a copy of the submission of a Category A and B polluting activity within seven days from the day of acceptance of the submission to:

23.1. the local government in the territory of which the relevant polluting activity is performed or planned, or the territory of which the relevant activity may substantially affect;

23.2. the Health Inspectorate; and

23.3. other interested State and local government institutions, as well as associations and foundations, if the referred to institutions, associations and foundations have applied to the Board and indicated an e-mail address where information is to be sent.

[8 July 2003; 3 February 2004; 26 July 2005; 6 January 2009; 13 October 2009]

24. The institutions referred to in Sub-paragraphs 23.1 and 23.2 shall submit their proposals to the Board regarding the issuance of a permit and the conditions thereof within 30 days after the receipt of a submission. The institutions referred to in Sub-paragraph 23.2 may also submit their proposals to the Board within the referred to time period. Paragraphs 34 and 35 of these Regulations apply to the proposals of the institutions referred to in a Sub-paragraph 23.3 of these Regulations.

[8 July 2003; 3 February 2004]

25. If on the basis of the information provided in a submission of a Category A polluting activity it may be concluded that the polluting activity may have a transboundary impact or the relevant state requests information regarding such activity, the Board shall inform the operator and the relevant local government regarding such conclusion or request, as well as shall send a copy of the submission to the Environment State Bureau (hereinafter – Bureau).

[26 July 2005]

26. In the case referred to in Paragraph 25 of these Regulations, an operator shall ensure the translation of the text of a submission into the necessary language and the Bureau shall prepare and send information regarding the polluting activity not later than two months before issuing a Category A permit, as well as proposals regarding joint monitoring measures to the competent institution of the relevant state upon prior co-ordination with the Ministry of Environment and informing the Ministry of Foreign Affairs. The information to be provided to the relevant state must be as detailed as the information to be provided to the public.

[8 July 2003; 3 February 2004]

27. If the state that may be affected by transboundary pollution organises public consultations, the Bureau shall ensure the participation of the relevant experts in such consultations. An operator shall cover the expenses related to the participation of experts.

[3 February 2004]

28. An operator shall prepare a submission of a Category C polluting activity in accordance with Annex 6 to these Regulations. The operator shall submit the submission in printed form (the relevant information shall be submitted also electronically) or in the form of electronic document. If the operator obtains or plans to extract also surface water or groundwater, the operator shall receive a permit for the use of water resources in accordance with the procedures specified by the regulatory enactments regulating the management of water resources.

[26 July 2005; 6 January 2009]

III. Public Participation in the Discussion of a Submission

29. The information included in a submission, including additional submitted documents, shall be available to the public. If an operator considers that a part of the information is restricted access information, such information shall be submitted separately with the relevant reference. The operator shall indicate the reasons why the information is considered restricted access information.

29.1 If the Board does not agree that the relevant information is to be considered restricted access information, the Board shall take a reasoned decision and shall notify the operator thereof in writing. The decision of the Board may be appealed before the Bureau within one month. In such case, the day on which the final decision regarding whether the relevant information is to be considered restricted access information entered into effect shall be considered the day of acceptance of a submission.

[26 July 2005]

29.2 The Board shall, within ten working days from the day when a submission of Category A or B polluting activity has been accepted, insert the relevant submission on the Internet homepage of the State Environmental Service.

[6 January 2009]

29.3 The Board shall, within five working days from the day when a submission of Category A polluting activity has been accepted, insert the following information on the Internet homepage of the State Environmental Service:

29.31. the name of the operator;

29.32. the name of the polluting activity and address of the place of performance thereof;

29.33. the place and time when it is possible to get acquainted with the information included in the submission, including additional materials submitted (if public consultations are determined, also with materials which have become available only after informing of the public in accordance with Paragraph 30 of these Regulations or after public consultations); and

29.34. the essence of the possible decision (a decision regarding the issuance of a permit or regarding refusal to change conditions of a permit) and the time of taking of the decision.

[6 January 2009]

30. Within seven days after the acceptance of a submission in the Board for the commencement of an activity, the continuing of an existing activity or substantial changes in the existing activity with regard to a Category A or Category B polluting activity, an operator shall inform the public regarding the polluting activity:

30.1. publicly – providing an opportunity to receive information at the office of the operator and from the local government;

30.2. individually – by sending notices to owners (possessors) whose immovable properties border the place of the declared polluting activity or are located in direct area of impact thereof;

30.3. in the newspaper Latvijas Vēstnesis [the official Gazette of the Government of Latvia], as well as in at least one local newspaper;

30.4. on the Internet – on the homepage of the operator or the relevant Board; and

30.5. regarding new polluting activities – also on the First Programme of the Latvian Radio or on local radio.

[8 July 2003; 3 February 2004; 26 July 2005]

31. An operator shall provide the following information in a notice to the public:

31.1. the name and type of polluting activity or information regarding the changes of activity;

31.2. waste that is intended to be incinerated (for waste incineration plants);

31.3. information regarding the owner and the operator of an installation;

31.4. the location (address) of the intended polluting activity;

31.5. the place where the public may receive a summary of a submission or acquaint themselves with the submission and documents attached thereto;

31.6. the place and time of public consultations;

31.7. the date by which the public may submit written proposals to the Board;

31.8. the Board that is responsible for the issuance of a permit, and the address thereof; and

31.9. where appropriate, – an indication regarding the possible transboundary impact.

[3 February 2004]

31.1 Within five working days after the information is provided to the public, the operator shall send to the Board documents certifying the fulfilment of the requirements referred to in Paragraphs 30 and 31 of these Regulations.

[19 December 2006]

32. If the Board determines that a Category B polluting activity, which is not referred to in Paragraph 30 of these Regulations, may significantly and negatively affect the environment, the Board shall assign the operator to inform the public within 14 days after the receipt of a submission in accordance with the procedures specified in Paragraphs 30 and 31 of these Regulations.

32.1 An operator shall organise a public consultation at least five days after the publication of the notice referred to in Paragraph 31 of these Regulations in a local newspaper. The Board shall permit the operator not to organise a public consultation meeting if such meeting has already taken place during the environmental impact assessment of the intended activity and not more than one year has passed since such meeting, as well as if no new negative impact on human health or the environment has been discovered while evaluating the submission.

[3 February 2004]

32.2 Public consultations shall be managed by an authorised representative of the local government or the operator. The operator shall ensure the course of the meeting and the taking of minutes, as well as provide sufficient information to the public regarding the submission and the intended activity. The persons present during a public consultation shall be provided with an opportunity to ask questions and to express their opinion.

[3 February 2004]

32.3 Within three working days after the public consultation meeting an operator shall send the meeting minutes to the local government in the territory of which the relevant polluting activity is performed or planned. The minutes shall be available to the public at the local government and the Board. Each member of the meeting has the right to submit an individual opinion within seven working days, to be appended to the minutes, regarding the issues examined at the public consultation meeting.

[3 February 2004]

32.4 In addition to the information specified in Paragraph 31 of these Regulations, information that may be used in the taking of a decision but which became available only after informing the public in accordance with Paragraph 30 of these Regulations or after the public consultation shall also be available to the public at the Board.

[3 February 2004]

33. The public may submit proposals thereof or an opinion regarding the issuance of a permit or the conditions thereof to the Board in writing within 40 days after the day of publication of the notice referred to in Sub-paragraph 30.3 of these Regulations.

[3 February 2004]

34. The Board shall inform an operator regarding received proposals and shall indicate the deadline by which an explanation of the operator regarding such proposals is to be submitted.

35. If the informed institutions or the public propose not to issue a permit for the performance of a declared polluting activity, the Board shall issue the permit or take a substantiated decision regarding a refusal to issue the permit only after the operator has been given not less than 14 days to express his or her explanation in writing.

IV. Procedures for the Issuance of a Category A and Category B Permit and Category C Polluting Activity Confirmation

36. The Board shall evaluate the presented submission for a Category A polluting activity or Category B polluting activity and shall issue the relevant permit for the performance of the polluting activity or substantial changes in the polluting activity, or shall take a decision regarding change of conditions of the permit or regarding a refusal to issue the permit within 90 days after the day of acceptance of the submission for a Category A polluting activity and within 60 days after the day of acceptance of the submission for a Category B polluting activity.

[8 July 2003; 6 January 2009]

36.1 The Board may extend the time period referred to in Paragraph 36 of these Regulations for an existing Category B activity, as well as for a new Category B activity in cases where a public consultation is specified, for a time period that does not exceed 90 days, and shall inform an operator thereof in writing. If necessary, the time period of existing permits shall be extended accordingly.

[3 February 2004; 26 July 2005; 19 December 2006]

37. In issuing a permit or in taking a substantiated decision regarding a refusal to issue a permit, the Board shall repose on the submitted information and an evaluation thereof, taking into account the proposals of the relevant local government, the Health Inspectorate, other State and local government institutions, as well as the public, and the explanation of an operator. In issuing a Category A permit for a polluting activity, which may have a transboundary impact of pollution, the Board shall take into account the opinion of the competent institution of the relevant state, as well as the proposals of the public of such state.

[13 October 2009]

38. A permit shall be issued in accordance with the sample form indicated in Annex 7 of these Regulations.

[26 July 2005; 19 December 2006]

38.1 On the basis of the submission referred to in Paragraph 7.1 of these Regulations or on the basis of Section 24.1, Paragraph six of the Law On Pollution, if necessary, the conditions of a greenhouse gas emission permit shall be amended and the relevant permit shall be issued.

[22 April 2004]

39. The same tables that are used in a submission for a Category A and Category B polluting activity in accordance with Annex 4 of these Regulations shall be used in a permit.

40. If it is not possible to specify in a permit the emission limits for a certain installation, such limits shall be replaced with other conditions that ensure equivalent protection of human health and the environment. In determining the conditions of a permit for the installations referred to in Paragraph six, Clause 6 of the Annex to the Law On Pollution, the nature of such installations and the practical considerations for ensuring the quality of the environment shall be taken into account, including the requirements for manure pits.

41. Within three working days after issuance of the relevant permit referred to in Paragraph 36 of these Regulations or after taking of a decision, the Board shall send a copy of the permit or the decision (also electronically) to the Bureau. The relevant documents shall be sent electronically only to the local governments referred to in Sub-paragraph 23.1 of these Regulations, the Health Inspectorate, the institutions and organisations referred to in Sub-paragraph 23.3 of these Regulations that in accordance with the procedures and time periods specified in Chapter III of these Regulations have submitted written proposals, as well as, if necessary, to other State and local government institutions, if the relevant institution has not requested to provide the information in writing.

[26 July 2005; 19 December 2006; 13 October 2009]

42. The Board may cancel a permit if such Board determines that an operator has provided false or misleading information.

[6 January 2009]

43. The Board shall evaluate the submitted declaration of a Category C polluting activity and shall send to the operator a confirmation of a Category C polluting activity in accordance with Annex 8 to these Regulations within 14 days from the day of acceptance of the submission. The Board shall send electronically the confirmation of a Category C polluting activity to the institutions referred to in Paragraph 23 of these Regulations.

[8 July 2003; 6 January 2009]

43.1 The Board shall, in issuing a certification of Category C, include an operator in the list of performers of Category C polluting activities. The operator shall be excluded from the list in cases when Category C polluting activity is terminated or the relevant activity is deleted from Annex 2 to these Regulations.

[6 January 2009]

44. The issuance of a permit, a refusal to issue a permit or the conditions specified in a permit may be disputed in accordance with the procedures specified in the Law On Pollution.

[26 July 2005]

44.1 [6 January 2009]

[26 July 2005]

IV.1 Review of the Conditions of a Permit for Category A or Category B Polluting Activity

[6 January 2009]

44.2 An operator shall submit a submission to the Board regarding the review of conditions of a permit for Category A or Category B polluting activity. If it is necessary to review the conditions in separate sections of the permit, only the sections to which changes apply shall be completed in the submission.

[6 January 2009]

44.3 The Board has the right to request additional information from an operator regarding polluting activity in accordance with Paragraph 22 of these Regulations, as well as to invite experts.

[6 January 2009]

V. Information for the Public and for the European Commission

45. The conditions specified in a permit, as well as the results of pollution control and monitoring shall be freely available to the public at the relevant Board.

46. If in accordance with these Regulations a public consultation has taken place, an operator shall inform the public regarding a polluting activity in accordance with Sub-paragraphs 30.1 and 30.2 of these Regulations within eight days from the day on which the Board issued or extended a permit, or changed the conditions specified in the permit. The operator shall provide the following information in a notice to the public:

46.1. the type and name of the polluting activity;

46.2. information regarding the owner and the operator of the installation;

46.3. the location (address) of the intended polluting activity;

46.4. the Internet address where the conditions of the permit are available;

46.5. the address of the Board where it is possible to receive a copy of the permit;

46.6. the time period during which and the institution in which it is possible to dispute the issuance of the permit or the conditions specified therein; and

46.7. [19 December 2006].

[8 July 2003; 19 December 2006]

46.1 Within five working days after the information is provided to the public, the operator shall send to the Board documents certifying the fulfilment of the requirements referred to in Paragraph 46 of these Regulations.

[19 December 2006]

47. Within eight days after issuance of the permit referred to in Paragraph 36 of these Regulations or after taking of a decision, the Bureau shall place the permit or the decision on the Internet home page of the Bureau.

[8 July 2002; 19 December 2006]

48. If a Category A permit for a polluting activity is received which may have transboundary impact, the Board shall inform the competent institution of the relevant state regarding the conditions specified in the Category A permit, as well as shall provide the information prepared by the Board regarding the results of monitoring the polluting activity. An operator shall ensure the preparation of information in the necessary language.

[3 February 2004]

49. The State limited liability company “Latvian Environment, Geology and Meteorology Centre” shall prepare and submit to the European Commission information regarding emission limit values and emission limits that are specified for Category A polluting activities (installations) not less than once every three years, indicating also the guidelines of the best available techniques utilised to determine the emission limits or limit values.

[13 October 2009]

49.1 The State limited liability company “Latvian Environment, Geology and Meteorology Centre” shall be the authority responsible for the provision of information in accordance with Regulation (EC) No 166/2006 of the European Parliament and of the Council of 18 January 2006 concerning the establishment of a European Pollutant Release and Transfer Register and amending Council Directives 91/689/EEC and 96/61/EC.

[6 January 2009; 13 October 2009]

VI. Closing Provisions

50. Cabinet Regulation No. 324 of 17 July 2001, Procedures for the Application for, Issuance and Review of Category A Permits and Category B Permits for Waste Incineration Plants and the Directions for Use of the Best Available Techniques (Latvijas Vēstnesis, 2001, No. 111), is repealed.

51. The maximum deadline for the implementation of the best available techniques for a Category A polluting activity shall be 31 October 2007, except the cases where a longer time period for an installation has been specified in the Treaty of Accession to the European Union.

[26 July 2005]

52. On the basis of a programme for the introduction of the best available techniques submitted by an operator, the Board shall determine in a Category A permit the time periods for the implementation of certain technological solutions, which do not exceed the time periods referred to in Paragraph 51 of these Regulations.

53. After 1 January 2003, a Category B polluting activity may not be commenced if the operator has not received a Category B permit in accordance with the procedures specified in these Regulations.

54. After 1 January 2003, a Category C polluting activity may not be commenced if the operator has not presented a submission for a Category C polluting activity in accordance with the procedures specified in these Regulations.

55. The timetable by which Sections 22, 23 and 24 of the Law On Pollution shall come into force, as well as the conditions of Chapter V in relation to existing Category B polluting activities is specified in Annex 1 of these Regulations.

56. If a Category C polluting activity was commenced until 1 January 2003, an operator shall present a submission for a Category C polluting activity by 1 January 2004.

57. [6 January 2009]

[26 July 2005]

58. An operator that performs a polluting activity and has presented a submission in order to receive a permit for the performance of the polluting activity in accordance with the procedures specified in these Regulations or in order to receive a permit for substantial changes in the polluting activity shall, until the receipt of the relevant permit within the time periods specified in these Regulations, if necessary, present a submission to the Board regarding the extension of the time period of a permit for a Category A or B polluting activity. The Board shall extend the relevant permit for a time period not exceeding 60 days. In cases when the Board has taken a decision regarding the application of public consultation procedures for the receipt of a permit for Category B polluting activity, the relevant permit shall be extended for a time period not exceeding 90 days from the day of taking of the decision.

[26 July 2005; 6 January 2009]

59. A category A or Category B permit shall replace other permits for the emission of air polluting substances, for the use of water and waste management specified by other regulatory enactments, except the permit for the transport of waste.

[26 July 2005]

60. Category A and Category B permits that have been issued up to the day of the coming into force of these Regulations shall be valid until the expiration of the time period thereof, except the case where, in accordance with the Law On Pollution, permit conditions shall be reviewed earlier.

61. The Board shall examine in accordance with these Regulations the submissions regarding Category A polluting activities and Category B polluting activities in a waste incineration plant, which the operators have submitted to the Board up to the day of the coming into force of these Regulations.

62. Paragraph 49 of these Regulations shall come into force with a special Cabinet Regulation.

63. These Regulations shall come into force on 30 July 2002.

Informative Reference to European Union Directives

[26 July 2005]

These Regulations contain legal norms arising from:

1) Council Directive 75/442/EEC of 15 July 1975 on waste;

2) Council Directive 80/68/EEC of 17 December 1979 on the protection of groundwater against pollution caused by certain dangerous substances;

3) Council Directive of 21 May 1991 concerning urban waste water treatment (91/271/EEC);

4) Council Directive 91/689/EEC of 12 December 1991 on hazardous waste;

5) Directive 2008/1/EC of the European Parliament and of the Council of 15 January 2008 concerning integrated pollution prevention and control (Codified version);

6) Council Directive 1999/31/EC of 26 April 1999 on the landfill of waste;

7) Directive 2000/60/EC of the European Parliament and of the Council of 23 October 2000 establishing a framework for Community action in the field of water policy;

8) Directive 2000/76/EC of the European parliament and of the Council of 4 December 2000 on the incineration of waste; and

9) Directive 2003/53/EC of the European Parliament and of the Council of 26 May 2003 providing for public participation in respect of the drawing up of certain plans and programmes relating to the environment and amending with regard to public participation and access to justice Council Directives 85/337/EEC and 96/61/EC.

[6 January 2009]

Note. Paragraph 1.9 of these Regulations shall come into force on 1 January 2010.

[6 January 2009]

Acting for the Prime Minister,

Minister for Culture 					K. Pētersone

Acting for the Minister for Environmental Protection

and Regional Development – Minister for the Interior

M. Segliņš

Annex 1

Cabinet Regulation No. 294

9 July 2002

[8 July 2003; 3 February 2004; 22 April 2004; 26 July 2005; 25 July 2006; 19 December 2006; 6 January 2009]

Category B Polluting Activities

| Sector and type of activity | NACE code |  |
| --- | --- | --- |
| 1 | 2 |  |
| 1. Energy industry: |  |  |
| 1.1. combustion installations with rated thermal input: | 35 |  |
| 1.1.1. from 5 to 50 megawatts, if biomass (also wood and peat) or gaseous fuels are used in the combustion installation; |  |  |
| 1.1.2. from 0.5 to 50 megawatts, if liquid fuels are used in the combustion installation, except fuel oil (heavy fuel oil); |  |  |
| 1.1.3. from 5 to 50 megawatts, if liquid fuel or fuel oil, is utilised in the combustion installation that is used in a grain dryer; |  |  |
| 1.1.4. from 0.2 to 50 megawatts, if coal is utilised in the combustion installation |  |  |
| 1.2. combustion installations in which fuel oil (heavy fuel oil) is utilised |  |  |
| 1.3. oil depots and terminals with 5 000 or more tonnes of fuel per year (the largest total amount of fuel pumped per year during the last three years) | 46.12 |  |
| 1.4. petrol stations with 2 000 or more cubic metres of fuel per year (the total largest amount of fuel pumped during the last three years) | 47.3 |  |
| 1.5. gas storage installations with a capacity of 100 m3 or more and underground storage sites of natural gas | 35 |  |
| 1.6. coal and brown-coal briquetting equipment | 35 |  |
| 1.7. the production of charcoal | 35 |  |
| 2. Production and processing of metals: |  |  |
| 2.1. installations for the production of pig iron or steel, also for continuous casting, with a capacity not exceeding 2.5 tonnes per hour | 24.1 |  |
| 2.2. installations for the processing of ferrous metals; |  |  |
| 2.2.1. hot-rolling mills which process less than 20 tonnes of crude steel per hour | 24 |  |
| 2.2.2. installations for the application of protective fused metal coats, which treat less than 2 tonnes of crude steel per hour | 24 |  |
| 2.3. ferrous metal foundries with a production capacity of up to 20 tonnes per day | 24.5 |  |
| 2.4. installations for smelting, also fusion, of non-ferrous metals, including metals to be used for recycling, the melting capacity of which does not exceed four tonnes of molten lead or cadmium per day or 20 tonnes of other metals per day, except installations that are used in crafts and sculpture, including for the processing of gold and silver | 24 |  |
| 2.5. installations in which electrolysis or chemical processes are used for surface treatment of metals and plastic materials and the total volume of the treatment vats of which does not exceed 30 m3 | 25.6 |  |
| 2.6. installations for surface treatment during the operation of which dust is created, including the polishing of iron, steel or other metallic objects, cleaning by sand blasting and powder painting, if the total discharge of the installation is 10 000 or more cubic metres per hour | 25.6 |  |
| 2.7. floating docks and dry docks of a steel shipyard | 30.1 |  |
| 2.8. other installations for industrial processing of iron, steel or other metals with a production area of 1 000 m2 or more | 25.6 |  |
| 2.9. installations for the production of cables | 27.3 |  |
| 2.10. installations for the production of accumulators and batteries | 27 |  |
| 2.11. electro-technical equipment for the production of transformer and printed circuits | 27 |  |
| 3. Production of mineral products: |  |  |
| 3.1. installations for the production of cement clinker in rotary kilns the production capacity of which does not exceed 500 tonnes per day or installations for the production of lime in rotary kilns with a production capacity that does not exceed 50 tonnes per day, or in other furnaces with a production capacity that does not exceed 50 tonnes per day | 23 |  |
| 3.2. installations for the manufacture of glass, including glass fibre, with a melting capacity that does not exceed 20 tonnes per day, except craftsmanship | 23 |  |
| 3.3. installations for melting mineral substances, including the production of mineral wool, with a melting capacity that does not exceed 20 tonnes per day | 23 |  |
| 3.4. installations for the manufacture of ceramic products by firing, including roofing tiles, bricks, refractory bricks, tiles, stove tiles, pottery, faience or porcelain, in which up to 75 tonnes of finished products may be manufactured per day, except for craftsmanship | 23 |  |
| 3.5. cement production units with a production capacity of 20 000 or more tonnes per year and installations for the production of concrete and concrete products with a capacity of 20 000 or more cubic metres per year | 23.5 |  |
| 3.6. installations for the production of plaster products, except craftsmanship | 23.5 |  |
| 4. Chemical industry and activities with chemical substances and chemical products: |  |  |
| 4.1. installations for the production of organic and inorganic substances, products or intermediary products, including enzymes, in which physical production processes (for example, dilution and mixing) are utilised | 20 |  |
| 4.2. installations for the storage of unpacked organic or inorganic chemical substances, chemical products or intermediary products, if one tonne or more is stored, for the storage of enzymes – 20 tonnes or more |  |  |
| 4.3. installations for the production of pharmaceutical products, in which physical processes (for example, dilution and mixing) are utilised | 21.1 |  |
| 4.4. installations for the production of explosives, in which physical production processes (for example, mixing) are utilised | 20.51 |  |
| 4.5. installations for the production of munitions | 25.4 |  |
| 4.6. installations for industrial production of colorants, additives and ancillary substances (also usable in food industry), in which physical processes are utilised (for example, dilution and mixing), except retail trade | 10.84+20.12 |  |
| 4.7. installations for the production of chemical substances and chemical products and also for the production of plant protection products and biocides using physical methods (for example, dilution and mixing), packing and filling |  |  |
| 4.8. installations for the production of soaps, detergents and cleaning agents with a production capacity of one tonne or more tonnes per year | 20.4 |  |
| 4.9. installations for the production of paints, varnishes or glue | 20.3 |  |
| 4.10. installations for the production of goods with teflon thermo-coating, thermoplastic materials moulded by extrusion or by performing recycling of fibrous thermoplastic composite materials, if 100 or more kilograms of plastic are used per day | 22. |  |
| 4.11. installations for the production of plastic goods, using injection moulding from alloy, the extrusion process, including calendering or thermal moulding, if five or more tonnes of plastic are used per day Installations for the production of plastic goods from expanded polystyrene, if five or more tonnes of plastic are used per day | 22.2 |  |
| 4.12. installations for the production of goods of rubber with a production capacity above 500 tonnes per year | 22.1 |  |
| 4.13. installations for the production of regenerated pulp | 17.1 |  |
| 4.14. installations for the production of gelatine and glue from the skin and bones of animals | 20.5 |  |
| 4.15. installations for the production of organic chemical products via chemical, biological or physical process that are not included in the Annex to the Law On Pollution or in another sector of this Annex |  |  |
| 4.16. installations for the production of asphalt and road surfacing materials |  |  |
| 4.17. installations for the production of roof covering, using tar and bitumen |  |  |
| 4.18. installations for the distillation of tar |  |  |
| 4.19. gas and coke plants | 32.5 + 19 |  |
| 4.20. weaveries, spinneries and knitwear production units, if the production capacity is 100 or more kilograms per day | 14.3 |  |
| 4.21. dry-cleaners | 96 |  |
| 4.22. laundries with a capacity exceeding 1 000 kilograms per day | 96 |  |
| 5. Waste management: |  |  |
| 5.1. installations for the disposal or processing of hazardous waste, including petroleum product waste, the capacity of which does not exceed 10 tonnes per day | 38.2 |  |
| 5.2. installations for the incineration of municipal waste and other waste that may not be classified as hazardous waste, if the capacity of the installation does not exceed three tonnes per hour | 38 |  |
| 5.3. installations for biological or physico-chemical treatment of municipal waste, the capacity of which does not exceed 50 tonnes per day, except composting installations with an intake capacity not exceeding 100 tonnes per year and composting installations for animal manure | 38 |  |
| 5.4. installations for the treatment of municipal waste for purposes of disposal in which the biological or physico-chemical treatment method is not utilised | 38 |  |
| 5.5. landfills for the processing of municipal waste | 38 |  |
| 5.6. landfills that can receive up to 10 tonnes of waste per day or with a total capacity not exceeding 25 000 tonnes, excluding landfills of inert waste | 38 |  |
| 5.7. landfills of inert waste | 38 |  |
| 5.8. places for the disposal, storage or composting of wastewater sludge and waste that may not be classified as hazardous waste | 36 |  |
| 5.9. installations for the processing of discarded vehicles and for the recycling and storage of ship wrecks | 38.3 |  |
| 5.10. installations for the sorting or temporary storage of municipal waste, including reloading stations with a receiving capacity of 30 or more tonnes of waste per day | 38 |  |
| 5.11. installations for the storage of, recycling or treatment of waste of animal origin, also installations for composting and bio-gas installations with a receiving capacity of waste of animal or vegetable origin, including animal droppings and waste from slaughterhouses, of 30 or more tonnes per day | 38 |  |
| 5.12. installations for the storage of hazardous waste (including at the places of creation) for more than one year | 38 |  |
| 5.13. installations for temporary (not more than one year) storage of hazardous waste, for example, reloading stations and container warehouses, excluding storage of waste for such a short period of time or in such an insignificant amount that the waste does not cause a risk to human health or the environment | 38 |  |
| 5.14. installations for the recycling of electric and electronic waste | 38 |  |
| 6. Agriculture, forestry and wood processing: |  |  |
| 6.1. slaughterhouses with a carcass production capacity from 5 to 50 tonnes per day |  |  |
| 6.2. installations for the disposal or recycling of animal carcasses and waste of animal origin, the capacity of which is from 1 to 10 tonnes per day |  |  |
| 6.3. slaughterhouses with a production capacity of products of poultry origin of 5 000 or more tonnes per year |  |  |
| 6.4. the production of matches |  |  |
| 6.5. the production of plywood |  |  |
| 6.6. the production of furniture, if the production area is 1 000 m2 or more | 31 |  |
| 7. Food industry: |  |  |
| 7.1. installations for the collection, pre-treatment and processing of milk, in which the quantity of milk received is from 10 to 200 tonnes per day (average value on an annual basis) | 10.51 |  |
| 7.2. installations for the production of food products, in which products of animal origin (other than milk) are treated and processed and which produce from 1 to 75 tonnes of finished product per day, or which treat and process vegetable products and produce from 10 to 300 tonnes of finished product per day (average value on a quarterly basis), including: | 10 |  |
| 7.2.1. the production of oils and fats of vegetable and animal origin | 10.4 |  |
| 7.2.2. the production of beer and malt | 11.05 |  |
| 7.2.3. the production and bottling of non-alcoholic beverages | 11.07 |  |
| 7.2.4. installations for industrial production of starch and potato starch | 10.6 |  |
| 7.2.5. fish meal and fish oil production units | 10 |  |
| 7.2.6. sugar production units | 10.81 |  |
| 7.2.7. the production of coffee, tea and food additives | 10.83 |  |
| 7.2.8. grain processing | 10.6 |  |
| 7.2.9. the production of yeast | 10 |  |
| 7.2.10. the production and bottling of alcohol and alcoholic beverages | 11 |  |
| 7.2.11. the conservation, filling and packaging of products of animal and vegetable origin | 10.39 |  |
| 7.2.12. other food product production installations in which vegetables are treated and processed | 10.3 |  |
| 7.2.13. installations for the production of fish and crustacean products, including for the production of canned, smoked and frozen products | 10.2 |  |
| 7.3. facilities for the production of meat meal, including bone meal, blood meal, blood plasma and feather meal production units | 10 |  |
| 7.4. the production of protein and pectin | 10 |  |
| 7.5. installations for the production of tobacco products | 12 |  |
| 8. Other sectors: |  |  |
| 8.1. in manufacturing: |  |  |
| 8.1.1. installations for the production of paper and cardboard with a production capacity not exceeding 20 tonnes per day | 17.1 |  |
| 8.1.2. installations for the pre-treatment of fibres and fabric (washing, bleaching, mercerisation) or dyeing, the treatment capacity of which is from 0.5 to 10 tonnes per day | 13.3 |  |
| 8.1.3. installations for the tanning of hides and skins, in which less than 12 tonnes of finished products are produced per day | 14.2 |  |
| 8.1.4. installations that emit volatile organic compounds and for which a permit is required in accordance with the regulatory enactments regulating emission from stationary sources of pollution |  |  |
| 8.2. crematoria |  |  |
| 8.3. airports and airfields |  |  |
| 8.4. railway depot and stations that perform the functions of a freight station, marshalling station or district station | 49.1 |  |
| 8.5. berths of ports for the loading and unloading of cargoes into ships with gross tonnage not less than 450 tonnes |  |  |
| 8.6. hospitals with the number of beds above 100 | 86.1 |  |
| 8.7. washing installations intended for the cleaning of storage and transportation receptacles and containers of chemical substances |  |  |
| 8.8. other polluting activities for the performance of which a permit for emission of pollutants into the environment is required in accordance with the requirements of regulatory enactments |  |  |
| 8.9. waste water treatment plants with capacity of 20 or more cubic metres per day that drain the treated waste water in the environment | 37 |  |

# Acting for the Minister for Environmental Protection

and Regional Development – Minister for the Interior

M. Segliņš

Annex 2

Cabinet Regulation No. 294

9 July 2002

[8 July 2003; 3 February 2004; 22 April 2004; 26 July 2005; 19 December 2006; 6 January 2009]

## Category C Polluting Activities

| Sector and type of activity | NACE code |
| --- | --- |
| 1 | 2 |
| 1. Power industry: |  |
| 1.1. combustion installations with rated thermal input of more than 0.2 megawatts, if a permit is not required for the combustion installation in accordance with Sub-paragraph 1.1 or 1.2 of Annex 1 to the Cabinet Regulation No. 294 of 9 July 2002, Procedures by which Polluting Activities of Category A, B and C shall be Declared and Permits for the Performance of Category A and B Polluting Activities shall be Issued | 35 |
| 1.2. wind power stations or power station parks with the total capacity of more than 125 kilowatts | 35.1 |
| 1.3. petrol stations with fuel amount of up to 2000 m3 per year (the total largest amount of fuel pumped during the last three years) | 47.3 |
| 1.4. gas filling stations | 35 |
| 1.5. oil depots with fuel amount of less than 5000 tonnes per year | 46.12 |
| 1.6. installations for the production of heating fuel from the remains of timber | 20 |
| 1.7. installations for the production of heating fuel from peat |  |
| 2. Production and processing of metals: |  |
| 2.1. installations for surface treatment that create dust during operation, including the polishing of iron, steel or other metallic objects, sand blasting (cleaning by sand blasting) and powder painting, if the total emission of the installation is from 300 to 10 000 m3/hour | 26.5 |
| 2.2. other installations for the processing of iron, steel or other metals with a production area from 100 m2 to 1 000 m2 | 26.5 |
| 2.3. foundries usable in craftsmanship, also for the casting of gold and silver |  |
| 2.4. production facilities for electro-technical products, except installations for the production of transformers or printed circuits | 27.9 |
| 2.5. bonding of plastic products | 22.29 |
| 3. Manufacturing of mineral products (processing of mineral substances): |  |
| 3.1. cement production units with a production capacity from 2 to 20 000 tonnes per year and installations for the production of concrete and concrete products with a capacity from 2 to 20 000 cubic metres per year | 23.5 |
| 3.2. installations for the production and mixing of gravel or lime mortar and installations for the crushing of stones, which are not installed at the places where the stones are obtained | 23 |
| 3.3. stationary installations for the production of aerated concrete, coal dust or lime-and-sand bricks | 23 |
| 4. Agriculture, forestry and wood processing: |  |
| 4.1. animal housings in which 10 or more animal units are bred for commercial purposes (including the storage, collection and drainage of solid manure, liquid manure, slurry and silage juice); animal housings which in accordance with Cabinet Regulation No. 531 of 18 December 2001, Regulations regarding Protection of Water and Soil from Pollution with Nitrates Caused by Agricultural Activity, are located in a highly sensitive territory and in which five or more animal units are bred for commercial purposes, if the animal housing is not included in Annex 1 to the Law On Pollution | 01.4 |
| 4.2. sawmills and wood-processing installations in which timber-cutting machinery is utilised and which process 2 000 m3 or more round wood and timber per year; installations in which industrial chemical treatment of timber is performed, also pressure impregnation (high-pressure impregnation), vacuum impregnation (low-pressure impregnation) and protection of timber against blue stain and mould | 16 |
| 4.3. fish farms | 01.49 |
| 5. Food industry: |  |
| 5.1. installations for the collection, pre-treatment and processing of milk, in which the quantity of milk received is from one to ten tonnes per day (average value on an annual basis) | 10.51 |
| 5.2. installations for the production of food, in which products of animal origin (excluding milk) are processed and which produce from 0.1 to 1 tonne of finished products per day, and in which products of vegetable origin are processed and from 0.5 to 10 tonnes of finished products are produced per day (average value on a quarterly basis) | 10 |
| 5.3. installations for the production of fish and crustacean products, including for the production of canned, smoked and frozen products, in which less than one tonne of finished products is produced per day | 10.2 |
| 5.4. smoke-houses, meat and gastronomy production units (also in shops) in which 500 or more kilograms of food products are produced per day | 10 |
| 5.5. installations for the baking of bread and industrial production of confectionery products with a production capacity that exceeds two tonnes per day | 10.7 |
| 5.6. slaughterhouses with a production capacity of carcasses less than five tonnes per day |  |
| 6. Other sectors |  |
| 6.1. repair and maintenance shops for mechanical land vehicles of all categories (L, M, N, O), mobile agricultural machinery and mobile non-road machinery, and other movable aggregates, where the activities provided for in Paragraph 2 of Cabinet Regulation No. 380 of 22 April 2004, Environmental Requirements for the Establishment and Operation of Repair Shops for Mechanical Vehicles, are performed |  |
| 6.2. chemical and biological laboratories (except study laboratories) |  |
| 6.3. waste water treatment plants with a capacity from 5 to 20 cubic metres per day, if waste water is drained in the environment | 74.2 |
| 6.4. installations and photographic laboratories in which 1000 or more square metres of photographic films are processed per year |  |
| 6.5. installations for the storage of salt and salt mixtures, if one tonne or more of salt or salt mixtures is stored, installations for obtaining salt and salt mixtures (with a capacity of one tonne or more per day) for scattering on roads during winter conditions |  |
| 6.6. installations for the production of casement windows and doors |  |
| 6.7. installations for the storage of packed organic and inorganic chemical substances, chemical products or intermediary products, if more than 10 tonnes of chemical substances, chemical products or intermediary products are stored |  |

Acting for the Minister for Environmental Protection

and Regional Development -

Minister for the Interior					M. Segliņš

Annex 3

Cabinet Regulation No. 294

9 July 2002

[8 July 2003; 3 February 2004]

Time Periods for the Presentation of a Submission for Existing Category A Polluting Activities

| No. | Number in accordance with the Annex to the Law On Pollution. | Sector of the Polluting Activity | Deadline by which submission for a permit shall be presented |
| --- | --- | --- | --- |
| 1. | 1. | Energy industry | 31 December 2005 |
| 2. | 2. | Production and processing of metals | 30 June 2004 |
| 3. | 3. | Production of mineral products | 31 December 2004 |
| 4. | 4. | Chemical industry | 30 June 2004 |
| 5. | 5.1., 5.2., 5.3., 5.5. | Waste management | 28 December 2004 |
| 6. | 5.4. | Waste management | 1 March 2007 |
| 7. | 6.1. | Production of pulp, timber, paper and cardboard products | 31 December 2005 |
| 8. | 6.2. | Initial treatment of fibres or fabric | 30 June 2003 |
| 9. | 6.3. | Treatment of hides and skins | 30 June 2003 |
| 10. | 6.4.a, 6.4.c, 6.5., 6.6. | Food production | 30 June 2004 |
| 11. | 6.4.b | Food industry (vegetable raw materials) | 30 June 2003 |
| 12. | 6.7. | Installations for surface treatment of substances, objects or products | 31 December 2005 |
| 13. | 6.8. | Installations for the production of carbon or electrographite | 31 December 2005 |

Note.

A permit for installations in which waste is incinerated or co-incinerated shall be issued by 28 December 2002.

Acting for the Minister for Environmental Protection

and Regional Development – Minister for the Interior

M. Segliņš

Annex 4

Cabinet Regulation No. 294

9 July 2002

[8 July 2003; 3 February 2004; 22 April 2004; 26 July 2005; 19 December 2006; 6 January 2009]

Submission for the Receipt of a Permit for Category A or Category B Polluting Activity

(the operator shall complete only such parts of the submission that correspond to the particular polluting activity)

| Regional Environmental Board |  |  |  |  |  |  |  | of the State Environmental Service |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Firm name (name) of the merchant (or another person), the name of the operator or |  |  |  |  |  |  |  |  |  |  |  |  |
| given name, surname |  |  |  |  |  |  |  |  |  |  |  |  |
| Address, telephone number, fax number, e-mail address |  |  |  |  |  |  |  |  |  |  |  |  |
| Single registration number of the merchant (or another person) |  |  |  |  |  |  |  |  |  |  |  |  |
| Date of registration in the Enterprise Register |  |  |  |  |  |  |  |  |  |  |  |  |
| Date of registration in the Commercial Register of the Enterprise Register |  |  |  |  |  |  |  |  |  |  |  |  |
| The given name, surname and address of such land owner on whose land the installation is located |  |  |  |  |  |  |  |  |  |  |  |  |
| or the polluting activity is performed (if different from the address of the merchant) |  |  |  |  |  |  |  |  |  |  |  |  |
| The given name, surname and address of the owner of buildings, ancillary buildings and producer goods |  |  |  |  |  |  |  |  |  |  |  |  |
| (if different from the addresses referred to above) |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | " |  |

Part A

General Description

1. Information regarding the location of the polluting activity or installation:

1.1. name, address, telephone number, fax number, e-mail address;

1.2.  the name, surname and position of the contact person;

1.3. the territory code;

1.4. a map of the location of the installation on a scale of 1:25 000 or 1:10 000, or 1:5 000, or 1:500 (to be attached in an Annex);

1.5. the position of buildings and production units in the territory (to be indicated in a map on a scale of 1:500, 1:1 000 or 1:5 000 suitable for the installations and attached in an Annex);

1.6. the conformity of the location of the installation with the permitted (planned) land use in accordance with territorial planning; and

1.7. the hydrological and geological characterisation of the location.

2. Information regarding the immediate vicinity and the type of land use:

2.1. populated area, one-storey building, multi-storey building, industrial area, public area, trade area; and

2.2. whether the installation is located in a protection zone, in the sensitive territory specified by the Cabinet to which higher requirements for the protection of water and soil from pollution with nitrates caused by agricultural activity apply, in a territory in which an assessment of air quality indicates that the concentration of air polluting substances exceeds the lowest threshold of pollution assessment (if the installation is located in a protection zone, a map in which the protection zones are indicated shall be attached).

3. Planning, design and construction work:

3.1. the name, address, telephone and fax number of the construction board of the relevant local government, which supervises the intended or existing activity (examination of the construction submission, acceptance of designs and acceptance for operation);

3.2. [6 January 2009]; and

3.3. planning, design, construction work, information regarding design and acceptance for operation (information regarding the date of issuance, number and term of validity of planning and architectural task and construction licence shall be attached).

[6 January 2009]

4. Classification of the polluting activity:

4.1. the types of activity in accordance with the Annex to the Law On Pollution or Annex 1 or 2 to Cabinet Regulation No. 294 of 9 July 2002, Procedures by which Polluting Activities of Category A, B and C shall be Declared and Permits for the Performance of Category A and B Polluting Activities shall be Issued; and

4.2. the direction (directions) of the activity in accordance with the economic activity classification (NACE code), as well as with the PRODCOM code, shall be indicated.

5. The number of employees in existing and planned production units:

5.1. the current number of employees (at the particular workplace) and the planned number of employees after the receipt of a permit shall be indicated for the existing installations; and

5.2. the planned number of employees shall be indicated for new installations.

6. Description of the polluting activity;

6.1. the reason for presenting a submission (mark as appropriate):

| 6.1.1. |  | the obtaining of a permit for commencement of a polluting activity; |
| --- | --- | --- |
| 6.1.2. |  | the obtaining of a permit for performance of substantial changes in an existing polluting activity; |
| 6.1.3. |  | [6 January 2009]; or |
| 6.1.4. |  | the obtaining of a permit for an existing polluting activity; |

6.2. working hours (the duration of work in normal operating mode shall be indicated, as well as whether the installation is working outside of normal working hours);

6.3. the intended time for the commencement and completion of construction or the reconstruction of industrial installations;

6.4. the intended time for the commencement of the polluting activity;

6.5. the production capacity requested for a permit and the planned annual amount of output for an existing installation, for a new installation – the designed capacity; and

6.6. for waste landfills – the planned capacity of the landfill, the planned duration of operation, the territory to be serviced, for municipal waste landfills – the number of inhabitants to be serviced.

6.1 Information regarding the documents issued by environmental institutions for the polluting activity (if such documents are necessary for an installation in accordance with the regulatory enactments regulating the field of environmental protection):

6.1 1. in relation to the commencement of the polluting activity or substantial changes in an existing polluting activity – the number and date of the final report regarding environmental impact assessment, the institution that accepted the intended activity, the decision number and the date of taking thereof, the number and date of the technical regulations issued by the Board;

6.1 2. in relation to an existing polluting activity – the number, date of issue and period of validity of the last permits issued for the emission of polluting substances into the air, for the use of water or for waste management (also permits for waste transport); and

6.1 3. the date of submission of an industrial accident prevention programme or a safety report and an emergency readiness plan of an object to the Environment State Bureau.

7. The list of contracts entered into with other natural persons or legal persons (given name, surname and address shall be indicated for a natural person, for a legal person – the single registration number and address of the merchant) in accordance with Table 7.1, indicating the main conditions:

7.1. regarding water supply;

7.2. regarding waste water treatment;

7.3. regarding waste management; and

7.4. other contracts, if they are applicable to the activity performed by an operator.

Information regarding contracts entered into*

Table 7.1.

| No. | Contract number | Subject of the contract | Parties to the contract | Capacity indicated in the contract (for example, the amount of waste water, waste) | Term of validity of a contract |
| --- | --- | --- | --- | --- | --- |

# Note. * An operator shall present originals of the contracts upon the request of the Board.

[6 January 2009]

Part B

Production Processes and Technologies

8. Detailed description of the installation and the polluting activity:

8.1. a description of the installation and the production processes (a history of industrial operation, production capacity, products and production processes of the installation shall be described, attaching flow charts for Category A installations and, if necessary, additional information that characterises the polluting activity). The Regional Environmental Board shall clarify the level of detail of the information in accordance with the submission of the operator.

8.2. cleaner production measures, the best available techniques (for Category A installations) and waste reduction (indicate how the reduction of raw materials, chemical substances or chemical products, water and energy consumption, the replacement of hazardous chemical substances, the utilisation or processing of re-usable raw materials is ensured);

8.3. the implementation of environmental protection requirements – the operator shall prepare a plan for existing installations for ensuring the fulfilment of the environmental protection requirements specified by regulatory enactments and for the introduction of cleaner production. The plan for Category A installations shall indicate how the achievement of the conditions specified by the guidelines of the best available techniques is intended. The operator shall not submit the plan if the installation fully complies with all of the environmental protection requirements specified by regulatory enactments, but a Category A installation – also with the conditions specified by the guidelines of the best available techniques; and

8.4. the possible accidents and the reduction of the consequences thereof – indicate what are the means of liquidation of an emergency situation, fire-fighting regulations and how the storage of water intended for fire-fighting is ensured. It shall be indicated if the installation does not require an industrial accident prevention programme or a safety report and an emergency readiness plan of an object.

Part C

Raw Materials and Chemical Substances, Energy and Water

9. Information regarding raw materials, ancillary materials chemical substances and chemical products, fuel and heating fuel in accordance with Tables 9.1, 9.2 and 9.3 of this Annex, information regarding storage tanks – in accordance with Table 9.4 of this Annex (for an installation with different production processes, information regarding raw materials and ancillary materials shall be provided separately for each production process. Tables shall be completed only for such materials, chemical substances or groups thereof, which are essential for the production process. Tables may be applied to the relevant installation. Raw materials and ancillary materials may be grouped accordingly. The Regional Environmental Board shall clarify the level of detail of the information in accordance with the submission of the operator before the final version of the submission is presented. Safety data sheets for dangerous chemical substances or chemical products shall be presented upon the request of the State Environmental Service).

Chemical Substances, Chemical Products and other Materials Utilised during Production Process as Raw Materials or Ancillary Materials and which are not Classified as Dangerous

Table 9.1

| No. or code | Chemical substance or chemical product (or groups thereof) | Chemical substances or the type of chemical product(1) | Way of utilisation | Amount stored (tonnes, type of storage(2) | Amount utilised per year (tonnes) |
| --- | --- | --- | --- | --- | --- |

Notes.

(1) Types of raw materials or ancillary materials: metal, wood, plastic, clay, sand, petroleum products, organic substances, inorganic substances, fruit, vegetables, animals, paints in which volatile organic compounds (hereinafter – VOC) are less than 5 per cent, detergents, filter materials.

(2) Storage: in barrels, tanks, underground, outdoors, indoors and in other places. The maximum and the average amount that is stored. Provide a reference to a map.

Dangerous Chemical Substances and Chemical Products Utilised in Production as Raw Materials, Ancillary Materials or which Develop in Intermediate or Finished Products

Table 9.2

| No. or code | Chemical substance or chemical product(1) (or groups thereof) | Chemical substances or the type of product(2) | Way of utilisation | EC number | CAS number(3) | Class of danger(4) | Designation of danger by letter | Characterisation of risk impact (R-phrase)4) | Designation of safety requirements (S-phrase)(4) | Amount stored (tonnes, type of storage(5) | Amount utilised (tonnes/per year) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Notes.

(1) The dangerous chemical substances specified in the EU Member States are included in the Cabinet Regulation No. 949 of 13 December 2005, Regulations Regarding List of Dangerous Chemical Substances. A chemical substance shall be considered as dangerous if it complies with the danger criteria defined in Cabinet Regulation No. 107 of 12 March 2002, Procedures for Classification, Labelling and Packaging of Chemical Substances and Chemical Products.

(2) The type of raw materials: petroleum products, tar products, inorganic compounds, organic compounds, paints with more than 5 per cent VOC content and other

(3) CAS number – substance index in the Chemical Abstracts Service (Chemical Abstracts).

(4) Characterisation of the impact of a substance (R–phrase) – a risk phrase characterises the impact of a dangerous chemical substance; a designation of safety requirements (S–phrase) – a safety phrase characterises the necessary safety requirements in accordance with Cabinet Regulation No. 107 of 12 March 2002, Procedures for Classification, Labelling and Packaging of Chemical Substances and Chemical Products.

(5) Storage in barrels, tanks (the type of tank shall be indicated), underground, outdoors, indoors and in other places. Provide a reference to a map.

[6 January 2009]

Utilisation of Heating Fuel or Fuel for Heat Energy, Electricity and Transport in the Installation

Table 9.3

|  | Amount utilised in one year | Sulphur content (%) | Utilised |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | for production processes | for heating(1) | for the transport within the territory of the installation | for the generation of electricity |
| Fuel oil (t) |  |  |  |  |  |  |
| Natural gas (1 000 m3) |  |  |  |  |  |  |
| Coal (t) |  |  |  |  |  |  |
| Diesel fuel (t) |  |  |  |  |  |  |
| Gasoline (t) |  |  |  |  |  |  |
| Furnace fuel (t) |  |  |  |  |  |  |
| Shale oil (t) |  |  |  |  |  |  |
| Timber (t) |  |  |  |  |  |  |
| Peat (t) |  |  |  |  |  |  |
| Other types of heating fuel (t) |  |  |  |  |  |  |

Note.

(1) For space heating and hot water supply for domestic or economic needs (does not apply to production process).

## List of Storage Tanks

Table 9.4

| Code(1) | Contents of the storage tank(2) | Dimensions of the tank (m3) | Age of the tank (in years) | Positioning of the tank(3) | Date of examination |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | previous | next |

Notes.

(1) Each storage tank shall be identified with a unique internal code B1, B2, B3, etc.

(2) The name of the substance stored in the tank.

(3) Located underground, above ground or in buildings.

10. Information regarding the utilisation of electricity and heat energy in accordance with Tables 10.1 and 10.2 of this Annex.

Utilisation of Electricity (per year)

Table 10.1

| Electricity, MWh/a (kWh/a) |  |
| --- | --- |
| utilised | total amount |
| For production installations |  |
| For lighting |  |
| For chilling and refrigeration |  |
| # For ventilation |  |
| For heating |  |
| For other purposes |  |
| Total |  |

## Utilisation of Heat Energy per Year

Table 10.2

| Supplier | Heat energy MWh/per year |  |  |
| --- | --- | --- | --- |
|  | in production processes | for heating(1) | for other purposes |

Note.

(1) For space heating and hot water supply for domestic or economic needs (does not apply to production process).

11. Information regarding the abstraction of surface water, groundwater and sea water shall be submitted in accordance with Table 11.1 of this Annex. An operator shall, in accordance with Table 11.2. of this Annex, indicate whether he or she has the following at his or her disposal:

11.1. technical inventory files of external water supply networks and structures;

11.2. the schematic plan of the water supply system;

11.3. the technical passport (for an existing installation for which such passport has been developed);

11.4. the water supply bore-hole passport; or

11.5 the passport for mineral resource deposit (groundwater).

## Water Abstraction

Table 11.1

| Identification number of the water abstraction source(*) | Water abstraction source (water body or bore-hole) |  |  |  |  | Water quantity |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | name and location (address) | geographic co-ordinates |  | code of the water supply district | territory code | cubic metres per day | cubic metres per year |
|  |  | N latitude | E longitude |  |  |  |  |

Note. (*) In accordance with the classification of the State limited liability company “Latvian Environment, Geology and Meteorology Centre”.

[6 January 2009]

11.1 Water intake wells or points, locations of water meters, mains, external water supply networks with a diameter of 100 mm or more, hydrants, gate valves, inspection pits, water towers, clean water reservoirs, water treatment plants and disinfection facilities shall be indicated in the schematic plan of a water supply system (at a scale of 1:500 or 1:5 000, or 1:10 000).

[6 January 2009]

11.2 An operator shall provide the information regarding devices that prevent the ingress of fish into technical equipment, as well as information regarding the water abstraction regime, the specified category for the water abstracted and the water intake points for protection zones in accordance with the Protection Zone Law. If the abstraction of drinking water is planned, a water quality testing report shall be attached upon the request of the Board.

Information Regarding Water Supply System and Mineral Resource (Groundwater) Deposits*

Table 11.2.

| No. | Document | Date of drawing up | Mark regarding the presence of the document |
| --- | --- | --- | --- |
| 1. | Technical inventory files of external water supply networks and structures |  |  |
| 2. | The schematic plan of the water supply system |  |  |
| 3. | The technical passport |  |  |
| 4. | The water supply bore-hole passport |  |  |
| 5. | The passport for mineral resource deposit (groundwater) |  |  |

Note. * An operator shall, upon the request of the Board, present the originals of the documents.

[6 January 2009]

12. Information regarding the use of water in the installation in accordance with Table 12 of this Annex.

## Use of Water

Table 12

| Water abstraction sources and ways of utilisation | Total water consumption (cubic metres per year) | For chilling (cubic metres per year) | For production processes (cubic metres per year) | For domestic needs (cubic metres per year) | For other purposes (cubic metres per year) |
| --- | --- | --- | --- | --- | --- |
| 1. From external suppliers |  |  |  |  |  |
| 2. From a bore-hole owned by an owner |  |  |  |  |  |
| 3. Lake or river |  |  |  |  |  |
| 4. Sea water |  |  |  |  |  |
| 5. Other sources |  |  |  |  |  |
| Total |  |  |  |  |  |

Part D

Environmental Pollution

13. Emission of polluting substances into the air:

13.1. a description of air pollution sources in accordance with Table 13.1 of this Annex;

## Physical Characteristics of Emission Sources

Table 13.1

| Emission source code(1) | Description of the emission source | Characterisation of the emission source and the emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | geographic co-ordinates(2) |  | chimney height | internal diameter of chimney | flow | emission temperature(3) | emission duration(4) |
|  |  | N latitude | E longitude | m | mm | Nm3/h | oC |  |

Notes.

1 Each chimney or other emission source, if such source is not considered a diffuse emission source, shall be identified with an internal code A1, A2, A3, etc.

(2) Geographic co-ordinates are specified with an accuracy of up to one second.

(3) Emission temperature at the place of the flow measurement.

(4) If the emission is not continuous, information regarding the duration thereof shall be provided - minutes/hour, hours/day and days/year.

13.2. information regarding emission sources in accordance with Table 13.2 of this Annex. (Complete in accordance with the procedures by which the emission of air polluting substances from stationary air pollution sources shall be prevented, restricted and controlled specified by regulatory enactments. All activities and substances, including such activities and substances that are not treated or purified, shall be indicated in the Table. Information regarding pollution detected in purification or treatment installations shall be provided in the Section “Waste Management” (Paragraph 17 of this Annex).

## Substances Emitted into the Air from Emission Sources

Table 13.2

| Installation, process, production unit, name of the production site |  |  |  |  | Polluting substance |  | Characteristics of emissions before purification |  |  | Gas purification installations |  |  | Characterisation of emissions after purification(5) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| name | type | emission source code(1) | duration of emission (h) |  | substance code(2) | name | g/s(3) | mg/m3 (3) | tonnes/year (3) | name, type | Efficiency |  | g/s(4) | mg/m3 (4) | tonnes/year (4) |
|  |  |  | dnn | per year |  |  |  |  |  |  | designed | actual |  |  |  |

Notes.

(1) The internal code of the emission source reference as in Table 13.1 of this Annex.

(2) Substance code specified by the State limited liability company “Latvian Environment, Geology and Meteorology Centre”.

(3), (4) Oxygen content shall be indicated for combustion installations. The content of polluting substances shall be indicated for a normal cubic metre (273 K 101.3 kPa). Moisture conditions (moist/dry) shall match the conditions provided in other tables, unless such conditions are specified separately.

(5) The content of the polluting substance (concentration and amount) under standard conditions (273 K 101.3 kPa), unless such content is specified separately.

Moisture conditions (dry/moist) shall be comparable with the data provided in other tables, unless such conditions are specified separately.

[6 January 2009]

13.3. information regarding other emissions into the air (smells and emission from non-organised emission sources) in accordance with Table 13.3;

## Emission from Non-Organised Emission Sources and Smells

Table 13.3

| Emission source | Substance | Concentration of the smell (ouE/m3)(1) | Reduction measures | Characterisation of the smell(2) |
| --- | --- | --- | --- | --- |

Note.

(1) The number of smell units (ouE) in one cubic metre of gas under standard conditions.

(2) For example, the smell of rotten eggs. It shall be completed if the intensity of the smell from the source is lower than the threshold of the smell detection method.

[6 January 2009]

13.4. impact of emissions into the air on air quality (results of calculations and models of the performed calculations shall be indicated). The operator shall calculate an emissions limit project that ensures the fulfilment of air quality requirements, using a pollution-dispersion calculation computer program that ensures the compliance with the local building, geographical and meteorological conditions and which ensures the fulfilment of such requirements that are specified by the regulatory enactments regarding the procedures by which the emission of air pollutants from stationary air pollution sources shall be prevented, restricted and controlled, regarding waste incineration and regarding air quality and methods for determining the smells caused by a polluting activity, as well as the procedures by which the spread of smells is restricted;

13.4.1 air quality may be modelled indicatively for polluting activities which have the same stationary emission source by using the ADMS Screen computer program in accordance with the procedures specified in Cabinet Regulation No. 1015 of 14 December 2004, Environmental Requirements for the Management of Small Boiler Houses. If the indicative calculations attest that exceeding of air quality norms is not possible, the development of an emission limit project for stationary pollution sources shall not be necessary for an installation in accordance with Cabinet Regulation No. 200 of 22 April 2003, Regulations regarding the Development of Emission Limit Project for Stationary Pollution Sources.

## Emission Limit Project for Polluting Substances

Table 13.4

| Emission source |  |  |  | Polluting substance |  |  |  |  | O2 % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | name | geographic co-ordinates |  | name | code | g/s | mg/m3 ouE/m3 (2) | t/a |  |
|  |  | N latitude | E longitude |  |  |  |  |  |  |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10(1) |

Note.

(1) Shall be completed for the installations for which oxygen content in waste gases or exhaust gases is specified by regulatory enactments.

(2) Columns 6, 7, 9 and 10 shall not be completed regarding emission of smells.

13.5. the following data shall be provided regarding incineration plants in addition to the data requested in Table 13.4 of this Annex:

13.5.1. the theoretical amount of air required for incineration – V0 m3/kg (m3/nm3);

13.5.2. the theoretical amount of waste gases – V0d m3/kg (m3/nm3); and

13.5.3. the amount of waste gases in accordance with the specified O2 – Vd m3/kg (m3/nm3).

14. Waste water discharge into surface waters or collecting systems and into waste water treatment plants (polluting substances in waste water shall be indicated, a description of waste water treatment plants shall be provided. A plan for the implementation of improvements with specific goals, time periods for fulfilment, as well as possible modifications, or reconstruction, or the replacement of dangerous chemical substances or products, shall be attached for existing installations in order to ensure that the emissions do not exceed the limits specified by regulatory enactments. Information regarding the collection and drainage of rain water and snow and melted ice water from the site of the installation shall be attached):

14.1 a description of the discharge of polluting substances in accordance with Table 14.1, 14.2 and 14.3 of this Annex (information regarding the receiving water body shall be indicated and a schematic plan of discharge points shall be attached, and all substances, including such substances that are not purified before drainage into the water body, shall be indicated in Table 14.1).

## Polluting Substances in Waste Water

Table 14.1

| Identification number of the point of discharge(1) | Polluting substance, parametre code (3 | Concentration that may not be exceeded(mg/l)(2) | Before treatment |  | Short description of the treatment utilised and the efficacy thereof (%) | After treatment |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | mg/l in 24 hours (on average) | tonnes per year (on average) |  | mg/l in 24 hours (on average) | tonnes per year (on average) |

Notes.

(1) In accordance with the classification of the State limited liability company “Latvian Environment, Geology and Meteorology Centre”.

(2) Indicate in the permit only.

(3) Substance code in accordance with the list approved by the State limited liability company “Latvian Environment, Geology and Meteorology Centre”.

Direct Discharge of Waste Water and Rain Water in Water Bodies (in a Ditch, River, Lake or Sea)

Table 14.2

| Name and address (place) of the point of discharge | Identification number of the point of discharge(1) | Geographic co-ordinates of the point of discharge |  | Receiving water body |  |  | Amount of waste water |  | The duration of discharge (2) (hours per day or days per year) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | N latitude | E longitude | name | code of the water supply district(1) | water flow (m3/h) | m3/d (on average) | cubic metres per year (on average) |  |

Notes.

(1) In accordance with the classification of the State limited liability company “Latvian Environment, Geology and Meteorology Centre”.

(2) For irregular discharges the period of discharge shall be indicated in hours, days, months and years (also periods that are related to the setting up, maintenance and repair of the system).

## Waste Water Discharge to the Treatment Plants of Another Operator

Table 14.3

| Number and address of the point of discharge | Identification number of the point of discharge(2) | Geographic co-ordinates of the point of discharge |  | Name of the operator of another water treatment plant, number of the disconnecting chamber | Amount of waste water (to external waste water treatment plants in accordance with contract) |  | Duration of discharge(3) (hours per day or days per year) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | N latitude | E longitude |  | m3/d | cubic metres per year |  |

Notes.

(1) In accordance with the technical inventory file of external sewage networks and structures or the technical passport of the collecting system.

(2) In accordance with the classification of the State limited liability company “Latvian Environment, Geology and Meteorology Centre”.

(2) If the discharge is not continuous, the duration of the discharge periods shall be indicated (also periods that are related to the setting up, maintenance and repair of the system).

14.2. a characterisation of the collecting system (the age of pipes, the date of the last inspection, information regarding the performer of the inspection and the results thereof, as well as information regarding the maintenance of the system operation shall be indicated). An operator shall indicate whether the technical inventory file of external sewage networks and structures or the technical passport of the schematic plan of the collecting system in accordance with Table 14.4 of this Annex is at his or her disposal.

Documents Characterising the Collecting System which are at the Disposal of an Operator*

Table 14.4.

| No. | Document | Date of drawing up | Mark regarding the presence of the document |
| --- | --- | --- | --- |
| 1. | The technical inventory file of external sewage networks and structures |  |  |
| 2. | The technical passport of the collecting system |  |  |

Note. * An operator shall, upon the request of the Board, present the originals of the documents.

[6 January 2009]

14.3. balance of water use (a schematic plan of the water supply system and collecting system in which the correspondence between the amount of water abstraction and the quantity of waste water transferred to other operators or discharged into the environment is shown); and

14.4. an external sewage network with a diameter of 150 mm or more, inspection pits, spillway chambers, sewage pumping stations and waste water treatment stations shall be indicated in the schematic plan of the collecting system (on a scale of 1:5 000 or 1:10 000).

[6 January 2009]

15. Emission of polluting substances into the soil and ground, as well as groundwater pollution:

15.1. a characterisation of pollution sources (provide a report regarding all soil, ground, subterranean depths or groundwater pollution known to the operator, attach the results of research, if such research has been carried out); and

15.2. soil and groundwater pollution caused by waste.

16. Noise:

16.1. information regarding noise sources in accordance with Table 16.1 of this Annex (indicate such noise sources that create equivalent continuous A-weighted sound pressure level (LAeq. dB(A)), more than 40 dB(A) at night (from 23.00 to 7.00), 45 dB(A) in the evening (from 19.00 to 23.00) and 50 dB(A) during the day (from 7.00 to 19.00).

## Noise Sources and the Indicators thereof

Table 16.1

| No. | Characterisation of the noise source |  |  |  |  | Characterisation of the noise receiver |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | noise source code(1) | name and/or characterisation of the noise source | noise indicator of the noise source(2) (dB (A)) |  |  | noise receiver(3) | distance (m)(4) | noise indicator of the noise receiver(5) (dB (A)) |  |  |
|  |  |  | Lday | Levening | Lnight |  |  | Lday | Levening | Lnight |
| 1. |  |  |  |  |  |  |  |  |  |  |
| 2. |  |  |  |  |  |  |  |  |  |  |

Notes.

(1) Designation of each identifiable noise source with a unique internal code N1, N2, N3, etc.

(2) Noise indicators Lday, Levening and Lnight shall be evaluated in accordance with Cabinet Regulation No. 597 of 13 July, 2004, Procedures for Environmental Noise Evaluation, in the direction of the receiver beyond the border of the territory of the installation at such a distance as to comply with the conditions referred to in Standard LVS ISO 9613-2:2004 "Acoustics – Attenuation of Sound During Propagation Outdoors – Part 2: General Method of Calculation” (for example, not closer than the doubled largest linear size of the noise source).

(3) The utilisation function of the territory shall be indicated in accordance with Annex 2 of Cabinet Regulation No. 597 of 13 July, 2004, Procedures for Environmental Noise Evaluation.

(4)The distance between the noise source and noise receiver. If the noise receiver is located at a distance that is less than the doubled largest linear size of the noise source, the noise indicators shall be determined only at the noise receiver.

(5) The noise indicators Lday, Levening and Lnight at the noise receiver shall be evaluated in accordance with Cabinet Regulation No. 597 of 13 July, 2004, Procedures for Environmental Noise Evaluation.

16.2. transport (indicate the noise caused by the transport driving to and from the installation and measures that are taken and planned in order to reduce such noise at nights).”

17. Waste management:

17.1. information regarding the amount of municipal waste, hazardous waste and waste water sludge caused and currently in temporary storage in accordance with Table 17.1, as well as the amount of production residual material.

Waste Formation and Action with Such Waste

Table 17.1

| Class of waste(1) | Name of the state (2): | Degree of hazard of the waste(3) | Temporary storage (tonnes per year) | Incoming flow of waste(t/a) |  |  |  | Outgoing waste flow (t/a) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | produced |  | received from other undertakings (companies) | Total | processed |  | disposed |  | transferred to other undertakings (companies) | total |
|  |  |  |  | main source | tonnes per year |  |  | quantity | R-code | quantity | D-code |  |  |

Notes.

(1) , (2) , (3) In accordance with Cabinet Regulation No. 985 of 30 November 2004, Regulations regarding Waste Classification and Characteristics that Make Waste Hazardous, and Cabinet Regulation No. 362 of 2 May 2006, Regulations regarding the Utilisation, Monitoring and Control of Sewage Sludge and the Compost Thereof.

(4) A reference shall be provided regarding the main activity and process for each type of waste.

(5)R-code – the type of waste recycling in accordance with Cabinet Regulation No. 365 of 20 April 2004, Regulations regarding the Types of Waste Recycling, Recovery and Disposal.

(6)D-code – the type of waste disposal in accordance with Cabinet Regulation No. 365 of 20 April 2004, Regulations regarding the Types of Waste Recycling, Recovery and Disposal.

17.2. the maximum and minimum flows of hazardous waste, the maximum and minimum thermal input, the maximum permissible pollution with dangerous chemical substances, including with polychlorinated biphenyl, polychlorinated terphenyl, chlorine, fluorine, sulphur and heavy metals; and

17.3. information regarding waste collection, transportation and disposal shall be provided in accordance with Table 17.2 and 17.3 of this Annex.

### Waste Collection and Transportation

Table 17.2

| Class of waste(1) | Name of waste (2) | Degree of hazard of the waste(3) | Collection type(4) | Quantity of transported waste tonnes/ per year | Transportation type(5) | Merchant that carries out transportation of waste (or waste holder) | Merchant that receives the waste |
| --- | --- | --- | --- | --- | --- | --- | --- |

Notes.

(1) , (2) , (3) In accordance with Cabinet Regulation No. 985 of 30 November 2004, Regulations regarding Waste Classification and Characteristics that Make Waste Hazardous.

(4) Containers, barrels, sacks and others.

(5) Road transport, rail, sea transport.

## Waste Disposal

Table 17.3

| Class of waste(1) | Name of waste (2) | Degree of hazard of the waste(3) | Maximum amount of waste for disposal requested in a permit, tonnes per year (or tonnes per quarter) |
| --- | --- | --- | --- |

Notes.

(1) , (2) , (3) In accordance with Cabinet Regulation No. 985 of 30 November 2004, Regulations regarding Waste Classification and Characteristics that Make Waste Hazardous.

17.4. the existing and planned protective measures;

17.5. specific information regarding landfills (types and amount of waste to be disposed, a description of the landfill operation, methods for the prevention or reduction of pollution, a plan for the closure of the landfill and a management plan after closure);

17.6. temporary placement of waste in the territory of the installation (conditions for the selection of the location, a geological and hydrogeological description of the place, a waste management plan, indicating the control and monitoring measures for gases and infiltrate, as well as the control of operation after the closure of the location of the placement and attaching the schematic plan of the placement of waste); and

17.7. specific information regarding incineration and co-incineration plants (plant capacity, a list of waste to be incinerated in accordance with a waste catalogue, for hazardous waste the amount and categories thereof shall be indicated, as well as the maximum and minimum flows of such hazardous waste, the maximum and minimum thermal input thereof, as well as the maximum permissible pollution with dangerous chemical substances, including with polychlorinated biphenyl, polychlorinated terphenyl, chlorine, fluorine, sulphur and heavy metals.

Part E

Monitoring

18. A description of air, water, ground, noise and waste monitoring in accordance with Table 18 of this Annex. The accounting journals specified in the regulatory enactments regarding the utilisation of water resources shall be used for accounting of the use of water.

Monitoring

Table 18

| Code(1) | Parameters subject to monitoring | Sampling method | Analytical method and technology | Frequency of control | Laboratory performing the analyses |
| --- | --- | --- | --- | --- | --- |

Note.

(1) Emission source code, discharge code or waste code. Use the same codes as in Tables 13.2, 14.1, 14.2, 14.3, 16.1 and 17.1 of this Annex.

Part F

Measures to be Taken upon Termination of Operation of the Installation or a Part thereof in order to Reduce the Impact on the Environment

19. The measures that shall be taken in order to reduce the impact on the environment after the termination of operation of a part of or the entire installation, indicating the intended activities with potentially polluting residual materials. The way in which the operator will finance the closure of the landfill, including the management of the landfill after closure, shall be indicated for waste landfills.

Part G

Summary

20. The following shall be indicated in a summary for informing the public regarding the installation:

20.1. the name of the installation, information regarding the operator, owner and location of the installation;

20.2. a short description of production and the reason why a permit is necessary;

20.3. a description of the polluting activity (the resources to be utilised and the impact of emissions on the environment shall be indicated). The following information shall be provided in the description:

20.3.1. water consumption (annual amount – for an existing installation) and measures for reduction of water consumption;

20.3.2. the main source material (also heating fuel and fuel) and utilisation thereof;

20.3.3. the utilisation of dangerous chemical substances and the planned measures for the replacement thereof;

20.3.4. the most significant emissions into the air and water (concentration and annual amount);

20.3.5. the formation and management of waste; and

20.3.6. noise emission level;

20.4. [3 February 2004]

20.5. [3 February 2004]

20.6. [3 February 2004]

20.7. [3 February 2004]

20.8. [3 February 2004]

20.9. [3 February 2004]

20.10. prevention of possible accidents; and

20.11. future plans – the planned expansion of the installation, the modernisation of individual parts or processes.

[19 December 2006]

Part H

I certify that the information provided in the submission for a permit is true and accurate.

Operator or the responsible person:

| Given name, surname |  |  | Date |  |
| --- | --- | --- | --- | --- |
|  |  |  | Seal |  |
| (signature) |  |  |  |  |
| Telephone number |  |  |  |  |

Notes of the Regional Environmental Board of the State Environmental Service

| Date of receipt |  |  |  |
| --- | --- | --- | --- |

Official of the Regional Environmental Board of the State Environmental Service:

| Given name, surname |  |  | Date |  |
| --- | --- | --- | --- | --- |
|  |  |  | Seal |  |
| (signature) |  |  |  |  |

Note. The details “paraksts” [signature], “datums” [date] and “zīmoga vieta” (“z.v.”) [place for a seal] of the document shall not be completed, if the electronic document has been prepared in conformity with the regulatory enactments regarding drawing up of electronic documents.

[6 January 2009]

Acting for the Minister for Environmental Protection

and Regional Development – Minister for the Interior			M. Segliņš

Annex 5

Cabinet Regulation No. 294

of 9 July 2002

Particularly Dangerous Polluting Substances and Substances with a Particularly Significant Environmental Impact that must be Taken into Account in Determining the Permitted Emission

I. Emission into the Air

1. Sulphur dioxide and other sulphur compounds.

2. Nitrogen oxides and other nitrogen compounds.

3. Carbon monoxide.

4. Volatile organic compounds.

5. Metals and compounds thereof.

6. Solid particles.

7. Asbestos (solid particles, fibres).

8. Chlorine and compounds thereof

9. Fluorine and compounds thereof

10. Arsenic and compounds thereof.

11. Cyanides.

12. Chemical substances and chemical products that possess carcinogenic or mutagenic properties or that may affect fertility or reproduction.

13. Polychlorinated dibenzo-dioxins and polychlorinated dibenzofurans.

II. Emission into Water

14. Organohalogen compounds and substances that may form such compounds in the aquatic environment.

15. Organophosphorous compounds.

16. Organotin compounds.

17. Chemical substances and chemical products that possess carcinogenic or mutagenic properties or that may affect the fertility of aquatic organisms.

18. Hydrocarbons and other persistent organic substances that are toxic and accumulate in live organisms.

19. Cyanides.

20. Metals and compounds thereof.

21. Arsenic and compounds thereof.

22. Biocides and plant protection products.

23. Materials in suspension.

24. Substances that facilitate eutrophication (including nitrates and phosphates).

25. Substances that have a negative effect on the content of oxygen in the aquatic environment (expressed by the biochemical consumption of oxygen and the chemical consumption of oxygen).

Acting for the Minister for Environmental Protection

and Regional Development – Minister for the Interior			M. Segliņš

Annex 6

Cabinet Regulation No. 294

9 July 2002

[8 July 2003; 26 July 2005; 6 January 2009]

Submission for a Category C Polluting Activity

| Regional Environmental Board |  |  |  |  |  |  | of the State Environmental Service |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Firm name (name) of the merchant (or another person), name of the operator or |  |  |  |  |  |  |  |  |  |  |  |  |  |
| given name, surname |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Address, telephone number, fax number, e-mail address |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Single registration number of the merchant (or another person) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Date of registration in the Enterprise Register |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Date of registration in the Commercial Register of the Enterprise Register |  |  |  |  |  |  |  |  |  |  |  |  |
|  | The given name, surname and address of such land owner on whose land the installation is located |  |  |  |  |  |  |  |  |  |  |  |  |
|  | or the polluting activity is performed (if different from the address of the merchant) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | The given name, surname and address of the owner of buildings, ancillary buildings and producer goods |  |  |  |  |  |  |  |  |  |  |  |  |
|  | (if different from the addresses referred to above) |  |  |  |  |  |  |  |  |  |  |  |  |
| Place of the performance of polluting activity |  |  |  |  |  |  |  |  |  |  |  |  |  |

## Information Regarding the Polluting Activity

1. The polluting activity performed in the installation in accordance with Annex 2 to Cabinet Regulation No. 294 of 9 July 2002, Procedures by which Polluting Activities of Category A, B and C shall be Declared and Permits for the Performance of Category A and B Polluting Activities shall be Issued

2. Activities performed in the installation, or a characterisation of the intended changes in the activity (a short description of the polluting activity, for example, the type of activity, amount of production produced per day and per year, the type and capacity of installations, the type and consumption of fuel, packaging and management thereof).

3. Information regarding the commencement of the operation of the installation or the continuation of operation, as well as the time period for the operation of the installation, if the operation of the installation is temporary in nature.

4. Information regarding emissions and waste caused by the installation. Description of the intended measures for the prevention or reduction of the emission and waste. Information regarding the waste management (for example, municipal, production and dangerous waste management type, amount of waste per year (in tonnes), place and type of storage thereof, waste manager).

5. Information regarding the received permits (a permit for the use of water resources, a permit for the waste management). The type, number, date of issue and period of validity of the permit shall be indicated.

6. Characterisation of the location of the installation:

6.1. information regarding the immediate vicinity;

6.2. name of the populated area;

6.3. a reference regarding one-storey or multi-storey construction;

6.4. a reference regarding an industrial, public or trade area; and

6.5. a reference regarding protection zones.

7. Information regarding water abstraction (for example, water abstraction source, amount of water consumption (m3/day; m3/year) shall be indicated) and waste water drainage (the type of collection and drainage of municipal waste water and rain water). Information regarding existing waste water purification installations, the capacity thereof (m3/day, m3/year) (the identification number of the waste water discharge point and the code of the water supply district shall be indicated). If there are no purification installations – the amount of waste water caused (m3/day, m3/year) and the type of management thereof shall be indicated.

8. Information regarding water abstraction and waste water drainage (the identification number of water abstraction source and the waste water discharge point and the code of the water supply district).

9. List of chemical substances used.

I certify that the information provided for in this submission is true and precise.

Operator or the responsible person:

| Given name, surname |  |  | Date |  |
| --- | --- | --- | --- | --- |
|  |  |  | Seal |  |
| (signature) |  |  |  |  |
| Telephone number |  |  |  |  |

Note. The details “paraksts” [signature], “datums” [date] and “zīmoga vieta” (“z.v.”) [place for a seal] of the document shall not be completed, if the electronic document has been prepared in conformity with the regulatory enactments regarding drawing up of electronic documents.

Acting for the Minister for Environmental Protection

and Regional Development – Minister for the Interior			M. Segliņš

Annex 7

Cabinet Regulation No. 294

9 July 2002

[8 July 2003; 3 February 2004; 22 April 2004; 26 July 2005; 19 December 2006; 6 January 2009]

Permit for Category A or Category B Polluting Activity

Category ____ Permit No. _________

| Regional Environmental Board |  |  |  | _____________________ |  |  | of the State Environmental Service |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Address | _____________________________________________________________ |  |  |  |  |  |  |  |  |
| Telephone number |  | _____________________________ |  |  |  |  |  |  |  |
| Firm name (name) of the merchant (or another person) |  |  |  |  |  |  |  | _______________________ |  |
| Legal address |  |  | ____________________________________________________ |  |  |  |  |  |  |
| Single registration number |  |  |  |  | _________________________________________ |  |  |  |  |
| Date of registration in the Enterprise Register |  |  |  |  |  | ___________________________________ |  |  |  |
| Date of registration in the Commercial Register of the Enterprise Register___________________________________________________ |  |  |  |  |  |  |  |  |  |
| Installations, operator |  |  | ____________________________________ |  |  |  |  |  |  |
| Address | _______________________________________________ |  |  |  |  |  |  |  |  |
| Territory code |  | _________________________________________ |  |  |  |  |  |  |  |

The type of the intended polluting activity in accordance with the Annex to the Law On Pollution or Annexes 1 and 2 to Cabinet Regulation No. 294 of 9 July 2002, Procedures by which Polluting Activities of Category A, B and C shall be Declared and Permits for the Performance of Category A and B Polluting Activities shall be Issued

| NACE code |  |
| --- | --- |
| PRODCOM code (codes) |  |

| Date of acceptance of the submission for the permit |  |  |  |
| --- | --- | --- | --- |
| The permit has been issued for a new polluting activity |  |  |  |
| The permit has been issued for an existing polluting activity |  |  |  |
| The permit has been issued for an existing polluting activity with substantial changes |  |  |  |

| Date of issuance |  |  |  |
| --- | --- | --- | --- |
|  |  |  | name of the place |

| Director of the Regional Environmental Board of the State Environmental Service |  |
| --- | --- |
|  | (signature and full name) |

|  | Date |  |
| --- | --- | --- |
| Seal |  |  |

A decision regarding the issuance of the permit or the permit conditions may be disputed before the Environmental State Bureau within one month from the day the decision comes into effect.

Permit conditions may be reviewed during the entire period of validity of the permit on the basis of Section 32, Paragraph 3.1 of the Law On Pollution.

Note. The details “paraksts” [signature], “datums” [date] and “zīmoga vieta” (“z.v.”) [place for a seal] of the document shall not be completed, if the electronic document has been prepared in conformity with the regulatory enactments regarding drawing up of electronic documents.

Part A

General Information regarding the Permit

1. Regulatory enactments, on the basis of which the permit has been issued.

2. The period of validity of the permit and the time period for the presentation of a new submission.

3. Information regarding to whom copies of the permit have been sent.

4. Indication regarding restricted access information.

5. [26 July 2005]

6. Other received permits and permits that are replaced by this permit.

Part B

Declared Activity, Evaluation of the Submission and Justification for Issuance of a Permit

7. A short description of the declared activity.

8. Evaluation of the location.

9. Proposals submitted during the process of the taking of a decision (indicating how such proposals have been taken into account):

9.1. proposals by State or local government institutions;

9.2. proposals by the competent institutions of other states in case of a transboundary impact;

9.3. proposals by the public; and

9.4. explanations of an operator.

10. Evaluation of a submission:

10.1. the best available techniques implemented and planned for Category A polluting activities;

10.2. implemented and planned cleaner production measures;

10.3. utilisation of resources (water, energy and chemical substances);

10.4. emission into the air and the environmental impact of such emission;

10.5. the formation of smells;

10.6. emission into water and the environmental impact of such emission;

10.7. the formation and management of waste;

10.8. noise emission;

10.9. soil protection; and

10.10. risk of accidents and action plans for emergency situations.

Part C

Permit Conditions

11. Conditions for the operation of an undertaking:

11.1. operation and management;

11.2. working hours;

11.3. [8 July 2003]

12. Utilisation of resources:

12.1. water

12.2. energy; and

12.3. raw and auxiliary materials.

13. Air protection:

13.1. emission form point sources, emission limits;

13.2. emission from diffuse emission sources, emission limits;

13.3. operation of the process and treatment plants;

13.4. smells;

13.5. monitoring and measurement of emission (places, regularity and methods of measurements)

13.6. supervision of such types of emission that are generated by diffuse emission sources;

13.7. air monitoring;

13.8. maintenance and calibration of measuring equipment; and

13.9. information to be provided to environmental protection institutions.

14. Waste water:

14.1. discharges, emission limits;

14.2. the course of the process and the operation of treatment plants;

14.3. monitoring and measurements (places, regularity and methods of measurements);

14.4. measurements in the receiving body of water;

14.5. maintenance and calibration of measuring equipment; and

14.6. information to be provided to environmental protection institutions.

15. Noise:

15.1. noise sources and conditions for the operation of noise-generating installations;

15.2. noise emission limits;

15.3. monitoring and measurements (places, regularity and methods of measurements); and

15.4. information to be provided to environmental protection institutions.

16. Waste:

16.1. formation of waste;

16.2. conditions for the waste management (collection, treatment, regeneration and disposal);

16.3. monitoring and measurements (places, regularity and methods of measurements);

16.4. information to be provided to environmental protection institutions;

16.5. for a waste incineration or co-incineration plant – the capacity of the plant, the categories of waste to be incinerated in the plant, the amount of waste; and

16.6. for waste landfills – landfill category, capacity, the duration of operation, the types and categories of waste to be disposed, the requirements for the arrangement, exploitation, monitoring and control procedures of the landfill, the requirements for the closure of the landfill and for management after closure.

17. Requirements for the protection of soil, ground, as well as groundwater.

18. Conditions for the operation of a plant in atypical conditions.

18.1 Conditions interrupting the operation of the installation or a part thereof in order to reduce impact on the environment.

19. Conditions for the prevention of accidents and activities in emergency situations.

20. Requirements for the information to be submitted to environmental protection institutions, if permit conditions have been violated or an accident has occurred, as well as requirements for the information to be submitted to environmental protection institutions in accordance with European Pollutant Release and Transfer Register as prescribed by Regulation (EC) No. 166/2006 of the European Parliament and of the Council of 18 January 2006 concerning the establishment of a European Pollutant Release and Transfer Register and amending Council Directives 91/689/EEC and 96/61/EC.

21. Conditions for regular controls by State environmental inspectors.

22. [8 July 2003].

Annexes

23. The attached documents (references regarding dates, including the dates of presentation of submissions and clarifications thereof or additions thereto, proposals by the public, local governments and other institutions and explanations by operators, minutes regarding meetings with the operator and representatives of institutions, minutes of a public consultation) – Annex 1.

24. Submission summary – Annex 2.

25. If necessary, other annexes shall also be attached (schematic plans that show the location of a plant, a copy of a final report of an environmental impact assessment or technical regulations, copies of contracts entered into with managers of external treatment plants, copies of a programme for the prevention of industrial accidents or safety report and an emergency readiness plan and a civil defence plan of the plant).

Acting for the Minister for Environmental Protection

and Regional Development – Minister for the Interior 			M. Segliņš

[ 26 July 2005; 6 January 2009]

Annex 8

Cabinet Regulation No. 294

9 July 2002

Confirmation of Category C Polluting Activity

| Regional Environmental Board |  |  | of the State Environmental Service |
| --- | --- | --- | --- |
| Address |  |  |  |
| Telephone number |  |  |  |

The Regional Environmental Board of the State Environmental Service certifies that a submission prepared in accordance with Cabinet Regulation No. 294 of 9 July 2002, Procedures by which Polluting Activities of Category A, B and C shall be Declared and Permits for the Performance of Category A and B Polluting Activities shall be Issued, has been received from the operator and the operator is included in the list of performers of Category C polluting activities.

| Firm name (name) of the merchant (or another person) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Legal address |  |  |  |  |  |  |  |
| Single registration number |  |  |  |  |  |  |  |
| Date of registration in the Enterprise Register |  |  |  |  |  |  |  |
| Date of registration in the Commercial Register of the Enterprise Register |  |  |  |  |  |  |  |
| Address where the polluting activity is performed |  |  |  |  |  |  |  |
| Type and production volume of the polluting activity |  |  |  |  |  |  |  |

The type of activity in accordance with Annex 2 to Cabinet Regulation No. 294 of 9 July 2002, Procedures by which Polluting Activities of Category A, B and C shall be Declared and Permits for the Performance of Category A and B Polluting Activities shall be Issued

| Submission accepted on |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (date) |  |  |  |  |  |
| Information of the Board |  |  |  |  |  |  |  |
| Director of the Regional Environmental Board of the State Environmental Service |  |  |  |  |  |  |  |
|  |  |  |  | (signature and full name) |  |  |  |
|  |  |  | Date |  |  |  |  |

Place for a seal.

Note. The details “paraksts” [signature], “datums” [date] and “zīmoga vieta” (“z.v.”) [place for a seal] of the document shall not be completed, if the electronic document has been prepared in conformity with the regulatory enactments regarding drawing up of electronic documents.