Text consolidated by Valsts valodas centrs (State Language Centre) with amending regulations of:

2 October 2007 (No. 658).

If a whole or part of a paragraph has been amended, the date of the amending regulation appears in square brackets at the end of the paragraph. If a whole paragraph or sub-paragraph has been deleted, the date of the deletion appears in square brackets beside the deleted paragraph or sub-paragraph.

Republic of Latvia

Cabinet

Regulation No. 370

Adopted 14 August 2001

Regulations Regarding Latvian Building Code LBN 202-01 “Content and Drawing up of Construction Design”

Issued pursuant to Section 2,

Paragraph four of the Construction Law

1. These Regulations approve the Latvian Building Code LBN 202-01 “Content and Drawing up of Construction Design”.

2. Revision of a construction design pursuant to the requirements specified in the Latvian Construction Standard LBN 202-01 “Content and Drawing up of Construction Design” shall not be mandatory for construction designs which have been accepted by 31 October 2001 in accordance with the procedures specified by law and the technical solutions of which conform to the requirements of regulatory enactments applied during the respective period of time.

3. These Regulations shall come into force on 1 November 2001.

Prime Minister 								A. Bērziņš

Minister for Environmental Protection and Regional Development 		V. Makarovs

Approved by

Cabinet Regulation

No. 370

14 August 2001

Latvian Building Code LBN 202-01

“Content and Drawing up of Construction Design”

I. General Provisions

1. The Building Code prescribes the requirements for the content of text pages of a construction design and for the drawing up of drawings.

2. The Building Code shall apply to any construction design at a sketch design stage or a technical design stage, including a structure fixation design and detailed drawings, except the draft of a building idea, other pre-design graphic or text materials, as well as demonstration materials.

3. A construction design shall be developed in not less than three copies:

3.1. the first copy – the original of the construction design – shall be kept by the designer in accordance with the procedures specified in the Law On Archives;

3.2. the second copy – the copy of the construction design harmonised with the originals – shall be submitted by the designer to the building authority for storage in the archives; and

3.3. the third copy – the copy of the construction design harmonised with the originals – shall be transferred by the designer to the customer.

4. Structural and other calculations used for the development of drawings shall not be included in the construction design. The designer shall keep such materials in accordance with the procedures specified in the Law On Archives and present them to the customer or the authorities controlling the construction and institutions of the respective local government upon their request.

5. The institution, which issues the technical or specific provisions for the development of a construction design, may specify which drawings (in one copy) of a part of the construction design must be submitted for storage in the archives of the institution after acceptance of the construction design.

6. A construction design shall contain text pages, including documents and drawings, which are bound or fastened together in one or several books (hereinafter – volume) depending on the number of parts or sections.

II. Content of Text Pages of a Construction Design

7. Text pages of a construction design shall contain the necessary information and explanations regarding the construction design, the solutions, the designer, the customer, the time and place thereof.

8. Text pages of a construction design shall be as follows:

8.1. the title page of the construction design;

8.2. the composition pages of the construction design;

8.3. a table of contents (a table of contents of the text pages and drawings of the construction design included in the volume with page references);

8.4. the documents necessary for construction design works, including reports and technical statements of engineering research;

8.5. an explanatory memorandum; and

8.6. a report on fire safety measures (for socially important structures).

[2 October 2007]

9. The following information shall be specified on the title page:

9.1. the name, the registration number in the Enterprise Register and the licence number of the designer – for legal persons or the given name, surname of the designer and the number of the certificate – for natural persons;

9.2. the name, the registration number in the Enterprise Register and legal address of the customer – for legal persons or the given name, surname and address – for natural persons;

9.3. the number of the commission;

9.4. the name and address of the object of construction;

9.5. the stage of construction design works;

9.6. the name of the part or section of the construction design;

9.7. the volume number and the make of the part or section included therein;

9.8. the given name, surname, signature and seal of the responsible person of the designer’s undertaking if the designer is a legal person;

9.9. the given name, surname and signature of the manager of the construction design;

9.10. the given name, surname and signature of the author of the construction design; and

9.11. the place and year of development of the construction design.

10. The following information shall be specified in the composition page of a construction design:

10.1. titles of parts and sections; and

10.2. marks and sequence numbers in the construction design by volumes accordingly.

11. Parts or sections of a construction design and the page of a table of contents of a volume with page references shall be inserted after the title page and the composition page of the construction design in each volume.

12. The explanatory memorandum of a construction design shall contain general information regarding the location, architectonic solution and the overall engineering solutions of the object of construction, the main technical indicators of the designed structure, as well as other data characterising the structure pursuant to LBN 006-00 “The Essential Requirements for Structures”.

12.1 A review of fire safety measures shall be included in the explanatory memorandum of a construction design according to Cabinet Regulation No. 112 of 1 April 1997, General Construction Regulations.

[2 October 2007]

13. The explanatory memorandum of a part or section of a construction design in the respective part or section shall contain information regarding the solutions included in an individual part or section of the construction design.

III. Drawing up of Drawings of a Construction Design

14. Drawings of a construction design shall be drawn up on drawing sheets of specific size pursuant to the standard LVS EN ISO 5457:1999, Technical Product Documentation. Sizes and Layout of Drawing Sheets.

15. If there are not specific conditions in a contract for construction design works, drawings of a construction design shall conform to the requirements of the standards referred to in Annex 1 to this Construction Standard.

16. The scale of drawings shall be accepted pursuant to the requirements specified in the standard LVS EN ISO 5455:2002, Technical Drawings – Scales.

[2 October 2007]

17. Graphic designations shall be accepted pursuant to the requirements specified in the standard LVS ISO 128-23:1999, Technical Drawings – General Principles of Presentation – Part 23: Lines On Construction Drawings.

18. Drawings of a part or section of a construction design shall be introduced by a page of general indicators, which includes the following:

18.1. a list of drawings of the part or section and marks thereof;

18.2. a list of the used and appended documents;

18.3. a list of construction products necessary for construction, including technical information – designations, type, number;

18.4. indications (may be joined with the explanatory memorandum) and non-standard designations; and

18.5. an attestation of the manager of the part or section of the construction design in accordance with Section 67 of Cabinet Regulation No. 112 of 1 April 1997, General Construction Regulations.

19. A title block shall be created in the lower right corner of the drawing sheet, in which the following information is indicated:

19.1. the name of the designer – for legal person or the surname and address – for natural persons;

19.2. the name of the object of construction;

19.3. the title or content of the drawing sheet;

19.4. the number of the commission;

19.5. the stage of construction design works;

19.6. the mark and number of the drawing;

19.7. the total number of pages in the section, only on the page of general references;

19.8. scale;

19.9. the surname, signature, date of the manager of the part or section of the construction design;

19.10. the surname, signature, date of the technical development engineer of the page; and

19.11. the archive registration number.

20. For repeatedly used drawings a title block shall be applied, in which the following information is indicated:

20.1. the number of the commission;

20.2. the surname, signature, date of the person attaching the page;

20.3. the surname, signature, date of the manager of the part or section;

20.4. the mark and number of the drawing; and

20.5. the archive registration number.

21. Titles of stages of construction design works, as well as of individual parts, sections, works or drawings of a construction design shall be abbreviated, labelling with a special mark of drawing. Labels of marks of a drawing are specified in Annex 2 to this Construction Standard.

22. The ministry, which supervises specialised construction, shall determine the labels of marks of drawings not included in this Construction Standard in relation to specialised construction.

Minister for Environmental Protection and Regional Development 		V. Makarovs

[2 October 2007]

Annex 1

to Latvian Construction Standard LBN 202-01

“Content and Drawing up of Construction Design”

(approved with Cabinet Regulation

No. 370 of 14 August 2001)

List of Standards Applicable to the Drawing up of Drawings of a Construction Design

| No. | Code of the standard | Title of the standard |
| --- | --- | --- |
| 1 | 2 | 3 |
| 1. | LVS ISO 128-20 : 2001 | Technical Drawings – General Principles of Presentation – Part 20: Basic Conventions for Lines |
| 2. | LVS ISO 128-21 : 2001 | Technical Drawings – General Principles of Presentation – Part 21: Preparation of Lines by CAD Systems |
| 3. | LVS ISO 128-22 : 1999 | Technical Drawings – General Principles of Presentation – Part 22: Basic Conventions and Applications for Leader Lines and Reference Lines |
| 4. | LVS EN ISO 3098-0 : 1997 | Technical Product Documentation – Lettering – Part 0: General Requirements |
| 5. | LVS EN ISO 3098-5 : 1997 | Technical Product Documentation – Lettering – Part 5: CAD Lettering of the Latin Alphabet, Numerals and Symbols |
| 6. | LVS EN ISO 3766 : 2004 | Construction Drawings – Simplified Representation of Concrete Reinforcement |
| 7. | LVS EN ISO 4066 : 1999 | Construction Drawings – Bar Scheduling |
| 8. | LVS EN ISO 4157-1 : 1998 | Construction Drawings – Designation Systems – Part 1: Buildings and Parts of Buildings |
| 9. | LVS EN ISO 4157-2 : 1998 | Construction Drawings – Designation Systems – Part 2: Room Names and Numbers |
| 10. | LVS EN ISO 4157-3 : 1998 | Construction Drawings – Designation Systems – Part 3: Room Identifiers |
| 11. | LVS EN ISO 4172 : 1996 | Technical Drawings – Construction Drawings – Drawings for the Assembly of Prefabricated Structures |
| 12. | LVS EN ISO 5261 : 1999 | Technical Drawings – Simplified Representation of Beams and Profile Sections |
| 13. | LVS EN ISO 5456-1 : 1999 | Technical Drawings – Projection Methods – Part 1: Synopsis |
| 14. | LVS EN ISO 5456-2 : 1999 | Technical Drawings – Projection Methods – Part 2: Orthographic Representations |
| 15. | LVS EN ISO 5456-3 : 1999 | Technical Drawings – Projection Methods – Part 3: Axonometric Representations |
| 16. | LVS EN ISO 5456-4 : 2002 | Technical Drawings – Projection Methods – Part 4: Central Projection |
| 17. | LVS EN ISO 5845-1 : 1999 | Technical Drawings – Simplified Representation of the Assembly of Parts with Fasteners – Part 1: General Principles |
| 18. | LVS EN ISO 6284 : 1999 | Construction Drawings – Indication of Limit Deviations |
| 19. | LVS EN 60617-8 : 1996 | Graphical Symbols for Circuit Diagrams – Part 8: Measuring Instruments, Lamps and Signalling Devices |
| 20. | LVS EN 60617-11 : 1996 | Graphical Symbols for Circuit Diagrams – Part 11: Architectural and Topographical Installation Plans and Diagrams |
| 21. | LVS EN ISO 6411 : 1997 | Technical Drawings – Simplified Representation of Centre Holes |
| 22. | LVS EN ISO 6412-3 : 1996 | Technical Drawings – Simplified Representation of Pipelines – Part 3: Terminal Features of Ventilation and Drainage Systems |
| 23. | LVS EN ISO 6428 : 1999 | Technical Drawings – Requirements for Micro-copying |
| 24. | LVS ISO 7296-2 : 1996 | Cranes – Graphical Symbols – Part 2: Mobile Cranes |
| 25. | LVS EN ISO 7437 : 2003 | Technical Drawings – Construction Drawings – General Rules for Execution of Production Drawings for Prefabricated Structural Components |
| 26. | LVS EN ISO 7518 : 1999 | Technical Drawings – Construction Drawings – Simplified Representation of Demolition and Rebuilding |
| 27. | LVS EN ISO 7519 : 1996 | Technical Drawings – Construction Drawings – General Principles of Presentation for General Arrangement and Assembly Drawings |
| 28. | LVS EN ISO 8560 : 1999 | Technical Drawings – Construction Drawings – Representation of Modular Sizes, Lines and Grids |
| 29. | LVS EN ISO 9431 : 2003 | Construction Drawings – Spaces for Drawing and for Text, and Title Blocks on Drawing Sheets |
| 30. | LVS ISO 10628 : 2003 | Flow Diagrams for Process Plants – General Rules |
| 31. | LVS EN ISO 11091 : 1999 | Construction Drawings – Landscape Drawing Practice |
| 32. | LVS ISO 13567-1 : 2002 | {0>Tehniskā dokumentācija.<}94{>Technical Product Documentation <0} Organization and Naming of Layers for CAD – Part 1: Overview and Principles |
| 33. | LVS ISO 13567-2 : 2003 | Technical Product Documentation – Organization and Naming of Layers for CAD – Part 2: Concepts, Format and Codes Used in Construction Documentation |
| 34. | LVS ISO 14985 : 1999 | Hard-copy Output of Engineering Drawings – Specification for the Structure of Control Files |
| 35. | LVS EN 22553 : 1994 | Welded Brazed and Soldered Joints – Symbolic Representation on Drawings |
| 36. | LVS EN 60617-3 : 1996 | Graphical Symbols for Diagrams – Part 3: Conductors and Connecting Devices |

Annex 2

to Latvian Construction Standard LBN 202-01

“Content and Drawing up of Construction Design”

(approved with Cabinet Regulation

No. 370 of 14 August 2001)

Marks of Drawings of a Construction Design

| No. | Mark | Name | Notes |
| --- | --- | --- | --- |
| 1 | 2 | 3 | 4 |
| 1. General part |  |  |  |
| 1. | ti | Topographic research |  |
| 2. | Ģi | Geotechnical research |  |
| 3. | tis | Opinion of the technical research | For existing buildings |
| 4. | Vas | Opinion of the visual survey | For existing buildings |
| 5. | ur | Survey drawings | For existing buildings |
| 6. | FF | Photo fixation | For existing buildings |
| 7. | VI | Historical research | For existing buildings |
| 8. | AMI | Architectonic and artistic research | For existing buildings |
| 9. | AA | Landscape analysis | For landscape projects |
| 2. Architectural part |  |  |  |
| 10. | TS | Territorial section |  |
| 11. | ĢP | General plan of the construction design |  |
| 12. | ar | Architectural solutions |  |
| 13. | ard | Architectural solutions, in details |  |
| 14. | ie | Layout of facilities | For projects of public (community) buildings |
| 15 | in | Interior |  |
| 3. Section of engineering solutions |  |  |  |
| 16. | bk | Building structures | If the amount is small, may be combined with MK, KK, DZK |
| 17. | dzk | Reinforced concrete structures |  |
| 18. | DZKD | Reinforced concrete structures, detailed |  |
| 19. | mk | Metal structures |  |
| 20. | mkd | Metal structures, detailed |  |
| 21. | kk | Wood structures |  |
| 22. | kkd | Wood structures, detailed |  |
| 23. | pk | Plastic structures |  |
| 24. | avk | Heating, ventilation and air conditioning | Also cold supply, anti-smoke protection |
| 25. | Ūk | Water supply and sewerage | Internal networks |
| 26. | ga | Gas supply | Internal networks |
| 27. | sm | Heating equipment | Boilers, installations |
| 28. | el | Electricity supply | Internal networks |
| 29. | vs | Low intensity current systems | Also communications and alarm system Centralised notification systems |
| 30. | UAS | Automatic fire-fighting systems | Automatic fire-fighting systems Automatic fire-fighting alarm systems |
| 31. | vas | Control systems and automated systems |  |
| 32. | Ūkt | Water supply and sewerage, external networks |  |
| 33. | dt | Draining networks |  |
| 34. | lkt | Rainwater sewerage networks |  |
| 35. | gat | Gas supply, external networks |  |
| 36. | sat | Heating supply, external networks |  |
| 37. | elt | Electricity supply, external networks |  |
| 38. | vst | Law intensity currents, external networks |  |
| 39. | var | Environmental protection measures |  |
| 4. Technological part |  |  |  |
| 40. | tn | Technological part |  |
| 5. Economic part |  |  |  |
| 41. | is | Summary of equipment, structures and materials | Specifications |
| 42. | ba | List of the amount of construction works |  |
| 43. | Dop | Organisational design plan |  |
| 44. | Dvp | Execution design | DVP may be included in the composition of the construction design upon special commission, but the designer shall develop it in case of a “build-construct” design |
| 45. | t | Cost estimate | Financial estimates |
| 6. Other designations |  |  |  |
| 46. | sp | Draft sketches |  |
| 47. | tp | Technical design |  |
| 48. | dr | Detailed work drawings |  |

Minister for Environmental Protection and Regional Development 		V. Makarovs