Republic of Latvia

Cabinet Regulation No. 44

Adopted 8 February 2000

Regulation On the Minimum Work Safety and Health protection Requirements when Using Work Equipment.

Issued pursuant to Section 16

of the Law On Labour Protection

I. General Provisions

1. These Regulations determine the minimum requirements for devices (machinery, mechanisms), apparatus, tools or equipment (hereinafter – work equipment) in order that their use does not present a risk to the safety or health of workers, as well as the obligations of the employer when implementing such requirements.

2. These Regulations shall apply to any operation with work equipment (putting into operation (starting up), stopping, utilisation, movement, repair, modification, technical servicing (maintenance), servicing, cleaning) (hereinafter – use) with the exception of specific operations in such areas as, for example, the armed forces, the police, fire-fighting and emergency services, if the nature of the relevant specific activities are in conflict with the requirements of these Regulations.

3. The employer shall be responsible for the observance of these Regulations.

4. The observance of these Regulations shall be controlled by the State Labour Inspection.

- II. Minimum Requirements for Work Equipment

5. Control devices of work equipment that affect the safety of its use shall be clearly visible, appropriately marked, and their meaning shall be readily comprehensible.

6. Control devices of work equipment shall, as far as possible, be located outside danger zones (an area within work equipment or surrounding it, where the safety or health of a worker is endangered) so that the use of such control devices does not cause additional danger. Unforeseen operations with control devices shall not present any risk.

7. A worker who uses work equipment (hereinafter – operator) shall ensure that there are no people in the danger zone.

8. If it is impossible to meet the requirements of Paragraph 7 of these Regulations, work equipment shall have a safety system with sound and/or light signals which automatically signal the work equipment being put into operation (started up) in order that a worker who is fully or partially located in the danger zone may in sufficient time avoid the risk that arises when starting or shutting off the work equipment.

9. Control devices of work equipment shall be safe. They shall be selected by assessing the possibility of accidents, breakdowns and interference under intended conditions of use.

10. It shall only be possible to put work equipment into operation (started up) by a conscious action, utilising a control device intended for such purpose.

11. The requirements set out in Paragraph 10 of these Regulations shall also apply to repeated putting into operation (starting up) of work equipment after bringing it to a halt or a change in the operating regimen (for example, speed, pressure), but repeated putting into operation (starting up) or change in the operating regimen shall not present risk to workers. These requirements shall not apply to equipment operating in an automatic mode.

12. Work equipment requires a safe stopping device for bringing the work equipment to a complete halt and – depending on the danger level of the work equipment – also a stopping device for stopping the dangerous (for example, rotary, mobile) parts of such.

13. The stopping device for work equipment shall prevail over the starting device. When work equipment or dangerous parts of such have been brought to a halt, the power supply to the work equipment shall also switch off.

14. Depending on the danger level and the normal time required for the stopping of work equipment, it shall also be equipped with an emergency stopping device.

15. Work equipment which is dangerous because of falling objects or protuberances shall be equipped with appropriate safety devices.

16. Work equipment which is dangerous due to the release of gases, steam, liquids or dust shall be equipped with appropriate collecting and/or suction devices near the relevant source of risk.

17. Work equipment and parts thereof shall be secured with special clamps or other means if such are required for the safety or health of workers.

18. If mobile parts of work equipment present a risk to the safety or health of workers, special enclosures or protective devices are required which prevent access of workers to the danger zone and comply with the following requirements:

18.1. the structure of such enclosures or protective devices shall be rigid in order that they may not be easily removed or made ineffective;

18.2. they shall be durable and shall not cause additional danger themselves;

18.3. they shall be located at a sufficient distance from the danger zone;

18.4. they shall not restrict visibility in the area of operation of the work equipment; and

18.5. they shall not obstruct the carrying out of repairs and technical servicing (maintenance) of the work equipment. If possible, the protective enclosures or protective devices shall not be removed during repairs and technical servicing (maintenance).

19. The working or technical servicing area at the work equipment or an area surrounding it shall be illuminated in accordance with the nature of the work.

20. If breaking or fragmentation of parts of the work equipment is possible, presenting risk to the safety or health of a worker, the work equipment shall require special protective devices (for example, lattice, screen).

21. If during operation work equipment or separate parts of such become hot or cold to such temperature as presents a risk to the safety or health of workers, such parts shall be isolated or enclosed.

22. Warning (alarm) devices for work equipment shall be easily visible and obvious, and their meaning shall be readily comprehensible and unambiguous.

23. Work equipment shall be appropriately marked and labelled with safety notices and warning signs. The warning signs and notices on the work equipment shall be sufficiently informative, obvious and comprehensible.

24. Technical servicing (maintenance) of work equipment shall be carried out when the equipment is shut off and disconnected from the power source. If the equipment can not be shut off, the technical servicing (maintenance) shall be carried out outside the danger zone or by observing special protective measures.

25. Workers shall regularly complete the maintenance log for machinery, apparatus and equipment. If necessary, the log shall be presented to the State Labour Inspection.

26. Work equipment may only be used under the conditions intended for it and for intended operations.

27. Work equipment requires clearly visible and easily accessible power supply cut-off devices. The isolation or disconnection of the work equipment from the source of power, as well as repeated connection to the source of power shall not present a risk to the safety or health of workers.

28. Workers shall be provided with safe access to work equipment, as well as a safe environment in all areas of production, assembly and technical servicing (maintenance).

29. Work equipment by means of which or in which substances are manufactured, utilised or accumulated shall be secured against the risk of explosion of the work equipment or of the relevant substances.

30. Work equipment shall be fireproof and secured against overheating, as well as equipped in such a way as to protect workers against the risks related to the release in the surrounding environment of gases, liquids, steam, dust or other substances which are produced, used or stored by it or in it.

31. Workers shall not come into direct or indirect contact with such parts of work equipment as may present an electrical type risk.

32. Work equipment shall be installed, located and used so that the risk presented to the safety or health of the operator and other workers (for example, mobile and fixed parts or mobile parts of the work equipment are sufficiently distant) is reduced to a minimum, and safe supply and/or relocation of power and substances utilised and manufactured is ensured.

33. Work equipment shall be installed or dismantled under safe conditions observing all the instructions indicated by the manufacturer.

34. Work equipment that may be struck by lightning shall be specifically protected against the effect of lightning by grounding or by ensuring the work equipment with other protective devices.

- III. Additional Minimum Requirements for Mobile Work Equipment

35. The requirements of this Chapter shall apply to mobile work equipment, also to self-propelled vehicles (hereinafter – equipment), which may present risk due to the mobility of the equipment.

36. Equipment on which workers are present shall be equipped so that, when moving, it does not present risk to workers of coming into contact with or being crushed by parts (for example, wheels, track mechanisms) of the equipment.

37. Equipment shall be equipped or adapted so that the coupler between the equipment and its accessories and/or items that are being pulled do not get jammed (blocked). If these requirements cannot be met fully, relevant measures shall be taken in order to eliminate the risk to the safety or health of workers.

38. Power transmission mechanisms between mobile parts of equipment shall be secured and protected from sand, dirt and mechanical damage.

39. Equipment on which workers are present must be so designed as to prevent the equipment from overturning, or it shall be necessary to equip such with:

39.1. safety structures that prevent the equipment from tilting more than one quarter;

39.2. a structure which ensures a sufficiently large free area around workers, if the tilting is more than one quarter; and

39.3. other equivalent equipment.

40. If it is impossible to entirely eliminate the risk of equipment overturning, the equipment shall be equipped with a safety structure. Such safety structure may be a part of the equipment.

41. If equipment is stabilised during operation or the equipment is so designed as to prevent it from overturning, safety structures shall not be necessary.

42. If it is possible that during the operation of equipment intended for lifting of loads – a fork-lift – may tilt, the equipment shall be adapted accordingly or equipped with:

42.1. a driver’s cab;

42.2. a structure which prevents the equipment from tilting;

42.3. a structure which, when a platform tilts, ensures for a worker present on the equipment a sufficiently large distance between the ground and parts of the equipment; and

42.4. a protective structure which prevents the worker sitting in the driver’s cab from being crushed by parts of the platform.

43. In order that the use of a self-propelled vehicle does not present risk to the safety or health of workers, the following conditions shall be observed:

43.1. the equipment shall be so equipped as to prevent its unauthorised putting into operation (starting up);

43.2. if several self-propelled vehicles run on rails simultaneously, safety measures shall be taken in order to reduce the consequences of a possible collision;

43.3. the equipment shall be equipped with a braking and stopping device, and, if necessary, also with an emergency device with an automatic system which brings the equipment to a halt if the main braking and stopping device is damaged;

43.4. if the direct field of vision of the driver is not sufficient for safe performance of work, appropriate accessories (mirrors or other optical or electronic devices) shall be fitted to improve visibility;

43.5. if the equipment is used at night or in dark places, sufficient lighting appropriate for the nature of the work shall be provided;

43.6. if the equipment or freight transported by such is flammable, fire extinguishing devices shall be placed near the location where the equipment is used. If it is impossible to ensure this, the equipment shall be provided with fire extinguishing devices;

43.7. remote control equipment, when leaving the area of control, shall stop automatically; and

43.8. remote control equipment shall be so equipped as to prevent the risk of impact or collision when work is being performed. This condition shall not apply to remote control equipment that has already been equipped with adequate protective devices for preventing the risk of collision.

44. Only workers who have been appropriately trained shall be permitted to drive self-propelled vehicles.

45. Workers shall observe the safety instructions formulated by the employer for the operation of self-propelled vehicles moving in a work area.

46. Workers may not be present in an area where self-propelled vehicles are used. If the work cannot be done without the presence of workers, relevant measures are required in order that the self-propelled vehicles do not present a risk to the safety or health of the workers.

47. Workers may be carried on mechanically driven equipment only in cases where the equipment is specially equipped for such. The speed of movement of the equipment shall be regulated in conformity with the work to be done.

48. Equipment with combustion engines may be operated and used only where a sufficient amount of air can be ensured in order that exhaust gases do not present a risk to the safety or health of workers.

- IV. Additional Minimum Requirements for Work Equipment Provided for Lifting Loads

49. The requirements of this Chapter shall apply to work equipment provided for lifting loads (hereinafter – lifting equipment).

50. If lifting equipment is permanently installed, its solidity and stability during use shall be ensured by taking into account the size of the load to be lifted and the stress on the mounting or fixing points of the structure.

51. The nominal lifting capability and, if necessary, a table of the lifting capability for each configuration of the mechanism shall be clearly indicated on the lifting equipment mechanisms.

52. Capacities for the safe use of accessories shall be unambiguously indicated on the accessories of lifting equipment.

53. If lifting equipment is not intended for lifting people, a clear notice of such prohibition shall appear on the equipment.

54. Permanently installed lifting equipment shall be installed so as to reduce the risk that the load may:

54.1. fall on workers or strike them;

54.2. slide dangerously or fall freely; or

54.3. be released unintentionally.

55. People may only be lifted with lifting equipment and accessories of such intended for that purpose. In exceptional cases people may also be lifted with equipment which, in conformity with the requirements of regulatory enactments, is adapted to the lifting of people and which does not present a risk to their safety or health.

56. If lifting equipment is provided for lifting and movement of workers, it shall comply with the following requirements:

56.1. it shall be equipped with a safety device that eliminates the possibility of the car falling. If this can not be ensured because of height or due to the specific nature of the work site, a special rope and a safety harness, which shall be examined daily, are required to prevent a worker falling from a height;

56.2. the possibility of a worker falling from the car shall be eliminated;

56.3. the crushing, trapping or getting stuck of a worker shall be prevented, in particular if unintentional contact with objects is possible; and

56.4. the release of a worker from the car shall be ensured in case of an accident.

57. If the lifting equipment is mobile or can be dismantled, the stability of the equipment during use in all intended working conditions shall be ensured by taking into consideration the specific nature of the base.

58. When lifting workers with lifting equipment, the following requirements shall be complied with:

58.1. an operator shall be at the control panel throughout the performance of work;

58.2. workers who are being lifted shall be equipped with quality means of communication; and

58.3. if a dangerous situation arises, workers shall be provided with safe means of evacuation.

59. Workers shall not be present under suspended or raised load. Loads shall not be moved over unprotected work sites where workers are normally present. If this cannot be ensured due the specific nature of the work, the employer shall take appropriate measures to eliminate the risk to the safety or health of workers.

60. Lifting accessories shall be selected by taking into consideration the specific nature of the load to be moved, the gripping points, attachment tackle and weather conditions, as well as the type and configuration of the movement.

61. If the lifting accessory tackle is not dismantled after use, a clear notice of such, as well as a clear notice of its characteristics, must appear on the lifting accessory tackle.

62. Lifting accessories shall be so stored as to prevent such being damaged.

63. When lifting loads not directly controllable (loads which after being lifted could, due to wind or other conditions, start to move uncontrollably – for example, rotate, oscillate – and such movement can not be stopped immediately), the following requirements shall be observed:

63.1. if two or more pieces of lifting equipment, the activity radius of which overlap, have been installed at a work site, the possibility of a collision between loads and/or parts of the lifting equipment shall be eliminated;

63.2. if mobile lifting equipment is utilised, appropriate measures shall be taken to eliminate the possibility of tilting, overturning, moving or sliding, and examination shall be carried out to determine whether such measures have been taken appropriately;

63.3. if the movement path of a load is not in full view, the operator shall communicate, directly or with the help of means of communication, with a competent worker who shall give the operator instructions and shall take relevant measures to eliminate the collision of loads and risk to the safety or health of workers;

63.4. manual attaching and disengaging of loads shall not present a risk to the safety or health of workers; during the course of such actions the worker shall directly or indirectly control the lifting equipment;

63.5. the work of lifting shall be planned in advance and appropriate supervision shall be provided for in order to eliminate risk to the safety or health of workers;

63.6. if the load is lifted by two or more pieces of lifting equipment simultaneously, the operator, for better work co-ordination, shall observe the special safety rules for such work formulated by the employer;

63.7. if the lifting equipment does not ensure the holding of loads when electricity supply is fully or partially interrupted, relevant measures shall be taken in order to eliminate risk to the safety or health of workers;

63.8. equipment and raised loads shall not be left without supervision, except in cases when the danger zone is enclosed and the load is secured and held safely; and

63.9. if weather conditions deteriorate and use of the lifting equipment is not completely safe, the use of the lifting equipment in open air shall be discontinued and measures shall be taken to ensure that the lifting equipment does not overturn and present a risk to the safety or health of workers.

- Obligations of Employers

64. The employer shall observe instructions provided by the manufacturer of work equipment in order to ensure that use of the work equipment is without harm to the safety or health of workers.

65. The employer shall ensure:

65.1. the conformity of work equipment to the work process and proper use of the work equipment in order to eliminate risk to the safety or health of workers. If it is impossible to eliminate the risk fully, the employer shall take measures to reduce the risk factors;

65.2. that, when selecting work equipment, the working conditions and related risks to the safety or health of workers in the undertaking (company) and at each work site be taken into consideration;

65.3. the conformity of the work equipment to the requirements of these Regulations and appropriate technical servicing (maintenance) during the whole period of operation of such;

65.4. the examination of work equipment after installation and use of such, as well as after assembly in a new place or new location if safety of the work equipment is determined by the conditions of its installation;

65.5. the following examinations of the work equipment which is exposed to conditions that may cause its deterioration, presenting a risk to the safety or health of workers:

65.5.1. periodic examination and also testing if necessary;

65.5.2. special examination if exceptional circumstances have arisen (for example, breakdowns, accidents, unfavourable weather conditions), also when work equipment has been modified or has not been used for a long time;

65.6. that work equipment which may present an increased risk to the safety or health of workers (for example, dangerous equipment) is only used by specifically designated and trained workers; and

65.7. that the work equipment has a document regarding the last safety examination, if it is used outside the undertaking (company).

66. The examinations referred to in Subparagraphs 65.4 and 65.5 shall be carried out by a competent worker designated by the employer, or a competent inspection authority if this is prescribed by regulatory enactments or if the employer is unable to ensure inspection.

67. Records of examination shall be available to the employer and the inspection authority (if an inspection authority has carried out the examination). The owner of the work equipment and the employer, as well as the inspection authority, shall keep records of examination for 10 years, and shall thereafter deposit them in archives.

68. In order to ensure the conformity of work equipment to the minimum requirements for safety and health, the employer shall take into account the specific nature of the workplace, distribution of workers, body position and principles of ergonomics, when using the work equipment.

69. The employer shall provide workers with comprehensible necessary information and written instructions regarding work equipment.

70. The information and written instructions regarding work equipment shall contain the requirements for safety and health at work, the instructions for use of the work equipment, a description of the possible specific situations and conclusions resulting from the experience gained in using the work equipment.

71. Workers shall be informed regarding work equipment at or near the work site and of any changes in the work equipment even if workers do not use it, and explanation shall be provided regarding the danger of the relevant work equipment.

72. The employer shall ensure that instruction to workers is provided (also, information regarding any risk related to use of the work equipment) in accordance with procedures prescribed by regulatory enactments.

73. The employer shall provide special training to the workers referred to in Subparagraph 65.6 of these Regulations

74. The employer shall ensure that workers and their representatives are consulted and participate in all matters related to safety and health safeguards at work, especially in relation to work equipment.

VI. Closing Provisions

75. These Regulations shall come into force on 1 July 2000.

76. Work equipment, the use of which was begun prior to 1 July 2000, shall be modified by 1 July 2004 in compliance with the requirements of these Regulations.

Prime Minister									A. Šķēle

Minister for Welfare									P. Jurdžs