Text consolidated by Tulkošanas un terminoloģijas centrs (Translation and Terminology Centre) with amending regulations of:

5 October 2004 (No. 831);

2 May 2007 (No. 294).

If a whole or part of a paragraph has been amended, the date of the amending regulation appears in square brackets at the end of the paragraph. If a whole paragraph or sub-paragraph has been deleted, the date of the deletion appears in square brackets beside the deleted paragraph or sub-paragraph.

Republic of Latvia

Cabinet

Regulation No. 628

27 July 2004

Special Environmental Requirements for Performance of Polluting Activities in Animal Housing

Issued pursuant to

Section 24.2, Paragraph two

of the Law On Pollution

I. General Provisions

1. These Regulations prescribe special environmental requirements for the performance of polluting activities in animal housing.

2. These Regulations shall apply to:

2.1. polluting activities of category A;

2.2. polluting activities of category C, if they are performed in animal housing where:

2.2.1. 10 or more livestock units are located;

2.2.2. five or more livestock units are located, if the housing is located in a vulnerable zones, to which increased requirements for the protection of water and soil from pollution with nitrates caused by agricultural activity (hereinafter – vulnerable zones) are applicable.

[2 May 2007]

3. The following terms are used in these Regulations:

3.1. manure:

3.1.1. farmyard (solid) manure – solid excrements or mixture of solid excrements and litter where there is at least 15% of dry matter;

3.1.2. slurry – any mixture of solid excrements and urine or water where the content of dry matter is from 3-15%;

3.1.3. urine – liquid, which flows out from farmyard (solid) manure, as well as precipitation water, which reaches the location of the heap of manure and mixes with the manure;

3.2. silage juice – liquid, which flows out from silage during preparation and storage thereof; and

3.3. animal housing – animal lodging, also storage facilities for the storage of farmyard (solid) manure, slurry, urine and silage juice in any form.

II. Requirements for Restriction and Control of Pollution

4. In order to prevent the pollution of ground water and surface water, the following requirements shall be specified for the drainage, storage and dispersal of manure and silage juice:

4.1. collection and drainage systems in animal lodgings (except deep cattle-sheds where up to 20 animals are bred) shall be built from a waterproof material, which is resistant to the effects of equipment used in animal lodgings;

4.2. the floor and walls of the storage facility shall be built from a waterproof material, which is resistant to the effects of the equipment;

4.3. an appropriate system for the collection and drainage of slurry or urine to the relevant storage facilities shall be established in animal lodgings;

4.4. in storing farmyard (solid) manure in a pile, the following requirements shall be complied with:

4.4.1. temporary storage of farmyard (solid) manure shall be permissible on a field in piles. The keeping of piles on a field at the same site for a period exceeding 18 months is prohibited;

4.4.2. in forming the base for a pile of farmyard (solid) manure on a field, material protecting liquid fractions of manure from run-off shall be used. The surface and base of the pile of farmyard (solid) manure shall be covered with a protective layer, which hinders run-off or evaporation;

4.4.3. a pile of farmyard (solid) manure shall be formed at sites where the terrain does not facilitate the formation of surface run-off. The pile of farmyard (solid) manure shall be placed in accordance with the requirements specified in the regulatory enactments regarding environment protection in relation to the protective zones of surface water objects, but not closer than 30 m from a river, brook, ditch, wells of amelioration systems or a well, from which water for the household is being taken;

4.4.4. [2 May 2007]

4.5. manure shall not be dispersed in the period of time from 15 December up to 1 March; and

4.6. manure and silage juice shall not be dispersed on frozen, wet or snow-covered soil.

[2 May 2007]

5. The requirements referred to in Sub-paragraph 4.4 of these Regulations shall not be applicable to polluting activities in vulnerable zones.

6. The following requirements shall be specified for the capacity of storage facilities of farmyard (solid) manure, slurry and silage juice:

6.1. the volume of storage facilities of farmyard (solid) manure established in animal housing shall ensure accumulation of farmyard (solid) manure for at least six months. The volume of storage facilities of farmyard (solid) manure shall ensure accumulation of slurry for at least seven months. If the volume of storage facilities of farmyard (solid) manure or slurry does not conform to the referred to requirements, the operator of the animal housing shall enter into a contract with another natural person or legal person regarding the storage or use of farmyard (solid) manure or slurry outside the animal housing;

6.2. urine from open and closed storage facilities of farmyard (solid) manure shall be collected in a separate facility, the capacity of which ensures accumulation of the amount of urine collected within a period of seven months;

6.3. storage of silage juice in a urine or slurry facility shall be permissible if the amount of silage juice does not exceed 5% from the amount of slurry. Discharge of silage juice in the environment shall not be permissible.

6.1 The structures referred to in Paragraph 6 of these Regulations shall be built in accordance with the regulatory enactments regulating building.

[2 May 2007]

7. The following requirements shall be specified for the storage of silage in a trench and piles and use thereof:

7.1. the base of a silage trench shall be made of a waterproof material and resistant to the effects of silage and potential mechanical damage during filling in or emptying;

7.2. the base of a silage trench shall be made with a slant towards the direction of unloading of silage. A cross-channel for drainage of silage juice into an accumulation tank shall be arranged in the lowest part of the trench in the slant;

7.3. when storing silage in piles on a field, a film shall be spread under the pile. The spread film shall be connected with the film covering the pile;

7.4. a silage pile shall be formed at sites where the terrain does not facilitate the formation of surface run-off and the run-off of silage juice. The silage pile shall be placed in accordance with the requirements specified in the regulatory enactments regarding environment protection in relation to protective zones of surface water objects, but not closer than 30 m from a river, brook, ditch, wells of amelioration systems or a well, from which water for the household is being taken;

7.5. if a silage pile is formed at the same place every year, the base of the pile shall be covered with concrete. The base for a field shall be formed with a slant towards the part of the tank for collection of silage juice. A small ditch for the collection of silage juice or an edge with a height of 0.2-0.3 m shall be formed around the field of the pile;

7.6. silage juice shall not be dispersed during the period of time from 1 December to 1 March; and

7.7. manure and silage juice shall be used as organic manure in accordance with Cabinet Regulation No. 531 of 18 December 2001, Regulations regarding Protection of Water and Soil from Pollution with Nitrates Caused by Agricultural Sources.

8. The requirement referred to in Sub-paragraph 7.6 of these Regulations shall not be applicable to activities in vulnerable zones.

9. The following requirements shall be specified for the storage of slurry:

9.1. the storage facilities of slurry may be closed-type or covered with a permanent natural or artificial floating covering layer, which reduces evaporation. The floating covering layer or cover shall cover the surface of the storage facility at all times and, where appropriate, supplemented with a new material; and

9.2. slurry shall be infused in storage facilities under the level of manure in the storage facility in order to prevent the possibility of reverse flow. The filling system shall be established so that the floating covering layer would not be disturbed.

III. Monitoring and Control Performed by an Operator

10. An operator shall indicate data in an inventory journal (on paper or in electronic form) in accordance with Paragraph 11 of these Regulations. The inventory journal shall be presented for examination upon the request of the State environmental inspector. The operator shall keep the relevant information for at least five years. If accounting is performed in electronic form, data of the journal shall be printed out once every six months.

11. The following data shall be indicated in the inventory journal:

11.1. the date when the covering of the floating covering layer (if any) was supplemented; and

11.2. the date when manure or silage juice was dispersed on a field or transferred to another natural person or legal person, and the amount of dispersed or transferred manure or silage juice.

12. If an operator has entered into a contract with another private person regarding waste collection, wastewater treatment, storage or use of farmyard (solid) manure or slurry outside the housing, the operator shall present the relevant contract upon the request of the State environmental inspector.

13. State environmental inspectors shall control the conformity to the requirements of Regulations.

[2 May 2007]

IV. Closing Provisions

14. If animal housing has been put into service before the coming into force of these Regulations and does not conform to the requirements of these Regulations, the operator shall submit a plan of measures to the regional environmental board of the State Environmental Service within a time period of 12 months after coming into force of these Regulations in order to ensure the conformity to the requirements of these Regulations (hereinafter – plan of improvements). If animal housing requires reconstruction pursuant to the plan of improvements, it shall be performed not longer than within a time period of 10 years after the coming into force of Regulations.

[2 May 2007]

15. The regional environmental board of the State Environmental Service shall take a decision regarding accepting of the plan of improvements within a time period of one month after receipt of the plan of improvements and shall inform the operator of the animal housing about it in writing. If the plan of improvements does not ensure the fulfilment of the requirements specified in these Regulations, the regional environmental board of the State Environmental Service shall take a justified decision regarding refusal to approve the plan. The referred to decision may be contested to the State Environmental Impact Assessment Bureau within a time period of one month.

[2 May 2007]

16. In designing new animal housing, building of a manure storage facility shall be intended in the building design.

Prime Minister 								I. Emsis

Minister for the Environment 							R. Vējonis