Republic of Latvia

Cabinet

Regulation No. 63

Adopted 13 February 2001

Regulations Regarding Minimum Requirements for the Safety and Health Protection of Workers in Workplaces

Issued pursuant to Section 16 of the

Law On Protection at Work

I. General Provisions

1. These Regulations determine the minimum requirements for the safety and health protection of workers in all existing workplaces, as well as in workplaces to be newly organised or modified – in places where a person (hereinafter – the worker) performs his or her work, as well as in other places accessible to the worker in the course of his or her work.

2. The requirements of these Regulations do not apply to workplaces:

2.1. in automotive, river, sea, air and railway means of transport;

2.2. mineral-extracting industries;

2.3. fishing boats;

2.4. agriculture or forestry undertakings, if such are located separate from the buildings of these undertakings; and

2.5. construction sites.

3. These Regulations do not determine the minimum requirements for the safety and health protection of workers in workplaces in emergency situations.

4. The employer shall be liable for the observance of these Regulations.

5. Compliance with these Regulations shall be controlled by the State Labour Inspection.

II. Minimum Requirements for the Safety and

Health Protection of Workers in Workplaces

6. The requirements specified in this Chapter shall be applied, observing the characteristics of the workplace, nature of the work, working conditions, risk of the working environment and other conditions.

7. Electric appliances shall be designed, installed and maintained so as:

7.1. not to create the risk of fire or explosion;

7.2. workers may be protected from electrical trauma caused by direct or indirect contact with an electric appliance; and

7.3. selection of materials and protective devices shall be adequate for the voltage, working conditions and competence of the workers who have access to electric appliances or parts thereof.

8. Emergency routes and exits shall be designed, constructed and maintained in compliance with the following requirements:

8.1. emergency routes and exits, as well as traffic routes and doors leading to such, shall be free and ensure the most rapid passage to a safe area;

8.2. emergency routes and exits shall ensure rapid and, as safe as possible, evacuation of workers from all workplaces in case of danger;

8.3. emergency doors shall open outwards;

8.4. it is prohibited to use sliding doors or revolving doors as special emergency doors;

8.5. emergency doors shall not be locked during the presence of workers, and shall be easily opened in case of danger;

8.6. emergency routes and exits shall be marked by signs in accordance with the requirements for the use of safety signs at workplaces;

8.7. the number, location and size of emergency routes and exits shall be specified depending on the number, equipment and size of the workplaces, as well as on the maximum potential number of workers; and

8.8. emergency routes and exits requiring illumination shall be provided with emergency lighting of adequate intensity in cases when the lighting fails.

9. Workplaces shall be ensured, in accordance with procedures prescribed in regulatory enactments, with suitable automatic fire-fighting equipment, automatic fire-detectors, fire alarm systems and fire-fighting devices in adequate quantities, taking account of the size of the buildings and the intended use of such, the work equipment installed therein, the physical and chemical properties of the substances in use, and the maximum potential number of workers.

10. Fire-fighting devices, automatic fire-fighting equipment, automatic fire detectors and fire alarm systems shall be easily accessible and simple to use, regularly inspected and maintained, and the location shall be identified by signs located in accordance with procedures prescribed by regulatory enactments.

11. In planning, organising and maintaining a territory, the following requirements shall be complied with:

11.1. the territory shall be, as much as possible, planted with greenery, and with well-organised traffic routes;

11.2. waste shall be collected, sorted and temporarily stored in specially arranged places; and

11.3. warehouses for fine (dust-like) material shall be located at least 25 metres from other buildings (if the size of the territory of the undertaking allows it).

12. Requirements regarding ventilation of enclosed work premises:

12.1. sufficient supply of fresh air shall be ensured in workplaces, taking into account the nature of the work and the physical demands placed on workers;

12.2. concentration of any such substance which , upon coming into contact with a human body, causes an injury, an occupational disease or other health problems (hereinafter – harmful substance), may not exceed the maximum permissible concentration of harmful substances in the working environment, which shall not cause, for the duration of a whole working life of eight-hour work days (or any other duration of exposure, but not more than 40 hours a week), contracting of a disease or deterioration of health which can be determined through modern investigative techniques (hereinafter – maximum permissible level of occupational exposure to harmful substances);

12.3. air-conditioning or ventilation systems shall be maintained in good order and regularly cleaned, and the efficiency of operation of such shall be inspected;

12.4. a ventilation control system which detects disturbances in the operation of a ventilation system shall be installed (if the necessity for such a control system for the safety and health protection of workers at the workplace is determined by an employer or by the trusted representatives of the workers);

12.5. the operation of mechanical ventilation systems and air-conditioning equipment shall not cause draughts;

12.6. in organising supply - exhaust ventilation and air heating, it shall be permissible to have no more than 10% air re-circulation of the total air supply volume. It is prohibited to have air re-circulation from premises in which work involves chemical substances, chemical products, asbestos, bacteria, viruses, radioactive substances, as well as from premises in which work presenting a fire hazard or an explosion hazard is performed;

12.7. noise caused by ventilation and air heating devices shall not exceed the admissible noise level;

12.8. if it is not possible to prevent or reduce the concentration of harmful substances to the limit values of occupational exposure of harmful substances by existing technical means, local exhaust ventilation shall be installed in the workplaces where harmful substances are discharged in the air, in order that the harmful substances shall be excluded from entering the breathing zone of the workers, a neighbouring workplace, other premises or the surrounding environment; and

12.9. the ventilation system shall be linked with fire-detectors or fire-fighting equipment, so that, in case of fire, the ventilation shall be switched off, preventing the fire from spreading to other premises.

13. Production waste and chemical substances or chemical products that may pollute the atmosphere of the working environment or atmosphere as a whole, and create a hazard to the safety and health of the workers, shall be removed without delay.

14. Requirements regarding the microclimate of premises:

14.1. it shall be ensured that the microclimate of work premises conforms with the nature of the work and the physical demands placed on the workers (temperature, relative air humidity, air movement rate, and reflected heat); and

14.2. the microclimate in rest areas, rooms for duty staff, changing rooms, lavatories, showers, canteens and first aid rooms shall be appropriate for the intended use for such areas.

15. Requirements regarding the illumination of premises:

15.1. workplaces shall be ensured with natural light and equipped with artificial lighting in such a way that the total intensity of the lighting is adequate for the safety and health of workers;

15.2. illuminating devices in workplaces and exits shall be located in such a manner that workers are protected from risk of accidents and occupational diseases due to insufficient lighting; and

15.3. workplaces in which sudden switching off of light may cause harm to the safety and health of the workers, shall be equipped with sufficient emergency lighting.

16. Windows and skylights shall be designed, installed and maintained in compliance with the following requirements:

16.1. windows, skylights and glass partitions shall allow for avoidance of excessive effects of sunlight in workplaces, taking into account the nature of the work and the characteristics of the workplace.

16.2. workers shall have the possibility to open, close, adjust or secure windows, skylights and ventilators in a safe manner;

16.3. open windows, skylights or ventilators shall not create a hazard to the workers; and

16.4. windows and skylights shall be designed together with facilities for cleaning thereof, or shall be fitted with devices allowing such to be cleaned without risk to the workers carrying out this work or to workers in proximity of such windows.

17. Floors, walls, ceilings and roofs shall be designed, installed and maintained in compliance with the following requirements:

17.1. the floors of the premises shall be stable; they must not be slippery or have dangerous bumps, holes or slopes that may endanger the safety and health of workers;

17.2. workplaces shall have adequate thermal insulation, taking into account the nature of the work and the physical demands placed on workers;

17.3. the surfaces of floors, walls and ceilings in the premises shall be easily cleaned or refurbished in conformity with standards of hygiene;

17.4. transparent or translucent walls (in particular all-glass partitions in work premises and in proximity to traffic routes) shall be clearly marked and made of safety material or be demarcated in such a way as to prevent workers from coming into contact with walls or being injured in case the walls shatter; and

17.5. measures shall be taken to prevent performance of work on or access to roofs by workers, if the roofs are made of insufficiently durable materials or may otherwise create a hazard to the safety and health of workers (for example, if not fenced off), unless such equipment is provided which permits the work to be carried out in a safe manner.

18. Doors and gates shall be designed, installed and maintained in compliance with the following requirements:

18.1. the location, number and dimensions of doors and gates, and the materials used in construction thereof, shall be determined by the characteristics and use of the premises;

18.2. transparent doors shall be appropriately labelled at a conspicuous level;

18.3. revolving doors and gates shall be transparent or have translucent elements;

18.4. transparent or translucent parts in doors and gates that are not made of durable material or may otherwise create a risk of incurring injuries or traumas, shall be protected against breakage;

18.5. sliding doors shall be fitted with a safety device to prevent them from being derailed and falling over;

18.6. doors and gates opening upwards shall be fitted with a mechanism to prevent such from falling back;

18.7. doors and gates opening to the emergency routes, passages and exits shall be marked by safety signs and it must be possible to open them from the inside without assistance at any time;

18.8. doors for pedestrians shall be organised in proximity to the gates intended for means of transport, if passage of pedestrians through such gates can not be ensured;

18.9. doors for pedestrians shall be clearly marked and kept unobstructed;

18.10. the operation of mechanical doors and gates shall not endanger workers, and the doors and gates shall be fitted with easily identifiable and accessible emergency shut-down devices, which allow such to be opened manually if due to failure of the power supply the doors and gates do not open automatically; and

18.11. gates intended for means of transport from inside shall be marked in warning colours.

19. Traffic routes and areas in which the life or health of a worker may be endangered (hereinafter – danger area) shall be designed, installed and maintained in compliance with the following requirements:

19.1. traffic routes shall be located in such a manner as to ensure easy and safe access for pedestrians, as well as means of transport without creating hazards to the workers employed in proximity to such traffic routes;

19.2. pedestrian walkways and traffic routes shall correspond to the maximum potential number of users;

19.3. if traffic routes are utilised by both means of transport and pedestrians, pedestrian walkways shall be installed or a sufficient safety clearance shall be left for pedestrians, ensuring also safe access to doors or gates, passages for pedestrians, corridors and staircases;

19.4. traffic routes shall be clearly identified and marked;

19.5. if workplaces contain danger areas in which, due to the characteristics of the work, there is a risk of workers falling or being injured by falling objects, such places shall be fitted with devices preventing unauthorised entrance of workers into such areas;

19.6. workers who work in the danger areas shall be ensured with collective or personal protective equipment; and

19.7. danger areas shall be clearly identified and marked with safety signs in compliance with the requirements of regulatory enactments.

20. Escalators and travelators shall function safely and shall be fitted with safety devices as well as with easily identifiable and accessible emergency shutdown devices.

21. Loading ramps and platforms shall be designed, installed and maintained in compliance with the following requirements:

21.1. they must comply with the dimensions and loading weight of the freight to be transported;

21.2. they must have at least one exit point;

21.3. if technically possible, platforms exceeding the length specified in regulatory enactments shall have two exit points; and

21.4. loading ramps shall be fitted in such a way as to prevent workers from falling off.

22. Work premises shall be designed, installed and maintained in compliance with the following requirements:

22.1. they shall be sufficiently spacious, of such height and such air space which allows workers to perform their work without risk to their safety and health;

22.2. the dimensions of the free unoccupied area at the workstation shall allow workers sufficient freedom of movement to perform their work, but if this is not possible for reasons specific to the workplace, each worker shall be ensured sufficient freedom of movement in the immediate proximity of his or her workstation.

23. Rest rooms shall be designed, installed and maintained in compliance with the following requirements:

23.1. comfortable and accessible rest rooms shall be ensured depending on the specific nature of the work and the number of workers;

23.2. the rooms shall be sufficiently spacious and with an adequate number of tables and seats with backrests;

23.3. if the specific nature of work requires regular and frequent interruptions, and no rest rooms have been organised, other rooms shall be provided in which workers can stay during such interruptions; and

23.4. the requirements of this Paragraph need not apply to persons employed in offices or other similar workplaces, who have adequate possibilities for rest in their work premises during interruptions of work.

24. If pregnant women and nursing mothers are employed, they shall be ensured the opportunity to lie down and rest in suitable conditions.

25. Changing rooms and lockers shall be designed, installed and maintained in compliance with the following requirements:

25.1. changing rooms shall be installed if special work clothes are intended to be worn during the work process and where, for reasons of health or propriety, workers cannot be expected to change in another place;

25.2. changing rooms shall be easily accessible, sufficiently spacious, and equipped with lockers and seating.

25.3. separate changing rooms for men and women shall be provided or, if not possible, separate use of changing rooms for men and women shall be provided;

25.4. if working conditions are related to activation of harmful substances (for example humidity or dirt), separate lockers shall be installed for storing the workers` own clothing and work clothes;

25.5. air exchange or ventilation shall be ensured in lockers for the workers own clothing and work clothes, if such a necessity is determined by an employer or the trusted representatives of workers; and

25.6. if changing rooms are not installed in accordance with Sub-paragraph 25.1 of these Regulations, each worker shall be ensured with a place to lock away and store his or her own clothes and personal effects.

26. Showers, washbasins and lavatories shall be designed, installed and maintained in compliance with the following requirements:

26.1. an employer or the trusted representatives of the workers shall determine the necessity for installation of shower rooms depending on the nature of work or the health protection of workers. If possible, separate shower rooms for men and women shall be installed or, if this is not possible, separate use of shower rooms for men and women shall be provided;

26.2. shower rooms shall be sufficiently spacious to permit workers to wash without hindrance to each other in conformity with hygiene requirements;

26.3. showers shall be equipped with a supply of hot and cold running water;

26.4. if shower rooms are not installed in accordance with Sub-paragraph 26.1, suitable washbasins with a supply of hot and cold running water shall be installed in the proximity of the workrooms and the changing rooms;

26.5. in cases specified in Sub-paragraph 26.4 of these Regulations, if necessary, the washbasins shall be demarcated or used separately by men and women;

26.6. if shower rooms or washbasins have been installed separately from changing rooms, easy communication from one room to the other shall be ensured;

26.7. the number of lavatories and washbasins shall be sufficient, taking into account the number of workers, and such shall be equipped with personal hygiene products (toilet paper, soap, towels or other types of devices for hand drying);

26.8. lavatories and washbasins shall be installed near workrooms and rest rooms, changing rooms and shower rooms;

26.9. ventilation shall be ensured for lavatories; and

26.10. separate lavatories or separate use of lavatories for men and women shall be ensured, as well as a separate cabin for handicapped persons, ensuring separate use of such for men and women.

27. First aid rooms shall be designed, installed and maintained in compliance with the following requirements:

27.1. depending on the type of activity, extent of work and risk of the working environment to the safety and health of workers, at least one or more first aid rooms shall be organised that are fitted with first aid equipment and facilities;

27.2. easy accessibility with stretchers shall be ensured for these rooms;

27.3. these rooms shall be clearly marked in conformity with the requirements regarding the use of safety signs at workplaces; and

27.4. first aid kits shall be available at all places where, due to working conditions, such are necessary, and the location thereof shall be clearly marked.

28. Outdoor workplaces shall be designed, constructed and maintained in compliance with the following requirements:

28.1. workstations, traffic routes and other areas or installations outdoors which are used during the course of work or where workers are located, shall be organised in such a way that pedestrians and means of transport can relocate in a safe manner;

28.2. Paragraphs 19, 20 and 21 of these Regulations shall apply also to traffic routes leading to specific workstations and to routes used for regular maintenance and supervision of installations and loading platforms;

28.3. Section 18 of these Regulations shall apply also to workplaces outdoors;

28.4. where necessary, workplaces outdoors shall be provided with artificial lighting;

28.5. workers shall be protected against unfavourable weather conditions and from falling objects;

28.6. workers shall be provided with suitable protective equipment against slipping and falling;

28.7. workers shall have the possibility to leave their workstation in cases of accident or fire, or other situations dangerous to health and life, as well as the rendering of first aid shall be ensured; and

28.8. workers shall neither be exposed to harmful noise levels or to harmful external influences (for example gases, vapours or dust).

29. In reconstruction and modernisation of workplaces the following requirements shall be complied with:

29.1. harmful substances shall be replaced, as far as possible, by less harmful or harmless substances;

29.2. concentrations of harmful substances in the working environment shall not exceed the maximum permissible level for occupational exposure to harmful substances;

29.3. the effects of risk in the working environment shall be reduced to the possible minimum; and

29.4. the physical demands placed on the workers shall be reduced as much as possible, and the requirements of ergonomics shall be taken into account.

III. Obligations of Employers

30. For the safety and health of workers in workplaces the employer shall ensure:

30.1. unobstructed passage for pedestrians and means of transport to emergency routes and exits, as well as unobstructed emergency routes and exits;

30.2. technical maintenance of work equipment, rectifying without delay defects which may negatively affect the safety and health of workers;

30.3. cleanliness in workplaces and regular cleaning of workplaces (in particular ventilation equipment), in conformity with hygiene requirements and by not creating a hazard to the safety and health of workers;

30.4. regular inspection and supervision of the operation of safety equipment and devices which are intended for the reduction or prevention of the effects of such dangerous or harmful factors (chemical, biological, physical, psycho physiological, technological and other), the effects of which may cause,(depending on the level and duration of exposure) occupational diseases, traumas, loss of ability to work or a reduction in the ability to work.

30.5. regularly informing workers and their trusted representatives regarding all safety and health protection measures that are taken at workplaces;

30.6. consultations with workers and their trusted representatives, and their participation in all matters related to safety and health protection at work;

30.7. training of workers regarding safety and health protection at work; and

30.8. organising of workplaces (in particular with respect to doors, pedestrian walkways, stairs, showers, washbasins, and lavatories) in order to allow for, if necessary, employment of handicapped persons.

IV. Closing Provisions

31. These Regulations shall come into force on 1 August 2001.

32. The workplaces that were established by 1 August 2001 shall be modified, in compliance with the requirements of these Regulations, by 31 December 2004.

Prime Minister 					A.Bērziņš

Minister for Welfare 					A. Požarnovs

These Regulations shall come into force on 1 August 2001.